3GPP TSG-RAN5 Meeting #89-e	R5-206174

# Online, , 9th Nov 2020 - 20th Nov 2020

Title:						On n259 MU and testability limit

Source:			Anritsu

Agenda Item:			5.3.2.17

Document for:			Endorsement

# 1.	Introduction

In RAN5#88-e, it was endorsed to call for the input on the n259 MU and testability limit [1] for some remaining test cases listed in Table 1.

Table 1 Status of n259 MU and relaxation upper limit [1]

| TC | Upper limit (MU for TT calculation) | Upper limit (relaxation due to testability issue) |
| --- | --- | --- |
| MOP (Max TRP) | 5.5 dB [2] | TBD |
| REFSENS | 5.8 dB [2] | TBD |
| Frequency Error | 0.01 PPM [2] | TBD |
| SEM | 5.8 dB [2] | TBD |
| OFF power | TBD (Treated in [3]) | TBD |
| OBW | Not required because TT = 0 dB [4] | TBD |
| ACLR | TBD | TBD |
| General spurious | Not required because TT = 0 dB [4] | TBD |
| Spurious co-existence | Not required because TT = 0 dB [4] | TBD |
| Inband blocking | Not required because TT = 0 dB [4] | TBD |
| ACS | Not required because TT = 0 dB [4] | TBD |
| Rx spurious | Not required because TT = 0 dB [4] | TBD |

# 2.	Discussion

# Our view on the upper limit assumption for n259 MU and relaxation used for TT determination  is provided in Table 2.

Table 2 Status of n259 MU and relaxation upper limit [1]

| TC | Proposal for max MU assumption for TT determination | Proposal for max relaxation for TT determination | Note, Consideration |
| --- | --- | --- | --- |
| MOP (Max TRP) | - | No relaxation required. |  |
| REFSENS | - | No relaxation required. |  |
| Frequency Error | - | No relaxation required. |  |
| SEM | - | No relaxation required. | (DSNR=1.0) |
| OFF power | Same as FR2b MU | FR2b relaxation(33.1) + 0.73dB = 33.83dB | (0.73: FSPL increase, keep DSNR=1.0) |
| OBW | << Under calculation now… >> | << Under calculation now… >> |  |
| ACLR | Same as FR2b MU | MPR of test point minus Max Testable MPR in Table2 |  |
| General spurious | - | No relaxation required | (possible DSNR increase) |
| Spurious co-existence | - | This was already discussed in RAN5#88-e and fixed. |  |
| Inband blocking | - | << Under calculation now… >> |  |
| ACS | - | << Under calculation now… >> |  |
| Rx spurious | - | f <= 66GHz : Same as existing value. 66GHz <= f <= 87GHz 66GHz relaxation(33.1) + 2.4 dB = 35.5dB | (2.4 : FSPL increase against 66GHz) |

# ACLR

# Maximum Testable SNR without relaxation(threshold SNR=6.0dB) for FR2 ACLR test case is listed in Table 2. FR2c values are derived based on 0.73dB worse floor noise than FR2b. 0.73dB is corresponding to FSPL difference from FR2b.

Table 3 FR2c max testable MPR for ACLR test case (single carrier)

|  | FR2a | FR2b | FR2c |
| --- | --- | --- | --- |
| ChBW (50MHz) | >9.5 | 7.5 | 6.0 |
| ChBW (100MHz) | 7.0 | 5.0 | 4.0 |
| ChBW (200MHz) | 5.0 | 3.5 | 3.0 |
| ChBW (400MHz) | 3.0 | 2.0 | 0.5 |

# Blocking Test

# << Checking now >>>

- Conclusion

Proposal 1 : Use Table 1 for determination of n259 MU and relaxation upper limit assumption for TT determination.

# 4.	References

[1] R5-204851, “Update of WF on Band n259”, NTT Docomo, RAN5#88e