3GPP TSG-RAN5 Meeting #89-e	R5-206175

# Online, , 9th Nov 2020 - 20th Nov 2020

Title:						On FR2 OBW MU

Source:			Anritsu

Agenda Item:			5.3.2.17

Document for:			Endorsement

# 1.	Introduction

OBW MUs have been discussed in [1-17] but no conclusion was reached as of now. This paper is in principle a resubmission of [17] from RAN5#88e, but with additional consideration on FR2c and MPR value for CA case.

# 2.	Discussion

2.1 Simulation

During the RAN5#88-e meeting, it is shown that the center frequency can be 27998.4 MHz rather than 28000.0MHz. This section provides the updated simulation result for center frequency of 27998.4MHz.

The simulation procedure is specified in Annex 1. Upon the simulation, the origin reference spectrum is resampled with 50kHz uniform interval.

OBW measurement error is calculated with (OBWmeas - OBWsignal) / CBW *100 [%CBW], where OBWsignal is the OBW without any distortion (noise/ripple) from TE, and OBWmeas is the measured OBW with TE having some noise and ripple.

TxBW assumption to derive noise level is as per the following table.

Table 1 Tx BW assumption in the simulation

| 100MHz | 400MHz (1) | 400MHz (2) |
| --- | --- | --- |
|  | Tx BW Assumption = Single Carrier | Tx BW Assumption = Intra-band CA class I |
| 95.04MHz (66*120*12/1000) | 380.16MHz(264*120*12/1000) | 394.92(95.04 + 99.96*3) |
|  |  | * 99.96MHz is channel spacing defined in 38.508-1 |

| Figure 5 BW=400MHz(SC) Error [%CBW] | Figure 5 BW=400MHz(CA) Error [%CBW] | Figure 6 BW=100MHz Error [%CBW] |
| --- | --- | --- |
| Figure 7 BW=400MHz, Absolute Error[MHz] | Figure 7 BW=400MHz, Absolute Error[MHz] | Figure 8 BW=100MHz, Absolute Error[MHz] |

Table 2 shows the OBWsignal (w/o noise/ripple)  and error limit for the reference spectrum and is used as a basis for the measurement error. Error limit is defined as the OBW measurement error above which measured OBW of reference spectrum is judged as FAIL, which is depicted as red line in the graphs above.

Table 2 OBWsiginal and Error Limit of the reference spectrum with center frequency = 27998.4 MHz

|  | OBW meas span = 1.5 x CBW | Error Limit %CBW |
| --- | --- | --- |
| 100MHz | 94.15 MHz | 5.85 %CBW |
| 400MHz Single Carrier | 394.71 MHz | 1.32 %CBW |
| 400MHz CA class I | 394.71 MHz | 1.32 %CBW |

2.2 Observations

Based on the analysis in 2.1, the required SNR to achieve the measurement error less than error limit is shown in the below table.

Table 3 OBWsiginal of the reference spectrum with center frequency = 27998.4 MHz

|  | Required SNR for error limit |
| --- | --- |
| 100MHz | 21 dB |
| 400MHz (Single Carrier Assumption) | 25 dB (24.3dB with 0.1 granularity) |
| 400MHz (Intra-band CA Assumption) | 25 dB (24.1dB with 0.1 granularity) |

Achievable SNR based on R5-200868 is given in the below table. In the right most column, red color means the OBW error becomes bigger than error limit even with TE achievable SNR, otherwise green colored.

Table 4 Achievable SNR by TEV

|  | BW | SNR with R5-200868 (w/o MBR,MPR,T(MPR)) | MBR | MPR | T(MPR) | SNR[dB] |
| --- | --- | --- | --- | --- | --- | --- |
| FR2a | 100 | 36 | 0.75 | 2 | 1.5 | 31.75 |
|  | 400 | 30 | 0.75 | 3 | 2.0 | 24.25 |
|  | 100(CA) | 36 | 0.75 | 5 | 4.0 | 26.25 |
|  | 400(CA) | 30 | 0.75 | 5 | 4.0 | 20.25 |
| FR2b | 100 | 32.1 | 0.75 | 2 | 1.5 | 27.85 |
|  | 400 | 26.1 | 0.75 | 3 | 2.0 | 20.35 |
|  | 100(CA) | 32.1 | 0.75 | 5 | 4.0 | 22.35 |
|  | 400(CA) | 26.1 | 0.75 | 5 | 4.0 | 16.35 |
| FR2c | 100 | 32.1-0.73-1.9 | 0.75 | 2 | 1.5 | 26.22 |
| (NOTE2) | 400 | 26.1-0.73-1.9 | 0.75 | 3 | 2.0 | 18.72 |
|  | 100(CA) | 32.1-0.73-1.9 | 0.75 | 5 | 4.0 | 20.72 |
|  | 400(CA) | 26.1-0.73-1.9 | 0.75 | 5 | 4.0 | 14.72 |
| NOTE1 : MPR values are those corresponding to full RB allocation with CP-OFDM QPSK. NOTE2 : 0.73 : FSPL increase against FR2b. 1.9 : Min peak EIRP requirement difference from FR2b. |  |  |  |  |  |  |

Observation 1 : Testability of FR2 OBW is analyzed as in Table 4.

# 3.	Conclusion

In this paper FR2 OBW MU and testability limit is studied, which is summarized in Table 4.

Observation 1 : Testability of FR2 OBW is analyzed as in Table 4.

# 4.	References

[1] R5-188189, “On the FR2 MU for occupied BW and ACLR”, Anritsu, RAN5#81

[2] R5-190390, “On QoQZ MU Impact on OBW”, Keysight, RAN5 NR#4 AdHoc

[3] R5-192660, “On the FR2 OBW MU”, Anritsu, RAN5#82

[4] R5-191617, “On OBW measurement uncertainty”, Keysight, RAN5#82

[5] R5-194353, “On the noise impact to FR2 OBW”, Anritsu, RAN5#83

[6] R5-192984, “On the SNR for FR2 TRx test cases”, R&S, Rohde & Schwarz, RAN5 NR#5 AdHoc

[7] R5-193187, “On noise impact”, Keysight, RAN5 NR#5 AdHoc

[8] ITU-R SM 443-4, “SM.443 : Bandwidth measurement at monitoring stations”, https://www.itu.int/rec/R-REC-SM.443-4-200702-I/en

[9] R5-197626, “Impact of noise for FR2 OBW”, Anritsu, RAN5#83

[10] R5-198175, “Discussion on AP#84.22 Tx Spectrum Shape for FR2 OBW MU”, Qualcomm, RAN5#85

[11] R5-200874, “On FR2 OBW MU”, Kesight, RAN5#86e

[12] R5-200869, “On the MU of FR2 OBW”, Rohde&Shwarltz, RAN5#86e

[13] R5-200739, “On FR2 OBW MU “, Anritsu, RAN5#86e

[14] R5-202318, “On FR2 OBW MU”, Anritsu, RAN5#87e

[15] R5-202458, “On the MU of FR2 OBW”, ROHDE & SCHWARZ, RAN5#87e

[16] R5-202460, “On FR2 OBW MU”, Keysight Technologies UK Ltd, RAN5#87e

[17] R5-204849, “On FR2 OBW MU”, Anritsu, RAN5#88e

Annex A : OBW evaluation algorithm

- f, Ptheta, Pphi = Raw data from reference spectrum, N:Number of samples

- P = 10log10( 10^( Ptheta /10) + 10^( Pphi /10))

- M = 1.5*CBW/0.05 # latter : Fixed 50kHz interval resampling. M=N*2 was used in original simulation

- f’ = linspace(Fcenter -1.5*CWB/2, Fcenter + 1.5*CBW/2,  M)

- P ‘ = lininterp(f’, f, P) # Make a uniformly sampled data within OBW measurement window by linear interpolation

- Pnoise = Average Power per sample of P’ within TxBW centered on Fcenter – SNR.

- For each realization 1…10000

  - flatness_d = 3 independent random variables of normal distribution (sigma =2.19)

  - flatness_f = linspace(Fcenter -1.5* CBW/2, Fcenter + 1.5*CBW/2, 3)

  - flatness = lininterp(f’, flatness_f, flatness_d) # linear interpolate flatness over OBW measurement window

  - S = 10log10(10^(P’/10) + 10^(Pnoise/10)) + flatness

  - OBWmeas = Frequency width(centered on Fcenter) where the power within it (in W) equals to 0.99 * total power(in W) of S

- Obtain 2.5%-tile and 97.5%-tile value from CDF of 10000 OBWmeas

linspace(x0, x1, N) : N evenly spaced samples, calculated over the interval [x0, x1].

lininterp(x’, x, y) : Linearly interpolated data of (x,y) sampled at x’