3GPP TSG-RAN5 Meeting #90-e	R5-210071

# Electronic Meeting, 22nd Feb 2021 - 5th Mar 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.508 | CR | 1340 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to emergency bearer service over S1 for eCall test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK, Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, EIEI-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Currently emergency bearer services over S1 is indicated only for rel-9, there is a need to add the support for rel-14 also for eCall test cases |  |  |  |  |  |  |  |  |
| Summary of change: |  | “EPS network feature support” in the table 4.7.2-1 of spec 36.508 says “IMS voice over PS session in S1 mode supported, emergency bearer services in S1 mode supported” and indicated rel-9, the support for rel-14 has also been added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the eCall over IMS test cases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.7.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | Associated with TTCN CR R5s210009 |  |  |  |  |  |  |  |  |

<Start of modified section>

### 4.7.2	Contents of EMM messages

#### -	ATTACH ACCEPT

This message is sent by the SS to the UE.

Table 4.7.2-1: ATTACH ACCEPT

| Derivation Path: 24.301 clause 8.2.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | EMM |  |  |
| Security header type |  | '0000'B | Plain NAS message, not security protected |  |
| Attach accept message identity |  | '0100 0010'B | Attach accept |  |
| EPS attach result |  | '0001'B | EPS only | EPS_only |
|  |  | '0010'B | combined EPS/IMSI attach | combined_EPS_IMSI |
| Spare half octet |  | '0000'B |  |  |
| T3412 value |  |  |  |  |
| Timer value |  | '0 0000'B |  |  |
| Unit |  | '111'B | value indicates that the timer is deactivated. |  |
| TAI list |  |  |  |  |
| Length of tracking area identity list contents |  | '0000 0110'B | 6 octets |  |
| Partial tracking area identity list 1 |  |  |  |  |
| Number of elements |  | '0 0000'B | 1 element |  |
| Type of list |  | '00'B | list of TACs belonging to one PLMN, with non-consecutive TAC values |  |
| MCC |  | See table 4.4.2-2 in this document | For NAS test cases, see table 6.3.2.2-1. |  |
| MNC |  | See table 4.4.2-2 in this document | For NAS test cases, see table 6.3.2.2-1. |  |
| TAC 1 |  | See table 4.4.2-2 in this document | For NAS test cases, see table 6.3.2.2-1. |  |
| ESM message container |  | ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message to activate the default bearer |  |  |
| ESM message container |  | ESM DUMMY MESSAGE |  | CIoT_Attach_WithoutPDN |
| GUTI |  |  |  |  |
| Length of EPS mobile identity contents |  | '0000 1011'B | 11 octets |  |
| Type of identity |  | '110'B | GUTI |  |
| Odd/even indication |  | '0'B | even number of identity digits and also when the GUTI is used |  |
| MCC |  | See table 4.4.2-2 in this document | For NAS test cases, see table 6.3.2.2-1. |  |
| MNC |  | See table 4.4.2-2 in this document | For NAS test cases, see table 6.3.2.2-1. |  |
| MME Group ID |  | See table 4.4.2-2 in this document | For NAS test cases, see table 6.3.2.2-1. |  |
| MME Code |  | See table 4.4.2-2 in this document | For NAS test cases, see table 6.3.2.2-1. |  |
| M-TMSI |  | See table 4.4.2-2 in this document | For NAS test cases, see table 6.3.2.2-1. |  |
| Location area identification |  | Not present |  | EPS_only |
| Location area identification |  |  |  | combined_EPS_IMSI |
| MCC |  | MCC of the EUTRA cell from which this message is sent |  |  |
| MNC |  | MCC of the EUTRA cell from which this message is sent |  |  |
| LAC |  | 1 |  |  |
| MS identity |  | Not present |  | EPS_only |
| MS identity |  |  |  | combined_EPS_IMSI |
| Length of mobile identity contents |  | '0000 0101'B | 5 octets |  |
| Type of identity |  | '100'B | TMSI/P-TMSI/M-TMSI |  |
| Odd/even indication |  | '0'B | even number of identity digits and also when the TMSI/P-TMSI or TMGI and optional MBMS Session Identity is used |  |
| TMSI |  | TMSI-1 |  |  |
| EMM cause |  | Not present |  |  |
| T3402 value |  | Not present |  |  |
| T3423 value |  | Not present |  |  |
| Equivalent PLMNs |  | Not present |  |  |
| Emergency number list |  | Not present |  |  |
| EPS network feature support |  | '0000 0001'B | IMS voice over PS session in S1 mode supported |  |
| EPS network feature support |  | '0000 0011'B | IMS voice over PS session in S1 mode supported, emergency bearer services in S1 mode supported | Rel-9, Rel-14 |
| EPS network feature support |  | ‘1000 0011 0000 1111’B | IMS voice over PS session in S1 mode, emergency bearer services in S1 mode, CP CIoT, ePCO, HC-CP CIoT, S1-U data and UP CIoT supported | CIoT |
| EPS network feature support |  | ‘1100 0000 0000 1100’B | CIoT control plane optimization indicated, Attach Without PDN supported, ePCO supported, Header Compression for CIoT control plane supported | CIoT_CP AND pc_NB |
| EPS network feature support |  | ‘1000 0000 0000 1011’B | CP CIoT supported, ePCO supported, S1-U data transfer supported, CIoT user plane optimization indicated | CIoT_UP AND pc_NB |
| EPS network feature support |  | ‘1000 0000 0000 1010’B | CP CIoT supported, ePCO supported, S1-U data transfer only indicated | S1-U Only AND pc_NB |
| EPS network feature support |  | ‘0010 0000 0000 0000’B | EXTENDED SERVICE REQUEST for packet services indicated | ESRPS |
| EPS network feature support |  | '0000 0011 0000 1000'B | - IMS voice over PS session in S1 mode supported - emergency bearer services in S1 mode supported - ePCO supported - Interworking without N26 interface not supported | NR |
| Additional update result |  | Not present |  |  |
| Additional update result |  | ‘0010’ B | “SMS only” | additional_update_result_SMS OR CIoT_SMSOnly |
| T3412 extended value |  | Not present |  | Rel-10 |
| T3324 value |  | Not present |  |  |
| Extended DRX parameters |  | Not present |  |  |
| DCN-ID |  | Not present |  |  |
| SMS services status |  | Not present |  |  |
| Non-3GPP NW provided policies |  | Not present |  |  |
| T3448 value |  | Not present |  |  |
| Network policy |  | Not present |  |  |
| T3447 value |  | Not present |  |  |
| Extended emergency number list |  | Not present |  |  |
| Ciphering key data |  | Not present |  |  |

<End of modified section>