3GPP TSG-RAN WG5 Meeting #90-e	R5-210191

# Online, , 22nd Feb 2021 - 5th Mar 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 1949 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of NR test case 9.1.5.1.8 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated, ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Cell I is configured as Suitable neighbour cell at step 11A and not turned off till the end of test case. When UE is powered on and user selects PLMN at step 17, as both Cell E and Cell I of same PLMN are available, UE may select Cell I instead of Cell E causing test case failure. Similar issue during preamble is fixed with R5-205866 in RAN5#89-e. From 38.304, as this is cell selection process, UE may perform it by leveraging stored information of frequencies, previously received measurement control information elements or from previously detected cells. Due to this UE can sometimes camp on Cell I instead of Cell E during preamble causing test case failure. [TS 38.304] ### 5.2.3 Cell Selection process #### 5.2.3.1 Description Cell selection is performed by one of the following two procedures: a) Initial cell selection (no prior knowledge of which RF channels are NR frequencies): 1. The UE shall scan all RF channels in the NR bands according to its capabilities to find a suitable cell. 2. On each frequency, the UE need only search for the strongest cell, except for operation with shared spectrum channel access where the UE may search for the next strongest cell(s). 3. Once a suitable cell is found, this cell shall be selected. b) Cell selection by leveraging stored information: 1. This procedure requires stored information of frequencies and optionally also information on cell parameters from previously received measurement control information elements or from previously detected cells. 2. Once the UE has found a suitable cell, the UE shall select it. 3. If no suitable cell is found, the initial cell selection procedure in a) shall be started. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Cell I is configured as Non-suitable Off cell at step 15. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE may fail the test case by selecting Cell I at step 18. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.5.1.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN Impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section>

##### 9.1.5.1.8	Initial registration / Rejected / Serving network not authorized

9.1.5.1.8.1	Test Purpose (TP)

(1)

with { The UE has sent initial REGISTRAION REQUEST message }

ensure that {

when { the UE receives a REGISTRATION REJECT with cause #73 (Serving network not authorized) }

then { the UE stores the PLMN identity in the "forbidden PLMN list" and does not attempt to register on a cell belong to that PLMN }

}

9.1.5.1.8.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.501, clauses 5.5.1.2.5. Unless otherwise stated these are Rel-15 requirements.

[TS 24.501, clause 5.5.1.2.5]

If the initial registration request cannot be accepted by the network, the AMF shall send a REGISTRATION REJECT message to the UE including an appropriate 5GMM cause value.

If the initial registration request is rejected due to general NAS level mobility management congestion control, the network shall set the 5GMM cause value to #22 "congestion" and assign a back-off timer T3346.

The UE shall take the following actions depending on the 5GMM cause value received in the REGISTRATION REJECT message.

#3	(Illegal UE); or

#6	(Illegal ME).

….

#73	(Serving network not authorized).

The UE shall set the 5GS update status to 5U2 NOT UPDATED, reset the registration attempt counter, store the PLMN identity in the "forbidden PLMN list" and enter state 5GMM-DEREGISTERED.PLMN-SEARCH in order to perform a PLMN selection according to 3GPP TS 23.122 [5].

9.1.5.1.8.3	Test description

9.1.5.1.8.3.1	Pre-test conditions

System Simulator:

-	3 NGC Cells NGC Cell E, NGC Cell I and NGC Cell A are configured as specified in TS 38.508-1[4] table 6.3.2.2-1 and table 6.3.2.2-3.

UE:

-	The UE is in Automatic PLMN selection mode.

Preamble:

-	NGC Cell E is set to “Serving Cell”.

-	NGC Cell I is set to “Non-Suitable cell”.

-	NGC Cell A is set to “Non-Suitable cell”.

-	The UE is in state 0N-B with a successful registration on NGC Cell E according to 38.508-1[4].

9.1.5.1.8.3.2	Test procedure sequence

Table 9.1.5.1.8.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE is switched on | - | - | - | - |
| 2-9 | Steps 2-9 of Table 4.5.2.2-2 in TS38.508-1 [4] are performed on NGC Cell E. | - | - | - | - |
| 10 | The SS transmits a REGISTRATION REJECT with cause #73 (Serving network not authorized). | <-- | REGISTRATION REJECT | - | - |
| 11 | The SS releases the RRC connection | - | - | - | - |
| 11A | The SS configures - NGC Cell I as “Suitable neighbour cell” | - | - | - | - |
| 12 | Check: Does the UE send in the next 30 sec a request for RRC connection establishment on Cell E or Cell I. | --> | NR RRC: RRCSetupRequest | 1 | F |
| 13 | The SS configures - NGC Cell A as “Serving Cell” | - | - | - | - |
| 14 | Check: Does the UE perform Registration procedure on NGC Cell A as specified in TS 38.508-1 [4] subclause 4.5.2? | - | - | 1 | P |
| 14A | Switch off procedure in RRC_IDLE specified in TS 38.508-1 subclause 4.9.6.1 is performed. | - | - | - | - |
| 15 | The SS configures - NGC Cell A as “Non-Suitable "Off" cell” - NGC Cell I as “Non-Suitable "Off" cell” | - | - | - | - |
| 15A | The UE is brought back to operation or the USIM is inserted. The UE is powered on or switched on. | - | - | - | - |
| 16 | Set the UE in manual PLMN selection mode or request a PLMN search. | - | - | - | - |
| 17 | The user selects the PLMN of NGC Cell E. | - | - | - | - |
| 18 | The UE performs Registration procedure on NGC Cell E as specified in TS 38.508-1 [4] subclause 4.5.2. | - | - | - | - |
| 19 | Set the UE in Automatic PLMN selection mode. | - | - | - | - |

9.1.5.1.8.3.3	Specific message contents

Table 9.1.5.1.8.3.3-1: REGISTRATION REJECT (step 10 Table 9.1.5.1.8.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.7.1-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| 5GMM cause |  | ‘01001001’B | Cause #73 (Serving network not authorized) |  |

<End of modifed section>