3GPP TSG-RAN WG5 Meeting #90-e	R5-210267

# Electronic Meeting, 22nd Feb 2021 – 5th March 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 1955 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to 5GC NAS TC 9.1.5.2.9 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | An AP# 89.01 was noted to remove Editors note and reintroduce TP 1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Readding TP1 and modified the test steps accordingly. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TTCN implementation will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.5.2.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN Impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 9.1.5.2.9	Mobility and periodic registration update / Abnormal / Change of cell into a new tracking area, collision with generic UE configuration update procedure

9.1.5.2.9.1	Test Purpose (TP)

(1)

with { UE initiates a Mobility and periodic registration procedure in 5GMM-REGISTERED state }

ensure that {

when { UE changes the cell into a new tracking area before the registration procedure for mobility and periodic registration update has been completed }

then { UE shall abort registration procedure for mobility and periodic registration update and re-initiate immediately }

}

(2)

with { UE initiates a Mobility and periodic registration procedure in 5GMM-REGISTERED state }

ensure that {

when { UE receives a CONFIGURATION UPDATE COMMAND message before the registration procedure for mobility and periodic registration update has been completed }

then { UE shall ignore the message and proceed with the mobility and periodic registration update procedure }

}

9.1.5.2.9.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.501, clauses 5.5.1.3.7 and 5.4.4.5. Unless otherwise stated these are Rel-15 requirements.

[TS 24.501, clause 5.5.1.3.7]

The following abnormal cases can be identified:

…

f)	Change of cell into a new tracking area.

If a cell change into a new tracking area occurs before the registration procedure for mobility and periodic registration update is completed, the registration procedure for mobility and periodic registration update shall be aborted and re-initiated immediately. The UE shall set the 5GS update status to 5U2 NOT UPDATED.

…

h)	Registration procedure for mobility and periodic registration update and generic UE configuration update procedure collision.

If the UE receives a CONFIGURATION UPDATE COMMAND message before the registration procedure for mobility and periodic registration update has been completed, UE shall behave as specified in subclause 5.4.4.5.

[TS 24.501, clause 5.4.4.5]

The following abnormal cases can be identified:

…

d)	Generic UE configuration update and registration procedure for mobility and periodic registration update collision

If the UE receives a CONFIGURATION UPDATE COMMAND message before the ongoing registration procedure for mobility and periodic registration update has been completed, and the Configuration update indication IE in the CONFIGURATION UDPATE COMMAND message indicates that the acknowledgement is requested, then the UE shall ignore the CONFIGURATION UPDATE COMMAND message and proceed with registration procedure for mobility and periodic update procedure. Otherwise the UE shall proceed with both the procedures.

9.1.5.2.9.3	Test description

9.1.5.2.9.3.1	Pre-test conditions

System Simulator:

-	2 NGC cells with system information combination NR-2 in accordance with TS 38.508-1[4] sub-clause 4.4.3.1.2.

-	NGC cell A configured as "Serving cell" according to TS 38.508-1 [4] Table 6.2.2.1-3 for FR1 or Table 6.2.2.2-2 for FR2, HPLMN, TAI-1

-	NGC cell B configured as "Non-Suitable cell" according to TS 38.508-1 [4] Table 6.2.2.1-3 for FR1 or Table 6.2.2.2-2 for FR2, HPLMN, TAI-2

UE:

None.

Preamble:

-	The UE is in state 1N-A on NGC cell A according to TS 38.508-1 [4] Table 4.4A.2-1.

9.1.5.2.9.3.2	Test procedure sequence

Table 9.1.5.2.9.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | The following messages are to be observed on NGC cell B unless explicitly stated otherwise. | - | - | - | - |
| 1 | Change NGC cell A to ''Non-Suitable ''Off'' cell''. Change NGC cell B to ''Serving cell''. | - | - | - | - |
| 2-4 | Steps 1-3 in TS 38.508-1 [4] Table 4.9.5.2.2-1 are peformed. | - | - | - | - |
| 5 | The SS transmits a CONFIGURATION UPDATE COMMAND message with the Configuration update indication IE indicating that the acknowledgement is requested. | <-- | CONFIGURATION UPDATE COMMAND | - | - |
| 6 | Check: Does the UE transmit a CONFIGURATION UPDATE COMPLETE message within the expiry of T3555? | --> | CONFIGURATION UPDATE COMPLETE | 2 | F |
| 7 | Step 4 in TS 38.508-1 [4] Table 4.9.5.2.2-1 are performed with a new assigned 5G-GUTI in the REGISTRATION ACCEPT message. | - | - | - | - |
| 8 | Check: Does the UE transmit a REGISTRATION COMPLETE message? | --> | REGISTRATION COMPLETE | 2 | P |
| 9 | Change NGC cell A to ''Serving cell''. | - | - | - | - |
| 10 | The SS performs an intra-frequency handover to NGC Cell A by sending a RRCReconfiguration message and receiving a RRCReconfigurationComplete message on NGC Cell A. | - | - | - | - |
| - | The following messages are to be observed on NGC Cell A unless explicitly stated otherwise | - | - | - | - |
| 11-16 | Steps 1–6a1 of Table 4.9.5.2.2-1 in TS 38.508-1 [4] are performed. | - | - | - | - |
| 17 | Wait for 2 minutes until T3512 expiry. | - | - | - | - |
| 18 | Step 4 of Table 4.9.5.2.2-1 of the generic procedure in TS 38.508-1 [4] is performed. The UE transmits a REGISTRATION REQUEST message. | --> | REGISTRATION REQUEST | - | - |
| 19 | The SS performs an intra-frequency handover to NGC Cell B by sending a RRCReconfiguration message and receiving a RRCReconfigurationComplete message on NGC Cell B. | - | - | - | - |
| - | The following messages are to be observed on NGC Cell B unless explicitly stated otherwise. | - | - | - | - |
| 20 | Check: Does the UE The UE transmits a REGISTRATION REQUEST message with IE 5GS registration type set to "Mobility registration updating" on NGC Cell B?? | --> | REGISTRATION REQUEST | 1 | -P |
| 21-22 | Steps 4 to 5 of the generic procedure in TS 38.508-1 [4] subclause 4.9.5.2.2 | - | - | - | - |

9.1.5.2.9.3.3	Specific message contents

Table 9.1.5.2.9.3.3-1: REGISTRATION REQUEST (steps 4 Table 9.1.5.2.9.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.7.1-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| 5GS registration type |  |  |  |  |
| 5GS registration type value |  | '010'B | Mobility registration updating |  |

Table 9.1.5.2.9.3.3-2: CONFIGURATION UPDATE COMMAND (step 5 Table 9.1.5.2.9.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.7.1-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Configuration update indication |  | Not Present |  |  |
| ACK |  | '1'B | acknowledgement requested |  |

Table 9.1.5.2.9.3.3-3: REGISTRATION ACCEPT (step 14 Table 9.1.5.2.9.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.7.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| T3512 value |  |  |  |  |
| Timer value |  | '0 0010'B | 2 minutes |  |
| Unit |  | '101'B | value is incremented in multiples of 1 minute |  |

Table 9.1.5.2.9.3.3-4: REGISTRATION REQUEST (step 20 Table 9.1.5.2.9.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.7.1-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| 5GS registration type |  |  |  |  |
| 5GS registration type value |  | '011'B | Periodic registration updating |  |