3GPP TSG-RAN5 Meeting #90-e	R5-210314

# Online, , 22nd Feb 2021 - 5th Mar 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 1685 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updating Contents of RRC messages for Logged MDT test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The default contents of RRC message should be introduced for Logged MDT test cases. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated message contents of RRC messages LoggedMeasurementConfiguratio, UEInformationRequest and UEInformationResponse. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The default contents of RRC message for logged MDT test cases are incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Table 4.6.1-5AA, Table 4.6.1-32A, Table 4.6.1-32B |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

–	LoggedMeasurementConfiguration

Table 4.6.1-5AA: LoggedMeasurementConfiguration

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| LoggedMeasurementConfiguration-r16 ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| loggedMeasurementConfiguration-r16 SEQUENCE { |  |  |  |  |
| TraceReference-r16 ::= SEQUENCE { |  |  |  |  |
| plmn-Identity-r16 |  | plmn-Identity in USIM | MCC/MNC=MCC/MNC in USIM |  |
| traceId-r16 |  | ‘0EF’H | OCTET STRING (SIZE (3)) |  |
| } |  |  |  |  |
| traceRecordingSessionRef-r16 |  | ‘1A’H | OCTET STRING (SIZE (2)) |  |
| tce-Id-r16 |  | ‘5’H | OCTET STRING (SIZE (1)) |  |
| absoluteTimeInfo-r16 |  | Set to value corresponding to the absolute time when the message is sent | BIT STRING (SIZE (48)) |  |
| areaConfiguration-r16 |  | Not present |  |  |
| plmn-IdentityList-r16 |  | Not present |  |  |
| bt-NameList-r16 |  | Not present |  |  |
| wlan-NameList-r16 |  | Not present |  |  |
| sensor-NameList-r16 |  | Not present |  |  |
| loggingDuration-r16 |  | min120 |  |  |
| reportType CHOICE { |  |  |  |  |
| LoggedPeriodicalReportConfig-r16 SEQUENCE { |  |  |  | periodical |
| loggingInterval-r16 |  | ms2560 | 2.56s |  |
| } |  |  |  |  |
| LoggedEventTriggerConfig-r16 SEQUENCE { |  |  |  | eventTriggered |
| eventType-r16 CHOICE { |  |  |  |  |
| outOfCoverage |  | NULL |  | outOfCoverage |
| eventL1 SEQUENCE { |  |  |  | eventL1 |
| l1-Threshold CHOICE { |  |  |  |  |
| rsrp |  | Thres1 | Thres is an entry value into a mapping table in TS 38.133 [13]. |  |
| } |  |  |  |  |
| hysteresis |  | 0 |  |  |
| timeToTrigger |  | ms100 | 0.1s |  |
| } |  |  |  |  |
| loggingInterval-r16 |  | ms2560 |  |  |
| } |  |  | 2.56s |  |
| } |  |  |  |  |
| } |  |  |  |  |
| nonCriticalExtension SEQUENCE {} |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

< Unchanged sections omitted >

–	UEInformationRequest

Table 4.6.1-32A: UEInformationRequest

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UEInformationRequest-r16 ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| UEInformationRequest-r16-IEs ::= SEQUENCE { |  |  |  |  |
| idleModeMeasurementReq-r16 |  | True |  | Idle Meas |
| logMeasReportReq-r16 |  | True |  | Logged MDT |
| connEstFailReportReq-r16 |  | True |  | connEst Fail |
| ra-ReportReq-r16 |  | True |  | Ra-report |
| rlf-ReportReq-r16 |  | True |  | Rlf-report |
| mobilityHistoryReportReq-r16 |  | True |  | Mobility History |
| lateNonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension SEQUENCE {} |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| CriticalExtensionsFuture SEQUENCE { } |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| Idle Meas | Used for idle mearsurement test case |
| Logged MDT | Used for Logged MDT test case |
| connEst Fail | Used for connestFail test case |
| Ra-report | Used for ra-report test case |
| Rlf-report | Used for rlf-report test case |
| Mobility History | Used for mobility history test case |

–	UEInformationResponse

Table 4.6.1-32B: UEInformationResponse

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UEInformationResponse-r16 ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| UEInformationResponse-r16-IEs ::= SEQUENCE { |  |  |  |  |
| measResultIdleEUTRA-r16 SEQUENCE {} |  | Not checked |  |  |
| measResultIdleNR-r16 SEQUENCE {} |  | Not checked |  |  |
| logMeasReport-r16 SEQUENCE {} |  | Not checked |  |  |
| connEstFailReport-r16 SEQUENCE {} |  | Not checked |  |  |
| ra-ReportList-r16 SEQUENCE {} |  | Not checked |  |  |
| rlf-Report-r16 SEQUENCE {} |  | Not checked |  |  |
| mobilityHistoryReport-r16 SEQUENCE {} |  | Not checked |  |  |
| lateNonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension SEQUENCE {} |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| criticalExtensionsFuture SEQUENCE {} |  | Not present |  |  |
| } |  |  |  |  |