3GPP TSG-RAN5 Meeting #90-e	R5-210322

# Electronic Meeting, 22nd Feb 2021 - 5th Mar 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 1959 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to 5GMM Initial Registration test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK, Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | After the last step in the test cases 9.1.5.1.2, 9.1.5.1.8 and 9.1.5.1.10 where UE is switched to automatic PLMN selection mode, UE immediately enters into the postamble where UE is switched off without having the time to switch the PLMN selection mode causing test case to fail in postamble as UE is not able to perform deregistration. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added a delay of 300 sec to allow UE to switch to automatic selection mode |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.5.1.2, 9.1.5.1.8, 9.1.5.1.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | No TTCN impact, it is already aligned with the proposed change |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 9.1.5.1.2	Initial registration / 5GS services / Equivalent PLMN list handling

9.1.5.1.2.1	Test Purpose (TP)

(1)

with { The UE in 5GMM-REGISTERED-INITIATED state }

ensure that {

when { UE receives a REGISTRATION ACCEPT with a new set of Equivalent PLMNs, then after Switch OFF and Switch ON }

then { The UE deletes the old equivalent PLMN list and uses the new equivalent PLMN list }

}

(2)

with { The UE in 5GMM-REGISTERED-INITIATED state }

ensure that {

when { UE receives a REGISTRATION ACCEPT that does not include Equivalent PLMNs, then after Switch OFF and Switch ON }

then { UE deletes the old equivalent PLMN list }

}

(3)

with { The UE in 5GMM-REGISTERED-INITIATED state }

ensure that {

when { the UE receives a REGISTRATION ACCEPT that includes equivalent PLMN list, and the UE detects that an entry in the equivalent PLMN list is also present in forbidden PLMN list }

then { UE deletes the entry in equivalent PLMN list that is also present in forbidden PLMN list }

}

9.1.5.1.2.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.501, clauses 5.5.1.2.4 and 5.5.1.3.4. Unless otherwise stated these are Rel-15 requirements.

[TS 24.501, clause 5.5.1.2.4]

…

The AMF may also include a list of equivalent PLMNs in the REGISTRATION ACCEPT message. Each entry in the list contains a PLMN code (MCC+MNC). The UE shall store the list as provided by the network, and if the initial registration procedure is not for emergency services, the UE shall remove from the list any PLMN code that is already in the list of "forbidden PLMNs". In addition, the UE shall add to the stored list the PLMN code of the registered PLMN that sent the list. The UE shall replace the stored list on each receipt of the REGISTRATION ACCEPT message. If the REGISTRATION ACCEPT message does not contain a list, then the UE shall delete the stored list.

…

[TS 24.501, clause 5.5.1.3.4]

…

The AMF may also include a list of equivalent PLMNs in the REGISTRATION ACCEPT message. Each entry in the list contains a PLMN code (MCC+MNC). The UE shall store the list as provided by the network, and if there is no emergency PDU session established, the UE shall remove from the list any PLMN code that is already in the list of "forbidden PLMNs". If the UE is not registered for emergency services and there is an emergency PDU session established, the UE shall remove from the list of equivalent PLMNs any PLMN code present in the "forbidden PLMNs list" when the emergency PDU session is released. In addition, the UE shall add to the stored list the PLMN code of the registered PLMN that sent the list. The UE shall replace the stored list on each receipt of the REGISTRATION ACCEPT message. If the REGISTRATION ACCEPT message does not contain a list, then the UE shall delete the stored list.

…

9.1.5.1.2.3	Test description

9.1.5.1.2.3.1	Pre-test conditions

System Simulator:

-	NGC Cell A and NGC Cell E and NGC Cell F are configured according to Table 6.3.2.2-1 and Table 6.3.2.2-3 in TS 38.508-1 [4].

-	NGC Cell F is configured with NR frequency NRf3.

-	System information combination NR-4 as defined in TS 38.508 [4] clause 4.4.3.1.2 is used.

UE:

None.

Preamble:

-	The UE is in state Switched OFF (State 0-A) as per TS 38.508-1 [4] Table 4.4A.2-0.

9.1.5.1.2.3.2	Test procedure sequence

Table 9.1.5.1.2.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS configures: - NGC Cell F as the "Serving cell". - NGC Cell E as the "Non-Suitable "off" cell". - NGC Cell A as the “Non-Suitable “off" cell". | - | - | - | - |
| 2 | The UE is Switched ON |  | - |  |  |
| - | The following messages are to be observed on NGC Cell F unless explicitly stated otherwise |  | - |  |  |
| 3-14 | Steps 2-13 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed. | - | - | - | - |
| 15 | SS transmits a REGISTRATION ACCEPT message with PLMN ID of NGC Cell A included in the Equivalent PLMNs IE | <-- | REGISTRATION ACCEPT | - | - |
| 16-20a1 | Steps 15-19a1 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed. | - | - | - | - |
| 21 | The UE is switched off by executing generic procedure in Table 4.9.6.3-1 in TS 38.508-1 [4] | - | - | - | - |
| 22 | The UE is Switched ON | - | - | - | - |
| 23-34 | Steps 2-13 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed. | - | - | - | - |
| 35 | SS transmits a REGISTRATION ACCEPT message with PLMN ID of NGC Cell E included in the Equivalent PLMNs IE | <-- | REGISTRATION ACCEPT | - | - |
| 36-40a1 | Steps 15-19a1 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed. | - | - | - | - |
| 41 | The UE is switched off by executing generic procedure in Table 4.9.6.3-1 in TS 38.508-1 [4]. | - | - | - | - |
| 42 | The SS configures: - NGC CelFl as the "Non-Suitable "off" cell". - NGC Cell E as the "Suitable neighbour cell". - NGC Cell A as the "Serving cell". | - | - | - | - |
| 43 | The UE is Switched ON. | - | - | - | - |
| - | The following messages are to be observed on NGC Cell E unless explicitly stated otherwise | - | - | - | - |
| 44-62a1 | Steps 2-20a1 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed. | - | - | 1 | P |
| 63 | The UE is switched off by executing generic procedure in Table 4.9.6.1-1 in TS 38.508-1 [4]. | - | - | - | - |
| 63A | The SS configures: - NGC Cell F as the "Serving cell". - NGC Cell E as the "Non-Suitable "off" cell". - NGC Cell A as the "Suitable neighbour cell". | - | - | - | - |
| 64 | The UE is Switched ON | - | - | - | - |
| - | The following messages are to be observed on NGC Cell A unless explicitly stated otherwise | - | - | - | - |
| 64A | Steps 2-19a1 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed. | - | - | 2 | P |
| 64B | The UE is switched off by executing generic procedure in Table 4.9.6.3-1 in TS 38.508-1 [4]. | - | - | - | - |
| 64C | The SS configures: - NGC Cell A as the "Non-Suitable "off" cell". | - | - | - | - |
| 64D | The UE is Switched ON. | - | - | - | - |
| 65-76 | Steps 2-13 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed. | - | - | 2 | P |
| 76A | Void. | - | - | - | - |
| 77 | SS transmits a REGISTRATION REJECT with cause #11 (PLMN not allowed) | <-- | REGISTRATION REJECT | - | - |
| 78 | The SS releases the RRC connection | - | - |  |  |
| 79 | The SS configures: - NGC Cell A as the "Serving cell". - NGC Cell E as the “Non-Suitable "off" cell ".. - NGC Cell F as the “Non-Suitable "off" cell ". | - | - | - | - |
| - | The following messages are to be observed on NGC Cell A unless explicitly stated otherwise | - | - | - | - |
| 80-93 | Steps 2-13 of Table 4.5.5.2.2- 2 of the generic procedure in TS 38.508-1 [4] are performed. | - | - | - | - |
| 94 | SS transmits a REGISTRATION ACCEPT message with PLMN ID of NGC Cell E and NGC Cell F included in the Equivalent PLMNs IE | <-- | REGISTRATION ACCEPT | - | - |
| 95-99a1 | Steps 15-19a1 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed. | - | - | - | - |
| 100 | The UE is switched off by executing generic procedure in Table 4.9.6.3-1 in TS 38.508-1 [4]. | - | - | - | - |
| 101 | The SS configures: - NGC Cell A as the “Non-Suitable "off" cell ". - NGC Cell E as the "Suitable neighbour cell". - NGC Cell F as the “Serving cell". | - | - | - | - |
| - | The following messages are to be observed on NGC Cell E unless explicitly stated otherwise | - | - | - | - |
| 102 | The UE is Switched ON | - | - | - | - |
| 103-121a1 | Steps 2-20a1 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed. | - | - | 3 | P |
| 122 | The UE is switched off by executing generic procedure in Table 4.9.6.1-1 in TS 38.508-1 [4]. | - | - | - | - |
| 123 | The SS configures: - NGC Cell A as the “Non-Suitable "off" cell ". - NGC Cell E as the "Non-Suitable "off" cell ". - NGC Cell F as the “Serving cell". | - | - | - | - |
| 124 | The UE is Switched ON | - | - | - | - |
| - | The user is prompted to manually select PLMN of NGC Cell F | - | - | - | - |
| - | The following messages are to be observed on NGC Cell F unless explicitly stated otherwise | - | - | - | - |
| 125-143a1 | Steps 2-20a1 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed. | - | - | - | - |
| 144 | The user sets the UE in Automatic PLMN selection mode. | - | - | - | - |
| 145 | Wait for 300s to allow the UE to switch to automatic PLMN selection mode. | - | - | - | - |

9.1.5.1.2.3.3	Specific message contents

Table 9.1.5.1.2.3.3-1: REGISTRATION ACCEPT (step 15 Table 9.1.5.1.2.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.7.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Equivalent PLMNs |  | MCC=003, MNC=101 | PLMN ID of NGC Cell A |  |

Table 9.1.5.1.2.3.3-2: REGISTRATION ACCEPT (step 35 Table 9.1.5.1.2.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.7.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Equivalent PLMNs |  | MCC=002, MNC=101 | PLMN ID of NGC Cell E |  |

Table 9.1.5.1.2.3.3-3: REGISTRATION ACCEPT (step 94 Table 9.1.5.1.2.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.7.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Equivalent PLMNs |  | MCC=003, MNC=101 and MCC=002, MNC=101 | PLMN ID of NGC Cell F and PLMN ID of NGC Cell E |  |

Table 9.1.5.1.2.3.3-4: REGISTRATION REJECT (step 77 Table 9.1.5.1.2.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.7.1-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| 5GMM Cause |  | ‘0000 1011’B | PLMN not allowed |  |

<<<<<<<<<SKIPPED TEXT>>>>>>>>>>>>

##### 9.1.5.1.8	Initial registration / Rejected / Serving network not authorized

9.1.5.1.8.1	Test Purpose (TP)

(1)

with { The UE has sent initial REGISTRAION REQUEST message }

ensure that {

when { the UE receives a REGISTRATION REJECT with cause #73 (Serving network not authorized) }

then { the UE stores the PLMN identity in the "forbidden PLMN list" and does not attempt to register on a cell belong to that PLMN }

}

9.1.5.1.8.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.501, clauses 5.5.1.2.5. Unless otherwise stated these are Rel-15 requirements.

[TS 24.501, clause 5.5.1.2.5]

If the initial registration request cannot be accepted by the network, the AMF shall send a REGISTRATION REJECT message to the UE including an appropriate 5GMM cause value.

If the initial registration request is rejected due to general NAS level mobility management congestion control, the network shall set the 5GMM cause value to #22 "congestion" and assign a back-off timer T3346.

The UE shall take the following actions depending on the 5GMM cause value received in the REGISTRATION REJECT message.

#3	(Illegal UE); or

#6	(Illegal ME).

….

#73	(Serving network not authorized).

The UE shall set the 5GS update status to 5U2 NOT UPDATED, reset the registration attempt counter, store the PLMN identity in the "forbidden PLMN list" and enter state 5GMM-DEREGISTERED.PLMN-SEARCH in order to perform a PLMN selection according to 3GPP TS 23.122 [5].

9.1.5.1.8.3	Test description

9.1.5.1.8.3.1	Pre-test conditions

System Simulator:

-	3 NGC Cells NGC Cell E, NGC Cell I and NGC Cell A are configured as specified in TS 38.508-1[4] table 6.3.2.2-1 and table 6.3.2.2-3.

UE:

-	The UE is in Automatic PLMN selection mode.

Preamble:

-	NGC Cell E is set to “Serving Cell”.

-	NGC Cell I is set to “Non-Suitable cell”.

-	NGC Cell A is set to “Non-Suitable cell”.

-	The UE is in state 0N-B with a successful registration on NGC Cell E according to 38.508-1[4].

9.1.5.1.8.3.2	Test procedure sequence

Table 9.1.5.1.8.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE is switched on | - | - | - | - |
| 2-9 | Steps 2-9 of Table 4.5.2.2-2 in TS38.508-1 [4] are performed on NGC Cell E. | - | - | - | - |
| 10 | The SS transmits a REGISTRATION REJECT with cause #73 (Serving network not authorized). | <-- | REGISTRATION REJECT | - | - |
| 11 | The SS releases the RRC connection | - | - | - | - |
| 11A | The SS configures - NGC Cell I as “Suitable neighbour cell” | - | - | - | - |
| 12 | Check: Does the UE send in the next 30 sec a request for RRC connection establishment on Cell E or Cell I. | --> | NR RRC: RRCSetupRequest | 1 | F |
| 13 | The SS configures - NGC Cell A as “Serving Cell” | - | - | - | - |
| 14 | Check: Does the UE perform Registration procedure on NGC Cell A as specified in TS 38.508-1 [4] subclause 4.5.2? | - | - | 1 | P |
| 14A | Switch off procedure in RRC_IDLE specified in TS 38.508-1 subclause 4.9.6.1 is performed. | - | - | - | - |
| 15 | The SS configures - NGC Cell A as “Non-Suitable "Off" cell” | - | - | - | - |
| 15A | The UE is brought back to operation or the USIM is inserted. The UE is powered on or switched on. | - | - | - | - |
| 16 | Set the UE in manual PLMN selection mode or request a PLMN search. | - | - | - | - |
| 17 | The user selects the PLMN of NGC Cell E. | - | - | - | - |
| 18 | The UE performs Registration procedure on NGC Cell E as specified in TS 38.508-1 [4] subclause 4.5.2. | - | - | - | - |
| 19 | Set the UE in Automatic PLMN selection mode. | - | - | - | - |
| 20 | Wait for 300s to allow the UE to switch to automatic PLMN selection mode. | - | - | - | - |

9.1.5.1.8.3.3	Specific message contents

Table 9.1.5.1.8.3.3-1: REGISTRATION REJECT (step 10 Table 9.1.5.1.8.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.7.1-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| 5GMM cause |  | ‘01001001’B | Cause #73 (Serving network not authorized) |  |

<<<<<<<<<SKIPPED TEXT>>>>>>>>>>>>

##### 9.1.5.1.10	Initial registration / Rejected / PLMN not allowed

9.1.5.1.10.1	Test Purpose (TP)

(1)

with { the UE in 5GMM-REGISTERED-INITIATED state }

ensure that {

when { the SS sends a REGISTRATION REJECT message to the UE including an appropriate 5GMM cause value #11 (PLMN not allowed) }

then { the UE deletes any 5G-GUTI, last visited registered TAI and ngKSI, deletes the list of equivalent PLMNs, stores the PLMN identity in the "forbidden PLMN list" and performs a PLMN selection }

}

9.1.5.1.10.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.501 clauses 5.5.1.2.5. Unless otherwise stated these are Rel-15 requirements.

[TS 24.501, clause 5.5.1.2.5]

If the initial registration request cannot be accepted by the network, the AMF shall send a REGISTRATION REJECT message to the UE including an appropriate 5GMM cause value.

…

The UE shall take the following actions depending on the 5GMM cause value received in the REGISTRATION REJECT message.

…

#11	(PLMN not allowed).

The UE shall set the 5GS update status to 5U3 ROAMING NOT ALLOWED (and shall store it according to subclause 5.1.3.2.2) and shall delete any 5G-GUTI, last visited registered TAI, TAI list and ngKSI. The UE shall delete the list of equivalent PLMNs and reset the registration attempt counter and store the PLMN identity in the "forbidden PLMN list". The UE shall enter state 5GMM-DEREGISTERED.PLMN-SEARCH and perform a PLMN selection according to 3GPP TS 23.122 [5]. If the message has been successfully integrity checked by the NAS, the UE shall set the PLMN-specific attempt counter and the PLMN-specific attempt counter for non-3GPP access for that PLMN to the UE implementation-specific maximum value.

9.1.5.1.10.3	Test description

9.1.5.1.10.3.1	Pre-test conditions

System Simulator:

-	NGC cell G and NGC cell I.

-	The NGC cells are configured according to Table 6.3.2.2-1 and Table 6.3.2.2-3 in 38.508-1 [4], except replacing NRf3 with NRf1;

UE:

-	the UE is previously registered on NGC, and when on NGC, the UE is last authenticated and registered on NGC cell G using default message contents according to TS 38.508-1 [4];

Preamble:

-	The UE is in state 0N-B according to TS 38.508-1 [4].

9.1.5.1.10.3.2	Test procedure sequence

Table 9.1.5.1.10.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U – S | Message |  |  |
| 1 | The SS configures: - NGC Cell G as the "Serving cell". - NGC Cell I as a "Non-Suitable Off cell". | - | - | - | - |
| 2 | Void |  |  |  |  |
|  | The following messages are to be observed on Cell G unless explicitly stated otherwise. | - | - | - | - |
| 3 | The UE is switched on. | - | - | - | - |
| 4-11 | Steps 2-9 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed. | - | - | - | - |
| 12 | The SS transmits an REGISTRATION REJECT message including an appropriate 5GMM cause value #11 (PLMN not allowed). | <-- | 5GMM: REGISTRATION REJECT | - | - |
| 13 | The SS releases the RRC connection. | - | - | - | - |
| 14 | Check: Does the UE transmit an REGISTRATION REQUEST message on NGC cell G in the next 90 seconds? | --> | 5GMM: REGISTRATION REQUEST | 1 | F |
| 15 | The generic test procedure in TS 38.508-1 Table 4.9.6.4-1 of Switch off procedure in State DEREGISTERED are performed. | - | - | - | - |
| 16 | The UE is brought back to operation or the USIM is inserted. The UE is powered on or switched on. | - | - | - | - |
| 17 | Check: Does the UE transmit an REGISTRATION REQUEST message in the next 30 seconds? | --> | 5GMM: REGISTRATION REQUEST | 1 | F |
| 18 | The SS configures: -NGC Cell G as the "Serving cell" -NGC Cell I as a "Suitable neighbour cell". | - | - | - | - |
| 19 | Void |  |  |  |  |
| - | The following messages are to be observed on NGC Cell I unless explicitly stated otherwise. | - | - | - | - |
| 20-22 | The UE establishes RRC connection by executing steps 2-4 of Table 4.5.2.2-2 in TS 38.508-1 [4]. | - | - | - | - |
| 23 | Check: Does the UE transmit an REGISTRATION REQUEST message as specified? | --> | 5GMM: REGISTRATION REQUEST | 1 | P |
| 24-39a1 | Steps 5-19a1 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed. | - | - | - | - |
| 40 | The generic test procedure in TS 38.508-1 Table 4.9.6.3-1 of Switch off procedure in RRC_CONNECTED are performed. | - | - | - | - |
| 40A | The SS configures: -NGC Cell G as the "Serving cell" -NGC Cell I as a "Non-Suitable "off" cell ". |  |  |  |  |
| 40B | The UE is brought back to operation or the USIM is inserted. The UE is powered on or switched on. | - | - | - | - |
| 40C | The user sets the UE in manual PLMN selection mode or requests a PLMN search. | - | - | - | - |
| 41 | The user selects PLMN of NGC Cell I. | - | - | - | - |
| 42-60a1 | Steps 2 to 20a1 of the registration procedure described in TS 38.508-1 [4] subclause 4.5.2 are performed on NGC Cell G. | - | - | - | - |
| 61 | The user sets the UE in Automatic PLMN selection mode. | - | - | - | - |
| 62 | Wait for 300s to allow the UE to switch to automatic PLMN selection mode. | - | - | - | - |

9.1.5.1.10.3.3	Specific message contents

Table 9.1.5.1.10.3.3-1: Message REGISTRATION REJECT (step 12, Table 9.1.5.1.10.3.2-1)

| Derivation path: TS 38.508-1 [4],table 4.7.1-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| 5GMM cause | '00001011'B | #11 "PLMN not allowed" |  |

Table 9.1.5.1.10.3.3-2: Message REGISTRATION REQUEST (step 23, Table 9.1.5.1.10.3.2-1)

| Derivation path: TS 38.508-1 [4], table 4.7.1-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| ngKSI |  |  |  |
| NAS key set identifier | '111'B | no key is available (UE to network) |  |
| TSC | Any allowed value | TSC does not apply for NAS key set identifier value "111" |  |
| 5GS mobile identity | The valid SUCI | . |  |
| Last visited registered TAI | Not present |  |  |