3GPP TSG-RAN WG5 Meeting #90-e	R5-210323

# Electronic Meeting, 22nd Feb 2021 - 5th Mar 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-1 | CR | 1456 | rev | - | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to annexure A.3.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK, MCC TF160, Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, EIEI-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - R5-174525 has introduced Call-Info, Accept and Recv-Info headers for the 200 OK message for eCall. These headers are specific for eCall (condition A12) and not needed in other cases. Nevertheless due to A.3.1 table cleanup (R5-193850) condition A12 is missing for these headers since V14.7.0. Furthermore A12 needs to be applied for the Message-body too. - Due to R5-193850 the Via header is "not present" even though the via-parm still is "same value as received in request"; as there is no justification in R5-193850 for the Via not being present and there is the contradiction of Via header and via-parm, it is assumed that "not present" has been added by mistake. - Due to R5-193850 condition A6 got lost for the P-Asserted-Identity header; again it is assumed that this happened by mistake. - Due to R5-193850 the Content-Id got lost from the Message-body for condition A14; again it is assumed that this happened by mistake. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Condition A12 added to Call-Info, Accept and Message-body and corrected for Recv-Info, Content-Type - "not present" removed from Via header - Condition A6 added to P-Asserted-Identity - Content-Id added to the Message-body for condition A14 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect test specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | No TTCN impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modification>

## A.3.1	200 OK for other requests than REGISTER or SUBSCRIBE

| Header/param | Cond | Value/remark | Rel | Reference |
| --- | --- | --- | --- | --- |
| Status-Line |  |  |  | RFC 3261 [15] |
| SIP-Version |  | SIP/2.0 |  |  |
| Status-Code |  | 200 |  |  |
| Reason-Phrase |  | OK |  |  |
| Via |  |  |  | RFC 3261 [15] |
| via-parm |  | same value as received in request |  |  |
| Record-Route |  | order of the parameters in this header must be like in the respective rows |  | RFC 3261 [15] |
| rec-route | A1 | <sip:pcscf.other.com;lr>, <sip:scscf.other.com;lr>, <sip:orig@scscf.3gpp.org;lr>, <sip:SS P-CSCF address: protected server port of SS;lr> |  |  |
|  | A3 | <sip:pcscf.other.com;lr>, <sip:scscf.other.com;lr>, <sip:orig@scscf.3gpp.org;lr>, <sip:SS P-CSCF address: unprotected server port of SS (optional);lr> |  |  |
|  | A2,A4, A5 | same value as received in the request (if present in the request) Note: for requests other than INVITE it is not regulated if and what the UE writes into this header in a response. |  |  |
|  | A6 | <sip:orig@ecscf.other.com;lr>, <sip:SS P-CSCF address:protected server port of SS;lr> |  |  |
|  | A7 | <sip:orig@ecscf.other.com;lr>, <sip:SS P-CSCF address:unprotected server port of SS;lr> |  |  |
| From |  |  |  | RFC 3261 [15] |
| addr-spec |  | same value as received in request |  |  |
| tag |  | same value as received in request |  |  |
| To |  |  |  | RFC 3261 [15] |
| addr-spec |  | same value as received in request |  |  |
| tag |  | same value as received in request or any value added if missing from request |  |  |
| P-Asserted-Identity | A6 |  |  | RFC 3325 [89] |
| addr-spec |  | A tel URI that can be recognized as valid emergency numbers if dialled by the user are specified in 3GPP TS 22.101 [39]. The emergency numbers 112 and 911 are stored on the ME, in accordance with 3GPP TS 22.101 [39] |  |  |
| uri-parameter |  | lr |  |  |
| Contact | A1,A2 A3,A4 |  |  | RFC 3261 [15] RFC 5627 [61 |
| addr-spec | A1,A3 | px_IMS_CalleeContactUri |  |  |
|  | A2 AND NOT A9 | SIP URI with IP address or FQDN and protected server port of UE |  |  |
|  | A4 AND NOT A9 | SIP URI with IP address or FQDN and unprotected server port of UE |  |  |
|  | A2 AND A9 | Public GRUU as obtained during registration as pub-gruu contact parameter of the 200 OK for REGISTER response |  |  |
|  | A4 AND A9 | Public GRUU as obtained during registration as pub-gruu contact parameter of the 200 OK for REGISTER response |  |  |
| feature-param | A10 | audio |  |  |
|  | A18 AND (A19 OR A20) | video |  | IR.94 [134] |
|  | A11 AND A14 AND (A15 OR A16) | audio |  |  |
| Call-ID |  |  |  | RFC 3261 [15] |
| callid |  | same value as received in request |  |  |
| Call-Info | A12 |  |  | RFC 8147 [149] |
| cid URL |  | psap@3gpp.org | Rel-14 |  |
| purpose |  | EmergencyCallData.Control |  |  |
| CSeq |  |  |  | RFC 3261 [15] |
| value |  | same value as received in request |  |  |
| P-Access-Network-Info | A8 |  |  | RFC 7315 [132] RFC 7913 [154] |
| access-net-spec | A21 | access network information and, if applicable, the cell ID |  |  |
|  | A22 | access network information for NR, containing access-class parameter with value "3GPP-NR" or access-type parameter with value "3GPP-NR-FDD" or "3GPP-NR-TDD", and also containing the cell ID |  |  |
| Accept | A12 |  |  | RFC 8147 [149] |
| media-range |  | application/sdp, application/pidf+xml, application/EmergencyCallData.Control+xml, application/emergencyCallData.eCall.MSD | Rel-14 |  |
| Recv-Info | A12 |  |  | RFC 8147 [149] |
| Info-package-type |  | emergencyCallData.eCall.MSD | Rel-14 |  |
| Content-Type | A12 |  |  | RFC 8147 [149] |
| media-type |  | multipart/mixed; boundary=boundary1 | Rel-14 |  |
| Content-Length | A10 |  |  | RFC 3261 [15] |
| value | NOT A12 | 0 |  |  |
|  | A12 | length of message body |  |  |
| Message-body | A12 |  |  |  |
|  | A13 | --boundary1 Content-Type: application/EmergencyCallData.eCall.Control+xml Content-ID: same cid as in Call-Info header Content-Disposition: by-reference <?xml version="1.0" encoding="UTF-8"?> <EmergencyCallData.control xmlns="urn:ietf:params:xml:ns:EmergencyCallData:control"> <ack received="true" ref=" cid URL of MIME body part containing the MSD sent by the UE in INVITE"/> </EmergencyCallData.control> --boundary1 |  | RFC 8147 [149] |
|  | A14 | --boundary1 Content-Type: application/EmergencyCallData.eCall.Control+xml Content-ID: same cid as in Call-Info header Content-Disposition: by-reference <?xml version="1.0" encoding="UTF-8"?> <emergencyCallData.Control xmlns="urn:ietf:params:xml:ns:EmergencyCallData:control"> <ack received="false" ref=" cid URL of MIME body part containing the MSD sent by the UE in INVITE "/> </EmergencyCallData.control> --boundary1 |  | RFC 8147 [149] |

| Condition | Explanation |
| --- | --- |
| A1 | Response sent by SS for INVITE/UPDATE (IMS security, A.6a/2 TS 34.229-2 [5])) |
| A2 | Response sent by UE for INVITE/UPDATE (IMS security, A.6a/2 TS 34.229-2 [5])) |
| A3 | Response sent by SS for INVITE/UPDATE (GIBA, A.6a/1 TS 34.229-2 [5])) |
| A4 | Response sent by UE for INVITE/UPDATE (GIBA, A.6a/1 TS 34.229-2 [5])) |
| A5 | Any response sent by the UE within a dialog |
| A6 | Response sent by SS for INVITE for emergency call or non-UE detectable emergency call |
| A7 | Response sent by SS for INVITE for emergency call without emergency registration |
| A8 | Any response sent by the UE within a dialog, except for CANCEL requests |
| A9 | obtaining and using GRUUs in the Session Initiation Protocol (SIP) (A.4/53 3GPP TS 34.229-2 [5]) |
| A10 | Response sent by SS |
| A11 | Response sent by UE |
| A12 | Response sent by SS for INVITE for eCall over IMS session with either ACK or NACK |
| A13 | Response sent by SS for INVITE for eCall over IMS session with ACK element = TRUE |
| A14 | Response sent by SS for INVITE for eCall over IMS session with ACK element = FALSE |
| A15 | UE supports audio media feature tag (A.12/56 3GPP TS 34.229-2 [5]) |
| A16 | UE uses E-UTRAN access and has received IMS voice over PS Session Supported Indication in the NAS ATTACH ACCEPT message as described in TS 24.301 [150], clauses 8.2.1 and 9.9.3.12A |
| A17 | UE uses UTRAN/GERAN access and has received IMS voice over PS Session Supported Indication in the NAS ATTACH ACCEPT message as described in TS 24.008 [12], clauses 9.4.2 and 10.5.5.23 |
| A18 | UE supports video feature tag (A.12/32 3GPP TS 34.229-2 [5]) |
| A19 | Response sent by SS for INVITE |
| A20 | Response sent by UE for INVITE |
| A21 | UE uses E-UTRAN access (A.18/1 3GPP TS 34.229-2 [5]) |
| A22 | UE uses NR access (A.18/5 3GPP TS 34.229-2 [5]) |

<End of modification>