3GPP TSG-RAN5 Meeting #90-e	R5-210461

# Electronic Meeting, 22nd February – 5th March 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 1702 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Editorial rework of the conditions for CSI-FrequencyOccupation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rohde & Schwarz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Table 4.6.3-33 CSI-FrequencyOccupation defines a long list of conditions that are actually already defined as Global conditions in 4.6.0. FR1, FR2 and the SCS are already defined. The Cell BW could also be defined as global condition |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removed all duplicated conditions from 4.6.3-33 Defined Cell BW as global condition and removed from 4.6.3-33 condition list |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Table 4.6.3-33 will have several duplicated conditions |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.0, 4.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# < Unchanged sections omitted >

### 4.6.0	General

#### 4.6.0.1	Global conditions

Groups of RRC conditions including always one condition set to true.

Table 4.6.0.1-1: Signalling, RF/RRM/Performance

| Condition | Explanation |
| --- | --- |
| SIG | Used for signalling test cases |
| RF | Used for RF//Performance test cases |
| RRM | Used for RRM test cases |

Table 4.6.0.1-2: NR operating bands

| Condition | Explanation |
| --- | --- |
| FR1 | 410 MHz – 7125 MHz |
| FR2 | 24250 MHz – 52600 MHz |

Table 4.6.0.1-3: FDD/TDD

| Condition | Explanation |
| --- | --- |
| FDD | Frequency Division Duplex |
| TDD | Time Division Duplex |

Table 4.6.0.1-4: Subcarrier spacing

| Condition | Explanation |
| --- | --- |
| SCS15 | 15kHz |
| SCS30 | 30kHz |
| SCS60 | 60kHz |
| SCS120 | 120kHz |
| SCS240 | 240kHz |

Table 4.6.0.1-5: Cell Bandwidth

| Condition | Explanation |
| --- | --- |
| BW5 | 5 MHz |
| BW10 | 10 MHz |
| BW15 | 15 MHz |
| BW20 | 20 MHz |
| BW25 | 25 MHz |
| BW30 | 30 MHz |
| BW40 | 40 MHz |
| BW50 | 50 MHz |
| BW60 | 60 MHz |
| BW80 | 80 MHz |
| BW100 | 100 MHz |

# < Unchanged sections omitted >

#### –	CSI-FrequencyOccupation

Table 4.6.3-33: CSI-FrequencyOccupation

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-FrequencyOccupation ::= SEQUENCE { |  |  |  |
| startingRB | 0 |  |  |
| nrofRBs | 160 |  | FR1 AND BW60 |
|  | 216 |  | FR1 AND BW80 |
|  | 108 |  | FR1 AND BW40 |
|  | 52 |  | FR1 AND BW10 |
|  | 276 |  | FR1 AND BW100 |
|  | 64 |  | FR2 AND BW100 |
|  | 52 |  | TRS |
|  | 40 |  | TRS AND ((FR1 AND BW15 AND SCS30) OR (FR1 AND BW30AND SCS60)) |
|  | 32 |  | TRS AND ((FR1 AND BW25 AND SCS60) OR (FR2 AND BW50 AND SCS120)) |
|  | 28 |  | TRS AND FR1 AND BW5 AND SCS15 |
|  | 24 |  | TRS AND ((FR1 AND BW5 AND SCS30) OR (FR1 AND BW10 AND SCS30) OR (FR1 AND BW10 AND SCS60) OR (FR1 AND BW15 AND SCS60) OR (FR1 AND BW20 AND SCS60)) |
|  | 64 |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| TRS | Tracking-Reference Signal |

< End of changes >