3GPP TSG-RAN5 Meeting #90-e	R5-210628

# Electronic Meeting, 22th Feb– 5th Mar 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2002 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR TC 6.1.1.6-PLMN Selection with MinimumPeriodicSearchTimer |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-2-6 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - According to TS 24.501 subclause 5.2.3.2.5 specified in Release 15, UE will perform initial registration procedure if a new PLMN is selected. Thus, the 5GS registration type shall be set as Initial Registration for R-15 UEs and Mobility Registration for R16 UEs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Voided steps 24-28a1 and 31-35a1. - Added steps 23A-23Cb3a1 and 30A-30Cb3a1. - Added Note 2. - Corrected PLMN vs. NR Cell mapping throughout the test case description (cl. 6.1.1.6.3).See E-UTRA test case 6.1.1.7 in TS 36.523-1 which covers similar test purposes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Normal UE may fail the TC |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.1.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

6.1.1.6	PLMN selection / Periodic reselection / MinimumPeriodicSearchTimer

6.1.1.6.1	Test Purpose (TP)

(1)

with { UE configured with “MinimumPeriodicSearchTimer” }

ensure that {

when { UE camps on an NG-RAN VPLMN cell upon switch on and cells of a higher priority NG-RAN PLMN available }

then { the MS shall make the first attempt to access the HPLMN or an EHPLMN or higher priority PLMN after a period of at least 2 minutes }

}

(2)

with { UE configured with “MinimumPeriodicSearchTimer”, having made first attempt to higher priority PLMN and camped on an NG-RAN VPLMN cell and cells of a higher priority NG-RAN PLMN available }

ensure that {

when { the higher priority PLMN search timer T stored in the USIM  or the default value for T is less than the “MinimumPeriodicSearchTimer” }

then { UE shall not use a value for T that is less than the “MinimumPeriodicSearchTimer” and selects and camps on a cell of the highest priority PLMN and attempts a location registration on the selected cell upon expiry of “MinimumPeriodicSearchTimer” }

}

6.1.1.6.2	Conformance requirements

References: The conformance requirements covered in the present test case are specified in: TS 23.122, clause 4.4.3.3.1 Unless otherwise stated these are Rel-15 requirements.

[TS 23.122, clause 4.4.3.3.1]

If the MS is in a VPLMN, the MS shall periodically attempt to obtain service on its HPLMN (if the EHPLMN list is not present or is empty) or one of its EHPLMNs (if the EHPLMN list is present) or a higher priority PLMN/access technology combinations listed in "user controlled PLMN selector" or "operator controlled PLMN selector" by scanning in accordance with the requirements that are applicable to i), ii) and iii) as defined in the Automatic Network Selection Mode in subclause 4.4.3.1.1. In the case that the mobile has a stored "Equivalent PLMNs" list the mobile shall only select a PLMN if it is of a higher priority than those of the same country as the current serving PLMN which are stored in the "Equivalent PLMNs" list. For this purpose, a value T minutes may be stored in the SIM, T is either in the range 6 minutes to 8 hours in 6 minute steps or it indicates that no periodic attempts shall be made. If no value is stored in the SIM, a default value of 60 minutes is used for T.

If the MS is configured with the MinimumPeriodicSearchTimer as specified in 3GPP TS 24.368 [50] or 3GPP TS 31.102 [40], the MS shall not use a value for T that is less than the MinimumPeriodicSearchTimer. If the value stored in the SIM, or the default value for T (when no value is stored in the SIM), is less than the MinimumPeriodicSearchTimer, then T shall be set to the MinimumPeriodicSearchTimer.

The attempts to access the HPLMN or an EHPLMN or higher priority PLMN shall be as specified below:

a)	The periodic attempts shall only be performed in automatic mode when the MS is roaming, and not while the MS is attached for emergency bearer services or has a PDN connection for emergency bearer services;

b)	After switch on a period of at least 2 minutes and at most T minutes shall elapse before the first attempt is made;

6.1.1.6.3	Test Description

6.1.1.6.3.1	Pre-test conditions

System Simulator:

-	3 NR cells: NR Cell 11, 12 and 13 as specified in TS 38.508-1[4] table 4.4.2-3 are configured as shown in Table 6.1.1.6.3.1–1.

-	PLMN settings are defined in TS 36.523-1 [13] table 6.0.1-1.

Table 6.1.1.6.3.1-1: PLMN identifiers

| NR Cell | PLMN names |
| --- | --- |
| NR Cell 11 | PLMN1 |
| NR Cell 12 | PLMN15 |
| NR Cell 13 | PLMN16 |

UE:

-	The UE is in Automatic PLMN selection mode.

-	The UE is configured with a value of MinimumPeriodicSearchTimer set to 7 minutes.

-	The UE is equipped with a USIM configuration 11 as per TS 38.508-1 [4] Table 6.4.1-11.

Preamble:

-	The UE performs a successful registration on PLMN15 after which it is switched OFF (State 0N-B) as per TS 38.508-1 [4] table 4.4A.2-0.

6.1.1.6.3.2	Test procedure sequence

Table 6.1.1.6.3.2-1 and Table 6.1.1.6.3.2-2 shows the cell configurations used during the test. The configuration T0 indicates the initial conditions. Subsequent configurations marked “T1” & “T2” are applied at the point indicated in the Main behaviour description in Table 6.1.1.6.3.2-3. Cell powers are chosen for a serving cell and a non-suitable [Off] cell as defined in TS 38.508-1[4] table 6.2.2.1-3.

Table 6.1.1.6.3.2-1: Time instances of cell power level and parameter changes for FR1

|  | Parameter | Unit | NR Cell 11 | NR Cell 12 | NR Cell 13 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | Off | -88 | Off |  |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | Off | -88 | -88 |  |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | -88 | -88 | -88 |  |

Table 6.1.1.6.3.2-2: Time instances of cell power level and parameter changes for FR2

|  | Parameter | Unit | NR Cell 11 | NR Cell 12 | NR Cell 13 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | Off | -82 | Off |  |
|  | Qrxlevmin | dBm | - | -91+ Delta(NRf2) | - |  |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | Off | -82 | -82 |  |
|  | Qrxlevmin |  | - | -91+ Delta(NRf2) | -91+ Delta(NRf3) |  |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | -82 | -82 | -82 |  |
|  | Qrxlevmin |  | -91+ Delta(NRf1) | -91+ Delta(NRf2) | -91+ Delta(NRf3) |  |

Table 6.1.1.6.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS adjusts cell levels according to row T1 of table 6.1.1.6.3.2-1/2. | - | - | - | - |
| 2 | Power on the UE. | - | - | - | - |
| 3-22a1 | Steps 1 to 20a1 of the registration procedure described in TS 38.508-1[4] table 4.5.2.2-2 are performed on NR Cell 12. | - | - | - | - |
| 23 | Check: Does the UE send an RRCSetupRequest on NR Cell 13 after 120 seconds, but before 420 seconds (“MinimumPeriodicSearchTimer”) from power on? (Note 1) | --> | NR RRC: RRCSetupRequest | 1 | P |
| 23A-23B | Steps 3-4 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed. (Note 2) | - | - | - | - |
| - | EXCEPTION: Steps 23Ca1 to 23Cb5 describe behaviour that depends on events happening prior to their execution; the "lower case letter" identifies a step sequence that take place if a specific prior event takes place, | - | - | - | - |
| 23Ca1-23Ca17a1 | IF 5GS registration type is set as Initial Registration in step 23B, THEN Steps 4 to 20a1of the generic test procedure in TS 38.508-1 Table 4.5.2.2-2 are performed on NR Cell 13. | - | - | - | - |
| 23Cb1-23Cb3a1 | IF 5GS registration type is set as Mobility Registration in step 23B, THEN Steps 4 to 6a1 of the generic test procedure in TS 38.508-1 Table 4.9.5.2.2-1 are performed on NR Cell 13. | - | - | - | - |
| 24-28a1 | Void | - | - | - | - |
| 29 | SS adjusts cell levels according to row T2 of table 6.1.1.6.3.2-1/2. | - | - | - | - |
| 30 | Check: Does the UE send an RRCSetupRequest on NR Cell 11 after 420 seconds (“MinimumPeriodicSearchTimer”) from step 24? (Note 1) | --> | NR RRC: RRCSetupRequest | 2 | P |
| 30A-30B | Steps 3-4 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed. (Note 2) | - | - | - | - |
| - | EXCEPTION: Steps 30Ca1 to 30Cb3a1 describe behaviour that depends on events happening prior to their execution; the "lower case letter" identifies a step sequence that take place if a specific prior event takes place, | - | - | - | - |
| 30Ca1-30Ca17a1 | IF 5GS registration type is set as Initial Registration in step 30B, THEN Steps 4 to 20a1of the generic test procedure in TS 38.508-1 Table 4.5.2.2-2 are performed on NR Cell 11. | - | - | - | - |
| 30Cb1-30Cb3a1 | IF 5GS registration type is set as Mobility Registration in step 30B, THEN Steps 4 to 6a1 of the generic test procedure in TS 38.508-1 Table 4.9.5.2.2-1 are performed on NR Cell 11. | - | - | - | - |
| 31-35a1 | Void | - | - | - | - |
| Note 1: Timers in Steps 23 and 30 are derived from the value defined by the “MinimumPeriodicSearchTimer”. Note 2: The 5GS registration type shall be only set as Mobility Registration for R-16 UEs according to TS 24.501 subclause 5.2.3.2.5 specified in Release 16. The EXCEPTION description applies only to R15 UEs. |  |  |  |  |  |

6.1.1.6.3.3	Specific message contents

None

<End of modified section 1>