3GPP TSG-RAN5 Meeting #90-e	R5-210634

# Electronic Meeting, 22th Feb– 5th Mar 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2008 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR TC 6.3.1.2-SoR ACK has NOT requested |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-2-6 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - If UE performs steps 23a1-23a15a1, due to the setting of SOR, it’s possible that the UE will perform the reselection after the N1 signaling releasing, which may cause an error before the postamble procedure. TS 23.122: 11) If the UE has a list of available PLMNs in the area and based on this list the UE determines that there is a higher priority PLMN than the currently camped chosen VPLMN and the UE is in automatic network selection mode, then the UE shall attempt to obtain service on a higher priority PLMN as specified in subclause 4.4.3.3 by acting as if timer T that controls periodic attempts has expired after the release of the N1 NAS signalling connection. Therefore, to solve this issue, other unused cells shall be turned off before completing the registration procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added step 22A to turn off the unused cells. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Normal UE may fail the TC |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section >

6.3.1.2	Steering of UE in roaming during registration/security check successful but SOR Transparent container indicates ACK has been NOT been requested

6.3.1.2.1	Test Purpose (TP)

(1)

with {UE being in automatic PLMN selection mode, current VPLMN not part of “PLMNs where registration was aborted due to SOR” List and not part of "User Controlled PLMN Selector with Access Technology" list and UE's USIM configured with indication that the UE is to receive Steering of Roaming information due to initial registration in a VPLMN}

ensure that {

when {SOR Transparent container indicates ACK has NOT been requested & List Type indicates PLMN ID and Access technology list in REGISTRATION ACCEPT and security check is successful and if the UE has a list of available and allowable PLMNs in the area and based on this list the UE determines that there is a higher priority PLMN than the currently camped chosen VPLMN}

then {the UE shall send the REGISTRATION COMPLETE message to the serving AMF without including an SOR transparent container and then either  releases N1 NAS Signaling connection locally and then attempt to obtain service on a higher priority PLMN or optionally wait for network release of the NAS signalling connection and then selects higher priority PLMN}

}

6.3.1.2.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 23.122, clause C.2, TS 24.501 clause 4.4.4.2 and 5.5.1.2.4. Unless otherwise stated these are Rel-15 requirements.

[TS 23.122, clause C.2]

6)	The VPLMN AMF to the UE: The VPLMN AMF shall transparently send the received steering of roaming information to the UE in the REGISTRATION ACCEPT message;

7)	If the steering of roaming information is received and the security check is successful, then:

a)	if the steering of roaming information contains a secured packet (see 3GPP TS 31.115 [67]):

-	if the UDM has not requested an acknowledgement from the UE the UE shall send the REGISTRATION COMPLETE message;

-	the ME shall upload the secured packet to the USIM using procedures in 3GPP TS 31.111 [41].

NOTE 1:	How the ME handles UICC responses and failures in communication between the ME and UICC is implementation specific and out of scope of this release of the specification.

-	if the UDM has not requested an acknowledgement from the UE and the ME receives a USAT REFRESH command qualifier (3GPP TS 31.111 [41]) of type "Steering of Roaming" it shall perform items a), b) and c) of the procedure for steering of roaming in subclause 4.4.6 and if the UE has a list of available and allowable PLMNs in the area and based on this list the UE determines that there is a higher priority PLMN than the currently camped chosen VPLMN and the UE is in automatic network selection mode, then the UE shall either:

i)	release the current N1 NAS signalling connection locally and then attempt to obtain service on a higher priority PLMN as specified in subclause 4.4.3.3 by acting as if timer T that controls periodic attempts has expired. In this case, steps 8 to 11 are skipped; or

ii)	not release the current N1 NAS signalling connection locally and skip steps 8 to 10;

b)	if the steering of roaming information contains the list of preferred PLMN/access technology combinations, the ME shall replace the highest priority entries in the "Operator Controlled PLMN Selector with Access Technology" list stored in the ME with the received list of preferred PLMN/access technology combinations. Additionally, if the UDM has not requested an acknowledgement from the UE and if the UE has a list of available and allowable PLMNs in the area and based on this list the UE determines that there is a higher priority PLMN than the currently camped chosen VPLMN and the UE is in automatic network selection mode, then the UE shall send the REGISTRATION COMPLETE message to the serving AMF without including an SOR transparent container and then either:

i)	release the current N1 NAS signalling connection locally and then attempt to obtain service on a higher priority PLMN as specified in subclause 4.4.3.3 by acting as if timer T that controls periodic attempts has expired. In this case, steps 8 to 11 are skipped; or

ii)	not release the current N1 NAS signalling connection locally and skip steps 8 and 10;

8)	If:

a)	the UE's USIM is configured with indication that the UE is to receive the steering of roaming information due to initial registration in a VPLMN, but neither the list of preferred PLMN/access technology combinations nor the secured packet nor the HPLMN indication that 'no change of the "Operator Controlled PLMN Selector with Access Technology" list stored in the UE is needed and thus no list of preferred PLMN/access technology combinations is provided' is received in the REGISTRATION ACCEPT message, when the UE performs initial registration in a VPLMN or if the steering of roaming information is received but the security check is not successful; and

b)	the current chosen VPLMN is not contained in the list of "PLMNs where registration was aborted due to SOR", not part of "User Controlled PLMN Selector with Access Technology" list, the UE is not in manual mode of operation and the PDU session for emergency services is not pending to be activated;

then the UE shall send the REGISTRATION COMPLETE message to the serving AMF without including an SOR transparent container, release the current N1 NAS signalling connection locally, store the PLMN identity in the list of "PLMNs where registration was aborted due to SOR" and attempt to obtain service on a higher priority PLMN as specified in subclause 4.4.3.3 by acting as if timer T that controls periodic attempts has expired, with an exception that the current PLMN is considered as lowest priority, and skip steps 9 to 12;

NOTE 2:	When the UE is in the manual mode of operation or the current chosen VPLMN is part of the "User Controlled PLMN Selector with Access Technology" list, the UE stays on the VPLMN.

9)	The UE to the VPLMN AMF: If the UDM has requested an acknowledgement from the UE:

the UE verified that the steering of roaming information has been provided by the HPLMN in step 7, the UE sends the REGISTRATION COMPLETE message to the serving AMF with an SOR transparent container including the UE acknowledgement; and

if the steering of roaming information contained a secured packet and the security check was successful, then when the UE receives the USAT REFRESH command qualifier of type "Steering of Roaming", it performs items a), b) and c) of the procedure for steering of roaming in subclause 4.4.6.

10)	The AMF to the UDM: If an SOR transparent container is received in the REGISTRATION COMPLETE message, the AMF uses the Nudm_SDM_Info service operation to provide the received SOR transparent container to the UDM. If the HPLMN decided that the UE is to acknowledge the successful security check of the received steering of roaming information in step 4, the UDM verifies that the acknowledgement is provided by the UE as specified in 3GPP TS 33.501 [66];

11)	If the UE has a list of available PLMNs in the area and based on this list the UE determines that there is a higher priority PLMN than the currently camped chosen VPLMN and the UE is in automatic network selection mode, then the UE shall attempt to obtain service on a higher priority PLMN as specified in subclause 4.4.3.3 by acting as if timer T that controls periodic attempts has expired after the release of the N1 NAS signalling connection. If the N1 NAS signaling connection is not released after implementation dependent time, the UE may locally release the N1 signaling connection except when the UE is registered for emergency services or the UE has an established emergency PDU session (see 3GPP TS 24.501 [64]); and

12)	The UE deletes the list of "PLMNs where registration was aborted due to SOR".

The list of "PLMNs where registration was aborted due to SOR" is deleted when the UE is switched off or the USIM is removed.

[TS 24.501 clause 4.4.4.2]

…

The network can provide the SOR transparent container IE during the registration procedure to the UE in the REGISTRATION ACCEPT message. The SOR transparent container IE is integrity protected by the HPLMN as specified in 3GPP TS 33.501 [24].

…

[TS 24.501 clause 5.5.1.2.4]

…

If the REGISTRATION ACCEPT message includes the SOR transparent container IE and the SOR transparent container IE successfully passes the integrity check (see 3GPP TS 33.501 [24]):

a)	the UE shall proceed with the behaviour as specified in 3GPP TS 23.122 [5] annex C; and

b)	if the registration procedure is performed over 3GPP access and the UE attempts obtaining service on another PLMNs as specified in 3GPP TS 23.122 [5] annex C, then the UE may locally release the established N1 NAS signalling connection after sending a REGISTRATION COMPLETE message. Otherwise the UE shall send a REGISTRATION COMPLETE message and not release the current N1 NAS signalling connection locally. If an acknowledgement is requested in the SOR transparent container IE of the REGISTRATION ACCEPT message, the UE acknowledgement is included in the SOR transparent container IE of the REGISTRATION COMPLETE message.

If the SOR transparent container IE successfully passes the integrity check (see 3GPP TS 33.501 [24]), indicates list of preferred PLMN/access technology combinations is provided and the list type indicates:

a)	"PLMN ID and access technology list", then the ME shall replace the highest priority entries in the "Operator Controlled PLMN Selector with Access Technology" list stored in the ME and shall proceed with the behaviour as specified in 3GPP TS 23.122 [5] annex C; or

b)	"secured packet", then the ME shall behave as if a SMS is received with protocol identifier set to SIM data download, data coding scheme set to class 2 message and SMS payload as secured packet contents of SOR transparent container IE. The SMS payload is forwarded to UICC as specified in 3GPP TS 23.040 [4A] and the ME shall proceed with the behaviour as specified in 3GPP TS 23.122 [5] annex C.

…

6.3.1.2.3	Test Description

6.3.1.2.3.1	Pre-test conditions

System Simulator:

-	Three inter-frequency multi-PLMN NR Cells as specified in TS 38.508-1 [4] Table 4.4.2-1 are configured broadcasting PLMNs as indicated in Table 6.3.1.2.3.1-1.

-	The PLMNs are identified in the test by the identifiers in Table 6.3.1.2.3.1-1. The MCC and MNC values corresponding to the PLMN identifier shall understood as those specified in TS 36.523-1 [42], Table 6.0.1-1.

Table 6.3.1.2.3.1–1: PLMN identifiers

| NR Cell | PLMN names |
| --- | --- |
| NR Cell 11 | PLMN2 |
| NR Cell 12 | PLMN13 |
| NR Cell 13 | PLMN14 |

NR Cell 11 is set to ''Serving Cell”;

NR Cell 12 is set to ''Serving Cell'';

NR Cell 13 is set to “Serving Cell'';

System Information Combination NR-4 as defined in TS38.508-1 [4] clause 4.4.3.1.3 is used in NR cells.

UE:

-	The UE is in Automatic PLMN selection mode.

-	USIM configuration as defined in Table 6.4.1-10 in TS 38.508-1 [4] will be used.

Preamble:

-	The UE is in Switched OFF (State 0-A) as defined in TS 38.508-1 [4] Table 4.4A.2-0.

-	UE’s Last Registered PLMN (RPLMN) needs to be cleared by using Registration Reject procedure as defined in TS 38.508-1 [4] Subclause 4.9.8.

6.3.1.2.4	Test procedure sequence

Table 6.3.1.2.4-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Power on the UE. | - |  | - | - |
| 2-14 | Steps 1 to 13 of the registration procedure described in TS 38.508-1 [4] subclause 4.5.2.2-2 are performed on NR Cell 13. | - | - | - | - |
| 15 | The SS transmits an DLInformationTransfer message and a REGISTRATION ACCEPT message containing steering of roaming information indicating that acknowledgment is not requested from the UE for successful reception. | <-- | NR RRC: DLInformationTransfer 5G MM: REGISTRATION ACCEPT | - | - |
| 16 | The SS also starts timer of tmax = (6 minutes + cell selection time) (Note 1, 2 and 3) | - | - | - | - |
| 17 | The UE transmits an ULInformationTransfer message and REGISTRATION COMPLETE message without Steering of Roaming Transparent container. | --> | NR RRC: ULInformationTransfer 5G MM: REGISTRATION COMPLETE | -1 | - |
| - | EXCEPTION: Step 18a1-18b1 describes behaviour depending UE implementation; the "lower case letter" identifies a step sequence that take place if the UE performs a specific action. | - | - | - | - |
| 18a1-18a5a1 | Steps 16-20a1 of the registration procedure described in TS 38.508-1 [4] Table 4.5.2.2-2 is performed. | - | - | - | - |
| 18b1 | The SS locally releases the RRC connection. | - | - | - | - |
| 19 | Void | - | - | - | - |
| 20 | Check: Does the UE transmits an RRCSetupRequest message on NR Cell 11 before tmax expires? (Note 1, 2 and 3) | --> | NR RRC: RRCSetupRequest | 1 | P |
| 21 | The SS transmits an RRCSetup message on NR Cell 11. | <-- | NR RRC: RRCSetup | - | - |
| 22 | The UE transmits an RRCSetupComplete message and a REGISTRATION REQUEST message on NR Cell 11. (Note 4) | --> | NR RRC: RRCSetupComplete 5GMM: REGISTRATION REQUEST | - | - |
| 22A | The SS configures: - NR Cell 12 as a "Non-Suitable "off" cell ". - NR Cell 13 as a "Non-Suitable "off" cell ". | - | - | - | - |
| - | EXCEPTION: Steps 23a1 to 23b5a1 describe behaviours which depend on procedure parameters; the "lower case letter" identifies a step sequence that take place if a procedure parameter has a particular value | - | - | - | - |
| 23a1-23a15a1 | IF 5GS registration type is set as Initial Registration in step 22, THEN steps 5 to 20a1 of the registration procedure described in TS 38.508-1 [4] Table 4.5.2.2-2 are performed on NR Cell 11. | - | - | - | - |
| 23b1-23b2 | Void | - | - | - | - |
| 23b3-23b5a1 | ELSE IF 5GS registration type is set as Mobility Registration in step 22, THEN steps 4 to 6a1 of the generic test procedure in TS 38.508-1 [4] Table 4.9.5.2.2-1 are performed on NR Cell 11. | - | - | - | - |
| 24-26 | Void | - | - | - | - |
| Note 1: Timer tmax in step 16 and 20 are derived from the high priority PLMN search timer T defined by EFHPPLMN Note 2: Following attempts to access the HPLMN/EHPLMN/higher priority PLMN in VPLMN is operator specific setting (Refer to TS 23.122 Rel-12). Hence, window between 120s to T+Tolerance is being used, where the high priority PLMN search timer T defined by EFHPPLMN Note 3: Tolerance of 5min is added to allow time for the UE to find the proper PLMN Note 4: The 5GS registration type shall be only set as Mobility Registration for Rel-16 UEs according to TS 24.501 subclause 5.2.3.2.5 specified in Release 16. The EXCEPTION description applies only to Rel-15 UEs. |  |  |  |  |  |

6.3.1.2.5	Specific message contents

Table 6.3.1.2.5-1: REGISTRATION ACCEPT for NR Cell 12 (step 15, Table 6.3.1.2.4-1)

| Derivation Path: 38.508-1 Table 4.7.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SOR Transparent Container | Present | The SOR transparent container carries steering of roaming information. |  |
| SoR-MAC-IAUS | Set to match the calculated SoR-MAC-IAUS as the way defined in TS 33.501 A.17 |  |  |
| CounterSOR | Value generated as per TS 33.501 Cl 6.14.2.3 |  |  |
| SOR data type | 0 | The SOR transparent container carries steering of roaming information. |  |
| List indication value | 1 | List of preferred PLMN/access technology combinations is provided |  |
| List type | 1 | The list type is a PLMN ID and access technology list |  |
| Acknowledgement (ACK) value | 0 | Acknowledgement is NOT requested |  |
| PLMN ID 1 | PLMN2 |  |  |
| Access Technology Identifier 1 | NG-RAN |  |  |
| PLMN ID 2 | PLMN13 |  |  |
| Access Technology Identifier 2 | NG-RAN |  |  |
| PLMN ID 3 | PLMN14 |  |  |
| Access Technology Identifier 3 | NG-RAN |  |  |

Table 6.3.1.2.5-2: Void

Table 6.3.1.2.5-3: REGISTRATION COMPLETE for NR Cell 13 (step 17, Table 6.3.1.2.4-1)

| Derivation Path: 38.508-1 Table 4.7.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SOR Transparent Container | Not Present | The SOR transparent container should not be present when ACK is not requested by UDM. |  |

Table 6.3.1.2.5-4: REGISTRATION ACCEPT for NR Cell 11 (step 23a10, Table 6.3.1.2.4-1)

| Derivation Path: 38.508-1 Table 4.7.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SOR Transparent Container | Present | The SOR transparent container carries steering of roaming information. |  |
| SoR-MAC-IAUS | Set to match the calculated SoR-MAC-IAUS as the way defined in TS 33.501 A.17 |  |  |
| CounterSOR | Value generated as per TS 33.501 Cl 6.14.2.3 |  |  |
| SOR data type | 0 | The SOR transparent container carries steering of roaming information. |  |
| List indication value | 1 | List of preferred PLMN/access technology combinations is provided |  |
| List type | 1 | The list type is a PLMN ID and access technology list |  |
| Acknowledgement (ACK) value | 0 | Acknowledgement is NOT requested |  |
| PLMN ID 1 | PLMN2 |  |  |
| Access Technology Identifier 1 | NG-RAN |  |  |
| PLMN ID 2 | PLMN13 |  |  |
| Access Technology Identifier 2 | NG-RAN |  |  |
| PLMN ID 3 | PLMN14 |  |  |
| Access Technology Identifier 3 | NG-RAN |  |  |

<End of modified section >