3GPP TSG-RAN5 Meeting #90-e	R5-210656

# Electronic Meeting, 22th Feb– 5th Mar 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-5 | CR | 0101 | rev | - | Current version: | 15.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR IMS TC 6.5-UE de-reg after ISIM refresh |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-2-8 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As described in the conformance requirements (24.229 Annex C.4), UE shall wait for the network-initiated deregistration, when changes are made to EFs other than the EFUICCIARI. It means, depending on specific scenarios, either UE-initiated or network-initiated deregistration might be valid. Thus, the test procedures need to be modified so that SS shall tolerate both UE behaviors. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Modified TP1 to consider both UE-initiated and network-initiated deregistration valid. And to change the check point from “UE does not de-register” to “UE initiates new IMS registration after the deregistration”. - Deleted TP2, which is overlapped with the modified TP1. - Modified the test procedures to consider both UE-initiated and network-initiated deregistration valid. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Legit UEs might fail the test. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

## 6.5	Refresh for ISIM parameters / 5GS

### 6.5.1	Test Purpose (TP)

(1)

with { UE being registered to IMS }

ensure that {

when { UE receives indication that contents of USIM/ISIM has been updated }

then { UE initiates new IMS registration after UE-initiated or network-initiated de-registration }

}

…

#### 6.5.3.2	Test procedure sequence

Table 6.5.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UICC is made to send a REFRESH command to the UE indicating that contents of ISIM has been updated. |  | REFRESH |  |  |
| 2 | UE de-register from IMS |  | REGISTER |  |  |
| 2a | SS responds to the UE-initiated IMS de-registration |  | 200 OK |  |  |
|  | EXCEPTION: If the ISIM change happened to EFUICCIARI , then step 2 and 2a happens. Otherwise, step 3 and 4 happens. |  |  |  |  |
| 3 | 10 seconds after step 1 the SS sends SIP NOTIFY for registration event package, containing full registration state information, with all previously registered IMS public user identities as "terminated" and "deactivated" | <-- | NOTIFY |  |  |
| 4 | Check: does the UE respond the NOTIFY with 200 OK? | --> | 200 OK |  |  |
| 5 | Check: does the UE initiate a new IMS registration sequence? For the Request-URI of SIP REGISTER request the UE uses the new value of home domain and/or IMS identities name as provided by ISIM after the update in step 1. | --> | REGISTER | 1 | P |
| 6-12 | Continue with Annex A.2 step 2-8 in order to get the UE in a stable registered state. | - |  |  |  |

#### 6.5.3.3	Specific message contents

Table 6.5.3.3-1: REFRESH (step 1, table 6.5.3.2-1)

Derivation Path: TS 31.111, subclause 6.4.7

Table 6.5.3.3-1a: REGISTER (step 2, table 6.5.3.2-1)

| Derivation Path: TS 34.229-1 [2], annex A1.1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Contact |  |  |  |  |
| addr-spec expires | A2 | SIP URI with IP address or FQDN and protected server port of the UE in case of IMS security, and if the UE supports GRUU, the following parameter: +sip.instance="<urn:gsma:imei: (gsma-specifier-defined-substring)>” or * 0 (if present) |  |  |
| Expires delta-seconds |  | (must be present if addr-spec is *) 0 (if present) |  |  |
| Supported Authorization |  | header may be missing or it may contain any value value not checked |  |  |

Table 6.5.3.3-1b: 200 OK (step 2a, table 6.5.3.2-1)

| Derivation Path: TS 34.229-1 [2], annex A1.3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Contact |  |  |  |  |
| addr-spec expires |  | same value as in REGISTER request if "*" is not included in the Contact header field of the REGISTER request in step 2 same value as in the Contact header field of the "200 OK" response to the initial registration if "*" is included in the Contact header field of the REGISTER request in step 1 (NOTE) 0 |  |  |
| NOTE: According to 3GPP TS 24.229 [7] clause 5.4.1.4.1 when the S-CSCF gets a wild-carded contact address for de-registration it shall include all de-registered contact addresses in the contact header of the 200 OK response  there is no “*” in DL. |  |  |  |  |

Table 6.5.3.3-2: NOTIFY for reg-event package (step 3, table 6.5.3.2-1)

Derivation Path: TS 34.229-1 [2], Table in subclause A.1.6, Conditions A1 AND ((A3 AND A6) OR (A4 AND A6))

Table 6.5.3.3-3: 200 OK for requests other than REGISTER or SUBSCRIBE (step 4, table 6.5.3.2-1)

Derivation Path: TS 34.229-1 [2], Table in subclause A.3.1, Conditions A5, A11 and A22

Table 6.5.3.3-4: REGISTER (step 5, table 6.5.3.2-1)

Derivation Path: TS 34.229-1 [2], Table in subclause A.1.1, Condition A1

<End of modified section 1>