3GPP TSG-RAN5 Meeting #90-e	R5-210806

# Electronic Meeting, 22nd February - 5th March 2021

# Source:	Rohde & Schwarz

# Title:	Discussion paper on confirming successful resource reservation

# Document for:	Endorsement

## 1.	Introduction

Generally we assume in MO voice call test cases that an UPDATE/200 OK handshake happens when preconditions are used, and that such handshake is not used when preconditions are not used. In the context of EPS fallback, this assumption is being challenged and should be put on more solid footing.

## 2.	Discussion

Let us consider the following test case scenarios:

- MO voice call with preconditions, see e.g. TS 34.229-5 test case 7.4
This test case lists UPDATE/200 OK as mandatory steps

- MO voice call without preconditions, see e.g. TS 34.229-5 test case 7.5
This test cases does not list UPDATE/200 OK

- EPS Fallback scenarios like TS 38.523-1 test case 11.1.1
This test cases relies on generic procedure A.9.1 and A.9.2 in TS 34.229-5. Preconditions are suppressed here, and UPDATE/200 OK is required.

Similar LTE test cases, e.g., test 12.12 in TS 34.229-1, evaded the issue by simply making UPDATE/200 OK optional. The obvious way out would be to apply such optionality again whenever test case executions fail. However, TS 24.229 cl 5.1.3.1 on call initiation the originating side contains the following text.

Upon successful reservation of local resources the UE shall confirm the successful resource reservation (see subclause 6.1.2) within the next SIP request.

NOTE 3:	In case of the precondition mechanism being used on both sides, this confirmation will be sent in either a PRACK request or an UPDATE request. In case of the precondition mechanism not being supported on one or both sides, alternatively a reINVITE request can be used for this confirmation after a 200 (OK) response has been received for the initial INVITE request, in case the terminating UE does not support the PRACK request (as described in RFC 3262 [27]) and does not support the UPDATE request (as described in RFC 3311 [29]).

Our test cases are engineered such that resources become available after a 183 Session Progress/PRACK/200 OK sequence. Assuming that UEs being subjected to IMS over 5GS testing all support PRACK and UPDATE, and 200 OK for INVITE not having been received yet, the above quote misses to cover the scenarios we want to test.

## 3.	Proposals

Liaise with CT1 on the above issue and ask for guidance. Until then make UPDATE/200 OK optional in scenarios where preconditions are not used and UPDATE/200 OK is currently required.