3GPP TSG-RAN5 Meeting #90-e	R5-210898

# Electronic Meeting, 22th Feb– 5th Mar 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 1754 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to test frequency parameters for band n84 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2020-02-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The parameter “OffsetToCarrier” should be updated as per table C.3.2-1. The frequency of PointA is updated as well. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updating test frequency parameters for band n84. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test frequencies for n84 is incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1.1.1.84 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

4.3.1.1	Test frequencies for NR operating bands in FR1

4.3.1.1.1	NR operating bands in FR1

<Unchanged Skipped>

4.3.1.1.1.84	Reference test frequencies for NR operating band n84 (SUL)

Table 4.3.1.1.1.84-1: Test frequencies for NR operating band n84 and SCS 15 kHz

| Bandwidth [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Uplink | Low | 1922.5 | 384500 | 1920.25 | 384050 | 0 |
|  |  |  | Mid | 1950 | 390000 | 1857.03 | 371406 | 504 |
|  |  |  | High | 1977.5 | 395500 | 1974.17 | 394834 | 6 |
| 10 | 52 | Uplink | Low | 1925 | 385000 | 1920.32 | 384064 | 0 |
|  |  |  | Mid | 1950 | 390000 | 1854.6 | 370920 | 504 |
|  |  |  | High | 1975 | 395000 | 1969.24 | 393848 | 6 |
| 15 | 79 | Uplink | Low | 1927.5 | 385500 | 1920.39 | 384078 | 0 |
|  |  |  | Mid | 1950 | 390000 | 1852.17 | 370434 | 504 |
|  |  |  | High | 1972.5 | 394500 | 1964.31 | 392862 | 6 |
| 20 | 106 | Uplink | Low | 1930 | 386000 | 1920.46 | 384092 | 0 |
|  |  |  | Mid | 1950 | 390000 | 1849.74 | 369948 | 504 |
|  |  |  | High | 1970 | 394000 | 1959.38 | 391876 | 6 |

Table 4.3.1.1.1.84-2: Test frequencies for NR operating band n84 and SCS 30 kHz

| Bandwidth [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Uplink | Low | 1925 | 385000 | 1920.68 | 384136 | 0 |
|  |  |  | Mid | 1950 | 390000 | 1764.24 | 352848 | 504 |
|  |  |  | High | 1975 | 395000 | 1968.52 | 393704 | 6 |
| 15 | 38 | Uplink | Low | 1927.5 | 385500 | 1920.66 | 384132 | 0 |
|  |  |  | Mid | 1950 | 390000 | 1761.72 | 352344 | 504 |
|  |  |  | High | 1972.5 | 394500 | 1963.5 | 392700 | 6 |
| 20 | 51 | Uplink | Low | 1930 | 386000 | 1920.82 | 384164 | 0 |
|  |  |  | Mid | 1950 | 390000 | 1759.38 | 351876 | 504 |
|  |  |  | High | 1970 | 394000 | 1958.66 | 391732 | 6 |

Table 4.3.1.1.1.84-3: Test frequencies for NR operating band n84 and SCS 60 kHz

| Bandwidth [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Uplink | Low | 1925 | 385000 | 1921.04 | 384208 | 0 |
|  |  |  | Mid | 1950 | 390000 | 1583.16 | 316632 | 504 |
|  |  |  | High | 1975 | 395000 | 1966.72 | 393344 | 6 |
| 15 | 18 | Uplink | Low | 1927.5 | 385500 | 1921.02 | 384204 | 0 |
|  |  |  | Mid | 1950 | 390000 | 1580.64 | 316128 | 504 |
|  |  |  | High | 1972.5 | 394500 | 1961.7 | 392340 | 6 |
| 20 | 24 | Uplink | Low | 1930 | 386000 | 1921.36 | 384272 | 0 |
|  |  |  | Mid | 1950 | 390000 | 1578.48 | 315696 | 504 |
|  |  |  | High | 1970 | 394000 | 1957.04 | 391408 | 6 |

<End of modified section 1>