Source:	Huawei

Title:	Discussion on test point selection for NS_3U UTRA-ACLR for UL-MIMO in FR1

# Background

The purpose of this paper is to provide an analysis for the parameters in test configuration table and a selection of test points for test environment, test frequencies, channel bandwidths, SCS, RB allocation and modulation for test point selection for NS_3U UTRA-ACLR for UL-MIMO in FR1.

The general agreement in RAN5 is that FR1 testing should follow what is being tested for LTE. This is also valid for test point selection for NS_3U UTRA-ACLR for UL-MIMO in FR1

# Discussion

Extracted from Clause 6.5D.2 [3]

6.5D.2	Out of band emission for UL-MIMO

For UE supporting UL-MIMO, the requirements for Out of band emissions resulting from the modulation process and non-linearity in the transmitters are specified at each transmit antenna connector.

For UEs with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the requirements in subclasuse 6.5.2 apply to each transmit antenna connector. The requirements shall be met with UL-MIMO configurations described in sub-clause 6.2D.1.

If UE is configured for transmission on single-antenna port, the requirements in subclause 6.5.2 apply.

Observation 1: Same ALCR requirements as specified for single-antenna port apply to UE supporting UL-MIMO.

Extracted from Table 6.2.3.1-1 in TS 38.101-1[4]

Table 6.2.3.1-1: Additional maximum power reduction (A-MPR)

| Network Signalling label | Requirements (subclause) | NR Band | Channel bandwidth (MHz) | Resources Blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_01 |  | Table 5.2-1 |  |  | N/A |
| NS_03 | 6.5.2.3.3 | n2, n25, n66, n70, n86 |  |  | Subclause 6.2.3.7 |
| NS_03U | 6.5.2.3.3, 6.5.2.4.2 | n2, n25, n66, n86 |  |  | Subclause 6.2.3.7 |
| NOTE 1: This NS can be signalled for NR bands that have UTRA services deployed NOTE 2: No A-MPR is applied for 5 MHz CBW where the lower channel edge is ≥ 1930 MHz,10 MHz CBW where the lower channel edge is ≥ 1950 MHz and 15 MHz CBW where the lower channel edge is ≥ 1955 MHz. NOTE 3: Applicable when the NR carrier is within 1447.9 – 1462.9 MHz |  |  |  |  |  |

Table 6.2.3.7-1 A-MPR for NS_03

| Modulation |  | A-MPR outer |
| --- | --- | --- |
| DFT-s-OFDM | PI/2 BPSK | ≤ 1.5 |
|  | QPSK | ≤ 2 |
|  | 16 QAM | ≤ 3 |
|  | 64 QAM | ≤ 3.5 |
|  | 256 QAM | ≤ 5.5 |
| CP-OFDM | QPSK | ≤ 4 |
|  | 16 QAM | ≤ 4 |
|  | 64 QAM | ≤ 4.5 |
|  | 256 QAM | ≤ 7.5 |
| NOTE 1: The backoff applied is max (MPR, A-MPR) where MPR is defined in Table 6.2.2-1 NOTE 2: Outer allocations are defined in clause 6.2.2 |  |  |

In case UE operates in a band where NS_03U applies and it receives additionalSpectrumEmission value of 3 then A-MPR values specified in Table 6.2.3.7-1 apply with an exception that DFT-s-OFDM PI/2 PBSK A-MPR is 2 dB.

Observation 2: For NS_3U, requirements for A-MPR is specified to meet requirements for A-SEM and UTRA ACLR.

Requirements for A-MPR applies for outer RB allocation, there will be no additional test points compared A-MPR test with MPR test.

As a result of observations 1, 2, it’s proposed to select test parameters for NS_03U UTRA-ACLR for UL-MIMO based on the same philosophy for ALCR test with the exception that inner RB allocation is precluded for NS_03U UTRA ACLR.

## Test Environment

Based on the baseline agreed in [1], the test environment for NS_03U UTRA ACLR for UL-MIMO measurement in FR1 can be defined as in LTE RF ALCR for UL-MIMO.

PROPOSAL 1: Select Test Environment as Normal Conditions (NC) for NS_03U UTRA ACLR for UL-MIMO measurement in FR1.

## Test Frequencies

The selection of test frequencies can be based on the agreed proposals for ACLR [2], i.e. Low and High frequency.

PROPOSAL 2: Select Low range and High range test frequencies for NS_03U UTRA ACLR for UL-MIMO in FR1.

## Test Channel Bandwidth

The selection of test channel bandwidth can be based on the agreed proposals for ACLR [2], i.e. Lowest and Highest channel bandwidth.]

PROPOSAL 3: Select Lowest and Highest test channel bandwidth for NS_03U UTRA ACLR for UL-MIMO in FR1.

## Test Subcarrier Spacing

The selection of test subcarrier spacing can be based on the agreed proposals for ACLR [2], i.e. lowest and highest supported subcarrier spacing.

PROPOSAL 4: Select Lowest and Highest supported subcarrier spacing for NS_03U UTRA ACLR for UL-MIMO in FR1.

## UL Modulation

UL waveform and modulation are selected based on the agreed proposals for ACLR [2] with the exception specified in [4] that the waveform of DFT-s-OFDM is precluded because CP-OFDM is mandatory in case UL-MIMO transmission.

Test all UL modulations supported by UE (CP-OFDM QPSK, CP-OFDM 16 QAM, CP-OFDM 64 QAM and CP-OFDM 256 QAM) for NS_03U UTRA ACLR measurement for UL-MIMO in FR1.

PROPOSAL 5: Test all modulations under CP-OFDM waveform(CP-OFDM QPSK, CP-OFDM 16 QAM, CP-OFDM 64 QAM and CP-OFDM 256 QAM) supported by the UE for NS_03U UTRA ACLR measurement for UL-MIMO in FR1.

For Rel-16 UEs supporting uplink full power transmission (ULFPTx) for UL MIMO, the maximum output power requirements specified in TS 38.101-1 [7] clause 6.2D.1 shall be met with the PUSCH configurations specified in Table 6.2D.1-3, based upon UE’s support of uplink full power transmission mode.

Table 6.2D.1-3: PUSCH Configuration for uplink full power transmission (ULFPTx)

| ULFPTx Mode | Transmission scheme | DCI format | Modulation | Number of layers | Number of Tx Port | TPMI index |
| --- | --- | --- | --- | --- | --- | --- |
| Mode-1 | Codebook based uplink | DCI format 0_1 | DFT-s-OFDM, CP-OFDM NOTE3 | 1 | 2 | 2 |
| Mode-2 | Codebook based uplink | DCI format 0_1 | DFT-s-OFDM, CP-OFDM | 1 | 2 | 0 or 1NOTE2 |
| Mode-full power | Codebook based uplink | DCI format 0_1 | DFT-s-OFDM, CP-OFDM | 1 | 2 | 0,1 |
| NOTE 1: The UE is configured with one SRS resource with the parameter nrofSRS-Ports set to 2. NOTE 2: TPMI index selected shall be based upon the full power TPMI reported by the UE [8, TS 38.213]. NOTE 3: For PUSCH configured with ULFPTxModes set to Mode-1, all the transmitter requirement for CP-OFDM based modulation is not needed to be verified if the requirement for UL MIMO has been validated. |  |  |  |  |  |  |

Observation 3: From Table 6.2D.1-3, it is observed that UL MIMO with ULFPTx supports both DFT-s-OFDM and CP-OFDM with 1-layer data. The singlular transmission test case for UTRA ACLR (6.5.2.4.2) is tested with both waveforms. The same approach can be used for ULFPTx UE, testing UTRA ACLR for UL-MIMO release 16.

PROPOSAL 6: For ULFPTx UE, select same UL modulation and waveform as singular transmission case for UTRA ACLR UL-MIMO.

## Uplink RB allocation

As a result of observations 1, 2, the selection of UL RB allocation can be based on the agreed proposals for ALCR [2], with the exception that outer RB allocations(Outer_full, Edge_1RB_left and  Edge_1RB_right), for which the NS_03U A-MPR applies, are selected.

PROPOSAL 7: Test outer RB allocation for NS_03U UTRA ACLR for UL-MIMO in FR1.

2.7.	Number of Test Points

The number of test points for NS_03U UTRA ACLR for UL-MIMO, release 15

| Number of Enviromental conditions | Number SCS | Maximum Number of ChBW | Maximum Number of Frequencies | Number of steps (mod and RB) | Maximum Number of Test Steps |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 2 | 2 | 12 | 96 |

The number of test points for NS_03U UTRA ACLR for UL-MIMO, release 16 and onward

| Number of Enviromental conditions | Number SCS | Maximum Number of ChBW | Maximum Number of Frequencies | Number of steps (mod and RB) | Maximum Number of Test Steps |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 2 | 2 | 27 | 216 |

# Proposal

It is proposed that RAN5 discusses and agrees the following proposals for open areas described in section 2 of this document to progress on SA FR1 NS_03U UTRA ACLR for UL-MIMO in FR1 test case definition:

PROPOSAL 1: Select Test Environment as Normal Conditions (NC) for NS_03U UTRA ACLR for UL-MIMO measurement in FR1.

PROPOSAL 2: Select Low range and High range test frequencies for NS_03U UTRA ACLR for UL-MIMO in FR1.

PROPOSAL 3: Select Lowest and Highest test channel bandwidth for NS_03U UTRA ACLR for UL-MIMO in FR1.

PROPOSAL 4: Select Lowest and Highest supported subcarrier spacing for NS_03U UTRA ACLR for UL-MIMO in FR1.

PROPOSAL 5: Test all modulations under CP-OFDM waveform(CP-OFDM QPSK, CP-OFDM 16 QAM, CP-OFDM 64 QAM and CP-OFDM 256 QAM) supported by the UE for NS_03U UTRA ACLR measurement for UL-MIMO in FR1.

PROPOSAL 6: For ULFPTx UE, select same UL modulation and waveform as singular transmission case for UTRA ACLR UL-MIMO.

PROPOSAL 7: Test outer RB allocation for NS_03U UTRA ACLR for UL-MIMO in FR1.

# References

- R5-180065, "Discussion about Test parameters for NR TRx TCs " (NTT DOCOMO)

- R5-180887," Discussion on test point selection for NR ACLR in FR1” (Keysight Technologies).

- TS 38.101-1 v15.5.0 User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone (Release 15)

- R5-192024,” Discussion on the waveform selection for UL MIMO tests”, Huawei, RAN5#82, Athens, Feb.2019