3GPP TSG-RAN5 Meeting #90-e                      	R5-211217

# Electronic Meeting, 22nd February – 5th March 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 1123 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of MTSU and applied TT in Annex F |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-01-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - For a number of FR1 RRM test cases TT analysis was corrected as a result of MU change to ±0.3 dB for Es/Noc accuracy over Meas PRBs with baseband combining of AWGN and signal assumed. - For FR2 RRM inter-freq event triggered test cases MTSU and applied TT is missing in 38.903 |  |  |  |  |  |  |  |  |
| Summary of change: |  | FR1 MTSU in tables F.1.1.2-3 and F.1.1.2-4 corrected FR2 MTSU in tables F.1.1.2-5 and F.1.1.2-6 added FR1 Applied TT in tables F.1.3.2-1 and F.1.3.2-2 corrected FR2 Applied TT in tables F.1.3.2-4 and F.1.3.2-4 added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MTSU and Applied TT will remain incorrect in annex F. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | F |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

///Start of change 1

Table F.1.1.2-3: Maximum test system uncertainty for RRM requirements for EN-DC FR1 test cases

| Subclause | Maximum Test System Uncertainty1 | Derivation of Test System Uncertainty |
| --- | --- | --- |
| ### ///Unchanged rows skipped |  |  |
| 4.6.2.1 EN-DC FR1-FR1 event-triggered reporting in non-DRX | Average Noc2 ±1.5 dB Average Noc3 ±1.5 dB Average Ês2 / Noc2 ±0.3 dB Average Ês3 / Noc3 ±0.3 dB Meas PRB Noc2 ±1.5 dB Meas PRB Noc3 ±1.5 dB Meas PRB Ês2 / Noc2 ±0.3 dB Meas PRB Ês3 / Noc3 ±0.3 dB | Noc2 is the AWGN on NR freq2 Noc3 is the AWGN on NR freq3 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc3 is the ratio of cell 3 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 4.6.2.2 EN-DC FR1-FR1 event-triggered reporting in DRX | Same as 4.6.2.1 | Same as 4.6.2.1 |
| 4.6.2.5 EN-DC FR1-FR1 event-triggered reporting in non-DRX with SSB time index detection | Same as 4.6.2.1 | Same as 4.6.2.1 |
| 4.6.2.6 EN-DC FR1-FR1 event-triggered reporting in DRX with SSB time index detection | Same as 4.6.2.1 | Same as 4.6.2.1 |
| 4.6.4.1 EN-DC FR1 SSB-based L1-RSRP measurement in non-DRX | Average Noc ±1.5 dB Average Ês0 / Noc ±0.3 dB Average Ês1 / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês0 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB | Noc is the AWGN on NR freq1 Ês0 / Noc is the SNR for the SSB#0 Ês1 / Noc is the SNR for the SSB#1 Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 4.6.4.2 EN-DC FR1 SSB-based L1-RSRP measurement in DRX | Same as 4.6.4.1 | Same as 4.6.4.1 |
| 4.6.4.3 EN-DC FR1 CSI-RS-based L1-RSRP measurement in non-DRX | Average Noc ±1.5 dB Average Ês0 / Noc ±0.3 dB Average Ês1 / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês0 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB | Noc is the AWGN on NR freq1 Ês0 / Noc is the SNR for the CSI-RS#0 Ês1 / Noc is the SNR for the CSI-RS#1 Meas PRB is the measurement PRB for CSI-RSRP #RB0 to RB274 |
| ///Unchanged rows skipped |  |  |

Table F.1.1.2-4 Maximum test system uncertainty for RRM requirements for SA FR1 test cases

| Subclause | Maximum Test System Uncertainty1 |  | Derivation of Test System Uncertainty |  |
| --- | --- | --- | --- | --- |
| ///Unchanged rows skipped |  |  |  |  |
| 6.6.2.1 NR SA FR1-FR1 event-triggered reporting in non-DRX |  | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Average Ês2s / Noc2 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês1 / Noc1 ±0.3 dB Meas PRB Ês2 / Noc2 ±0.3 dB |  | Noc2 is the AWGN on NR freq1 Noc3 is the AWGN on NR freq2 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 6.6.2.2 NR SA FR1-FR1 event-triggered reporting in DRX |  | Same as 6.6.2.1 |  | Same as 6.6.2.1 |
| 6.6.2.5 NR SA FR1-FR1 event-triggered reporting in non-DRX with SSB time index detection |  | Same as 6.6.2.1 |  | Same as 6.6.2.1 |
| 6.6.2.6 NR SA FR1-FR1 event-triggered reporting in DRX with SSB time index detection |  | Same as 6.6.2.1 |  | Same as 6.6.2.1 |
| 6.6.3.1 NR SA FR1 – E-UTRAN event-triggered reporting in non-DRX |  | Freq 1 Noc ±1.5 dB Freq 2 Noc ±1.5 dB Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB Fading ±0.5 dB |  | Note: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 6.6.3.2 NR SA FR1 – E-UTRAN event-triggered reporting in DRX |  | Same as 6.6.3.1 |  | Same as 6.6.3.1 |
| 6.6.4.1 NR SA FR1 SSB-based L1-RSRP measurement in non-DRX |  | Average Noc ±1.5 dB Average Ês0 / Noc ±0.3 dB Average Ês1 / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês0 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB |  | Noc is the AWGN on NR freq1 Ês0 / Noc is the SNR for the SSB#0 Ês1 / Noc is the SNR for the SSB#1 Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 6.6.4.2 NR SA FR1 SSB-based L1-RSRP measurement in DRX |  | Same as 6.6.4.1 |  | Same as 6.6.4.1 |
| 6.6.4.3 NR SA FR1 CSI-RS-based L1-RSRP measurement in non-DRX |  | Average Noc ±1.5 dB Average Ês0 / Noc ±0.3 dB Average Ês1 / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês0 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB |  | Noc is the AWGN on NR freq1 Ês0 / Noc is the SNR for the CSI-RS#0 Ês1 / Noc is the SNR for the CSI-RS#1 Meas PRB is the measurement PRB for CSI-RSRP #RB0 to RB274 |
| ///Unchanged rows skipped |  |  |  |  |

Table F.1.1.2-5: Maximum test system uncertainty for RRM requirements for EN-DC FR2 test cases

| Subclause | Maximum Test System Uncertainty1 | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 5.3.2.2.1 Contention based random access test in FR2 for PSCell in EN-DC | Average Ês ±6.0 dB Meas PRB Ês ±6.0 dB Uplink absolute power measurement ±6.0 dB Uplink relative power measurement ±FFS dB ±[48]Tc Uplink signal transmit timing relative to downlink | Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |
| 5.3.2.2.2 Non-contention based random access test in FR2 for PSCell in EN-DC | Same as 5.3.2.2.1 | Same as 5.3.2.2.1 |
| 5.4.1.1 EN-DC FR2 UE transmit timing accuracy | Noc ±6.0 dB Ês1 / Noc ±[0.3] dB ±[48]Tc Uplink signal transmit timing relative to downlink ±[40]Tc Relative during UE timing adjustment | Ês1 / Noc is the ratio of cell 1 signal / AWGN Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |
| 5.6.2.1 EN-DC FR2-FR2 event-triggered reporting in non-DRX | Average Ês ±5.65 dB Meas PRB Ês ±5.65 dB | Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 5.6.2.3 EN-DC FR2-FR2 event-triggered reporting in non-DRX with SSB time index detection | Same as 5.6.2.1 | Same as 5.6.2.1 |

Table F.1.1.2-6: Maximum test system uncertainty for RRM requirements for SA FR2 test cases

| Subclause | Maximum Test System Uncertainty1 | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 7.3.2.2.1 Contention based random access test in FR2 for NR standalone | Average Ês ±6.0 dB Meas PRB Ês ±6.0 dB Uplink absolute power measurement ±6.0 dB Uplink relative power measurement ±FFS dB ±[48]Tc Uplink signal transmit timing relative to downlink | Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |
| 7.3.2.2.2 Non-contention based random access test in FR2 for NR standalone | Same as 7.3.2.2.1 | Same as 7.3.2.2.1 |
| 7.4.1.1 SA FR2 UE transmit timing accuracy | Noc ±6.0 dB Ês1 / Noc ±[0.3] dB ±[48]Tc Uplink signal transmit timing relative to downlink ±[40]Tc Relative during UE timing adjustment | Ês1 / Noc is the ratio of cell 1 signal / AWGN Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |
| 7.6.2.1 NR SA FR2-FR2 event-triggered reporting in non-DRX | Average Ês ±5.65 dB Meas PRB Ês ±5.65 dB | Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 7.6.2.3 NR SA FR2-FR2 event-triggered reporting in non-DRX with SSB time index detection | Same as 7.6.2.1 | Same as 7.6.2.1 |

///End of change 1

///Start of change 2

Table F.1.3.2-1: Derivation of test requirements for EN-DC FR1 RRM tests

| Test |  | Minimum requirement in TS 38.133 [6] |  | Test tolerance (TT) |  | Test requirement in TS 38.533 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ///Unchanged rows skipped |  |  |  |  |  |  |  |
| 4.6.4.1 EN-DC FR1 SSB-based L1-RSRP measurement in non-DRX |  | During T1: Noc: -94.65dBm/15kHz Ês0 / Noc: +0.00dB Ês1 / Noc: -infinity During T2: Noc: -94.65dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.00dB Reported SSB#0 SS-RSRP values ± 10dB Reported SSB#1 SS-RSRP values ± 10dB Reported Differential SS-RSRP values ± 3dB |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0.5dB Via mapping |  | During T1: Noc: -94.65dBm/15kHz Ês0 / Noc: +0.00dB Ês1 / Noc: -infinity During T2: Noc: -94.65dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.50dB For configuration 1,2,4,5 SSB#0: RSRP_52 to RSRP_72 SSB#1: RSRP_55 to RSRP_75 DIFFRSRP_0 to DIFFRSRP_3 For configuration 3,6 SSB#0: RSRP_55 to RSRP_75 SSB#1: RSRP_58 to RSRP_78 DIFFRSRP_0 to DIFFRSRP_3 |  |
| 4.6.4.2 EN-DC FR1 SSB-based L1-RSRP measurement in DRX |  | Same as 4.6.4.1 |  | Same as 4.6.4.1 |  | Same as 4.6.4.1 |  |
| 4.6.4.3 EN-DC FR1 CSI-RS-based L1-RSRP measurement in non-DRX |  | Noc: -94.65dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.00dB Reported CSI-RSRP values ± 10dB Reported Differential CSI-RSRP values ± 3dB |  | 0dB 0dB 0.5dB Via mapping |  | Noc: -94.65dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.50dB For configuration 1,2,4,5 RSRP_55 to RSRP_75 DIFFRSRP_0 to DIFFRSRP_3 For configuration 3,6 RSRP_58 to RSRP_78 DIFFRSRP_0 to DIFFRSRP_3 |  |
| 4.6.4.4 EN-DC FR1 CSI-RS-based L1-RSRP measurement in DRX |  | Same as 4.6.4.3 |  | Same as 4.6.4.3 |  | Same as 4.6.4.3 |  |
| ///Unchanged rows skipped |  |  |  |  |  |  |  |

Table F.1.3.2-2: Derivation of test requirements for NR SA FR1 RRM tests

| Test |  | Minimum requirement in TS 38.133 [6] | Test tolerance (TT) | Test requirement in TS 38.533 |
| --- | --- | --- | --- | --- |
| ///Unchanged rows skipped |  |  |  |  |
| 6.6.4.1 NR SA FR1 SSB-based L1-RSRP measurement in non-DRX |  | During T1: Noc: -98dBm/15kHz Ês0 / Noc: +0.00dB Ês1 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.00dB Reported SSB#0 SS-RSRP values ± 10dB Reported SSB#1 SS-RSRP values ± 10dB Reported Differential SS-RSRP values ± 3dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0.5dB Via mapping | During T1: Noc: -94.65dBm/15kHz Ês0 / Noc: +0.00dB Ês1 / Noc: -infinity During T2: Noc: -94.65dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.50dB For configuration 1,2,4,5 SSB#0: RSRP_52 to RSRP_72 SSB#1: RSRP_55 to RSRP_75 DIFFRSRP_0 to DIFFRSRP_3 For configuration 3,6 SSB#0: RSRP_55 to RSRP_75 SSB#1: RSRP_58 to RSRP_78 DIFFRSRP_0 to DIFFRSRP_3 |
| 6.6.4.2 NR SA FR1 SSB-based L1-RSRP measurement in DRX |  | Same as 6.6.4.1 | Same as 6.6.4.1 | Same as 6.6.4.1 |
| 6.6.4.3 NR SA FR1 CSI-RS-based L1-RSRP measurement in non-DRX |  | Noc: -98dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.00dB Reported CSI-RSRP values ± 10dB Reported Differential CSI-RSRP values ± 3dB | 0dB 0dB 0.5dB Via mapping | Noc: -94.65dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.50dB For configuration 1,2,4,5 RSRP_55 to RSRP_75 DIFFRSRP_0 to DIFFRSRP_3 For configuration 3,6 RSRP_58 to RSRP_78 DIFFRSRP_0 to DIFFRSRP_3 |
| 6.6.4.4 NR SA FR1 CSI-RS-based L1-RSRP measurement in DRX |  | Same as 6.6.4.3 | Same as 6.6.4.3 | Same as 6.6.4.3 |
| ///Unchanged rows skipped |  |  |  |  |

Table F.1.3.2-4: Derivation of test requirements for EN-DC FR2 RRM tests

| Test | Minimum requirement in TS 38.133 [6] | Test tolerance (TT) | Test requirement in TS 38.533 |
| --- | --- | --- | --- |
| 5.3.2.2.1 Contention based random access test in FR2 for PSCell in EN-DC | Absolute uplink power: Pint ≥ P ≥ Pmin ±14.0dB PUMAX ≥ P > Pint ±12.0dB Relative uplink power step: Pint ≥ P ≥ Pmin ±6.0dB PUMAX ≥ P > Pint ±4.0dB Uplink timing: 120kHz SCS Te ±3.5*64*Tc | Not applicable due to UL calibration FFS dB FFS dB [48]Tc | Absolute uplink power: ±FFSdB Relative uplink power step: ±(6+FFS)dB, either PRACH being compared ≤ (Pmax – 6dB) ±(4+FFS)dB, both PRACHs being compared > (Pmax – 6dB) Uplink timing: 120kHz SCS Te ±224±[48]*Tc |
| 5.3.2.2.2 Non-contention based random access test in FR2 for PSCell in EN-DC | Same as 5.3.2.2.1 | Same as 5.3.2.2.1 | Same as 5.3.2.2.1 |
| 5.4.1.1 EN-DC FR2 UE transmit timing accuracy | TBD | TBD | TBD |
| 5.6.2.1 EN-DC FR2-FR2 event-triggered reporting in non-DRX | During T1: Es: -87 dBm/120kHz During T2: Es: -87 dBm/120kHz | During T1: 0dB During T2: 0dB | During T1: Es: -87 dBm/120kHz During T2: Es: -87 dBm/120kHz |
| 5.6.2.3 EN-DC FR2-FR2 event-triggered reporting in non-DRX with SSB time index detection | Same as 5.6.2.1 | Same as 5.6.2.1 | Same as 5.6.2.1 |

Table F.1.3.2-5: Derivation of test requirements for NR SA FR2 RRM tests

| Test | Minimum requirement in TS 38.133 [6] | Test tolerance (TT) | Test requirement in TS 38.533 |
| --- | --- | --- | --- |
| 7.3.2.2.1 Contention based random access test in FR2 for NR standalone | Absolute uplink power: Pint ≥ P ≥ Pmin ±14.0dB PUMAX ≥ P > Pint ±12.0dB Relative uplink power step: Pint ≥ P ≥ Pmin ±6.0dB PUMAX ≥ P > Pint ±4.0dB Uplink timing: 120kHz SCS Te ±3.5*64*Tc | Not applicable due to UL calibration FFS dB FFS dB [48]Tc | Absolute uplink power: ±FFSdB Relative uplink power step: ±(6+FFS)dB, either PRACH being compared ≤ (Pmax – 6dB) ±(4+FFS)dB, both PRACHs being compared > (Pmax – 6dB) Uplink timing: 120kHz SCS Te ±224±[48]*Tc |
| 7.3.2.2.2 Non-contention based random access test in FR2 for NR standalone | Same as 7.3.2.2.1 | Same as 7.3.2.2.1 | Same as 7.3.2.2.1 |
| 7.4.1.1 SA FR2 UE transmit timing accuracy | TBD | TBD | TBD |
| 7.6.2.1 NR SA FR2-FR2 event-triggered reporting in non-DRX | During T1: Es: -87 dBm/120kHz During T2: Es: -87 dBm/120kHz | During T1: 0dB During T2: 0dB | During T1: Es: -87 dBm/120kHz During T2: Es: -87 dBm/120kHz |
| 7.6.2.3 NR SA FR2-FR2 event-triggered reporting in non-DRX with SSB time index detection | Same as 7.6.2.1 | Same as 7.6.2.1 | Same as 7.6.2.1 |

///End of change 2