3GPP TSG RAN WG5 Meeting #90-e	R5-211350

# Electronic Meeting, 22nd February - 5th March 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5002 | rev | 1 | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Editorial updates of TS 36.523-1 Sections 15, 17 and 19 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI9_Test |  |  |  |  |  | Date: |  |  | 2021-02-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | There are redundant spaces in test case titles and TPs, and Capital Vs Small letters inconsistent usage in test case titles. In addition from the problem from standards editorial point of view they also represent inconsistency with the titles used in 36.523-2 where the issues do not exist. Other minor editorial issues have been found around the places with the issues described above (i.e. the entire spec was not deliberately searched for editorial issues): Test Purposes text does not follow the agreed RAN5 way of specifying a TP (PRD13). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Redundant spaces in test case titles and TPs have been removed, Capital Vs Small letters inconsistent usage has been corrected; plural replaced with singular. Test Purposes text has been modified to follow the agreed RAN5 way of specifying a TP. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Difficulties finding exact matches between tests definition and test applicability. Poor quality of 3GPP specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 15.1, 15.5, 17.2.4, 17.4.2a, 19.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The present CR is an update of R5-210149 followed by one intermediate revision r1. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 15.1	Discovery of the Home Agent via DNS

15.1.1	Test Purpose (TP)

(1)

with { UE has acquired an IP address and UE is configured with a DNS server address and UE is configured with the HA-APN Network Identifier }

ensure that {

when { UE is configured to discover IP address of Home Agent via DNS }

then { UE transmits a DNS Query with QNAME set to FQDN of the Home Agent }

}

[Text skipped here]

## 15.5	Security association establishment without Home Agent reallocation procedure

15.5.1	Test Purpose (TP)

(1)

with { UE has acquired an IP address }

ensure that {

when { UE has acquired the IP address of the Home Agent }

then { UE transmits an IKE_SA_INIT message addressed to the Home Agent to initiate security association establishment }

}

(2)

with { UE has transmitted an IKE_SA_INIT message addressed to the Home Agent to initiate security association establishment }

ensure that {

when { UE receives an IKE_SA_INIT response message }

then { UE transmits an IKE_AUTH Request message containing the configuration payload MIP6_HOME_PREFIX to receive the prefix to use for Home Address configuration }

}

(3)

with { UE has transmitted an IKE_AUTH Request message containing the configuration payload MIP6_HOME_PREFIX to receive the prefix to use for Home Address configuration }

ensure that {

when { UE receives an IKE_AUTH Response message including an EAP-Request/AKA Challenge }

then { UE transmits an IKE_AUTH Request message containing the correct EAP-Response/AKA-Challenge }

}

(4)

with { UE has transmitted an IKE_AUTH Request message containing an EAP-Response/AKA-Challenge }

ensure that {

when { UE receives an IKE_AUTH Response message including EAP-Success }

then { UE transmits an IKE_AUTH Request message with Authentication payload }

}

(5)

with { UE has transmitted an IKE_AUTH Request message with Authentication payload }

ensure that {

when { UE receives an IKE_AUTH Response message with configuration payload MIP6_HOME_PREFIX containing the Home Network Prefix HNP associated to the UE }

then { UE transmits a CREATE_CHILD_SA  Request  message including traffic selectors fields (TSi and TSr) that contain the parameters identifying the Binding Update (BU)/Binding Acknowledgments (BA)  messages }

}

[Text skipped here]

### 17.2.4	Reception of PDCCH DCI format 0 and PHICH in MBSFN subframes

17.2.4.1	Test Purpose (TP)

(1)

with { UE in E-UTRAN RRC_CONNECTED state and in a subframe configured as MBSFN subframe }

ensure that {

when { UE receives a PDCCH DCI format 0 }

then { UE performs uplink data transmission as per DCI format 0 received }

}

(2)

with { UE in E-UTRAN RRC_CONNECTED state and in a subframe configured as MBSFN subframe }

ensure that {

when { UE receives a PHICH for an uplink data transmission made }

then { UE acts upon the PHICH information }

}

[Text skipped here]

### 17.4.2a	Cell reselection to inter-band cell to start MBMS service reception

17.4.2a.1	Test Purpose (TP)

(1)

with { UE in E-UTRAN RRC IDLE state on a cell broadcasting SIB15 and interested to receive a MBMS service }

ensure that {

when { UE receives SIB15 indicating that the MBMS service is available on a frequency of a neighbour cell on a different frequency band }

then { UE performs cell reselection to the neighbour cell on the different frequency band even if the serving cell is better and starts MBMS reception }

}

[Text skipped here]

### 19.1.1	ProSe direct Communication /Pre-configured authorisation / UE in RRC_IDLE on an E-UTRAN cell operating on the carrier frequency provisioned for ProSe direct service / Utilisation of the resources of (serving) cells/PLMNs / Transmission

[End of changes]