3GPP TSG RAN WG5 Meeting #90-e	R5-211365

Electronic Meeting, February 22 – March 5, 2021

3GPP TSG RAN Meeting #91-e	RP-21xxxx

Electronic Meeting, March 22 - 26, 2021

# Source:	CMCC, Ericsson

# Title:	Revised WID on: UE Conformance Test Aspects for SON and MDT support for NR

# Document for:	Approval

Agenda Item:	7.4.2

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: 	UE Conformance Test Aspects on SON (Self-Organising Networks) and MDT (Minimization of Drive Tests) support for NR

Acronym: NR_ SON_MDT-UEConTest

| This WID includes a Testing part |  | X |
| --- | --- | --- |
| and it addresses the following 3GPP work area: | Radio Access | X |
|  | Core Network |  |
|  | Services |  |

1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X |  |  |  |
| No | X |  | X | X |  |
| Don't know |  |  |  |  |  |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a …

|  | Feature |
| --- | --- |
| X | Building Block |
|  | Work Task |
|  | Study Item |

2.2	Parent Work Item

The following lists of RAN3 led work items that will be tested in this RAN5 work item will be maintained in future revisions of this WI description:

| Parent Work Items |  |
| --- | --- |
| Unique ID | Title |
| 840191 | Core part: SON (Self-Organising Networks) and MDT (Minimization of Drive Tests) support for NR |

2.3	Other related Work Items and dependencies

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

3	Justification

Self-Organising Networks (SON) and Minimization of Drive Tests (MDT) were introduced in LTE to support deployment of the system and performance optimization. Many of the standardized SON/MDT features are implemented in LTE commercial networks and have provided means for a cost-efficient handing of the networks.

The core part of WI SON/MDT support for NR was introduced in Rel-16. It uses LTE solutions as baseline and takes the NR new architectures and features into account, e.g., EN-DC, beam, etc. In this WI, SON features Mobility Robustness Optimisation (MRO), RACH optimisation, L2 measurements and Minimization of Drive Tests (MDT) support for NR were introduced.

The Rel-16 WI NR_SON_MDT-Core has been completed at RAN#88 e-meeting. According to the requirements of NR network deployment and performance optimization, it is justified to now start the work on the corresponding UE conformance test specifications for NR MDT and SON features in 3GPP RAN WG5 to meet the market requirements in time.

4	Objective

4.1	Objective of SI or Core part WI or Testing part WI

The objective of this work item is to define the UE conformance requirements corresponding to WID on MDT support for NR with Unique identifier 840191. This work item will cover protocol conformance test specifications with NR SON and MDT.

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Remarks |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 38.508-1 | Definition of common environment for R16 NR SON and MDT. | TSG RAN#94 (Dec-21) |  |
| TS 38.508-2 | Introduction of common implementation conformance statements for R16 NR SON and MDT. | TSG RAN#94 (Dec-21) |  |
| TS 38.523-1 | Introduction of the SIG test cases for R16 NR SON and MDT. | TSG RAN#94 (Dec-21) |  |
| TS 38.523-2 | Introduction of test applicability for SIG test cases impacted by R16 NR SON and MDT. | TSG RAN#94 (Dec-21) |  |
| TS 38.523-3 | Introduction of test model for R16 NR SON and MDT requirements. | TSG RAN#94 (Dec-21) | Note: Progress of TTCN development is tracked in MCC TF160 reports to RAN5/RAN. |
| TS 38.509 | Introduction of UE Positioning test mode procedures for R16 NR SON and MDT requirements. | TSG RAN#94 (Dec-21) |  |

6	Work item Rapporteur(s)

Dong Wenjia (CMCC)

dongwenjia@chinamobile.com

Leif Mattisson (Ericsson)

leif.mattisson@ericsson.com

7	Work item leadership

RAN5

8	Aspects that involve other WGs

None

9	Supporting Individual Members

Supporting IM name

CMCC

Huawei

HiSilicon

ZTE

MediaTek Inc

Ericsson

DISH

Orange

Telecom Italia