3GPP TSG-RAN5 Meeting #90-e	R5-211806

# Online, , 22nd Feb 2021 - 5th Mar 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 1117 | rev | 1 | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of 6.5E.2.2.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Guangdong OPPO Mobile Telecom. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL_eV2XARC-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Additon of new TC 6.5E.2.2.1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Additon of new TC 6.5E.2.2.1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test plan will no be complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5E.2, 6.5E.2.2, 6.5E.2.2.1(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Outcome from R5-210422 and R5-210422r1 |  |  |  |  |  |  |  |  |

<< Unchanged sections omitted >>

6.5E.2	Out of band emission for V2X

6.5E.2.1	General

When UE is configured for NR V2X sidelink transmissions non-concurrent with NR uplink transmissions for NR V2X operating bands specified in Table 5.2E.1-1, the requirements in clause 6.5E.2 apply for NR V2X sidelink transmission.

For NR V2X UE with two transmit antenna connectors, the requirements specified for single carrier shall apply to each transmit antenna connector. The requirements shall be met with SL MIMO configurations described in clause 6.2D.1.

6.5E.2.2	Spectrum emission mask for V2X

6.5E.2.2.1	Spectrum emission mask for V2X / non-concurrent operation

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Test config table is FFS

- Uplink RMC is TBD in RAN4

- Connection diagram is TBD

- Preconfiguration is TBD in 38.508-1

- Test state and generic procedure are TBD in 38.508-1

6.5E.2.2.1.1	Test purpose

To verify that the power of any UE emission shall not exceed specified level for the specified channel bandwidth when UE is configured for NR V2X sidelink transmissions non-concurrent with NR uplink transmissions.

6.5E.2.2.1.2	Test applicability

This test case applies to all types of NR UE release 16 and forward that support NR V2X sidelink communication with non-concurrent operation.

6.5E.2.2.1.3	Minimum conformance requirements

For NR V2X UE, the existing NR general spectrum emission mask in clause 6.5.2.2 applied for all supporting NR V2X channel bandwidths. The spectrum emission mask of the UE applies to frequencies (ΔfOOB) starting from the  edge of the assigned NR channel bandwidth. For frequencies greater than (ΔfOOB), the power of any UE emission shall not exceed the levels specified in Table 6.5.2.2.3-1 for the specified channel bandwidth for NR V2X operating bands in Table 5.2E.1-1.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.5E.2.2.

6.5E.2.2.1.4	Test description

6.5E.2.2.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in table 6.5E.2.2.1.4.1-1. The details of the V2X reference measurement channels (RMCs) are specified in Annexes TBD.

Table 6.5E.2.2.1.4.1-1: Test Configuration Table

FFS

1.	Connect the SS and GNSS simulator to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure TBD for TE diagram and section TBD for UE diagram.

2.	The parameter settings for the V2X sidelink transmission over PC5 are pre-configured according to TS 38.508-1 [5] subclause TBD. Message content exceptions are defined in clause 6.5E.2.2.1.4.3.

3.	The V2X Reference Measurement Channel is set according to Table 6.5E.2.2.1.4.1-1.

4.	The GNSS simulator is configured for Scenario #1: static in Geographical area #1, as defined in TS36.508 [7] Table 4.11.2-2. Geographical area #1 is also pre-configured in the UE.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state TBD.

6.5E.2.2.1.4.2	Test procedure

1.	The V2X UE schedules the V2X RMC with transmission power at PUMAX level according to SL-V2X-Preconfiguration which is in line with the test configuration in Table 6.5E.2.2.1.4.1-1.

2.	Measure the mean power of the UE in the channel bandwidth of the radio access mode according to the test configuration, which shall meet the requirements described in [TBD] for Power Class 3 UEs. The period of the measurement shall be at least the continuous duration of one sub-frame (1ms). For TDD slots with transient periods are not under test.

3.	Measure the power of the transmitted signal with a measurement filter of bandwidths according to Table 6.5E.2.2.5-1. The centre frequency of the filter shall be stepped in continuous steps according to the same table. The measured power shall be recorded for each step. The measurement period shall capture the active TSs.

6.5E.2.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 and 5.4.

6.5E.2.2.1.5	Test requirement

The measured UE mean power in the channel bandwidth, derived in step 2, shall fulfil requirements in [TBD] as appropriate, and the power of any UE emission shall fulfil requirements in Table 6.5E.2.2.1.5-1.

Table 6.5E.2.2.1.5-1: General NR spectrum emission mask for V2X / non-concurrent operation

|  | Spectrum emission limit (dBm) / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Measurement bandwidth |
| ± 0-1 |  | -13 + TT | -13 + TT | -13 + TT | -13 + TT | -13 + TT | -13 + TT | -13 + TT |  |  |  |  |  | 1 % channel bandwidth |
| ± 0-1 |  |  |  |  |  |  |  |  | -24 + TT | -24 + TT | -24 + TT | -24 + TT | -24 + TT | 30 kHz |
| ± 1-5 |  | -10 + TT | -10 + TT | -10 + TT | -10 + TT | -10 + TT | -10 + TT | -10 + TT | -10 + TT | -10 + TT | -10 + TT | -10 + TT | -10 + TT | 1 MHz |
| ± 5-6 |  | -13 + TT | -13 + TT | -13 + TT | -13 + TT | -13 + TT | -13 + TT | -13 + TT | -13 + TT | -13 + TT | -13 + TT | -13 + TT | -13 + TT |  |
| ± 6-10 |  | -25 + TT |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 10-15 |  |  | -25 + TT |  |  |  |  |  |  |  |  |  |  |  |
| ± 15-20 |  |  |  | -25 + TT |  |  |  |  |  |  |  |  |  |  |
| ± 20-25 |  |  |  |  | -25 + TT |  |  |  |  |  |  |  |  |  |
| ± 25-30 |  |  |  |  |  | -25 + TT |  |  |  |  |  |  |  |  |
| ± 30-35 |  |  |  |  |  |  | -25 + TT |  |  |  |  |  |  |  |
| ± 35-40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 40-45 |  |  |  |  |  |  |  | -25 + TT |  |  |  |  |  |  |
| ± 45-50 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 50-55 |  |  |  |  |  |  |  |  | -25 + TT |  |  |  |  |  |
| ± 55-60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 60-65 |  |  |  |  |  |  |  |  |  | -25 + TT |  |  |  |  |
| ± 65-80 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 80-85 |  |  |  |  |  |  |  |  |  |  | -25 + TT |  |  |  |
| ± 85-90 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 90-95 |  |  |  |  |  |  |  |  |  |  |  | -25 + TT |  |  |
| ± 95-100 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 100-105 |  |  |  |  |  |  |  |  |  |  |  |  | -25 + TT |  |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel. Note 4: TT for each frequency and channel bandwidth is specified in Table 6.5E.2.2.1.5-2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.5E.2.2.1.5-2: Test Tolerance

FFS

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

<< End of changes >>