3GPP TSG-RAN WG5 Meeting #90-e	R5-211897

# Electronic Meeting, 22nd February – 5th March, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.905 | CR | 0382 | rev | 1 | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | TP analysis for DC_8A_n77A |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Test points are not analyzed for DC_8A_n77A |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added DC_8A_n77A in Table 4.3.3.1-1 to indicate it is completed Added test frequencies for DC_8A_n77A in Table D.2.8-1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not possible to test DC_8A_n77A |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.3, D.2.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR .38.521-3.. CR ..0927. |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | <CR implementation guideline: Table rows to be inserted in Table 4.3.3.1-1 after implementation of CR #0359. Table rows to be inserted in Table D.2.8-1 after implementation of CR #0360 TR rapporteur to guide MCC in order to implement the CRs impacting the new Table format> |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 4.3.3.1	Reference sensitivity test cases for EN-DC

This clause contains information on test point analysis and status for FR1 NR CA test cases in TS 38.521-3 [4] clause 7. The analyses are performed per CA configuration in Table 4.3.3.1-1, Table 4.3.3.1-2 and Table 4.3.3.1-3.

Table 4.3.3.1-1: Reference Sensitivity test cases per EN-DC configuration (2CC)

| Band or band configuration | Single UL | Refsens exception, victim band (Note 2) | Requirement coverage (Note 2) | Comments |
| --- | --- | --- | --- | --- |
| DC_(n)XAA |  |  |  |  |
| 5 | Only single UL requirements defined | 5 |  |  |
| 12 | Only single UL requirements defined | 12 |  |  |
| 38 | Only single UL requirements defined | N/A | NE | Added at RAN5#88-e |
| 41 | Yes | N/A | NE | Added at RAN5#88-e |
| 48 | Only single UL requirements defined | N/A | NE | Added at RAN5#88-e |
| 71 | Yes | 71 | Exception | Added at RAN5#88-e |
| DC_XA_nXA |  |  |  |  |
| DC_2A_n2A | Only single UL requirements defined | N/A | NE | Added at RAN5#88-e |
| DC_3A_n3A | Yes | 3 | Exception | Added at RAN5#88-e |
| DC_5A_n5A | Only single UL requirements defined | N/A | NE | Added at RAN5#88-e |
| DC_7A_n7A | Only single UL requirements defined | N/A | NE | Added at RAN5#88-e |
| DC_8A_n77A | No | 8 (HD4), 8 (IMD4) | NE (HD avoid), HD, IMD | Added at RAN5#90-e |
| DC_41A_n41A | Only single UL requirements defined | N/A | NE | Added at RAN5#88-e |
| DC_48(n)n48AA | Only single UL requirements defined | N/A | NE | Added at RAN5#88-e |
| DC_66A_n66A | Only single UL requirements defined | N/A | NE | Added at RAN5#88-e |
| DC_XA-nYA (Note 1) |  |  |  |  |
| DC_1A_n3A | Yes | 1 (IMD3) CBI | NE (Note 1) IMD3 CBI | Added at RAN5#89-e |
| DC_1A_n78A | No | 1 (IMD4) | IMD4 | Added at RAN5#88-e |
| DC_3A_n1A | Yes | 1 (IMD3) CBI | NE (Note 1) IMD3 CBI | Added at RAN5#89-e |
| DC_3A_n78A | Yes | 3 (IMD2) 3 (IMD4) 3 (HD) n78 (HM) | NE (Note 1) IMD2 PC2&PC3 IMD4 PC2&PC3 HD HM | Added at RAN5#89-e |
| DC_7A_n78A | No | N/A | NE | Added at RAN5#89-e |
| DC_20A_n3A | No | n3 (IMD4), 20 (IMD4) | IMD4, IMD4 | Added at RAN5#89-e |
| DC_20A_n78A | No | 20 (IMD4), n78A (HD) | IMD4, HD | Added at RAN5#88-e |
| DC_40A_n1A | No | N/A | NE | Added at RAN5#89-e |
| DC_40A_n78A | No | 40 (HM), | HM | Added at RAN5#89-e |
| NOTE 1: If single UL is allowed in a DC_XA-nYA configuration the IMD requirements does not apply for a single UL UE and non-exception requirements are tested. NOTE 2: Notations used NE: Non-exception as defined in TS 38.101-3 [7], meaning standalone LTE in TS 36.101 [8] and NR in 38.101-1 [5] requirements apply. HD: UL Harmonic Distortion, as defined in TS 38.101-3 [7], 7.3B.2.3.1 HM: RX Harmonic Mixing, as defined in TS 38.101-3 [7], 7.3B.2.3.2 IMD: Intermodulation Distortion, as defined in TS 38.101-3 [7], 7.3B.2.3.5 CBI: Cross Band Isolation, as defined in TS 38.101-3 [7], 7.3B.2.3.4 |  |  |  |  |

{Unchanged sections skipped}

## D.2.8	Test Frequencies selections for EN-DC

In E-UTRA DL CA and UL CA reference sensitivity testing, Low range and High range are selected for intra-band CA testing. Mid range is selected for inter band E-UTRA CA testing.

In standalone NR CA reference sensitivity testing, Low and High Range are tested as default for intra-band and Mid range for inter-band. For CA combinations containing intra-band configuration in an inter-band CA configuration, low and high range are tested for intra-band CA and mid channel for inter-band without intra-band component.

The same principle shall apply for EN-DC testing.

For EN-DC configurations affected by exceptions the test frequency cannot be freely chosen. One test frequency per exception requirement is sufficient to test the requirement. This is indicated in Table D.2.8-1. Exceptions in TS 38.101-3 clause 7.3A.5 (2UL intermodulation) are not specified since the test frequency and channel bandwidth is already specified in TS 38.101-1.

Table D.2.8-1 Test frequency selection per band pair for exceptions avoidable by test frequency setting (UL harmonics, Rx mixing, cross band isolation).

| Band set (Note 2) | Frequency | Channel BW [MHz] | Exception type | Comments |
| --- | --- | --- | --- | --- |
| 1A / n3A | Mid/Mid | Highest/Highest | - | Non-Exception. To be used in test cases 7.3B.2.3_1.0 to 7.3B.2.3_1.2 |
|  | Low/High | 20 / 40 | Cross band isolation | Exception in TS 38.101-3 table 7.3B.2.3.4-1 |
| 3A / n1A | Mid/Mid | Highest/Highest | - | Non-Exception. To be used in test cases 7.3B.2.3_1.0 to 7.3B.2.3_1.2 |
|  | High/Low | 20 / 20 | Cross band isolation | Exception in TS 38.101-3 table 7.3B.2.3.4-1 |
| 3A / n41A |  |  |  | CMCC CR R5-205929 |
| 3A / n78A | Mid/High | Highest/Highest | - | Non-Exception. To be used in test cases 7.3B.2.3_1.0 to 7.3B.2.3_1.2 |
|  | Mid/3495 | Highest/Highest | UL Harmonic Interference | Exception Note 2 of TS 38.101-3 table 7.3B.2.3.1-1 |
|  | Mid/3525 | 20/20 | UL Harmonic Interference | Exception Note 3 of TS 38.101-3 table 7.3B.2.3.1-1 |
| 3A / n78A | High /Mid | 20/20 | Receiver Harmonic Mixing Exception | Receiver Harmonic Mixing Exception in TS 38.101-3 Table 7.3B.2.3.2-1 |
| 8A / n77A | Mid (UL 897.5 MHz) / 3590 | Highest (10 MHz) / Highest (100 MHz) | UL Harmonic Interference HD4 | Exception Note 7 of TS38.101-3 table 7.3B.2.3.1-1 |
|  | Mid (UL 897.5 MHz) / 3520 | Highest (10 MHz) / Highest (100 MHz) |  | Non-Exception. Not overlapping interference since BWINT=40 MHz, FINT = (40+100)/2 = 70 |
| 20A / n78A | Mid/Mid | Highest/Highest | - | Non-Exception. To be used in test cases 7.3B.2.3_1.0 to 7.3B.2.3_1.2 |
|  | Mid/3388 | Highest/Highest | UL Harmonic Interference | Exception Note 7 of TS 38.101-3 table 7.3B.2.3.1-1 |
| 40A / n78A | Low/Mid | Highest/Highest | - | Non-Exception. To be used in test cases 7.3B.2.3_1.0 to 7.3B.2.3_1.2 |
|  | 2366.6/Mid | 20/20 | Receiver Harmonic Mixing Exception | Receiver Harmonic Mixing Exception Note 8 in TS38.101-3 Table 7.3B.2.3.2-1 |
| 66A / n78A |  |  |  | Qualcomm CR R5-206142 |
| NOTE 1: This selection is used in test case 7.3B.2.3 unless otherwise stated NOTE 2: Aggressor band in bold |  |  |  |  |  |

For EN-DC configurations affected by exceptions where the exception cannot be avoided and hence always apply, the test frequency and bandwidth are defined by RAN4, and not repeated here. There are no reference sensitivity requirements for other frequency/bandwidth.