3GPP TSG-RAN5 Meeting #91-e	R5-212050

# Online, , 17th May 2021 - 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 5277 | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of 41-48 combos to Table 7.3A.5.5-5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | C Spire Wireless |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_CA_R15-UEConTest |  |  |  |  |  | Date: |  |  | 2021-04-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | There are no reference sensitivity test requirements for 41-48 3DL combos. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add reference sensitivity for CA_41C-48A and CA_41A-48C to Table 7.3A.5.5-5 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Devices supporting CA_41C-48A and CA_41A-48C cannot be tested for reference sensitivity |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3A.5.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section>

Table 7.3A.5.5-5: Reference sensitivity QPSK PREFSENS for intra-band contiguous + inter-band

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | E-UTRA Band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
| CA_1A-3C (Note 9) | 1 | - | - | -99.3 | -96.3 | -94.5 | -93.3 | FDD |
|  |  | - | - | -102.016 | -99.016 | -97.216 | -96.016 |  |
|  | 3 | - | - | -93.3 | -90.8 | -89.3 | -88.3 |  |
| CA_1A-3C (Note 10) | 1 | - | - | -99.3 | -96.3 | -94.5 | -93.3 | FDD |
|  |  | - | - | -102.016 | -99.016 | -97.216 | -96.016 |  |
|  | 3 | - | - | -96.3 | -93.3 | -91.5 | -90.3 |  |
|  |  | - | - | -99.016 | -96.016 | -94.216 | -93.016 |  |
| CA_1A-3C (Note 9) | 1 | - | - | -99.3 | -96.3 | -94.5 | -93.3 | FDD |
|  |  | - | - | -102.016 | -99.016 | -97.216 | -96.016 |  |
|  | 3 | - | - | -93.3 | -90.8 | -89.3 | -88.3 |  |
| CA_1A-3C (Note 10) | 1 | - | - | -99.3 | -96.3 | -94.5 | -93.3 | FDD |
|  |  | - | - | -102.016 | -99.016 | -97.216 | -96.016 |  |
|  | 3 | - | - | -96.3 | -93.3 | -91.5 | -90.3 |  |
|  |  | - | - | -99.016 | -96.016 | -94.216 | -93.016 |  |
| CA_1A-41C | 1 | - | - | -99.3 | -96.3 | -94.5 | -93.3 | FDD |
|  |  | - | - | -102.016 | -99.016 | -97.216 | -96.016 |  |
|  | 41 | - | - | -97.3 | -94.3 | -92.5 | -91.3 | TDD |
|  |  | - | - | -100.016 | -97.016 | -95.216 | -94.016 |  |
| CA_1A-42C | 1 | - | - | -99.3 | -96.3 | -94.5 | -93.3 | FDD |
|  |  | - | - | -102.016 | -99.016 | -97.216 | -96.016 |  |
|  | 42 | - | - | -98.0 | -95.0 | -93.2 | -92.0 | TDD |
|  |  | - | - | -100.216 | -97.216 | -95.416 | -94.216 |  |
| CA_2A-5B | 2 | - | - | -97.3 | -94.3 | -92.5 | -91.3 | FDD |
|  |  | - | - | -100.016 | -97.016 | -95.216 | -94.016 |  |
|  | 5 | - | - | -97.3 | -94.3 | - | - |  |
| CA_2A-7C | 2 | - | - | -97.3 | -94.3 | -92.5 | -91.3 | FDD |
|  |  | - | - | -100.016 | -97.016 | -95.216 | -94.016 |  |
|  | 7 | - | - | -97.3 | -94.3 | -92.5 | -91.3 |  |
|  |  | - | - | -100.016 | -97.016 | -95.216 | -94.016 |  |
| CA_2A-12B | 2 | - | - | -97.3 | -94.3 | -92.5 | -91.3 | FDD |
|  |  | - | - | -100.016 | -97.016 | -95.216 | -94.016 |  |
|  | 12 | - | - | -96.3 | -93.3 | - | - |  |
| CA_2A-66C | 2 | - | - | -97.3 | -94.3 | -92.5 | -91.3 | FDD |
|  |  | - | - | -100.016 | -97.016 | -95.216 | -94.016 |  |
|  | 66 | - | - | -98.8 | -95.8 | -94.0 | -92.8 |  |
|  |  | - | - | -101.516 | -98.516 | -96.716 | -95.516 |  |
| CA_2C-5A | 2 | - | - | -97.3 | -94.3 | -92.5 | -91.3 | FDD |
|  |  | - | - | -100.016 | -97.016 | -95.216 | -94.016 |  |
|  | 5 | - | - | -97.3 | -94.3 | - | - |  |
| CA_2C-29A | 2 | - | - | -97.3 | -94.3 | -92.5 | -91.3 | FDD |
|  |  | - | - | -100.016 | -97.016 | -95.216 | -94.016 |  |
|  | 29 | - | - | -96.3 | -93.3 | - | - |  |
| CA_2C-30A | 2 | - | - | -97.3 | -94.3 | -92.5 | -91.3 | FDD |
|  |  | - | - | -100.016 | -97.016 | -95.216 | -94.016 |  |
|  | 30 | - | - | -98.3 | -95.3 | - | - |  |
|  |  | - | - | -101.016 | -98.016 | - | - |  |
| CA_2C-66A | 2 | - | - | -97.3 | -94.3 | -92.5 | -91.3 | FDD |
|  |  | - | - | -100.016 | -97.016 | -95.216 | -94.016 |  |
|  | 66 | - | - | -98.8 | -95.8 | -94.0 | -92.8 |  |
|  |  | - | - | -101.516 | -98.516 | -96.716 | -95.516 |  |
| CA_3C-5A | 3 | - | - | -96.3 | -93.3 | -91.5 | -90.3 | FDD |
|  |  | - | - | -99.016 | -96.016 | -94.216 | -93.016 |  |
|  | 5 | - | - | -97.3 | -94.3 | - | - |  |
| CA_3C-8A | 3 | - | - | -96.3 | -93.3 | -91.5 | -90.3 | FDD |
|  | 8 | - | -98.5 | -96.3 | -93.3 | - | - |  |
| CA_3A-7B | 3 | - | - | -96.3 | -93.3 | -91.5 | -90.3 | FDD |
|  |  | - | - | -99.016 | -96.016 | -94.216 | -93.016 |  |
|  | 7 | - | - | -97.3 | - | -92.5 | - |  |
|  |  | - | - | -100.016 | - | -95.216 | - |  |
| CA_3A-7C | 3 | - | - | -96.3 | -93.3 | -91.5 | -90.3 | FDD |
|  |  | - | - | -99.016 | -96.016 | -94.216 | -93.016 |  |
|  | 7 | - | - | -97.3 | -94.3 | -92.5 | -91.3 |  |
|  |  | - | - | -100.016 | -97.016 | -95.216 | -94.016 |  |
| CA_3A-40C (Note 12) | 3 | - | - | -96.3 | -93.3 | -91.5 | -90.3 | FDD |
|  |  | - | - | -99.016 | -96.016 | -94.216 | -93.016 |  |
|  | 40 | - | - | -94.7 | -92.2 | -90.6 | -89.5 | TDD |
| CA_3A-40C (Note 13) | 3 | - | - | -93.5 | -90.5 | -88.8 | -87.6 | FDD |
|  | 40 | - | - | -99.3 | -96.3 | -94.5 | -93.3 | TDD |
| CA_3A-41C (Note 12) | 3 | - | - | [-93.3] | [-90.3] | [-88.5] | [-87.2] | FDD |
|  | 41 | - | - | -96.811 | -93.811 | -9211 | -90.811 | TDD |
|  |  | - | - | -99.511,16 | -96.511,16 | -94.711,16 | -93.511,16 |  |
| CA_3A-41C (Note 14) | 3 | - | - | -96.3 | -93.3 | -91.5 | -90.3 | FDD |
|  |  | - | - | -99.016 | -96.016 | -94.216 | -93.016 |  |
|  | 41 | - | - | [-92.6]11 | [-90]11 | [-88.5]11 | [-87.4]11 | TDD |
| CA_3A-42C | 3 | - | - | -96.3 | -93.3 | -91.5 | -90.3 | FDD |
|  |  | - | - | -99.016 | -96.016 | -94.216 | -93.016 |  |
|  | 42 | - | - | -98.0 | -95.0 | -93.2 | -92.0 | TDD |
|  |  | - | - | -100.216 | -97.216 | -95.416 | -94.216 |  |
|  | 42 (Note 4) | - | - | -71.2 | -71.2 | -71.2 | -71.2 |  |
|  | 42 (Note 5) | - | - | -96.6 | -94.2 | -92.7 | -92.0 |  |
| CA_3A-42C (Note 17) | 3 | - | - | -67.0 | - | - | - | FDD |
|  |  | - | - | -67.06 | - | - | - |  |
|  | 42 | - | - | -71.7 | - | - | - | TDD |
|  |  | - | - | -88.8 | - | - | - |  |
|  | 3 | - | - | -88.26 | - | - | - | FDD |
|  |  | - | - | -97.1 | - | - | - |  |
|  | 42 | - | - | -67.0 | - | - | - | TDD |
|  |  | - | - | -67.06 | - | - | - |  |
|  | 3 | - | - | -96.3 | -93.3 | -91.5 | -90.3 | FDD |
|  |  | - | - | -99.016 | -96.016 | -94.216 | -93.016 |  |
|  | 42 | - | - | -98.0 | -95.0 | -93.2 | -92.0 | TDD |
|  |  | - | - | -100.216 | -97.216 | -95.416 | -94.216 |  |
| CA_3C-7A | 3 | - | - | -96.3 | -93.3 | -91.5 | -90.3 | FDD |
|  |  | - | - | -99.016 | -96.016 | -94.216 | -93.016 |  |
|  | 7 | - | - | -97.3 | -94.3 | -92.5 | -91.3 |  |
|  |  | - | - | -100.016 | -97.016 | -95.216 | -94.016 |  |
| CA_3C-20A | 3 |  |  | -96.3 | -93.3 | -91.5 | -90.3 |  |
|  |  |  |  | -99.016 | -96.016 | -94.216 | -93.016 |  |
|  | 20 |  |  | -96.3 | -93.3 | -90.5 | -89.3 |  |
|  |  |  |  | -99.016 | -96.016 | -93.216 | -92.016 |  |
| CA_3C-28A | 3 | - | - | -96.3 | -93.3 | -91.5 | -90.3 | FDD |
|  |  | - | - | -99.016 | -96.016 | -94.216 | -93.016 |  |
|  | 28 | - | - | -97.8 | -94.8 | -93.0 | -90.3 |  |
| CA_4A-7C | 4 | - | - | -99.3 | -96.3 | -94.5 | -93.3 | FDD |
|  |  | - | - | -102.016 | -99.016 | -97.216 | -96.016 |  |
|  | 7 | - | - | -97.3 | -94.3 | -92.5 | -91.3 |  |
|  |  | - | - | -100.016 | -97.016 | -95.216 | -94.016 |  |
| CA_5A-12B | 5 | - | - | -97.3 | -94.3 | - | - | FDD |
|  | 12 | - | - | -96.3 | -93.3 | - | - |  |
| CA_5A-40C | 5 | - | -99.5 | -97.3 | -94.3 | - | - | FDD |
|  | 40 | - | - | - | -96.3 | -94.5 | -93.3 | TDD |
|  |  | - | - | - | -99.016 | -97.216 | -96.016 |  |
| CA_5B-30A | 5 | - | - | -97.3 | -94.3 | - | - | FDD |
|  | 30 | - | - | -98.3 | -95.3 | - | - |  |
|  |  | - | - | -101.016 | -98.016 | - | - |  |
| CA_5B-66A | 5 | - | - | -97.3 | -94.3 | - | - | FDD |
|  | 66 | - | - | -98.8 | -95.8 | -94.0 | -92.8 |  |
| CA_7B-28A | 7 | - | - | -97.3 | -94.3 | -92.5 | -91.3 | FDD |
|  |  | - | - | -100.016 | -97.016 | -95.216 | -94.016 |  |
|  | 28 | - | - | -97.6 | -94.6 | -92.8 | -90.1 |  |
| CA_7C-28A | 7 | - | - | -97.3 | -94.3 | -92.5 | -91.3 | FDD |
|  |  | - | - | -100.016 | -97.016 | -95.216 | -94.016 |  |
|  | 28 | - | - | -97.6 | -94.6 | -92.8 | -90.1 |  |
| CA_8A-40C | 8 |  |  | -96.3 | -93.3 |  |  |  |
|  | 40 |  |  |  | -96.3 | -94.5 | -93.3 |  |
|  |  |  |  |  | -99.016 | -97.216 | -96.016 |  |
| CA_8A-42C | 8 | -101.3 | -98.3 | -96.1 | -93.1 | - | - | FDD |
|  | 42 | - | - | -98.0 | -95.0 | -93.2 | -92.0 | TDD |
|  |  | - | - | -100.216 | -97.216 | -95.416 | -94.216 |  |
|  | 42 (Note 8) | - | - | -83.8 | -83.7 | -83.6 | -83.5 |  |
| CA_11A-41C | 11 | - | - | -100.0 | -97.0 | - | - | FDD |
|  | 41 | - | - | -97.3 | -94.3 | -92.5 | -91.3 | TDD |
|  |  | - | - | -100.016 | -97.016 | -95.216 | -94.016 |  |
| CA_11A-42C | 11 | - | - | -100.0 | -97.0 | - | - | FDD |
|  | 42 | - | - | -97.5 | -94.5 | -92.7 | -91.5 | TDD |
|  |  | - | - | -99.716 | -96.716 | -94.916 | -93.716 |  |
| CA_18A-42C | 18 | - | - | -99.3 | -96.3 | -94.5 | - | FDD |
|  | 42 | - | - | -98.0 | -95.0 | -93.2 | -92.0 | TDD |
|  |  | - | - | -100.216 | -97.216 | -95.416 | -94.216 |  |
| CA_19A-42C | 19 | - | - | -99.3 | -96.3 | -94.5 | - | FDD |
|  | 42 | - | - | -98.0 | -95.0 | -93.2 | -92.0 | TDD |
|  |  | - | - | -100.216 | -97.216 | -95.416 | -94.216 |  |
| CA_21A-42C | 21 | - | - | -99.3 | -96.3 | -94.5 | - | FDD |
|  |  | - | - | -102.016 | -99.016 | -97.216 | - |  |
|  | 42 | - | - | -98.0 | -95.0 | -93.2 | -92.0 | TDD |
|  |  | - | - | -100.216 | -97.216 | -95.416 | -94.216 |  |
| CA_25A-41C | 25 | - | - | -95.8 | -92.8 | -91.0 | -89.8 | FDD |
|  |  | - | - | -98.516 | -95.516 | -93.716 | -92.516 |  |
|  | 41 | - | - | -97.3 | -94.3 | -92.5 | -91.3 | TDD |
|  |  | - | - | -100.016 | -97.016 | -95.216 | -94.016 |  |
| CA_26A-41C | 26 | - | - | -96.86 | -93.86 | -92.06 | - | FDD |
|  | 41 | - | - | -97.3 | -94.3 | -92.5 | -91.3 | TDD |
|  |  | - | - | -100.016 | -97.016 | -95.216 | -94.016 |  |
| CA_28A-40C (Note 15) | 28 | - | - | -97.8 | -94.8 | -93 | -90.3 | FDD |
|  | 40 | - | - | -94.4 | -92.2 | -90.7 | -89.8 | TDD |
| CA_28A-40C (Note 13) | 28 | - | - | -96.1 | -93.4 | -91.8 | -89.1 | FDD |
|  | 40 | - | - | -99.3 | -96.3 | -94.5 | -93.3 | TDD |
| CA_28A-41C | 28 | - | - | -97.8 | -94.8 | - | - | FDD |
|  | 41 | - | - | -97.3 | -94.3 | -92.5 | -91.3 | TDD |
|  |  | - | - | -100.016 | -97.016 | -95.216 | -94.016 |  |
| CA_28A-42C | 28 | - | - | -97.3 | -94.3 | -92.5 | -89.8 | FDD |
|  | 42 | - | - | -96.5 | -93.5 | -91.7 | -90.5 | TDD |
|  | 42 (Note 7) | - | - | -84.9 | -84.6 | -84.3 | -84.1 | TDD |
| CA_29A-66C | 29 | - | - | -96.3 | -93.3 | - | - | FDD |
|  | 66 | - | - | -98.8 | -95.8 | -94.0 | -92.8 |  |
|  |  | - | - | -101.516 | -98.516 | -96.716 | -95.516 |  |
| CA_29A-70C | 29 | - | - | -96.3 | -93.3 | - | - | FDD |
|  | 70 | - | - | -99.3 | -96.3 | -94.5 | -93.3 |  |
| CA_38A-40C | 38 | - | - | - | -96.3 | -94.5 | -93.3 | TDD |
|  | 40 | - | - | - | -96.3 | -94.5 | -93.3 |  |
|  |  | - | - | - | -99.016 | -97.216 | -96.016 |  |
| CA_39A-41C | 39 | - | - | - | -96.3 | -94.5 | -93.3 | TDD |
|  |  | - | - | - | -99.016 | -97.216 | -96.016 |  |
|  | 41 | - | - | -97.3 | -94.3 | -92.5 | -91.3 |  |
|  |  | - | - | -100.016 | -97.016 | -95.216 | -94.016 |  |
| CA_39C-41A | 39 | - | - | -99.3 | -96.3 | -94.5 | -93.3 | TDD |
|  |  | - | - | -102.016 | -99.016 | -97.216 | -96.016 |  |
|  | 41 | - | - | - | - | - | -91.3 |  |
|  |  | - | - | - | - | - | -94.016 |  |
| CA_41A-42C | 41 | - | - | - | -93.9 | -92.1 | -90.9 | TDD |
|  |  | - | - | - | -96.616 | -94.816 | -93.616 |  |
|  | 42 | - | - | -97.5 | -94.5 | -92.7 | -91.5 |  |
|  |  | - | - | -99.716 | -96.716 | -94.916 | -93.716 |  |
| CA_41C-42A | 41 | - | - | - | -93.9 | -92.1 | -90.9 | TDD |
|  |  | - | - | - | -96.616 | -94.816 | -93.616 |  |
|  | 42 | - | - | -97.5 | -94.5 | -92.7 | -91.5 |  |
|  |  | - | - | -99.716 | -96.716 | -94.916 | -93.716 |  |
| CA_41A-48C | 41 | - | - | - | -94.3 | -92.5 | -91.3 | TDD |
|  | 48 | - | - | -97.5 | -94.5 | -92.7 | -91.5 |  |
| CA_41C-48A | 41 | - | - | - | -94.3 | -92.5 | -91.3 | TDD |
|  | 48 | - | - | -97.5 | -94.5 | -92.7 | -91.5 |  |
| CA_46C-66A | 46 | - | - | - | - | - | -88.5 | TDD |
|  | 66 | - | - | -98.8 | -95.8 | -94.0 | -92.8 | FDD |
|  |  | - | - | -101.516 | -98.516 | -96.716 | -95.516 |  |
| CA_46A-66C | 46 | - | - | - | - | - | -88.5 | TDD |
|  | 66 | - | - | -98.8 | -95.8 | -94.0 | -92.8 | FDD |
|  |  | - | - | -101.516 | -98.516 | -96.716 | -95.516 |  |
| CA_66A-70A | 66 | - | - | -98.8 | -95.8 | -94.0 | -92.8 | FDD |
|  |  | - | - | -101.516 | -98.516 | -96.716 | -95.516 |  |
|  | 70 | - | - | -99.3 | -96.3 | -94.5 | - |  |
| CA_66A-70C | 66 | - | - | -98.8 | -95.8 | -94.0 | -92.8 | FDD |
|  |  | - | - | -101.516 | -98.516 | -96.716 | -95.516 |  |
|  | 70 | - | - | -99.3 | -96.3 | -94.5 | -93.3 |  |
| CA_66C-70A | 66 | - | - | -98.8 | -95.8 | -94.0 | -92.8 | FDD |
|  |  | - | - | -101.516 | -98.516 | -96.716 | -95.516 |  |
|  | 70 | - | - | -99.3 | -96.3 | -94.5 | - |  |
| CA_66C-71A | 66 | - | - | -98.8 | -95.8 | -94.0 | -92.8 | FDD |
|  |  | - | - | -101.516 | -98.516 | -96.716 | -95.516 |  |
|  | 71 | - | - | -96.5 | -93.5 | -91.2 | -86.8 |  |
| CA_70C-71A | 70 | - | - | -99.3 | -96.3 | -94.5 | -93.3 | FDD |
|  | 70 | - | - | -89.318 | -88.818 | -88.518 | -88.318 |  |
|  | 71 | - | - | -96.5 | -93.5 | -91.2 | -86.8 |  |
| Note 1: The transmitter shall be set to maximum output power level (Table 7.3A.5.5-4) Note 2: The reference measurement channel is specified in A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 Note 3: The signal power is specified per port Note 4: Reference sensitivity for the high band for which the 2nd harmonic of the low band is within transmission bandwidth, as specified Note 9 and 10 in Table 7.3A.0-0a. Note 5: Reference sensitivity for the high band for which the 2nd harmonic of the low band is within transmission bandwidth, as specified Note 11 in Table 7.3A.0-0a. Note 6: 6indicates that the requirement is modified by -0.5 dB when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. Note 7: Reference sensitivity for the high band for which the 5th harmonic of the low band is within transmission bandwidth, as specified Note 15 and 16 in Table 7.3A.0-0a. Note 8: Reference sensitivity for the high band for which the 4th harmonic of the low band is within transmission bandwidth, as specified Note 12 and 13 in Table 7.3A.0-0a. Note 9: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz. For each channel bandwidth in Band 3, the requirement applies regardless of channel bandwidth in Band 1. Note 10: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. For each channel bandwidth in Band 3, the requirement applies regardless of channel bandwidth in Band 1. Note 11: The B41 requirements are modified by -0.5dB when carrier frequency of the assigned E-UTRA channel bandwidth is within 2545-2690 MHz. Note 12: These requirements apply when the uplink is active in Band 3. Note 13: These requirements apply when the uplink is active in Band 40. Note 14: These requirements apply when the uplink is active in Band 41. Note 15: These requirements apply when the uplink is active in Band 28. Note 16: Applicable only if operation with 4 antenna ports is supported in the band with carrier aggregation configured. NOTE 17: These requirements apply when UL CA is configured and the uplink is active in Band X and Band Y in CA_XA-YC. Note 18: Reference sensitivity for the high band for which the 3rd harmonic of the low band is within transmission bandwidth, as specified Note 5 and 6 in Table 7.3A.0-0a. |  |  |  |  |  |  |  |  |

<End of modified section>