3GPP TSG-RAN5 Meeting #91-e	R5-212062

# Online, , 17th May 2021 - 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-2 | CR | 0173 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add new UE capability for Rel-16 NR Mobility Enhancement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecommunications |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2021-04-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To align with the latest WIP,the UE capability to support inter-frequency DAPS handover feature needs to be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add UE capability to support inter-frequency DAPS handover |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The WIP and the related TCs will not be fully completed |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4.3.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

### A.4.3.8	Mobility Capabilities

Table A.4.3.8-1: UE Mobility Capabilities

| Item | UE Mobility Capabilities | Ref. | Release | Mnemonic | M | If indicated "Yes" the feature shall be implemented and successfully tested for the corresponding release | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Support inter-RAT Handover to EUTRA connected to EPC | 38.306, 4.2.9 | Rel-15 | pc_interRAT_EUTRA_Handover | Yes |  |  |
| 2 | Support inter-frequency Handover from the corresponding duplex mode or from the corresponding frequency range. | 38.306, 4.2.9 | Rel-15 | pc_handoverInterF | Yes |  |  |
| 3 | Support Handover between FR1 and FR2 | 38.306, 4.2.9 | Rel-15 | pc_FR1toFR2_Handover | Yes |  |  |
| 4 | Support Handover between FDD and TDD | 38.306, 4.2.9 | Rel-15 | pc_FDDtoTDD_Handover | Yes |  |  |
| 5 | Support inter-RAT Handover to E-UTRA connected to 5GC | 38.306, 4.2.9 | Rel-15 | pc_interRAT_eLTE_Handover | Yes |  |  |
| 6 | Support inter-RAT Handover to NR FR1 TDD from EUTRA connected to EPC | 36.306, 4.3.34.9 | Rel-15 | pc_eutra_EPC_HO_ToNR_TDD_FR1_r15 | Yes |  |  |
| 7 | Support inter-RAT Handover to NR FR1 FDD from EUTRA connected to EPC | 36.306, 4.3.34.8 | Rel-15 | pc_eutra_EPC_HO_ToNR_FDD_FR1_r15 | Yes |  |  |
| 8 | Support inter-RAT Handover to NR FR2 TDD from EUTRA connected to EPC | 36.306, 4.3.34.11 | Rel-15 | pc_eutra_EPC_HO_ToNR_TDD_FR2_r15 | Yes |  |  |
| 9 | Support intra-frequency DAPS handover | 38.306, 4.2.7.5 | Rel-16 | pc_intraFreqDAPS_r16 | No |  |  |
| 10 | Support inter-RAT Handover from NR to EN-DC | 38.306, 4.2.10 | Rel-16 | pc_interRAT_NR_ToENDC | CY |  | It is mandated if the UE supports EN-DC. |
| 11 | Support inter-frequency DAPS handover | 38.306, 4.2.7.4 | Rel-16 | pc_interFreqDAPS_r16 | No |  |  |

<End of modified section 1>