3GPP TSG RAN WG5 Meeting #91-e	R5-212151

# Electronic Meeting, 17th - 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-2 | CR | 0144 | rev | - | Current version: | 14.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new MCPTT test case 6.2.24 On-network / Remotely initiated private call / Client Originated (CO) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCenhUEConTest |  |  |  |  |  | Date: |  |  | 2021-05-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | MCPTT Remotely initiated private call is a new feature introduced as part of Rel-15. Testing of the relevant requirements is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new test case 6.2.24 'On-network / Private call / Remotely initiated private call / Client Originated (CO)'. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of testing of UE implementation for conformance to important requirements may lead to incorrectly implemented devices to enter the market and disrupt services or/and user experience. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.24 (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | With the agreement of the present CR, TS 36.579-2 needs to be upgraded to Rel-15. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

6.2.17.3.3	Specific message contents

Table 6.2.17.3.3-1: SIP INVITE (Step 1, Table 6.2.17.3.2-1; step 2, TS 36.579-1 [2], Table 5.3.4.3-1)

| Derivation Path: TS 36.579-1 [2], table 5.5.2.5.2-1, condition PRIVATE-CALL |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Answer-Mode | not present |  |  |  |
| Priv-Answer-Mode |  |  |  |  |
| answer-mode-value | "Auto" |  |  |  |
| answer-mode-param | “require” |  |  |  |
| Message-body |  |  |  |  |
| MIME body part |  | SDP Message |  |  |
| MIME-part-body | SDP Message as described in Table 6.2.17.3.3-3 |  |  |  |
| MIME body part |  | MCPTT-Info |  |  |
| MIME-part-body | MCPTT-Info as described in Table 6.2.17.3.3-2 |  |  |  |

Table 6.2.17.3.3-2: MCPTT-Info in SIP INVITE (Table 6.2.17.3.3-1)

| Derivation Path: TS 36.579-1 [2], table 5.5.3.2.2-1, condition PRIVATE-CALL |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcpttinfo |  |  |  |  |
| mcptt-Params |  |  |  |  |
| session-type | "ambient-listening" |  |  |  |
| anyExt |  |  |  |  |
| ambient-listening-type | "local-init" |  |  |  |

Table 6.2.17.3.3-3: SDP Message in SIP INVITE (Table 6.2.17.3.3-1)

| Derivation Path: TS 36.579-1 [2], table 5.5.3.1.2-1, condition PRIVATE-CALL |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| media attribute |  | a=line attribute=sendonly |  |  |
| sendonly |  |  |  |  |

Table 6.2.17.3.3-3A: SIP 200 (OK) (Step 2, Table 6.2.17.3.2-1; step 4, TS 36.579-1 [2] Table 5.3.4.3-1)

| Derivation Path: TS 36.579-1 [2], table 5.5.2.17.1.1-1 condition INVITE-RSP |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| SDP Message | As described in Table 6.2.17.3.3-3B |  |  |  |

Table 6.2.17.3.3-3B: SDP Message in SIP 200 OK (Table 6.2.17.3.3-3A)

| Derivation Path: TS 36.579-1 [2], table 5.5.3.1.1-1, condition PRIVATE-CALL |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| media attribute |  | a=line attribute=recvonly |  |  |
| recvonly |  |  |  |  |

Table 6.2.17.3.3-4: Floor Taken (Step 3, Table 6.2.17.3.2-1)

| Derivation Path: 36.579-1 [2], Table 5.5.6.3-7 condition ON-NETWORK. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Permission to Request the Floor |  |  |  |
| Permission to Request the Floor | "0" | The receiver is NOT permitted to request floor |  |

Table 6.2.17.3.3-5: Void

### 6.2.24	On-network / Private call / Remotely initiated private call / Client Originated (CO)

6.2.24.1	Test Purpose (TP)

(1)

with { UE (MCPTT Client) registered and authorized for MCPTT Service, including authorized to initiate Remotely initiated private call and to participate in private calls }

ensure that {

when { UE (MCPTT User) requests the establishment of a remotely initiated private call to a third party }

then { UE (MCPTT Client) sends a SIP MESSAGE message requesting the establishment of a remotely initiated private call }

}

(2)

with { UE (MCPTT Client) registered and authorized for MCPTT Service, including authorized to initiate Remotely initiated private call and to participate in private calls, and, having requested the establishment of a remotely initiated private call }

ensure that {

when { UE (MCPTT User) receives a SIP MESSAGE message indicating the outcome of the requested establishment of a remotely initiated private call }

then { UE (MCPTT Client) sends a SIP OK message to confirm the reception of the SIP MESSAGE }

}

6.2.24.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.379 clauses 11.1.7.2.1. Unless otherwise stated these are Rel-15 requirements.

[TS 24.379, clause 11.1.7.2.1]

Upon receiving a request from the MCPTT to send a remotely initiated private call request to the remote MCPTT user to originate a private call to an identified MCPTT user, the MCPTT client:

1)	if:

a)	the <allow-request-remote-init-private-call> element of the <ruleset> element is not present in the requesting MCPTT user's MCPTT user profile document (see the MCPTT user profile document in 3GPP TS 24.484 [50]) or is set to a value of "false";

...

2)	shall generate a SIP MESSAGE request in accordance with 3GPP TS 24.229 [4] and IETF RFC 3428 [33] with the following clarifications:

a)	shall include the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcptt" (coded as specified in 3GPP TS 24.229 [4]), in a P-Preferred-Service header field according to IETF RFC 6050 [9] in the SIP MESSAGE request;

b)	shall include an Accept-Contact header field with the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcptt" along with the "require" and "explicit" header field parameters according to IETF RFC 3841 [6];

c)	may include a P-Preferred-Identity header field in the SIP MESSAGE request containing a public user identity as specified in 3GPP TS 24.229 [4];

d)	shall include an application/vnd.3gpp.mcptt-info+xml MIME body as specified in clause F.1 with the <mcpttinfo> element containing the <mcptt-Params> element with the <anyExt> element containing:

i)	the <request-type> element set to a value of "remotely-initiated-private-call-request";

ii)	the <mcptt-called-party-id> element set to the MCPTT ID of the identified MCPTT user of the remotely initiated private call;

iii)	the <notify-remote-user> element set to a value of "true" if the requesting MCPTT user has indicated that the remote MCPTT user be notified of the remotely initiated private call request; or

iv)	the <notify-remote-user> element set to a value of "false" if the requesting MCPTT user has indicated that the remote MCPTT user not be notified of the remotely initiated private call request;

e)	shall insert in the SIP MESSAGE request a MIME resource-lists body with the MCPTT ID of the remote MCPTT user, according to rules and procedures of IETF RFC 5366 [20]; and

f)	shall set the Request-URI to the public service identity identifying the participating MCPTT function serving the MCPTT user; and

3)	shall send the SIP MESSAGE request towards the MCPTT server according to rules and procedures of 3GPP TS 24.229 [4].

...

Upon receiving a "SIP MESSAGE request for remotely initiated private call response for terminating client", the MCPTT client:

1)	shall determine the success or failure of the sent remotely initiated private call request from the value of the <remotely-initiated-call-outcome> element contained in the <anyExt> element of the <mcptt-Params> element  of the <mcpttinfo> element of the application/vnd.3gpp.mcptt-info+xml MIME body included in the received SIP MESSAGE request; and

2)	should indicate to the requesting MCPTT user the success or failure of the sent remotely initiated private call request.

6.2.24.3	Test description

6.2.24.3.1	Pre-test conditions

System Simulator:

-	SS (MCPTT server)

-	For the underlining "transport bearer" over which the SS and the UE will communicate Parameters are set to the default parameters for the basic E-UTRA Single cell network scenarios, as defined in TS 36.508 [24] clause 4.4. The simulated Cell 1 shall belong to PLMN1 (the PLMN specified for MCPTT operation in the MCPTT configuration document).

IUT:

-	UE (MCPTT client)

-	The test USIM set as defined in TS 36.579-1 [2], subclause 5.5.10 is inserted.

Preamble:

-	The UE has performed the Generic Test Procedure for MCPTT UE registration as specified in TS 36.579-1 [2], subclause 5.4.2.

-	The MCPTT User performs the Generic Test Procedure for MCPTT Authorization/Configuration and Key Generation as specified in TS 36.579-1 [2], subclause 5.3.2.

-	The MCPTT User is authorized to initiate Remotely initiated private call: the <allow-request-remote-init-private-call> element of the <ruleset> element is present in the MCPTT user profile document and is set to "true".

-	The MCPTT User is authorized to participate in private calls: the <allow-private-call-participation> element of the <ruleset> element is present in the MCPTT user profile document and is set to "true".

-	UE States at the end of the preamble

-	The UE is in E-UTRA Registered, Idle Mode state.

-	The MCPTT Client Application has been activated and User has registered-in as the MCPTT User with the Server as active user at the Client.

6.2.24.3.2	Test procedure sequence

Table 6.2.24.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Make the UE (MCPTT User) request the establishment of an MCPTT remotely initiated private call to a third party (i.e. not to itself). (NOTE 1) | - | - | - | - |
| - | EXCEPTION: Step 1a1 describes behaviour that depends on the E-UTRA RRC state at the time the present procedure is called. | - | - | - | - |
| 2a1 | IF in RRC_IDLE state, the E-UTRA/EPC actions which are related to the MCPTT call establishment described in clause 5.4.3 'Generic Test Procedure for MCPTT CO communication in E-UTRA' take place. | - | - | - | - |
| 3 | Check: Does the UE (MCPTT Client) send a SIP MESSAGE requesting the establishment of an MCPTT remotely initiated private call? | --> | SIP MESSAGE | 1 | P |
| 4 | The SS (MCPTT server) responds with a SIP 200 (OK) | <-- | SIP 200 (OK) | - | - |
| 5 | Wait for 3 sec (an arbitrary value to simulate the time needed for the establishment of the remote private call). | - | - | - | - |
| 6 | The SS (MCPTT server) sends a SIP MESSAGE confirming the progress of the MCPTT remotely initiated private call. | <-- | SIP MESSAGE | - | - |
| 7 | Check: Does the UE (MCPTT Client) respond with a SIP 200 (OK)? (NOTE 2) | --> | SIP 200 (OK) | 2 | P |
| 8 | The Generic Test Procedure for MCPTT CT call release as described in TS 36.579-1 [2] Table 5.3.12.3-1 to end the private call is performed. | - | - | - | - |
| NOTE 1: This is expected to be done via a suitable implementation dependent MMI. NOTE 2: The UE is expected to determine the success or failure of the sent remotely initiated private call request. Following this the UE client may indicate to the requesting MCPTT user the success or failure of the sent remotely initiated private call request. This is not verified because the indication to the User is not mandated. |  |  |  |  |  |

6.2.24.3.3	Specific message contents

Table 6.2.24.3.3-1: SIP MESSAGE (Step 3, Table 6.2.24.3.2-1)

| Derivation Path: TS 36.579-1 [2], table 5.5.2.7.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Content-Type | "multipart/mixed" |  |  |  |
| Content-Length | length of message body |  |  |  |
| Message-body |  |  |  |  |
| MIME-Content-Type | "application/vnd.3gpp.mcptt-info+xml" |  |  |  |
| MCPTT-Info | As described in Table 6.2.24.3.3-2 |  |  |  |

Table 6.2.24.3.3-2: MCPTT-Info in SIP MESSAGE (Table 6.2.24.3.3-1)

| Derivation Path: TS 36.579-1 [2], table 5.5.3.2.1-1, condition PRIVATE-CALL, INVITE_REFER |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcpttinfo |  |  |  |  |
| mcptt-Params |  |  |  |  |
| anyExt |  | The anyExt field may contain other values - these need not be checked |  |  |
| request-type | "remotely-initiated-private-call-request" |  |  |  |
| mcptt-called-party-id | px_MCPTT_ID_User_C | The MCPTT ID of the identified MCPTT user of the remotely initiated private call It is assumed that the UE has px_MCPTT_ID_User_A with the px_MCPTT_ID_User_B being the ID of the SS and the UE requests a call to a third party px_MCPTT_ID_User_C |  |  |
| notify-remote-user | Any value |  |  |  |

Table 6.2.24.3.3-3: SIP MESSAGE (Step 6, Table 6.2.24.3.3-1)

| Derivation Path: TS 36.579-1 [2], table 5.5.2.7.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Accept-Contact |  |  |  |  |
| ac-value[1] |  |  |  |  |
| feature-param | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcptt" |  |  |  |
| req-param | "require" |  |  |  |
| explicit-param | "explicit" |  |  |  |
| P-Preferred-Identity | If present |  |  |  |
| PPreferredID-value | same URI as in From-header |  |  |  |
| Content-Type | "multipart/mixed" |  |  |  |
| Content-Length | length of message body |  |  |  |
| Message-body |  |  |  |  |
| MIME-Content-Type | "application/vnd.3gpp.mcptt-info+xml" |  |  |  |
| MCPTT-Info | As described in Table 6.2.25.3.3-7 |  |  |  |

Table 6.2.24.3.3-4: MCPTT-Info in SIP MESSAGE (Table 6.2.24.3.3-3)

| Derivation Path: TS 36.579-1 [2], table 5.5.3.2.2-1, condition PRIVATE-CALL, INVITE_REFER |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcpttinfo |  |  |  |  |
| mcptt-Params |  |  |  |  |
| anyExt |  | The anyExt field may contain other values - these need not be checked |  |  |
| response-type | "remotely-initiated-private-call-response" |  |  |  |
| mcptt-called-party-id | px_MCPTT_ID_User_C | The MCPTT ID of the identified MCPTT user of the remotely initiated private call |  |  |
| remotely-initiated-call-outcome | "success" |  |  |  |