3GPP TSG-RAN WG5 Meeting #91-e	R5-212294

# Online, , 17th May 2021 - 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-1 | CR | 0134 | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to Resource List message content |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCPTT-ConTest |  |  |  |  |  | Date: |  |  | 2021-04-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - EMERGENCY_ALERT condition not necessary – will be handled in specific message content, was removed in general conditions - No conditions for retrieval of documents of further groups |  |  |  |  |  |  |  |  |
| Summary of change: |  | - EMERGENCY-ALERT condition removed - Conditions for retrieval of documents of further groups added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Wrong message content results in incorrect MCPTT test case implementation and test case verification not possible |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 36.579-1 CR 0144 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 5.5.3.3	Resource-lists

##### 5.5.3.3.1	Resource-lists from the UE

##### -	MCPTT

Table 5.5.3.3.1-1: Resource-lists from the UE for MCPTT

| Derivation Path: RFC 5366 [35] / RFC 4826 [83] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| resource-lists | encrypted (NOTE 4) |  |  |  |
| list[1] | encrypted (NOTE 4) |  |  |  |
| name attribute | Not present |  |  |  |
| display-name | Not present |  |  |  |
| entry[1] | NOTE 4,5 |  |  |  |
| uri attribute | px_MCPTT_ID_User_B | The MCPTT ID of the invited user |  |  |
|  | px_MCPTT_ID_User_B | the MCPTT ID contained in the <mcptt-calling-user-id> element in the application/ vnd.3gpp.mcptt-info+xml MIME body of the received SIP MESSAGE request |  | MSG_RSP |
|  | SIP-URI with px_MCPTT_Group_A_ID (NOTE 8) extended with SIP URI header fields as specified for the SIP REFER message | SIP-URI: prearranged MCPTT group identit or chat group identity extended with header fields |  | PRE-ESTABLISH AND (GROUP-CALL OR CHAT-GROUP-CALL) |
|  | SIP-URI with px_MCPTT_ID_User_B (NOTE 8) extended with SIP URI header fields as specified for the SIP REFER message | SIP-URI: MCPTT ID of the called user extended with header fields |  | PRE-ESTABLISH AND PRIVATE-CALL |
| display-name | not present |  |  |  |
| resource-lists | encrypted (NOTE 4) |  | TS 24.481 [11] TS 24.484 [14] | CONFIG OR GROUPCONFIG |
| list[1] | encrypted (NOTE 4) |  |  |  |
| name attribute | not present |  |  |  |
| display-name | Not present |  |  |  |
| entry[1] | NOTE 4, 5 |  | TS 24.484 [14] | CONFIG |
| uri attribute | AUID1 & “/users/” & XUID & “/” & MCSUEID & “/” | UE Configuration document (NOTE 1a, 2, 3) |  |  |
|  | “AUID1 & “/users/” & XUID & “/” | Editor’s note: It is not clear in the core specs whether both options are allowed or only one of both; if the UE is allowed not to include the MCSUEID, it is not clear where the MC server gets it from |  |  |
| display-name | Not present |  |  |  |
| entry[2] | NOTE 4, 5 |  | TS 24.484 [14] | CONFIG |
| uri attribute | AUID2 & “/users/” & XUID & “/” | UE User Profile document (NOTE 1b, 2) |  |  |
| display-name | Not present |  |  |  |
| entry[3] | NOTE 4, 5 |  | TS 24.484 [14] | CONFIG |
| uri attribute | AUID3 & “/global/service-config.xml” | UE Service Configuration document (NOTE 1c) |  |  |
| display-name | Not present |  |  |  |
| entry[1] | NOTE 4, 5 |  | TS 24.484 [14] | GROUPCONFIG |
| uri attribute | “org.openmobilealliance.groups/global/byGroupID/” & px_MCPTT_Group_A_ID | UE Group Configuration document |  |  |
| display-name | Not present |  |  |  |
| entry[1] | NOTE 4, 5 |  | TS 24.484 [14] | GROUPCONFIG_B |
| uri attribute | “org.openmobilealliance.groups/global/byGroupID/” & px_MCPTT_Group_B_ID | UE Group Configuration document |  |  |
| display-name | Not present |  |  |  |
| entry[1] | NOTE 4, 5 |  | TS 24.484 [14] | GROUPCONFIG_C |
| uri attribute | “org.openmobilealliance.groups/global/byGroupID/” & px_MCPTT_Group_C_ID | UE Group Configuration document |  |  |
| display-name | Not present |  |  |  |
| entry[1] | NOTE 4, 5 |  | TS 24.484 [14] | GROUPCONFIG_T |
| uri attribute | “org.openmobilealliance.groups/global/byGroupID/” & px_MCPTT_Group_T_ID | UE Group Configuration document |  |  |
| display-name | Not present |  |  |  |
| entry[2] | optional, NOTE 4, 5 |  | TS 24.481 [11] | GROUPCONFIG |
| uri attribute | Doc-Sel & “~~” & Node-Sel | MCPTT-GKTP document (NOTE 6, 7) |  |  |
| display-name | Not present |  |  |  |
| entry[2] | optional, NOTE 4, 5 |  | TS 24.481 [11] | GROUPCONFIG_B |
| uri attribute | Doc-Sel_B & “~~” & Node-Sel | MCPTT-GKTP document (NOTE 6, 7) |  |  |
| display-name | Not present |  |  |  |
| entry[2] | optional, NOTE 4, 5 |  | TS 24.481 [11] | GROUPCONFIG_C |
| uri attribute | Doc-Sel_C & “~~” & Node-Sel | MCPTT-GKTP document (NOTE 6, 7) |  |  |
| display-name | Not present |  |  |  |
| entry[2] | optional, NOTE 4, 5 |  | TS 24.481 [11] | GROUPCONFIG_T |
| uri attribute | Doc-Sel_T & “~~” & Node-Sel | MCPTT-GKTP document (NOTE 6, 7) |  |  |
| display-name | Not present |  |  |  |
| entry[1] | NOTE 4, 5 |  | TS 24.481 [11] | GROUPKEY |
| uri attribute | Doc-Sel & “~~” & Node-Sel | MCPTT-GKTP document (NOTE 6, 7) |  |  |
| display-name | Not present |  |  |  |
| entry[1] | NOTE 4, 5 |  | TS 24.481 [11] | GROUPKEY_B |
| uri attribute | Doc-Sel_B & “~~” & Node-Sel | MCPTT-GKTP document (NOTE 6, 7) |  |  |
| display-name | Not present |  |  |  |
| entry[1] | NOTE 4, 5 |  | TS 24.481 [11] | GROUPKEY_C |
| uri attribute | Doc-Sel_C & “~~” & Node-Sel | MCPTT-GKTP document (NOTE 6, 7) |  |  |
| display-name | Not present |  |  |  |
| entry[2] | optional, NOTE 4, 5 |  | TS 24.481 [11] | GROUPCONFIG_T |
| uri attribute | Doc-Sel_T & “~~” & Node-Sel | MCPTT-GKTP document (NOTE 6, 7) |  |  |
| display-name | Not present |  |  |  |
| NOTE 1a: AUID1 = “org.3gpp.mcptt.ue-config” NOTE 1b: AUID2 = “org.3gpp.mcptt.user-profile” NOTE 1c: AUID3 = “org.3gpp.mcptt.service-config” NOTE 2: XUID = “sip:” & px_MCPTT_ID_User_A NOTE 3: MCSUEID = Instance id of the UE (derived from the IMEI according to 23.003 [69] clause 13.8) NOTE 4: XML encryption may be done by - element content encryption of the root element <resource-lists> as described in Table 5.5.13.2-1 - element content encryption of (each) <list> element as described in Table 5.5.13.2-1 - attribute URI encryption of the entry’s uri attribute as described in Table 5.5.13.3-1 NOTE 5: When a resource-lists document contains more than one entry, the entries may be in any order NOTE 6: Doc-Sel = “org.3gpp.MCPTT-GKTP/global/byGroupID/” & px_MCPTT_Group_A_ID & “/” Doc-Sel_B=“org.3gpp.MCPTT-GKTP/global/byGroupID/” & px_MCPTT_Group_B_ID & “/” Doc-Sel_C=“org.3gpp.MCPTT-GKTP/global/byGroupID/” & px_MCPTT_Group_C_ID & “/” Doc-Sel_T=“org.3gpp.MCPTT-GKTP/global/byGroupID/” & px_MCPTT_Group_T_ID & “/” NOTE 7: Node-Sel = “/group/list-service/mgktp:GKTPs?xmlns(mgktp=urn:3gpp:ns:mcpttGKTP:1.0)” NOTE 8: TS 23.179 [8] specifies MCPTT ID and MCPTT group ID (clause 8.1.3.1) to be a URIs but does not mandate them to be a SIP URIs; nevertheless according to TS 24.379 [9] (clauses 10.1.1.2.2.1, 10.1.2.2.2.1) the URI in the uri attribute of the resource-lists’ <entry> element needs to be a SIP URI. |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| PRE-ESTABLISH | Call using a pre-established session |
| MSG_RSP | resource lists IE in SIP MESSAGE in response to a received SIP MESSAGE message |
| GROUPCONFIG_B | Message content within subscription to GROUP_B documents |
| GROUPCONFIG_C | Message content within subscription to GROUP_C documents |
| GROUPCONFIG_T | Message content within subscription to temporary GROUP_T documents |
| GROUPKEY_B | Message content within subscription to GROUP_B key material retrieval |
| GROUPKEY_C | Message content within subscription to GROUP_C key material retrieval |
| GROUPKEY_T | Message content within subscription to temporary GROUP_T key material retrieval |
| For further conditions see table 5.5.1-1 |  |

##### -	MCVideo

Table 5.5.3.3.1-2: Resource-lists from the UE for MCVideo

| Derivation Path: RFC 5366 [35] / RFC 4826 [83] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| resource-lists |  |  |  | PRIVATE-CALL GROUP-CALL EMERGENCY-CALL IMMPERIL-CALL |
| list[1] |  |  |  |  |
| name attribute | Not present |  |  |  |
| display-name | Not present |  |  |  |
| entry[1] | NOTE 5 |  |  |  |
| uri attribute | px_MCVideo_ID_User_B | The MCVideo ID of the invited user |  |  |
| display-name | Not present |  |  |  |
| resource-lists | encrypted (NOTE 4) |  | TS 24.481 [11] TS 24.484 [14] | CONFIG OR GROUPCONFIG |
| list[1] | encrypted (NOTE 4) |  |  |  |
| name attribute | "uri: mcvideo-op.gov:resource-lists" | Editor's note: to be removed |  | CONFIG |
| display-name | Not present |  |  |  |
| entry[1] | NOTE 5 |  | TS 24.484 [14] | CONFIG |
| uri attribute | AUID1 & "/users/" & XUID & "/" & MCSUEID & "/" | UE Configuration document (NOTE 1a, 2, 3) |  |  |
|  | "AUID1 & "/users/" & XUID & "/" | Editor's note: It is not clear in the core specs whether both options are allowed or only one of both; if the UE is allowed not to include the MCSUEID, it is not clear where the MC server gets it from |  |  |
| display-name | Not present |  |  |  |
| entry[2] | NOTE 5 |  | TS 24.484 [14] | CONFIG |
| uri attribute | AUID2 & "/users/" & XUID & "/" | UE User Profile document (NOTE 1b, 2) |  |  |
| display-name | Not present |  |  |  |
| entry[3] | NOTE 5 |  | TS 24.484 [14] | CONFIG |
| uri attribute | AUID3 & "/global/service-config.xml" | UE Service Configuration document (NOTE 1c) |  |  |
| display-name | Not present |  |  |  |
| entry[1] | NOTE 5 |  | TS 24.481 [11] | GROUPCONFIG |
| uri attribute | "org.openmobilealliance.groups/global/byGroupID/" & px_MCVideo_Group_A_ID | UE Group Configuration document |  |  |
| display-name | Not present |  |  |  |
| entry[2] | optional NOTE 5 |  | TS 24.481 [11] | GROUPCONFIG |
| uri attribute | Doc-Sel & "~~" & Node-Sel | MCPTT-GKTP document (NOTE 6, 7) |  |  |
| display-name | Not present |  |  |  |
| entry[1] | NOTE 5 |  | TS 24.481 [11] | GROUPKEY |
| uri attribute | Doc-Sel & "~~" & Node-Sel | MCPTT-GKTP document (NOTE 6, 7) |  |  |
| display-name | Not present |  |  |  |
| NOTE 1a: AUID1 = "org.3gpp.mcvideo.ue-config" NOTE 1b: AUID2 = "org.3gpp.mcvideo.user-profile" NOTE 1c: AUID3 = "org.3gpp.mcvideo.service-config" NOTE 2: XUID = "sip:" & px_MCVideo_ID_User_A NOTE 3: MCSUEID = Instance id of the UE (derived from the IMEI according to 23.003 [69] clause 13.8) NOTE 4: XML encryption may be done by - element content encryption of the root element <resource-lists> as described in Table 5.5.13.2-1 - element content encryption of (each) <list> element as described in Table 5.5.13.2-1 - attribute URI encryption of the entry's uri attribute as described in Table 5.5.13.3-1 NOTE 5: When a resource-lists document contains more than one entry, the entries may be in any order NOTE 6: Doc-Sel = “org.3gpp.MCPTT-GKTP/global/byGroupID/” & px_MCVideo_Group_A_ID & "/" NOTE 7: Node-Sel = "/group/list-service/mgktp:GKTPs?xmlns(mgktp=urn:3gpp:ns:mcpttGKTP:1.0)” |  |  |  |  |

##### -	MCData

Table 5.5.3.3.1-3: Resource-lists from the UE for MCData

| Derivation Path: RFC 5366 [35] / RFC 4826 [83] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| resource-lists | Editor's note: XML element content encryption to be added |  |  |  |
| list |  |  |  |  |
| entry | px_MCData_ID_User_B | The MCData ID of the target MCData user |  |  |

##### 5.5.3.3.2	Resource-lists from the SS

##### -	MCPTT

Table 5.5.3.3.2-1: Resource-lists from the SS for MCPTT

| Derivation Path: RFC 5366 [35] / RFC 4826 [83] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| resource-lists | Editor's note: XML element content encryption to be added |  |  |  |
| name attribute | Not present |  |  |  |
| display-name | Not present |  |  |  |
| list |  |  |  |  |
| entry[1] |  |  |  |  |
| uri attribute | px_MCPTT_ID_User_A | The MCPTT ID of the invited user |  |  |
| display-name | Not present |  |  |  |

##### -	MCVideo

Table 5.5.3.3.2-2: Resource-lists from the SS for MCVideo

| Derivation Path: RFC 5366 [35] / RFC 4826 [83] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| resource-lists | Editor's note: XML element content encryption to be added |  |  |  |
| list |  |  |  |  |
| entry | px_MCVideo_ID_User_A | The MCVideo ID of the invited user |  |  |

##### -	MCData

Table 5.5.3.3.2-3: Resource-lists from the SS for MCData

| Derivation Path: RFC 5366 [35] / RFC 4826 [83] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| resource-lists | Editor's note: XML element content encryption to be added |  |  |  |
| list |  |  |  |  |
| entry | px_MCData_ID_User_A | The MCData ID of the invited user |  |  |