3GPP TSG-RAN WG5 Meeting #91-e	R5-212297

# Online, , 17th May 2021 - 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-1 | CR | 0137 | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New generic test procedure for group creation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCPTT-ConTest |  |  |  |  |  | Date: |  |  | 2021-05-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - No generic test procedure for group creation yet |  |  |  |  |  |  |  |  |
| Summary of change: |  | - New generic test procedure for group creation and temporary group creation added References: RFC 2616, TS 23.280, TS 33.180, TS 24.481 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Wrong test sequence results in incorrect MCPTT test case implementation and test case verification not possible |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.26 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 36.579-1 CR 0126, 0134 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 5.3.26	Generic Test Procedure for MCPTT CO Group Creation

5.3.26.1	Initial conditions

As specified in the test case which calls the procedure.

5.3.26.2	Definition of system information messages

The E-UTRA default system information messages as defined in TS 36.508 [6] are used.

5.3.26.3	Procedure

Table 5.3.26.3-1: MCPTT CO Group Creation procedure

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
|  | EXCEPTION: Steps 1a1-1b2 describe behaviour that depends on Group Creation procedure started; the "lower case letter" identifies a step sequence that takes place when one or the other is the case. | - | - | - | - |
| 1a1 | IF a new temporary group shall be created THEN Check: Does the UE (MCPTT Client) send a HTTP POST to the SS to request for creation of a new temporary group? | --> | HTTP POST | - | P |
| 1a2 | The SS (MCPTT Server) sends a HTTP 200 (OK) containing the new group identifier. | <-- | HTTP 200 (OK) | - | - |
| 1b1 | IF a new group shall be created THEN Check: Does the UE (MCPTT Client) send a HTTP PUT to the SS to request for creation of the new group? | --> | HTTP PUT | - | P |
| 1b2 | The SS (MCPTT Server) sends a HTTP 201 (Created) containing the new group identifier. | <-- | HTTP 201 (Created) | - | - |
| - | EXCEPTION: In parallel to procedure of all steps below the behaviour of table 5.3.2.3-2B: Group document subscription and notification procedure and the behaviour of Table 5.3.2.3-2C: Group communication key retrieval procedure takes place | - | - | - | - |
| 2 | The SS sends a SIP NOTIFY to the UE informing that the user profile has changed, containing the XCAP-URI of the MCPTT User Profile Configuration Document. | <-- | SIP NOTIFY | - | - |
| 3 | The UE sends a SIP 200 (OK) message to the SS. | --> | SIP 200 (OK) | - | - |
| 4 | The UE (MCPTT client) sends an HTTP GET Request message to the SS t that contains the access token and the XCAP-URI of the MCPTT User Profile Configuration Document | --> | HTTP GET | - | - |
| 5 | The SS sends the HTTP 200 (OK) message including the MCPTT User Profile Configuration Document. | <-- | HTTP 200 (OK) | - | - |

5.3.26.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

Table 5.3.26.4-1: HTTP POST (Step 1a1, Table 5.3.26.3-1)

Derivation Path: Table 5.5.4.3-1, condition TEMPGROUP

Table 5.3.26.4-2: HTTP 200 (OK) (Step 1a2, Table 5.3.26.3-1)

| Derivation Path: Table 5.5.4.6-1, condition GROUPCONFIG |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| ue-group-configuration | As described in Table 5.5.7.1-3 |  |  |  |

Table 5.3.26.4-3: HTTP PUT (Step 1b1, Table 5.3.26.3-1)

| Derivation Path: Table 5.5.4.4-1, condition GROUPCONFIG |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Request-Line |  |  |  |  |
| Request-URI |  |  |  |  |
| uri | "/" & tsc_MCX_GMSXCAPRootURI & "/" & "org.openmobilealliance.groups/global/byGroupID/" & px_MCPTT_Group_C_ID |  |  | GROUP_C |
| Message-body |  |  |  |  |  |  |  |  |  |
| group |  | Group Creation Document as described in Table 5.3.26.4-4 |  |  |  |  |  | GROUP_C |  |

Table 5.3.26.4-4: Group Creation Document in HTTP PUT (Table 5.3.26.4-3)

| Derivation Path: Table 5.5.7.1-2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| group |  |  |  |  |
| list-service |  |  |  |  |
| uri-attribute | px_MCPTT_Group_C_ID | uri of the MCPTT group |  |  |
| display-name | px_MCPTT_Group_C_name | group display name |  |  |

Table 5.3.26.4-5: HTTP 201 (Created) Step 1b2 (Table 5.3.26.3-1)

| Derivation Path: Table 5.5.4.7-1, condition GROUPCONFIG |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Location |  |  | RFC 7231 [118] clauses 4.3.3, 6.3.2, 7.1.2 |  |
| Location-URI |  |  |  |  |
| uri | "/" & tsc_MCX_GMSXCAPRootURI & "/" & "org.openmobilealliance.groups/global/byGroupID/" & px_MCPTT_Group_B_ID | URI referring to the created MCPTT GROUP B document |  | GROUP_B |
|  | "/" & tsc_MCX_GMSXCAPRootURI & "/" & "org.openmobilealliance.groups/global/byGroupID/" & px_MCPTT_Group_C_ID | URI referring to the created MCPTT GROUP C document |  | GROUP_C |
| Message-body |  |  |  |  |
| ue-group-configuration | As described in Table5.3.26.4-6 with condition GROUP_B |  |  | GROUP_B |
| ue-group-configuration | As described in Table5.3.26.4-6 with condition GROUP_C |  |  | GROUP_C |

Table 5.3.26.4-6: UE group configuration in HTTP 201 (created) (Table 5.3.26.4-5)

| Derivation Path: Table 5.5.7.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| list-service[1] |  |  |  |  |  |  |  | GROUP_B |  |
| uri attribute |  | px_MCPTT_Group_B_ID |  |  |  |  |  |  |  |
| display-name |  | px_MCPTT_Group_B_Name |  |  |  |  |  |  |  |
| list-service[1] |  |  |  | GROUP_C |
| uri attribute | px_MCPTT_Group_C_ID |  |  |  |
| display-name | px_MCPTT_Group_C_Name |  |  |  |

Table 5.3.26.4-7: SIP NOTIFY (Step 2, Table 5.3.26.3-1)

| Derivation Path: Table 5.5.2.8-1 condition CONFIG |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |  |
| MIME body part |  |  | xcap-diff |  |  |
| MIME-part-body |  | Xcap-diff as described in Table 5.3.26.4-8 |  |  |  |

Table 5.3.26.4-8: Xcap-diff document in SIP NOTIFY (Table 5.3.26.4-7)

| Derivation Path: RFC 5874 [107] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| xcap-diff |  |  |  |  |
| document |  |  |  |  |
| sel | org.3gpp.mcptt.user-profile & "/users/" & "sip:" & px_MCPTT_ID_User_A & "/mcptt-user-profile-" & 0 & ".xml" | Editor’s note: to be clarified whether only the path to the additional attribute shall be sent |  |  |
| new-etag | same value as used in notification procedure for user profile change before - appendage increased by 1 |  |  |  |
| previous-etag | same as used in notification procedure for user profile change before |  |  |  |

Table 5.3.26.4-9: HTTP GET (Step 4, Table 5.3.26.3-1)

Derivation Path: Table 5.5.4.2-1, condition UEUSERPROF.

Table 5.3.26.4-10: HTTP 200 (OK) (Step 5, Table 5.3.26.3-1)

| Derivation Path: Table 5.5.4.6-1, condition UEUSERPROF |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| mcptt-user-profile | User Profile Document described in Table 5.3.26.4-11 | User Profile Document returned |  |  |
| mcptt-user-profile | User Profile Document described in Table 5.3.26.4-12 | User Profile Document returned |  | GROUP_T |

Table 5.3.26.4-11: User Profile Document in HTTP 200 (OK) (Table 5.3.26.4-10)

| Derivation Path: Table 5.5.8.3-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcptt-user-profile |  |  |  |  |
| OnNetwork |  |  |  |  |
| index attribute | "0" |  |  |  |
| MCPTTGroupInfo |  |  |  |  |
| entry[2] |  | Group 2 the MCPTT user is allowed to affiliate to |  |  |
| index attribute | "1" |  |  |  |
| uri-entry | px_MCPTT_Group_B_ID | The MCPTT group ID for the on-network MCPTT group that the MCPTT user is allowed to affiliate to. |  | GROUP_B |
| display-name | px_MCPTT_Group_B_Name | The display name for the group |  |  |
| entry[3] |  | Group 3 the MCPTT user is allowed to affiliate to |  | GROUP_C |
| index attribute | "2" |  |  |  |
| uri-entry | px_MCPTT_Group_C_ID | The MCPTT group ID for the on-network MCPTT group that the MCPTT user is allowed to affiliate to. |  |  |
| display-name | px_MCPTT_Group_C_Name | The display name for the group |  |  |

Table 5.3.26.4-12: User Profile Document in HTTP 200 (OK) (Table 5.3.26.4-10)

| Derivation Path: Table 5.5.8.3-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcptt-user-profile |  |  |  |  |
| OnNetwork |  |  |  |  |
| index attribute | "0" |  |  |  |
| MCPTTGroupInfo |  |  |  |  |
| entry[2] |  | Group 2 the MCPTT user is allowed to affiliate to |  |  |
| index attribute | "1" |  |  |  |
| uri-entry | px_MCPTT_Group_T_ID | The MCPTT group ID for the on-network MCPTT group that the MCPTT user is allowed to affiliate to. |  |  |
| display-name | px_MCPTT_Group_T_Name | The display name for the group |  |  |

Table 5.3.26.4-13: SIP SUBSCRIBE (Step 1, Table 5.3.2.3-2B)

| Derivation Path: Table 5.5.2.14-1, condition GROUPCONFIG |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | Resource-lists |  |  |
| MIME-part-body | Resource-lists as described in Table 5.5.3.3.1-1 with condition GROUPCONFIG_B |  |  | GROUP_B |
| MIME-part-body | Resource-lists as described in Table 5.5.3.3.1-1 with condition GROUPCONFIG_C |  |  | GROUP_C |
| MIME-part-body | Resource-lists as described in Table 5.5.3.3.1-1 with condition GROUPCONFIG_T |  |  | GROUP_T |

Table 5.3.26.4-14: SIP NOTIFY (Step 3, Table 5.3.2.3-2B)

| Derivation Path: Table 5.5.2.8-1, condition GROUPCONFIG |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  |  |  |  |
| MIME part body | xcap-diff document as described in Table 5.3.26.4-15 with condition GROUP_B |  |  | GROUP_B |
| MIME part body | xcap-diff document as described in Table 5.3.26.4-15 with condition GROUP_C |  |  | GROUP_C |
| MIME part body | xcap-diff document as described in Table 5.3.26.4-15 with condition GROUP_T |  |  | GROUP_T |

Table 5.3.26.4-15: Xcap-diff document in SIP NOTIFY (Table 5.3.26.4-14)

| Derivation Path: Table 5.5.3.12-2, condition GROUPCONFIG |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| xcap-diff |  |  |  |  |
| document[1] |  |  |  |  |
| sel | "org.openmobilealliance.groups/global/byGroupID/" & px_MCPTT_Group_B_ID |  |  | GROUP_B |
| sel | "org.openmobilealliance.groups/global/byGroupID/" & px_MCPTT_Group_C_ID |  |  | GROUP_C |
| sel | "org.openmobilealliance.groups/global/byGroupID/" & px_MCPTT_Group_T_ID |  |  | GROUP_T |

Table 5.3.26.4-16: HTTP GET (Step 5, Table 5.3.2.3-2B)

| Derivation Path: Table 5.5.4.2-1, condition GROUPCONFIG |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Request-Line |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Request-URI |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| uri |  |  |  | "/" & tsc_MCX_GMSXCAPRootURI & "/" & "org.openmobilealliance.groups/global/byGroupID/" & px_MCPTT_Group_B_ID |  |  | points to group configuration document |  |  |  |  |  | GROUP_B |  |  |
| uri |  |  |  | "/" & tsc_MCX_GMSXCAPRootURI & "/" & "org.openmobilealliance.groups/global/byGroupID/" & px_MCPTT_Group_C_ID |  |  | points to group configuration document |  |  |  |  |  | GROUP_C |  |  |
| uri | "/" & tsc_MCX_GMSXCAPRootURI & "/" & "org.openmobilealliance.groups/global/byGroupID/" & px_MCPTT_Group_T_ID | points to group configuration document |  | GROUP_T |

Table 5.3.26.4-17: HTTP 200 (OK) (Step 6, Table 5.3.2.3-2B)

| Derivation Path: Table 5.5.4.6-1, condition GROUPCONFIG. |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Message-body |  |  |  |  |  |  |  |  |  |  |  |
| ue-group-configuration |  |  |  | As described in Table5.3.26.4-18 with condition GROUP_B |  |  |  |  |  | GROUP_B |  |
| ue-group-configuration |  |  | As described in Table5.3.26.4-18 with condition GROUP_C |  |  |  |  |  | GROUP_C |  |  |
| ue-group-configuration | As described in Table5.3.26.4-18 with condition GROUP_T |  |  | GROUP_T |

Table 5.3.26.4-18: UE group configuration in HTTP 200 (OK) (Table 5.3.26.4-17)

| Derivation Path: Table 5.5.7.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| list-service[1] |  |  |  | GROUP_B |
| uri attribute | px_MCPTT_Group_B_ID |  |  |  |
| display-name | px_MCPTT_Group_B_Name |  |  |  |
| list-service[1] |  |  |  | GROUP_C |
| uri attribute | px_MCPTT_Group_C_ID |  |  |  |
| display-name | px_MCPTT_Group_C_Name |  |  |  |
| list-service[1] |  |  |  | GROUP_T |
| uri attribute | px_MCPTT_Group_T_ID |  |  |  |
| display-name | px_MCPTT_Group_T_Name |  |  |  |

Table 5.3.26.4-19: SIP SUBSCRIBE (Step 1, Table 5.3.2.3-2C)

| Derivation Path: Table 5.5.2.14-1, condition GROUPCONFIG |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Reference |  | Condition |  |
| Message-body |  |  |  |  |  |  |  |  |  |
| MIME body part |  |  |  | Resource-lists |  |  |  |  |  |
| MIME-part-body |  | Resource-lists as described in Table 5.5.3.3.1-1 with condition GROUPKEY_B |  |  |  |  |  | GROUP_B |  |
| MIME-part-body | Resource-lists as described in Table 5.5.3.3.1-1 with condition GROUPKEY_C |  |  | GROUP_C |
| MIME-part-body | Resource-lists as described in Table 5.5.3.3.1-1 with condition GROUPKEY_T |  |  | GROUP_T |

Table 5.3.26.4-20: SIP NOTIFY (Step 7a1, Table 5.3.2.3-2B and Step 2a3, Table 5.3.2.3-2C)

| Derivation Path: Table 5.5.2.14-1, condition GROUPCONFIG |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Reference |  | Condition |  |
| Message-body |  |  |  |  |
| xcap-diff document | xcap-diff document as described in Table 5.3.26.4-21 with condition GROUP_B |  |  | GROUP_B |
| xcap-diff document | xcap-diff document as described in Table 5.3.26.4-21 with condition GROUP_C |  |  | GROUP_C |
| xcap-diff document | xcap-diff document as described in Table 5.3.26.4-21 with condition GROUP_T |  |  | GROUP_T |

Table 5.3.26.4-21: Xcap-Diff Document in SIP NOTIFY (Table 5.3.26.4-20)

| Derivation Path: Table 5.5.3.12-2, condition GROUPKEY |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element |  |  | Value/remark |  |  | Comment |  |  | Reference |  |  | Condition |  |  |
| xcap-diff |  |  |  |  |
| element[1] |  |  | group key transport payloads (GKTP) document as described in Table 5.3.26.4-22 with condition GROUP_B |  |  |  |  |  |  |  |  | GROUP_B |  |  |
| element[1] |  |  | group key transport payloads (GKTP) document as described in Table 5.3.26.4-22 with condition GROUP_C |  |  |  |  |  |  |  |  | GROUP_C |  |  |
| element[1] | group key transport payloads (GKTP) document as described in Table 5.3.26.4-22 with condition GROUP_T |  |  | GROUP_T |

Table 5.3.26.4-22: group key transport payloads (GKTP) document in xcap-diff document (Table 5.3.26.4-21)

| Derivation Path: Table 5.5.3.14-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| GKTPs |  |  |  |  |
| GMK-GKTPs |  |  |  |  |
| GKTP[1] | MIKEY message as described in Table 5.3.26.4-23 with condition GROUP_B | MIKEY message, containing the GMK |  | GROUP_B |
| GKTP[1] | MIKEY message as described in Table 5.3.26.4-23 with condition GROUP_C | MIKEY message, containing the GMK |  | GROUP_C |
| GKTP[1] | MIKEY message as described in Table 5.3.26.4-24 | MIKEY message, containing the GMK |  | GROUP_T |

Table 5.3.26.4-23: MIKEY-SAKKE I_MESSAGE in group key transport payloads (Table 5.3.26.4-22)

| Derivation Path: Table 5.5.3.14-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| General Extension Payload { |  |  |  |  |
| Data { |  |  |  |  |
| Group IDs { |  |  |  |  |
| Number of Group IDs | '1' |  |  |  |
| Group ID | px_MCPTT_Group_B_ID | The ID for the group associated with the key. |  | GROUP_B |
| Group ID | px_MCPTT_Group_C_ID | The ID for the group associated with the key. |  | GROUP_C |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 5.3.26.4-24: group key transport payloads (GKTP) document (Table 5.3.26.4-22)

| Derivation Path: TS 24.481 [11] clause 7.7 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| GKTPs |  |  |  |  |
| GMK-GKTPs |  |  |  |  |
| GKTP[1] | MIKEY message as used in group communication key retrieval procedure | MIKEY message containing the GMK for Group A |  |  |
| id attribute | Same value as used in group communication key retrieval procedure |  |  |  |
| on-network-regrouped-GKTPs |  |  |  |  |
| temporary-MCPTT-group-ID attribute | px_MCPTT_Group_T_ID |  |  |  |
| GKTP[1] | MIKEY message as described in Table 5.3.26.4-25 | MIKEY message containing the GMK for Group T |  |  |
| id attribute | arbitrary value | unique charstring assigned by the SS |  |  |

Table 5.3.26.4-25: MIKEY-SAKKE I_MESSAGE (GMK distribution by the SS) (Table 5.3.26.4-24)

| Derivation Path: Table 5.5.9.1-3 |  |  |  |
| --- | --- | --- | --- |
| Field | Value/remark | Comment | Condition |
| General Extension Payload { |  |  |  |
| Data { |  |  |  |
| Group IDs { |  |  |  |
| Number of Group IDs | '1' |  |  |
| Group ID | px_MCPTT_Group_T_ID | The ID for the group associated with the key. |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition |  | Explanation |  |
| --- | --- | --- | --- |
| GROUP_B |  | Message content for creation of GROUP_B |  |
| GROUP_C |  | Message content for creation of GROUP_C |  |
| GROUP_C | Message content for creation of temporary GROUP_T |
| NOTE: For further conditions see table 5.5.1-1 |  |  |  |