3GPP TSG-RAN5 Meeting #91-e	R5-212345

# Electronic Meeting, 17th – 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-3 | CR | 0944 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Define MU and TT for test case 7.9B.3_1.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-05-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | MU and TT for test case 7.9B.3_1.1 are not defined in annex F. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define MU and TT for test case 7.9B.3_1.1 using SA test case 7.9A.1 as reference. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test specification will remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | F.1.3, F3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES >>>

## F.1.3	Measurement of receiver

Table F.1.3-1: Maximum Test System Uncertainty for receiver tests

| Clause |  | Maximum Test System Uncertainty |  | Derivation of Test System Uncertainty |  |
| --- | --- | --- | --- | --- | --- |
| 7.3B.2.1 Reference sensitivity for Intra-band Contiguous EN-DC (2 CCs) |  | Same as clause 7.3.2 in TS 38.521-1 [8] |  |  |  |
| 7.3B.2.2 Reference sensitivity for Intra-band non-contiguous EN-DC (2 CCs) |  | Same as clause 7.3.2 in TS 38.521-1 [8] |  |  |  |
| 7.3B.2.3 Reference sensitivity for Inter-band EN-DC within FR1 (2 CCs) |  | Same as clause 7.3.2 in TS 38.521-1 [8] |  |  |  |
| 7.3B.2.3_1.1 Reference sensitivity for Inter-band EN-DC within FR1 (3 CCs) |  | Same as clause 7.3A.1 in TS 38.521-1 [8] |  |  |  |
| 7.3B.2.4 Reference sensitivity for Inter-band EN-DC including FR2 |  | Same as clause 7.3.2 in TS 38.521-2 [9] |  |  |  |
| 7.3B.2.4_1.1 Reference sensitivity for Inter-band EN-DC including FR2 (3 CCs) |  | Same as clause 7.3A.2.1 in TS 38.521-2 [9] |  |  |  |
| 7.3B.2.4_1.2 Reference sensitivity for Inter-band EN-DC including FR2 (4 CCs) |  | Same as clause 7.3A.2.2 in TS 38.521-2 [9] |  |  |  |
| 7.3B.2.4_1.3 Reference sensitivity for Inter-band EN-DC including FR2 (5 CCs) |  | Same as clause 7.3A.2.3 in TS 38.521-2 [9] |  |  |  |
| 7.3B.2.4_1.4 Reference sensitivity for Inter-band EN-DC including FR2 (6 CCs) |  | Same as clause 7.3A.2.4 in TS 38.521-2 [9] |  |  |  |
| 7.3B.4 EIS Spherical Coverage for Inter-band EN-DC including FR2 |  | Same as clause 7.3.4 in TS 38.521-2 [9] |  |  |  |
| 7.4B.1 Maximum Input Level for Intra-Band Contiguous EN-DC (2 CCs) |  | MU for NR CC downlink power same as clause 7.4 in TS 38.521-1 [8]. Uplink power measurement same as 6.2B.1.1. |  |  |  |
| 7.4B.2 Maximum Input Level for Intra-Band Non-Contiguous EN-DC (2 CCs) |  | Same as clause 7.4 in TS 38.521-1 [8] Uplink power measurement same as 6.2B.1.2. |  |  |  |
| 7.4B.3 Maximum Input Level for Inter-band EN-DC within FR1 (2 CCs) |  | Same as clause 7.4 in TS 38.521-1 [8] Uplink power measurement same as 6.2B.1.3. |  |  |  |
| 7.4B.3_1.1 Maximum Input Level for Inter-band EN-DC within FR1 (3 CCs) |  | Same as clause 7.4A.1 in TS 38.521-1 [8] Uplink power measurement same as 6.2B.1.3. |  |  |  |
| 7.5B.1 Adjacent Channel Selectivity for intra-band contiguous EN-DC (2 CCs) |  | TBD Uplink power measurement same as 6.2B.1.1. |  |  |  |
| 7.5B.2 Adjacent Channel Selectivity for intra-band non-contiguous EN-DC (2 CCs) |  | TBD Uplink power measurement same as 6.2B.1.2. |  |  |  |
| 7.5B.3 Adjacent Channel Selectivity for inter-band EN-DC within FR1 (2 CCs) |  | Same as clause 7.5 in TS 38.521-1 [8] Uplink power measurement same as 6.2B.1.3. |  |  |  |
| 7.5B.3_1.1 Adjacent Channel Selectivity for EN-DC within FR1 (3 CCs) |  | Same as clause 7.5 in TS 38.521-1 [8] Uplink power measurement same as 6.2B.1.3. |  |  |  |
| 7.5B.3_1.2 Adjacent Channel Selectivity for EN-DC within FR1 (4 CCs) |  | Same as clause 7.5 in TS 38.521-1 [8] Uplink power measurement same as 6.2B.1.3. |  | The wanted signal level uncertainty applies for each CC |  |
| 7.5B.4 Adjacent Channel Selectivity for inter-band EN-DC including FR2 (2CCs) |  | Same as clause 7.5 in TS 38.521-2 [9] |  |  |  |
| 7.5B.4_1.1 Adjacent Channel Selectivity for inter-band EN-DC including FR2 (3 CCs) |  | Same as clause 7.5A in TS 38.521-2 [9] |  |  |  |
| 7.5B.4_1.2 Adjacent Channel Selectivity for inter-band EN-DC including FR2 (4 CCs) |  | Same as clause 7.5A in TS 38.521-2 [9] |  |  |  |
| 7.5B.4_1.3 Adjacent Channel Selectivity for inter-band EN-DC including FR2 (5 CCs) |  | Same as clause 7.5A in TS 38.521-2 [9] |  |  |  |
| 7.5B.4_1.4 Adjacent Channel Selectivity for inter-band EN-DC including FR2 (6 CCs) |  | Same as clause 7.5A in TS 38.521-2 [9] |  |  |  |
| 7.6B.2.1 Inband blocking for intra-band contiguous EN-DC in FR1 (2 CCs) |  | MU for NR CC downlink power same as clause 7.6.2 in TS 38.521-1 [8]. Uplink power measurement same as 6.2B.1.1. |  |  |  |
| 7.6B.2.2 Inband blocking for intra-band non-contiguous EN-DC in FR1 (2 CCs) |  | Same as clause 7.6.2 in TS 38.521-1 [8] Uplink power measurement same as 6.2B.1.2. |  |  |  |
| 7.6B.2.3 Inband blocking for inter-band EN-DC within FR1 (2 CCs) |  | Same as clause 7.6.2 in TS 38.521-1 [8] Uplink power measurement same as 6.2B.1.3. |  |  |  |
| 7.6B.2.3_1.1 Inband blocking for EN-DC within FR1 (3 CCs) |  | Same as clause 7.6A.2.1 in TS 38.521-1 [8] Uplink power measurement same as 6.2B.1.3. |  |  |  |
| 7.6B.2.3_1.2 Inband blocking for EN-DC within FR1 (4 CCs) |  | Same as clause 7.6.2 in TS 38.521-1 [8] for each component carrier. Uplink power measurement same as 6.2B.1.3. |  |  |  |
| 7.6B.2.3_1.3 Inband blocking for EN-DC within FR1 (5 CCs) |  | Same as clause 7.6.2 in TS 38.521-1 [8] for each component carrier. Uplink power measurement same as 6.2B.1.3. |  |  |  |
| 7.6B.2.4 Inband blocking for inter-band EN-DC including FR2 (2 CCs) |  | Same as 7.6.2 in TS 38.521-2 [9] |  |  |  |
| 7.6B.2.4_1.1 Inband blocking for inter-band EN-DC including FR2 (3 CCs) |  | TBD |  |  |  |
| 7.6B.2.4_1.2 Inband blocking for inter-band EN-DC including FR2 (4 CCs) |  | TBD |  |  |  |
| 7.6B.2.4_1.3 Inband blocking for inter-band EN-DC including FR2 (5 CCs) |  | TBD |  |  |  |
| 7.6B.2.4_1.4 Inband blocking for inter-band EN-DC including FR2 (6 CCs) |  | TBD |  |  |  |
| 7.6B.3.1 Out-of-band blocking for intra-band contiguous EN-DC in FR1 (2 CCs) |  | MU for NR CC downlink power same as clause 7.6.3 in TS 38.521-1 [8]. Uplink power measurement same as 6.2B.1.1. |  |  |  |
| 7.6B.3.2 Out-of-band blocking for intra-band non-contiguous EN-DC in FR1 (2 CCs) |  | Same as clause 7.6.3 in TS 38.521-1 [8] Uplink power measurement same as 6.2B.1.2. |  |  |  |
| 7.6B.3.3 Out-of-band blocking for inter-band EN-DC within FR1 (2 CCs) |  | Same as clause 7.6.3 in TS 38.521-1 [8]. Uplink power measurement same as 6.2B.1.3. |  |  |  |
| 7.6B.4.1 Narrow band blocking for intra-band contiguous EN-DC in FR1 (2 CCs) |  | MU for NR CC downlink power same as clause 7.6.4 in TS 38.521-1 [8]. Uplink power measurement same as 6.2B.1.1. |  |  |  |
| 7.6B.4.2 Narrow band blocking for intra-band non-contiguous EN-DC in FR1 (2 CCs) |  | Same as clause 7.6.4 in TS 38.521-1 [8] Uplink power measurement same as 6.2B.1.2. |  |  |  |
| 7.6B.4.3 Narrow band blocking for inter-band EN-DC within FR1 (2 CCs) |  | Same as clause 7.6.4 in TS 38.521-1 [8] Uplink power measurement same as 6.2B.1.3. |  |  |  |
| 7.6B.4.3_1.1 Narrow band blocking for EN-DC within FR1 (3 CCs) |  | Same as clause 7.6A.4.1 in TS 38.521-1 [8] Uplink power measurement same as 6.2B.1.3. |  |  |  |
| 7.6B.4.3_1.2 Narrow band blocking for EN-DC within FR1 (4 CCs) |  | Same as clause 7.6.4 in TS 38.521-1 [8] for each component carrier. Uplink power measurement same as 6.2B.1.3. |  |  |  |
| 7.6B.4.3_1.3 Narrow band blocking for EN-DC within FR1 (5 CCs) |  | Same as clause 7.6.4 in TS 38.521-1 [8] for each component carrier. Uplink power measurement same as 6.2B.1.3. |  |  |  |
| 7.7B.1 Spurious Response for intra-band contiguous EN-DC in FR1 (2 CCs) |  | MU for NR CC downlink power same as clause 7.7 in TS 38.521-1 [8]. Uplink power measurement same as 6.2B.1.1. |  |  |  |
| 7.7B.2 Spurious Response for intra-band non-contiguous EN-DC in FR1 (2 CCs) |  | Same as clause 7.7 in TS 38.521-1 [8] Uplink power measurement same as 6.2B.1.2. |  |  |  |
| 7.7B.3 Spurious Response for inter-band EN-DC within FR1 (2 CCs) |  | Same as clause 7.7 in TS 38.521-1 [8]. Uplink power measurement same as 6.2B.1.3. |  |  |  |
| 7.7B.3_1.1 Spurious Response for EN-DC within FR1 (3 CCs) |  | Same as clause 7.7 in TS 38.521-1 [8] for each component carrier. Uplink power measurement same as 6.2B.1.2. |  |  |  |
| 7.7B.3_1.2 Spurious Response for EN-DC within FR1 (4 CCs) |  | Same as clause 7.7 in TS 38.521-1 [8] for each component carrier. Uplink power measurement same as 6.2B.1.2. |  |  |  |
| 7.8B.2.1 Wideband Intermodulation for intra-band contiguous EN-DC in FR1 (2 CCs) |  | MU for NR CC downlink power same as clause 7.8.2 in TS 38.521-1 [8]. Uplink power measurement same as 6.2B.1.1. |  |  |  |
| 7.8B.2.2 Wideband Intermodulation for intra-band non-contiguous EN-DC in FR1 (2 CCs) |  | Same as clause 7.8.2 in TS 38.521-1 [8] Uplink power measurement for same as 6.2B.1.2. |  |  |  |
| 7.8B.2.3 Wideband Intermodulation for inter-band EN-DC within FR1 (2 CCs) |  | Same as clause 7.8.2 in TS 38.521-1 [8] Uplink power measurement for same as 6.2B.1.3. |  |  |  |
| 7.8B.2.6 Wideband Intermodulation for EN-DC within FR1 (3 CCs) (2 CCs) |  | Same as clause 7.8A.2.1 in TS 38.521-1 [8] Uplink power measurement for same as 6.2B.1.3. |  |  |  |
| 7.9B.1 Spurious Emissions for intra-band contiguous EN-DC within FR1 (2 CCs) |  | Same as clause 7.9 in TS 38.521-1 [8] |  |  |  |
| 7.9B.2 Spurious Emissions for intra-band non-contiguous EN-DC within FR1 (2 CCs) |  | Same as clause 7.9 in TS 38.521-1 [8] |  |  |  |
| 7.9B.3 Spurious Emissions for inter-band EN-DC within FR1 (2 CCs) |  | Same as clause 7.9 in TS 38.521-1 [8] |  |  |  |
| 7.9B.3_1.1 Spurious Emissions for EN-DC within FR1 (3 CCs) |  | Same as clause 7.9A.1 in TS 38.521-1 [8] |  |  |  |
| 7.9B.4 Spurious Emissions for inter-band EN-DC including FR2 (2 CCs) |  | Same as clause 7.9 in TS 38.521-2 [9] |  |  |  |

## <<< Skip unchanged sections >>>

## F.3.3	Measurement of receiver

Table F.3.3-1: Derivation of Test Requirements (Receiver tests)

| Sub clause |  | Test Tolerance (TT) |  | Formula for test requirement |  |
| --- | --- | --- | --- | --- | --- |
| 7.3B.2.1 Reference sensitivity for Intra-band Contiguous EN-DC (2 CCs) |  | Same as 7.3.2 in TS 38.521-1 [8] |  |  |  |
| 7.3B.2.2 Reference sensitivity for Intra-band non-contiguous EN-DC (2 CCs) |  | Same as 7.3.2 in TS 38.521-1 [8] |  |  |  |
| 7.3B.2.3 Reference sensitivity for Inter-band EN-DC within FR1 (2 CCs) |  | Same as 7.3.2 in TS 38.521-1 [8] |  |  |  |
| 7.3B.2.3_1.1 Reference sensitivity for Inter-band EN-DC within FR1 (3 CCs) |  | Same as 7.3A.1 in TS 38.521-1 [8] |  |  |  |
| 7.3B.2.4 Reference sensitivity for Inter-band EN-DC including FR2 |  | Same as 7.3.2 in TS 38.521-2 [9] |  |  |  |
| 7.3B.2.4_1.1 Reference sensitivity for Inter-band EN-DC including FR2 (3 CCs) |  | Same as 7.3A.2.1 in TS 38.521-2 [9] |  |  |  |
| 7.3B.2.4_1.2 Reference sensitivity for Inter-band EN-DC including FR2 (4 CCs) |  | Same as 7.3A.2.2 in TS 38.521-2 [9] |  |  |  |
| 7.3B.2.4_1.3 Reference sensitivity for Inter-band EN-DC including FR2 (5 CCs) |  | Same as 7.3A.2.3 in TS 38.521-2 [9] |  |  |  |
| 7.3B.2.4_1.4 Reference sensitivity for Inter-band EN-DC including FR2 (6 CCs) |  | Same as 7.3A.2.4 in TS 38.521-2 [9] |  |  |  |
| 7.3B.4 EIS Spherical Coverage for Inter-band EN-DC including FR2 |  | Same as 7.3.4 in TS 38.521-2 [9] |  |  |  |
| 7.4B.1 Maximum Input Level for Intra-Band Contiguous EN-DC (2 CCs) |  | Same as 7.4 in TS 38.521-1 [8] |  |  |  |
| 7.4B.2 Maximum Input Level for Intra-Band Non-Contiguous EN-DC (2 CCs) |  | Same as 7.4 in TS 38.521-1 [8] |  |  |  |
| 7.4B.3 Maximum Input Level for Inter-band EN-DC within FR1 (2 CCs) |  | Same as 7.4 in TS 38.521-1 [8] |  |  |  |
| 7.4B.3_1.1 Maximum Input Level for Inter-band EN-DC within FR1 (3 CCs) |  | Same as 7.4A.1 in TS 38.521-1 [8] |  |  |  |
| 7.5B.1 Adjacent Channel Selectivity for intra-band contiguous EN-DC (2 CCs) |  | Same as 7.5 in TS 38.521-1 [8] |  |  |  |
| 7.5B.2 Adjacent Channel Selectivity for intra-band non-contiguous EN-DC (2 CCs) |  | Same as 7.5 in TS 38.521-1 [8] |  |  |  |
| 7.5B.3 Adjacent Channel Selectivity for inter-band EN-DC within FR1 (2 CCs) |  | Same as 7.5 in TS 38.521-1 [8] |  |  |  |
| 7.5B.3_1.1 Adjacent Channel Selectivity for EN-DC within FR1 (3 CCs) |  | Same as 7.5A.1 in TS 38.521-1 [8] |  |  |  |
| 7.5B.3_1.2 Adjacent Channel Selectivity for EN-DC within FR1 (4 CCs) |  | Same as 7.5 in TS 38.521-1 [8] |  |  |  |
| 7.5B.4 Adjacent Channel Selectivity for inter-band EN-DC including FR2 (2CCs) |  | Same as clause 7.5 in TS 38.521-2 [9] |  |  |  |
| 7.6B.2.1 Inband blocking for intra-band contiguous EN-DC in FR1 (2 CCs) |  | Same as 7.6.2 in TS 38.521-1 [8] |  |  |  |
| 7.6B.2.2 Inband blocking for intra-band non-contiguous EN-DC in FR1 (2 CCs) |  | Same as 7.6.2 in TS 38.521-1 [8] |  |  |  |
| 7.6B.2.3 Inband blocking for inter-band EN-DC within FR1 (2 CCs) |  | Same as 7.6.2 in TS 38.521-1 [8] |  |  |  |
| 7.6B.2.3_1.1 Inband blocking for EN-DC within FR1 (3 CCs) |  | 0 dB |  | Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged |  |
| 7.6B.2.3_1.2 Inband blocking for EN-DC within FR1 (4 CCs) |  | 0 dB |  | Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged |  |
| 7.6B.2.3_1.3 Inband blocking for EN-DC within FR1 (5 CCs) |  | 0 dB |  | Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged |  |
| 7.6B.2.4 Inband blocking for inter-band EN-DC including FR2 (2 CCs) |  | Same as 7.6.2 in TS 38.521-2 [9] |  |  |  |
| 7.6B.2.4_1.1 Inband blocking for inter-band EN-DC including FR2 (3 CCs) |  | 0 dB |  | Wanted signal power + TT T-put limit unchanged |  |
| 7.6B.2.4_1.2 Inband blocking for inter-band EN-DC including FR2 (4 CCs) |  | 0 dB |  | Wanted signal power + TT T-put limit unchanged |  |
| 7.6B.2.4_1.3 Inband blocking for inter-band EN-DC including FR2 (5 CCs) |  | 0 dB |  | Wanted signal power + TT T-put limit unchanged |  |
| 7.6B.2.4_1.4 Inband blocking for inter-band EN-DC including FR2 (6 CCs) |  | 0 dB |  | Wanted signal power + TT T-put limit unchanged |  |
| 7.6B.3.1 Out-of-band blocking for intra-band contiguous EN-DC in FR1 (2 CCs) |  | Same as 7.6.3 in TS 38.521-1 [8] |  |  |  |
| 7.6B.3.2 Out-of-band blocking for intra-band non-contiguous EN-DC in FR1 (2 CCs) |  | Same as 7.6.3 in TS 38.521-1 [8] |  |  |  |
| 7.6B.3.3 Out-of-band blocking for inter-band EN-DC within FR1 (2 CCs) |  | Same as 7.6.3 in TS 38.521-1 [8] |  |  |  |
| 7.6B.3.3_1.1 Out-of-band blocking for EN-DC within FR1 (3 CCs) |  | 0 dB |  | Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged |  |
| 7.6B.3.3_1.2 Out-of-band blocking for EN-DC within FR1 (4 CCs) |  | 0 dB |  | Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged |  |
| 7.6B.4.1 Narrow band blocking for intra-band contiguous EN-DC in FR1 (2 CCs) |  | Same as 7.6.4 in TS 38.521-1 [8] |  |  |  |
| 7.6B.4.2 Narrow band blocking for intra-band non-contiguous EN-DC in FR1 (2 CCs) |  | Same as 7.6.4 in TS 38.521-1 [8] |  |  |  |
| 7.6B.4.3 Narrow band blocking for inter-band EN-DC within FR1 (2 CCs) |  | Same as 7.6.4 in TS 38.521-1 [8] |  |  |  |
| 7.6B.4.3_1.1 Narrow band blocking for EN-DC within FR1 (3 CCs) |  | 0 dB |  | Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged |  |
| 7.6B.4.3_1.2 Narrow band blocking for EN-DC within FR1 (4 CCs) |  | 0 dB |  | Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged |  |
| 7.6B.4.3_1.3 Narrow band blocking for EN-DC within FR1 (5 CCs) |  | 0 dB |  | Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged |  |
| 7.7B.1 Spurious Response for intra-band contiguous EN-DC in FR1 (2 CCs) |  | Same as 7.7 in TS 38.521-1 [8] |  |  |  |
| 7.7B.2 Spurious Response for intra-band non-contiguous EN-DC in FR1 (2 CCs) |  | Same as 7.7 in TS 38.521-1 [8] |  |  |  |
| 7.7B.3 Spurious Response for inter-band EN-DC within FR1 (2 CCs) |  | Same as 7.7 in TS 38.521-1 [8] |  |  |  |
| 7.6B.3.3_1.1 Out-of-band blocking for EN-DC within FR1 (3 CCs) |  | 0 dB |  | Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged |  |
| 7.6B.3.3_1.2 Out-of-band blocking for EN-DC within FR1 (4 CCs) |  | 0 dB |  | Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged |  |
| 7.8B.2.1 Wideband Intermodulation for intra-band contiguous EN-DC in FR1 (2 CCs) |  | Same as 7.8.2 in TS 38.521-1 [8] |  |  |  |
| 7.8B.2.2 Wideband Intermodulation for intra-band non-contiguous EN-DC in FR1 (2 CCs) |  | Same as 7.8.2 in TS 38.521-1 [8] |  |  |  |
| 7.8B.2.3 Wideband Intermodulation for inter-band EN-DC within FR1 (2 CCs) |  | Same as 7.8.2 in TS 38.521-1 [8] |  |  |  |
| 7.8B.2.6 Wideband Intermodulation for EN-DC within FR1 (3 CCs) (2 CCs) |  | Same as 7.8A.2.1 in TS 38.521-1 [8] |  |  |  |
| 7.9B.1 Spurious Emissions for intra-band contiguous EN-DC within FR1 (2 CCs) |  | Same as 7.9 in TS 38.521-1 [8] |  |  |  |
| 7.9B.2 Spurious Emissions for intra-band non-contiguous EN-DC within FR1 (2 CCs) |  | Same as 7.9 in TS 38.521-1 [8] |  |  |  |
| 7.9B.3 Spurious Emissions for inter-band EN-DC within FR1 (2 CCs) |  | Same as 7.9 in TS 38.521-1 [8] |  |  |  |
| 7.9B.3_1.1 Spurious Emissions for EN-DC within FR1 (3 CCs) |  | Same as 7.9A.1 in TS 38.521-1 [8] |  |  |  |
| 7.9B.4 Spurious Emissions for inter-band EN-DC including FR2 (2 CCs) |  | Same as 7.9 in TS 38.521-2 [9] |  |  |  |

## <<< END OF CHANGES >>>