3GPP TSG-RAN5 Meeting #91-e	R5- 212408

# Elctronic Meeting,  17th – 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 5297 | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to FDD PDSCH Single Antenna Port Performance for CA (7DL CA) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_CA_R15-UEConTest |  |  |  |  |  | Date: |  |  | 2021-04-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The following items are incomplete for FDD 7DL CA PDSCH Single Antenna Port Performance test case. - Editor’s note - Applicability spec |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update of TC 8.2.1.1.1_A.7 FDD PDSCH Single Antenna Port Performance for CA (7DL CA) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case is incomplete and the WI will not be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.1.1.1_A.7, F.1.4, F.3.4, G.3A.5, G.3A.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Unchanged sections omitted >>

8.2.1.1.1_A.7	FDD PDSCH Single Antenna Port Performance for CA (7 DL CA)

Editor’s note: The following aspects are either missing or not yet determined:

- Applicability spec.

8.2.1.1.1_A.7.1	Test purpose

To verify the UE's ability to receive a predefined test signal, representing a multi-path fading channel that is determined by the SNR with a percentage of the information bit throughput for a specified downlink Reference Measurement Channel (RMC) not falling below a specified value for transmission on a single-antenna port with different channel models and MCS.

8.2.1.1.1_A.7.2	Test applicability

This test case also applies to all types of E-UTRA FDD release 14 and forward UE of categories 8, category 11 or higher that support 7DL with CA configurations in Table [7.1-2e].

Note:	This test also applies to UE supporting 4Rx antenna ports.

<< Unchanged sections omitted >>

F.1.4	Measurement of performance requirements

Table F.1.4-1: Maximum Test System Uncertainty for Performance Requirements

| Subclause | Maximum Test System Uncertainty1 | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 8.2.1.1.1 Multiple PRBs - Propagation Condition EVA5 - Propagation Condition ETU70 - Propagation Condition ETU300 | ± 0.8 dB | Overall system uncertainty for fading conditions comprises three quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness Items 1, 2 and 3 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB for single Tx AWGN flatness and signal flatness ±2.0 dB |
| 8.2.1.1.1 Multiple PRBs - Propagation Condition HST | ± 0.6 dB | Overall system uncertainty for HST condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB |
| 8.2.1.1.1 Single PRB - Propagation Condition ETU70 | ± 0.8 dB | Overall system uncertainty for fading condition comprises three quantities: 1. Average Signal-to-noise ratio uncertainty 2. Signal-to noise ratio variation for single PRB 3. Fading profile power uncertainty Items 1, 2 and 3 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = SQRT (Average signal-to-noise ratio uncertainty 2 + Signal-to-noise ratio variation2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Signal-to-noise ratio variation ±0.5 dB Fading profile power uncertainty ±0.5 dB for single Tx |
| 8.2.1.1.1_A.1 | Same as 8.2.1.1.1 Multiple PRBs, for each CC | Same as 8.2.1.1.1 Multiple PRBs Calculation applies for each CC |
| 8.2.1.1.1_A.2 | Same as 8.2.1.1.1 Multiple PRBs, for each CC | Same as 8.2.1.1.1 Multiple PRBs Calculation applies for each CC |
| 8.2.1.1.1_A.4 | Same as 8.2.1.1.1 Multiple PRBs, for each CC | Same as 8.2.1.1.1 Multiple PRBs Calculation applies for each CC |
| 8.2.1.1.1_A.5 | Same as 8.2.1.1.1 Multiple PRBs, for each CC | Same as 8.2.1.1.1 Multiple PRBs Calculation applies for each CC |
| 8.2.1.1.1_A.6 | Same as 8.2.1.1.1 Multiple PRBs, for each CC | Same as 8.2.1.1.1 Multiple PRBs Calculation applies for each CC |
| 8.2.1.1.1_A.7 | Same as 8.2.1.1.1 Multiple PRBs, for each CC | Same as 8.2.1.1.1 Multiple PRBs Calculation applies for each CC |
| 8.2.1.3.1_A.5 | Same as 8.2.1.3.1 for each CC | Same as 8.2.1.3.1 Calculation applies for each CC |
| 8.2.1.1.1_1 Multiple PRBs - Propagation Condition EVA5 - Propagation Condition ETU70 - Propagation Condition ETU300 | Same as 8.2.1.1.1 Multiple PRBs Propagation EVA5, ETU70, ETU300 |  |
| 8.2.1.1.1_2 Multiple PRB - Propagation Condition EVA5 | Same as 8.2.1.1.1 Multiple PRBs Propagation EVA5 |  |
| 8.2.1.1.2 Single PRB | ± 0.8 dB | Same as 8.2.1.1.1 Single PRB |
| 8.2.1.2.1 - Propagation Condition EVA5 | ± 0.9 dB | Overall system uncertainty for fading conditions comprises three quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness Items 1, 2 and 3 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for Tx Diversity AWGN flatness and signal flatness ±2.0 dB |
| 8.2.1.2.1 - Propagation Condition HST | ± 0.6 dB | Overall system uncertainty for HST condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB |
| 8.2.1.2.1_1 - Propagation Condition EVA5 | Same as 8.2.1.2.1 Propagation EVA5 |  |
| 8.2.1.2.2 | ± 0.9 dB | Same as 8.2.1.2.1 Propagation Condition EVA5 |
| <<Too many rows skipped>> |  |  |
| 10.1 | ± 0.9 dB | Overall system uncertainty for fading conditions comprises three quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness Items 1, 2 and 3 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for Tx Diversity AWGN flatness and signal flatness ±2.0 dB |
| 10.2 | ± 0.9 dB | Same as 10.1 |
| 14.2 | ± 0.8 dB | Overall system uncertainty for fading conditions comprises three quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness Items 1, 2 and 3 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB for single Tx AWGN flatness and signal flatness ±2.0 dB |
| 14.3 | ± 0.8 dB | Same as 14.2 |
| 14.4 | ± 0.6 dB | Overall system uncertainty for AWGN condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB |
| 14.6 | ± 0.8 dB | Same as 14.2 |
| 14.7 | ± 0.6 dB | Same as 14.4 |
| 14.8 | Downlink absolute power uncertainty, averaged over BWConfig ±1.0 dB Downlink EVM ≤ 3% | 3% EVM is equivalent to a Test system downlink SNR of 30.5dB. The noise from the Test system is then sufficiently below that required for the UE to demodulate the signal with the required % success rate. Under these conditions the UE throughput is limited by the Reference measurement channel and the UE capability, and not by the Test system EVM. |
| 14.9 | E-UTRA: Downlink absolute power uncertainty, averaged over BWConfig ±1.0 dB Downlink EVM ≤ 3% Sidelink: ± 0.6 dB | E-UTRA: Same as 8.7.1.1 Sidelink: Same as 14.4 |
| [Other tests FFS] |  |  |
| In addition, the following Test System uncertainties and related constraints apply: |  |  |
| AWGN Bandwidth |  | ≥ 1.08MHz, 2.7MHz, 4.5MHz, 9MHz, 13.5MHz, 18MHz; NRB x 180kHz according to BWConfig |
| AWGN absolute power uncertainty, averaged over BWConfig Note 4 |  | ±3 dB |
| AWGN flatness and signal flatness, max deviation for any Resource Block, relative to average over BWConfig |  | ±2 dB |
| AWGN peak to average ratio |  | ≥10 dB @0.001% |
| Signal-to noise ratio uncertainty, averaged over downlink transmission Bandwidth |  | ±0.3 dB (includes uncertainty in precoding applied by the test system, where applicable) |
| Signal-to noise ratio variation for any resource block, relative to average over downlink transmission Bandwidth |  | ±0.5 dB |
| Noc2 absolute power uncertainty, averaged over BWConfig Note 5 |  | ±3 dB |
| Noc1 / Noc2 ratio uncertainty, averaged over BWConfig Note 5 |  | ±0.3 dB |
| Noc3 / Noc2 ratio uncertainty, averaged over BWConfig Note 5 |  | ±0.3 dB |
| Es / Noc2 ratio (SNR) uncertainty, averaged over downlink transmission Bandwidth Note 5 |  | ±0.3 dB (includes uncertainty in precoding applied by the test system, where applicable) |
| Fading profile power uncertainty |  | Test-specific |
| Fading profile delay uncertainty, relative to frame timing |  | ±5 ns (excludes absolute errors related to baseband timing) |
| CA, DC and LAA performance requirements only: Relative frequency error between carriers |  | 30Hz, measured over a 1ms period, and maximum carrier spacing 80MHz |
| enhanced performance requirement Type B only: Frequency error on the frequency offset to cell 1 |  | 30Hz, measured over a 1ms period |
| Note 1: Only the overall stimulus error is considered here. The effect of errors in the throughput measurements due to finite test duration is not considered. Note 2: The AWGN or N ocx parameters apply to all test cases except 8.7.1 and 8.7.2. The fading parameters apply to test cases using fading Note 3: In CA, DC and LAA test cases using multiple component carriers (CCs), the uncertainties and related constraints apply for each CC. Note 4: Applies for test cases which specify Noc, a single value that remains constant with time. Note 5: Applies for test cases which specify Noc1, Noc2 and Noc3, that are symbol or subframe specific. |  |  |

<< Unchanged sections omitted >>

F.3.4	Measurement of performance requirements

Table F.3.4-1: Derivation of Test Requirements (performance tests)

| Test |  |  | Minimum Requirement in TS 36.101 |  |  | Test Tolerance (TT) |  |  | Test Requirement in TS 36.521-1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8.2.1.1.1 Multiple PRBs - Prop’n Condition EVA5 - Prop’n Condition ETU70 - Prop’n Condition ETU300 |  |  | SNRs as specified |  |  | 0.8dB |  |  | Formula: SNR + TT T-put limit unchanged |  |  |
| 8.2.1.1.1 Multiple PRBs - Prop’n Condition HST |  |  | SNR as specified |  |  | 0.6dB |  |  | Formula: SNR + TT T-put limit unchanged |  |  |
| 8.2.1.1.1 Single PRB - Prop’n Condition ETU70 |  |  | SNRs as specified |  |  | 0.8dB |  |  | Formula: SNR + TT T-put limit unchanged |  |  |
| 8.2.1.1.1_1 Multiple PRBs - Prop'n Condition EVA5 - Prop'n Condition ETU70 - Prop'n Condition ETU300 |  |  | SNRs as specified |  |  | Same as 8.2.1.1.1 Multiple PRBs Propagation EVA5, ETU70, ETU300 |  |  | Formula: SNR + TT T-put limit unchanged |  |  |
| 8.2.1.1.1_2 Multiple PRB - Prop'n Condition EVA5 |  |  | SNRs as specified |  |  | Same as 8.2.1.1.1 Multiple PRBs Propagation EVA5 |  |  | Formula: SNR + TT T-put limit unchanged |  |  |
| 8.2.1.1.1_A.1 |  |  | SNRs as specified |  |  | Same as 8.2.1.1.1 Multiple PRBs, for each CC |  |  | Formula: SNR + TT T-put limit unchanged |  |  |
| 8.2.1.1.1_A.2 |  |  | SNRs as specified |  |  | Same as 8.2.1.1.1 Multiple PRBs for each CC |  |  | Formula: SNR + TT T-put limit unchanged |  |  |
| 8.2.1.1.1_A.4 |  |  | SNRs as specified |  |  | Same as 8.2.1.1.1 Multiple PRBs for each CC |  |  | Formula: SNR + TT T-put limit unchanged |  |  |
| 8.2.1.1.1_A.5 |  |  | SNRs as specified |  |  | Same as 8.2.1.1.1 Multiple PRBs for each CC |  |  | Formula: SNR + TT T-put limit unchanged |  |  |
| 8.2.1.1.1_A.6 |  |  | SNRs as specified |  |  | Same as 8.2.1.1.1 Multiple PRBs for each CC |  |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.1.1.1_A.7 |  |  | SNRs as specified |  |  | Same as 8.2.1.1.1 Multiple PRBs for each CC |  |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.1.1.2 Single PRB |  |  | SNR as specified |  |  | 0.8dB |  |  | Formula: SNR + TT T-put limit unchanged |  |  |
| 8.2.1.2.1 - Prop’n Condition EVA5 |  |  | SNR as specified |  |  | 0.9 dB |  |  | Formula: SNR + TT T-put limit unchanged |  |  |
| 8.2.1.2.1 - Prop’n Condition HST |  |  | SNR as specified |  |  | 0.6 dB |  |  | Formula: SNR + TT T-put limit unchanged |  |  |
| 8.2.1.2.1_1 - Prop'n Condition EVA5 |  |  | SNR as specified |  |  | Same as 8.2.1.2.1 |  |  | Formula: SNR + TT T-put limit unchanged |  |  |
| 8.2.1.2.2 |  |  | SNR as specified |  |  | 0.9 dB |  |  | Formula: SNR + TT T-put limit unchanged |  |  |
| 8.2.1.2.2_1 |  |  | SNR as specified |  |  | Same as 8.2.1.2.2 |  |  | Formula: SNR + TT T-put limit unchanged |  |  |
| 8.2.1.2.3_C.1 |  |  | Noc1: -102dBm/15kHz Noc2: -98dBm/15kHz Noc3: -94.8dBm/15kHz Ês1 / Noc2: +3.4dB Ês2 / Noc2: +6dB |  |  | -0.1dB 0dB -0.1dB +0.9dB -0.2dB |  |  | Noc1: -102.1dBm/15kHz Noc2: -98dBm/15kHz Noc3: -94.9dBm/15kHz Ês1 / Noc2: +4.3dB Ês2 / Noc2: +5.8dB Formulae for Test Tolerance values: Noc1: -(Ês1/Noc1 uncertainty - Ês1/Noc2 uncertainty) Noc2: unchanged Noc3: -(Ês1/Noc3 uncertainty - Ês1/Noc2 uncertainty) Ês1 / Noc2: +Ês1/Noc2 uncertainty Ês2 / Noc2: -(Ês2/Ês1 uncertainty - Ês1/Noc2 uncertainty) T-put limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |  |  |
| 8.2.1.2.3_E.1 - Prop’n Condition EVA5 |  |  | Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Noc3: -93dBm/15kHz Ês1 / Noc2: +3.4dB Ês2 / Noc2: +12dB Ês3 / Noc2: +10dB |  |  | 0dB 0dB -0.1dB +0.9dB -0.2dB -0.2dB |  |  | Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Noc3: -93.1dBm/15kHz Ês1 / Noc2: +4.3dB Ês2 / Noc2: +11.8dB Ês3 / Noc2: +9.8dB The analysis is recorded in 3GPP TR 36.904 [17] T-put limit unchanged |  |  |
| 8.2.1.2.4 |  |  | Noc: -98dBm/15kHz DIP 1: -2.23dB DIP 1: -8.06dB SINR: -1.10dB |  |  | 0dB +0.35dB +0.85dB +0.95dB |  |  | Noc: -98dBm/15kHz DIP 1: -1.88dB DIP 1: -7.21dB SINR: -0.15dB T-put limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |  |  |
| 8.2.1.2.5 |  |  | Noc: -98dBm/15kHz Cell 1 Ês/Noc: +15.5 dB Cell 2 Ês/Noc: +13.91 dB Cell 3 Ês/Noc: +3.34 dB Cell 1 Ês/Noc is defined as SNR in minimum requirement. |  |  | 0 dB +1.85 dB +0.8 dB +0.8 dB |  |  | Noc: -98dBm/15kHz Cell 1 Ês/Noc: +17.35 dB Cell 2 Ês/Noc: +14.71 dB Cell 3 Ês/Noc: +4.14 dB T-put limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |  |  |
| 8.2.1.2.6 |  |  | Noc: -98dBm/15kHz Cell 1 Ês/Noc: +8.4 dB Cell 2 Ês/Noc: +3.28 dB Cell 3 Ês/Noc: +0.74 dB Cell 1 Ês/Noc is defined as SNR in minimum requirement. |  |  | 0 dB +1.54 dB +0.8 dB +0.8 dB |  |  | Noc: -98dBm/15kHz Cell 1 Ês/Noc: +9.94 dB Cell 2 Ês/Noc: +4.08 dB Cell 3 Ês/Noc: +1.54 dB T-put limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |  |  |
| 8.2.1.3.1 |  |  | SNR as specified |  |  | 0.9 dB |  |  | Formula: SNR + TT T-put limit unchanged |  |  |
| 8.2.1.3.1_1 |  |  | SNR as specified |  |  | Same as 8.2.1.3.1 |  |  | Formula: SNR + TT T-put limit unchanged |  |  |
| 8.2.1.3.1_A.1 |  |  | SNR as specified |  |  | 0.9 dB |  |  | Formula: SNR + TT T-put limit unchanged |  |  |
| 8.2.1.3.1_A.2 |  |  | SNR as specified |  |  | 0.9 dB |  |  | Formula: SNR + TT T-put limit unchanged |  |  |
| 8.2.1.3.1_A.3 |  |  | SNR as specified |  |  | 0.9 dB |  |  | Formula: SNR + TT T-put limit unchanged |  |  |
| 8.2.1.3.1_A.4 |  |  | SNR as specified |  |  | 0.9 dB |  |  | Formula: SNR + TT T-put limit unchanged |  |  |
| 8.2.1.3.1_A.5 |  |  | SNR as specified |  |  | 0.9 dB |  |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.1.3.1A_A.1 |  |  | SNR as specified |  |  | 0.9 dB |  |  | Formula: SNR + TT T-put limit unchanged |  |  |
| 8.2.1.3.1B |  |  | SNR as specified |  |  | 0.9 dB |  |  | Formula: SNR + TT T-put limit unchanged |  |  |
| 8.2.1.3.1C |  |  | Noc: -98dBm/15kHz Cell 1 Ês/Noc: +19.9 dB Cell 2 Ês/Noc: +12.95 dB Cell 1 Ês/Noc is defined as SNR in minimum requirement. |  |  | 0 dB +1.9 dB +0.8 dB |  |  | Noc: -98dBm/15kHz Cell 1 Ês/Noc: +21.8 dB Cell 2 Ês/Noc: +13.75 dB T-put limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |  |  |
| 8.2.1.3.2 |  |  | SNR as specified |  |  | 0.9 dB |  |  | Formula: SNR + TT T-put limit unchanged |  |  |
| 8.2.1.3.3_C.1 |  |  | Noc1: -102dBm/15kHz Noc2: -98dBm/15kHz Noc3: -94.8dBm/15kHz Ês1 / Noc2: +13.3dB Ês2 / Noc2: +6dB |  |  | -0.1dB 0dB -0.1dB +0.9dB -0.2dB |  |  | Noc1: -102.1dBm/15kHz Noc2: -98dBm/15kHz Noc3: -94.9dBm/15kHz Ês1 / Noc2: +14.2dB Ês2 / Noc2: +5.8dB Formulae for Test Tolerance values: Noc1: -(Ês1/Noc1 uncertainty - Ês1/Noc2 uncertainty) Noc2: unchanged Noc3: -(Ês1/Noc3 uncertainty - Ês1/Noc2 uncertainty) Ês1 / Noc2: +Ês1/Noc2 uncertainty Ês2 / Noc2: -(Ês2/Ês1 uncertainty - Ês1/Noc2 uncertainty) T-put limit unchanged |  |  |
| 8.2.1.3.3_C.2 |  |  | Noc1: -102dBm/15kHz Noc2: -98dBm/15kHz Noc3: -94.8dBm/15kHz Ês1 / Noc2: +12.0dB Ês2 / Noc2: +6dB |  |  | -0.1dB 0dB -0.1dB +0.9dB -0.2dB |  |  | Noc1: -102.1dBm/15kHz Noc2: -98dBm/15kHz Noc3: -94.9dBm/15kHz Ês1 / Noc2: +12.9dB Ês2 / Noc2: +5.8dB Formulae for Test Tolerance values: Noc1: -(Ês1/Noc1 uncertainty - Ês1/Noc2 uncertainty) Noc2: unchanged Noc3: -(Ês1/Noc3 uncertainty - Ês1/Noc2 uncertainty) Ês1 / Noc2: +Ês1/Noc2 uncertainty Ês2 / Noc2: -(Ês2/Ês1 uncertainty - Ês1/Noc2 uncertainty) T-put limit unchanged |  |  |
| 8.2.1.3.3_E.1 - Prop’n Condition EVA5 |  |  | Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Noc3: -93dBm/15kHz Test 1 Ês1 / Noc2: + 13.9dB Ês2 / Noc2: + 9dB Ês3 / Noc2: + 7dB Test 2 Ês1 / Noc2: +22.6dB Ês2 / Noc2: +9dB +1dB Ês3 / Noc2: |  |  | 0dB 0dB -0.1dB +0.9dB -0.2dB -0.2dB +0.9dB -0.2dB -0.2dB |  |  | Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Noc3: - 93.1 dBm/15kHz Ês1 / Noc2: + 14.8 dB Ês2 / Noc2: + 8.8dB Ês3 / Noc2: + 6.8 dB Ês1 / Noc2: +23.5dB Ês2 / Noc2: +8.8dB Ês3 / Noc2: +0.8dB The analysis is recorded in 3GPP TR 36.904 [17] T-put limit unchanged |  |  |
| 8.2.1.4.1 |  |  | SNRs as specified |  |  | 0.9 dB |  |  | Formula: SNR + TT T-put limit unchanged |  |  |
| <<Too many rows skipped>> |  |  |  |  |  |  |  |  |  |  |  |
| 10.1 |  |  | SNR as specified |  |  | 0.9dB |  |  | Formula: SNR + TT T-put limit unchanged |  |  |
| 10.2 |  |  | SNR as specified |  |  | 0.9dB |  |  | Formula: SNR + TT T-put limit unchanged |  |  |
| 14.2 |  |  | SNRs as specified |  |  | 0.8dB |  |  | Formula: SNR + TT T-put limit unchanged |  |  |
| 14.3 |  |  | SNRs as specified |  |  | 0.8dB |  |  | Formula: SNR + TT T-put limit unchanged |  |  |
| 14.4 |  |  | SNRs as specified |  |  | 0.6dB |  |  | Formula: SNR + TT T-put limit unchanged |  |  |
| 14.6 |  |  | SNRs as specified |  |  | 0.8dB |  |  | Formula: SNR + TT T-put limit unchanged |  |  |
| 14.7 |  |  | SNRs as specified |  |  | 0.6dB |  |  | Formula: SNR + TT T-put limit unchanged |  |  |
| 14.8 |  |  | Downlink power -85dBm/15kHz |  |  | 0 dB |  |  | Formula: Downlink power + TT T-put limit unchanged |  |  |
| 14.9 |  |  | SNRs as specified |  |  | E-UTRA: 0dB Sidelink: 0.6dB |  |  | E-UTRA: Same as 8.7.1.1 Sidelink: Same as 14.7 |  |  |
| [Other tests FFS] |  |  |  |  |  |  |  |  |  |  |  |

<< Unchanged sections omitted >>

G.3A.5	Minimum test time

In contrast to G.3.5, where the minimum test time is derived from simulations, the test time here is selected without simulation utilising test time information from similar demodulation scenarios in non-CA test cases and while maintaining the reasonable testing time.

Note 	MNAS values in this clause may need to be changed in future if current MNAS values turn out to be too short for giving stable CA/DC throughput results.

Table G.3A.5-1: Minimum Test time FDD PDSCH Single Antenna Port Performance for CA (2DL CA)

| Clause 8.2.1.1.1_A.1 Test No | Demodulation scenario plain text: RMC (Bandwidth, allocated RBs, modulation, coding) Antenna (configuration, correlation) Propagation condition, Doppler [additional parameters, if applicable] (info only) | Minimum Number of Active Subframes in each CC | Minimum Number of Subframes (MNS) in each CC (MNS = active and inactive subframes) |
| --- | --- | --- | --- |
|  |  |  | FDD |
| 1 | R.2FDD (2x10 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 55 556 |
| 2 | R.42FDD (2x20 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 55 556 |
| 3 | R.42-3 FDD + R.42-2 FDD (15MHz+5MHz,full,QPSK,1/3) (1x2 Low) EVA5) | 50 000 | 55 556 |

Table G.3A.5-1A: Minimum Test time FDD PDSCH Single Antenna Port Performance for CA (3DL CA)

| Clause 8.2.1.1.1_A.2 Test No | Demodulation scenario plain text: RMC (Bandwidth, allocated RBs, modulation, coding) Antenna (configuration, correlation) Propagation condition, Doppler [additional parameters, if applicable] (info only) | Minimum Number of Active Subframes in each CC | Minimum Number of Subframes (MNS) in each CC (MNS = active and inactive subframes) |
| --- | --- | --- | --- |
|  |  |  | FDD |
| 1 | R.42FDD (3x20 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 55 556 |
| 2 | R.42FDD + R.42-3FDD (20+20+15 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 55 556 |
| 3 | R.42FDD + R.2FDD (20+20+10 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 55 556 |
| 4 | R.42FDD + R.42-3FDD (20+15+15 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 55 556 |
| 5 | R.42FDD + R.42-3FDD + R.2FDD (20+15+10 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 55 556 |
| 6 | R.42FDD + R.2FDD (20+10+10 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 55 556 |
| 7 | R.42-3FDD + R.2FDD (15+15+10 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 55 556 |
| 8 | R.42FDD + R.2FDD + R.42-2FDD (20+10+5 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 55 556 |
| 9 | R.42FDD + R.42-3FDD + R.42-2FDD (20+15+5 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 55 556 |

Table G.3A.5-1B: Minimum Test time FDD PDSCH Single Antenna Port Performance for CA (4DL CA)

| Clause 8.2.1.1.1_A.4 Test No | Demodulation scenario plain text: RMC (Bandwidth, allocated RBs, modulation, coding) Antenna (configuration, correlation) Propagation condition, Doppler [additional parameters, if applicable] (info only) | Minimum Number of Active Subframes in each CC | Minimum Number of Subframes (MNS) in each CC (MNS = active and inactive subframes) |
| --- | --- | --- | --- |
|  |  |  | FDD |
| 1 | R.42FDD (4x20 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 55 556 |
| 2 | R.2FDD + R.42FDD (10+20+20+20 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 55 556 |
| 3 | R.2FDD + R.42FDD (10+10+20+20 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 55 556 |
| 4 | R.42-2FDD + R.2FDD + R.42FDD (5+10+20+20 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 55 556 |
| 5 | R.42-2FDD + R.2FDD + R.42FDD (5+10+10+20 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 55 556 |

Table G.3A.5-1C: Minimum Test time FDD PDSCH Single Antenna Port Performance for CA (5DL CA)

| Clause 8.2.1.1.1_A.5 Test No | Demodulation scenario plain text: RMC (Bandwidth, allocated RBs, modulation, coding) Antenna (configuration, correlation) Propagation condition, Doppler [additional parameters, if applicable] (info only) | Minimum Number of Active Subframes in each CC | Minimum Number of Subframes (MNS) in each CC (MNS = active and inactive subframes) |
| --- | --- | --- | --- |
|  |  |  | FDD |
| 1 | R.42FDD (5x20 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 55 556 |
| 2 | R.42-3FDD + R.42FDD (15+4x20 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 55 556 |
| 3 | R.2FDD + R.42FDD (10+4x20 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 55 556 |
| 4 | R.2FDD + R.42FDD (2x10+3x20 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 55 556 |
| 5 | R.42-2FDD + R.2FDD + R.42FDD (5+10+3x20 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 55 556 |
| 6 | R.2FDD + R.42FDD (3x10+2x20 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 55 556 |
| 7 | R.2FDD + R.42FDD (4x10+20 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 55 556 |

Table G.3A.5-1D: Minimum Test time FDD PDSCH Single Antenna Port Performance for CA (6DL CA)

| Clause 8.2.1.1.1_A.6 Test No | Demodulation scenario plain text: RMC (Bandwidth, allocated RBs, modulation, coding) Antenna (configuration, correlation) Propagation condition, Doppler [additional parameters, if applicable] (info only) | Minimum Number of Active Subframes in each CC | Minimum Number of Subframes (MNS) in each CC (MNS = active and inactive subframes) |
| --- | --- | --- | --- |
|  |  |  | FDD |
| 1 | R.42FDD (6x20 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 55 556 |

Table G.3A.5-1E: Minimum Test time FDD PDSCH Single Antenna Port Performance for CA (7DL CA)

| Clause 8.2.1.1.1_A.7 Test No | Demodulation scenario plain text: RMC (Bandwidth, allocated RBs, modulation, coding) Antenna (configuration, correlation) Propagation condition, Doppler [additional parameters, if applicable] (info only) | Minimum Number of Active Subframes in each CC | Minimum Number of Subframes (MNS) in each CC (MNS = active and inactive subframes) |
| --- | --- | --- | --- |
|  |  |  | FDD |
| 1 | R.42FDD (7x20 MHz,full,QPSK,1/3) (1x2 Low) EVA5 | 50 000 | 55 556 |

Table G.3A.5-2: Minimum Test time FDD PDSCH Open Loop Spatial Multiplexing 2x2 (2DL CA)

| Clause 8.2.1.3.1_A.1 Test No | Demodulation scenario plain text: RMC (Bandwidth, allocated RBs, modulation, coding) Antenna (configuration, correlation) Propagation condition, Doppler [additional parameters, if applicable] (info only) | Minimum Number of Active Subframes in each CC | Minimum Number of Subframes (MNS) in each CC (MNS = active and inactive subframes) |
| --- | --- | --- | --- |
|  |  |  | FDD |
| 1 | R.11 FDD (2x10 MHz, full 16QAM, 1/2), (2x2 Low) EVA70 | 10 000 | 11 112 |
| 2 | R.30 FDD (2x20 MHz, full 16QAM, 1/2), (2x2 Low) EVA70 | 10 000 | 11 112 |
| 3 | R.11-2 FDD (2x5 MHz, full 16QAM, 1/2), (2x2 Low) EVA70 | 10 000 | 11 112 |
| 4 | FFS | FFS | FFS |
| 5 | R.11-7 FDD + R.11-2 FDD (15MHz+5MHz, full 16QAM, 1/2), (2x2 Low) EVA70 | 10 000 | 11 112 |

<< Unchanged sections omitted >>

G.3A.6	Test conditions

Table G.3A.6-1: Test conditions for CA/DC performance tests

| Test | Statistical independence | Number of components in the test vector, as specified in the test requirements and initial conditions of the applicable test | Over all Pass/Fail condition |
| --- | --- | --- | --- |
| 8.2.1.1.1_A.1 FDD PDSCH Single Antenna Port Performance for CA (2DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.2.1.1.1_A.2 FDD PDSCH Single Antenna Port Performance for CA (3DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.2.1.1.1_A.4 FDD PDSCH Single Antenna Port Performance for CA (4DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.2.1.1.1_A.5 FDD PDSCH Single Antenna Port Performance for CA (5DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.2.1.1.1_A.6 FDD PDSCH Single Antenna Port Performance for CA (6DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.2.1.1.1_A.7 FDD PDSCH Single Antenna Port Performance for CA (7DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.2.1.3.1_A.1 FDD PDSCH Open Loop Spatial Multiplexing 2x2 for CA (2DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.2.1.3.1_A.2 FDD PDSCH Open Loop Spatial Multiplexing 2x2 for CA (3DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.2.1.3.1_A.3 FDD PDSCH Open Loop Spatial Multiplexing 2x2 for CA (4DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.2.1.3.1_A.4 FDD PDSCH Open Loop Spatial Multiplexing 2x2 for CA (5DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.2.1.3.1_A.5 FDD PDSCH Open Loop Spatial Multiplexing 2x2 for CA (6DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.2.1.3.1A_A.1 FDD PDSCH Soft buffer management test for CA (2 DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.2.1.4.2_A.1 FDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4 x 2 for CA (2DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.2.1.4.2_A.2 FDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4 x 2 for CA (3DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.2.1.4.2_A.3 FDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4 x 2 for CA (4DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.2.1.4.2_A.4 FDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4 x 2 for CA (5DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.2.1.4.2_A.5 FDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for CA (6DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.2.1.4.3A FDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for Dual Connectivity | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.2.2.1.1_A.1 TDD PDSCH Single Antenna Port Performance for CA (2DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.2.2.1.1_A.2 TDD PDSCH Single Antenna Port Performance for CA (3DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.2.2.1.1_A.3 TDD PDSCH Single Antenna Port Performance for CA (4DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.2.2.1.1_A.4 TDD PDSCH Single Antenna Port Performance for CA (5DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.2.2.3.1_A.1 TDD PDSCH Open Loop Spatial Multiplexing 2x2 for CA (2DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.2.2.3.1_A.2 TDD PDSCH Open Loop Spatial Multiplexing 2x2 for CA (3DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.2.2.3.1_A.3 TDD PDSCH Open Loop Spatial Multiplexing 2x2 for CA (4DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.2.2.3.1_A.4 TDD PDSCH Open Loop Spatial Multiplexing 2x2 for CA (5DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.2.2.3.1A_A.1 TDD PDSCH Soft buffer management test (2 DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.2.2.4.2_A.1 TDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for CA (2DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.2.2.4.2_A.2 TDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for CA (3DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.2.2.4.2_A.3 TDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for CA (4DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.2.2.4.2_A.4 TDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for CA (5DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.2.2.4.4 TDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for Dual Connectivity | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.2.3.1.1.5 TDD FDD CA PDSCH Single Antenna Port Performance for FDD PCell (6DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.2.3.1.2.5 TDD FDD CA PDSCH Single Antenna Port Performance for TDD PCell (6DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.2.3.1.2.6 TDD FDD CA PDSCH Single Antenna Port Performance for TDD PCell (7DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.2.3.2.1.5 TDD FDD CA PDSCH Open Loop Spatial Multiplexing 2x2 for FDD PCell (6DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.2.3.2.1.6 TDD FDD CA PDSCH Open Loop Spatial Multiplexing 2x2 for FDD PCell (7DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.2.3.2.2.1 TDD FDD CA PDSCH Open Loop Spatial Multiplexing 2x2 for TDD PCell (2DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.2.3.2.2.2 TDD FDD CA PDSCH Open Loop Spatial Multiplexing 2x2 for TDD PCell (3DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.2.3.2.2.5 TDD FDD CA PDSCH Open Loop Spatial Multiplexing 2x2 for TDD PCell (6DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.2.3.2.2.6 TDD FDD CA PDSCH Open Loop Spatial Multiplexing 2x2 for TDD PCell (7DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.2.3.3.1.5 TDD FDD CA PDSCH Closed Loop Multi Lyaer Spatial Multiplexing 4x2 for FDD PCell (6DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.2.3.3.1.6 TDD FDD CA PDSCH Closed Loop Multi Lyaer Spatial Multiplexing 4x2 for FDD PCell (7DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.2.3.3.2.5 TDD FDD CA PDSCH Closed Loop Multi Lyaer Spatial Multiplexing 4x2 for TDD PCell (6DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.2.3.3.2.6 TDD FDD CA PDSCH Closed Loop Multi Lyaer Spatial Multiplexing 4x2 for TDD PCell (7DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.3.3.1.1 LAA Dual-Layer Spatial Multiplexing with DM-RS with FDD as Pcell | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.3.3.1.2 LAA Dual-Layer Spatial Multiplexing with DM-RS with TDD as Pcell | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.7.1.1_A.1 FDD Sustained data rate performance for CA (2DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.7.1.1_A.2 FDD Sustained data rate performance for CA (3DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.7.1.1_A.4 FDD Sustained data rate performance for CA (4DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.7.2.1_A.1 TDD Sustained data rate performance for CA (2DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.7.2.1_A.2 TDD Sustained data rate performance for CA (3DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.7.2.1_A.4 TDD Sustained data rate performance for CA (4DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 8.7.2.1_A.5 TDD Sustained data rate performance for CA (5DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |
| 9.6.1.1_A.3 FDD CQI Reporting under AWGN conditions – PUCCH 1-0 for CA (4DL CA) | subframes are independent | 1 | To pass the test case all applicable components in the test vector must pass |

<< Unchanged sections omitted >>