Title: 	Discussion on test point selection for NR Occupied Bandwidth in FR2

Source: 	Keysight Technologies, Rohde & Schwarz, TTA

- Introduction

At RAN5#78, it was agreed a way forward to define test point selection for Tx/Rx test cases in both FR1 and FR2. This agreement included test point selection analysis justifying test environment, test frequencies, test bandwidths, test subcarrier spacing and UL/DL configuration for each test case. This analysis will be captured in TR 38.905.

The purpose of this contribution is to provide test point selection proposal for NR Occupied Bandwidth measurement in FR2.

- Discussion

The definition of Occupied Bandwidth for FR2 can be found in [1]:

6.5.1	Occupied bandwidth

Occupied bandwidth is defined as the bandwidth containing 99 % of the total integrated mean power of the transmitted spectrum on the assigned channel. The occupied bandwidth for all transmission bandwidth configurations (Resources Blocks) shall be less than the channel bandwidth specified in Table 6.6.1-1.

The occupied bandwidth is defined as a directional requirement. The requirement is verified in beam locked mode on beam peak direction.

Table 6.6.1-1: Occupied channel bandwidth

|  | Occupied channel bandwidth / Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| Channel bandwidth (MHz) | 50 | 100 | 200 | 400 |

The test case stablishes that the maximum transmission bandwidth configuration transmitted by the UE shall not exceed the channel bandwidth for this transmission bandwidth configuration.

Following subclauses introduce study for test environment, test frequencies, test bandwidth, test subcarrier spacing and uplink configuration aspects.

  - Test Environment

Test environment for Occupied Bandwidth measurement for LTE and FR1 in [2] is Normal Conditions. This test environment can be also leveraged for FR2.

Proposal 1: Define Test Environment as Normal Conditions (NC) for Occupied Bandwidth measurement in FR2.

  - Test Frequencies

In LTE, Occupied Bandwidth measurement is considered an agnostic-frequency measurement, meaning that the maximum channel bandwidth transmitted by the UE remains the same with the independence of the EARFCN being tested.

However, in NR FR2, operating bands are defined with a high bandpass, spanning more than 30000 MHz at very high frequencies. In Table 5.2-1 in [1], the operating band definition for each band is presented. In the following table, band bandwidth per NR operating band is calculated:

Table 2.2-1 Bandwidth per operating band

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL_low – FUL_high | Downlink (DL) operating band BS transmit / UE receive FDL_low – FDL_high | Duplex Mode | Band Bandwidth (MHz) |
| --- | --- | --- | --- | --- |
| n257 | 26500 MHz – 29500 MHz | 26500 MHz – 29500 MHz | TDD | 3000 |
| n258 | 24250 MHz – 27500 MHz | 24250 MHz – 27500 MHz | TDD | 3250 |
| n260 | 37000 MHz – 40000 MHz | 37000 MHz – 40000 MHz | TDD | 3000 |

Similarly to [2] for NR FR1 type-2 bands, due to the wide band bandwidths, for NR FR2 bands it is needed to consider the frequency response of filters, amplifiers and other active elements in the UE, which can modify the amplitude and the phase of the signal being transmitted.

Observation 1: For NR FR2 bands there is a need to ensure UEs properly work in the whole band bandwidth.

Proposal 2: Select Low range, Mid range and High range as test frequencies.

  - Test Channel Bandwidth

Occupied Bandwidth minimum conformance requirement defined in [1] is specific for each channel bandwidth.

Furthermore, in LTE all channel bandwidths supported in a band are tested.

Observation 2: Based on Occupied Bandwidth minimum conformance requirement definition and LTE test channel bandwidth selection for Occupied Bandwidth, all available channel bandwidths in a band must be tested.

Proposal 3: Select all channel bandwidths defined in a band to measure Occupied Bandwidth.

  - Test Subcarrier Spacing

According to Table 5.3.2-1 in [1], there is a relation between channel bandwidth, subcarrier spacing and the maximum number of resource blocks that can be allocated in each channel bandwidth and SCS combination.

Table 5.3.2-1: Maximum transmission bandwidth configuration NRB

| SCS (kHz) | 50MHz | 100MHz | 200MHz | 400 MHz |
| --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB |
| 60 | 66 | 132 | 264 | N.A |
| 120 | 32 | 66 | 132 | 264 |

Considering the maximum number of Resource blocks that can be allocated in each channel bandwidth and subcarrier spacing as shown in Table 5.3.2-1 in [1], the maximum transmission bandwidth can be calculated. Table 2.4-1 shows this calculation.

Table 2.4-1 Maximum transmission bandwidth for each UE channel bandwidth and SCS (MHz)

| SCS (kHz) | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| --- | --- | --- | --- | --- |
| 60 | 47.52 | 95.04 | 190.08 | N/A |
| 120 | 46.08 | 95.04 | 190.08 | 380.16 |

Observation 3: Maximum transmission bandwidth for each channel bandwidth is not always achieved with lowest SCS values.

Table 5.3.3-1 in [1] shows the minimum guardband for a UE channel bandwidth and SCS.

Table 5.3.3-1: Minimum guardband for each UE channel bandwidth and SCS (kHz)

| SCS (kHz) | 50MHz | 100MHz | 200MHz | 400 MHz |
| --- | --- | --- | --- | --- |
| 60 | 1210 | 2450 | 4930 | N.A |
| 120 | 1900 | 2420 | 4900 | 9860 |

The guardband is the distance between the channel edge and the transmission bandwidth edge. Narrower guardband implies higher attenuation slope in the UE transmission filter, to avoid subcarriers transmitted out of the channel bandwidth.

Observation 4: Lower guardbands are achieved with lower SCS for 50 MHz channel bandwidth. For 100 MHz and 200 MHz channel bandwidth, similar guardband values are achieved.

In [2], maximum transmission bandwidth and minimum guardband defined the criteria to select the SCS to test in FR1, as they set the worst conditions for the UE channel bandwidth filtering stage. However, for FR2, these parameters are not decisive for selecting the SCS for all channel bandwidths, as different SCS have similar values for the same channel bandwidth.

In the case neither maximum transmission bandwidth nor minimum guardband define the SCS applicability, other parameters are considered.

OFDM subcarrier generation is a non-ideal process involving local oscillators to generate the subcarriers. Local oscillators are affected by phase noise, which increases with carrier frequency. The effect of the phase noise in the frequency domain is a non-ideal, wider pulse for each subcarrier. Depending on the distance between subcarriers, this spread spectrum for each subcarrier can affect the generation of the OFDM signal, increasing the transmission bandwidth.

Observation 5: Lower SCS are less robust for channel bandwidth transmission against phase noise from local oscillators.

The OFDM subcarrier generation non-ideal frequency response is also taken into account to calculate the number of secondary lobs that can be contained in the guardbands. These secondary lobs are not filtered, which can increase the power level within the channel bandwidth and contribute to the calculation of the occupied bandwidth. Table 2.4-2 shows the number of secondary lobs contained in the guardband for each channel bandwidth:

Table 2.4-2 Number of secondary lobs in the guardband for each channel bandwidth and SCS

| SCS (kHz) | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| --- | --- | --- | --- | --- |
| 60 | 20 | 40 | 82 | N/A |
| 120 | 15 | 20 | 40 | 82 |

Note 1: The number of secondary lobs have been calculated using following equation: floor(guardband / SCS), where guardband and SCS are from Table 5.3.3-1 in [1].

Observation 6: High number of secondary lobs can increase the power level contribution for occupied bandwidth calculation. The highest number of secondary lobs are obtained with lowest SCS values for each channel bandwidth.

Proposal 4: Select the lowest SCS supported for the current channel bandwidth to test.

  - Uplink configuration

Maximum transmission bandwidth configuration NRB for the selected channel bandwidth and SCS according to Table 5.3.2-1 in [1] allow the maximum spectrum utilization to measure the occupied bandwidth.

On the other hand, maximum RB allocation depends on OFDM waveform as it is stated in Table 4.5.3.1-1 and Table 4.5.3.2-1 in [3].

Table 4.5.3.1-1: Range 2 NR UE and BS maximum RB allocation for CP-OFDM

| SCS [kHz] | BS / UE Channel bandwidths [MHz] |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 | 100 | 200 | 400 |
| 60 | 66 | 132 | 264 | N.A |
| 120 | 32 | 66 | 132 | 264 |

Table 4.5.3.2-1: Range 2 NR UE maximum RB allocation for DFT-S-OFDM

| SCS [kHz] | BS / UE Channel bandwidths [MHz] |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 | 100 | 200 | 400 |
| 60 | 64 | 128 | 264 | N.A |
| 120 | 32 | 64 | 128 | 264 |

Analyzing above tables, maximum spectrum utilization per channel bandwidth and SCS is obtained for CP-OFDM waveform. RB allocation defined for DFT-s-OFDM is the closest number lower of equal to CP-OFDM satisfying the RB allocation is multiple of 2, 3 or 5.

Observation 7: CP-OFDM waveform allows to maximize the spectrum utilization for the same channel bandwidth and SCS compared to DFT-s-OFDM waveform.

Observation 8: CP-OFDM has a larger MPR than DFT-s-OFDM. In order to optimize the SNR during the measurement and to reduce the influence of the MU, RAN5 has endorsed the selection of DFT-s-OFDM in [4].

Proposal 5: Select maximum transmission bandwidth configuration NRB for the selected channel bandwidth and SCS.

Proposal 6: Select DFT-s-OFDM waveform in order to optimize the SNR and reduce the measurement uncertainty.

In LTE and NR FR1, the lowest modulation scheme is used to measure the occupied bandwidth, as it is the most robust modulation. Similarly to LTE and [2] for 5G NR FR1, the lowest modulation scheme supported for the CP-OFDM is selected.

Proposal 7: Select QPSK modulation to test Occupied Bandwidth.

  - Exceptions for OBW UL MIMO test

The requirements of OBW for UL MIMO are referring back to basic Occupied bandwidth test case 6.5.1, therefor the test point selection in proposal 1~5 can be reused without any change.

However according to [5], CP-OFDM is mandatory in case UL-MIMO transmission with two layer data, therefore CP-OFDM QPSK is selected as tested modulation type.

Proposal 8: For OBW for UL MIMO testing, test points selected in proposal 1~5 can be reused, and CP-OFDM QPSK is adopted.

- Conclusion

It is proposed that RAN5 discusses and agrees the following proposals for open areas described in section 2 of this document to progress on SA FR2 Occupied Bandwidth test case definition:

Proposal 1: Define Test Environment as Normal Conditions (NC) for Occupied Bandwidth measurement in FR2.

Proposal 2: Select Low range, Mid range and High range as test frequencies.

Proposal 3: Select all channel bandwidths defined in a band to measure Occupied Bandwidth.

Proposal 4: Select the lowest SCS supported for the current channel bandwidth to test.

Proposal 5: Select maximum transmission bandwidth configuration NRB for the selected channel bandwidth and SCS.

Proposal 6: Select DFT-s-OFDM waveform in order to optimize the SNR and reduce the measurement uncertainty.

Proposal 7: Select QPSK modulation to test Occupied Bandwidth.

Proposal 8: For OBW for UL MIMO testing, test points selected in proposal 1~5 can be reused, and CP-OFDM QPSK is adopted.

- References

[1] R4-1802404, “38.101-2 Implementation of endorsed CRs on to 38.101-2”, Qualcomm Incorporated, RAN4#86, Athens, Greece, Feb – March 2018.

[2] R5-180885, “Discussion on test point selection for NR Occupied Bandwidth in FR1”, Keysight Technologies, RAN5#78, Athens, Greece, Feb – March 2018.

[3] TR 38.817-01 v1.0.0, “General aspects for UE RF for NR”.

[4] R5-202458, “On the MU of FR2 OBW, Rohde & Schwarz”, 3GPP TSG RAN WG 5 Meeting #87-e, May 2020

[5] R5-192024,” Discussion on the waveform selection for UL MIMO tests”, Huawei, RAN5#82, Athens, Feb.2019