3GPP TSG-RAN WG5 Meeting #91-e	R5-212781

Online, 17th May 2021 – 28th May 2021

Source:	Qualcomm Incorporated

Title:	Discussion paper for Rel-15 NR Tests Applicability on SNPN Only UE

# Document for:	Agreement

Agenda Item:	6.3.18.6

## 1.	Introduction

Standalone Non Public networks (SNPN) were introduced in Rel-16 and work is currently commencing in RAN5 for conformance specifications, which are tracking core specifications. There may be a possiblility that a UE can be SNPN ONLY UE, i.e designed to operate on a Standalone Network only. For such UE, we may have to provide coverage for applicable legacy NR (Rel-15) test cases.

## 2.	Discussion

While RAN5 ensure coverage is provided for explicit requirements mentioned in core spec, for both SNPN and CAG based NPN, there may be a industry requirement for certification of SNPN only UE.

Folllowing procedures are not defined in Rel-16 for a UE operating in SNPN, for ex-

- PLMN Selection

- IRAT to LTE

- CAG

- Autonomous Search

- IMS

- Emergency

- HO, Roaming

Problem 1

However, there are still several core spec mandatory requirements which may have to be tested. For example, out of existing  NR conformance specifications [1] for R15 UE’s, how many of those tests can be run and what shall be enforced vs what is optional?

Problem 2

After applicability, testability needs to be resolved. We can derive a similar resemblace from the in past RAN5 WI for Rel-13 eMTC (CAT M1) where applicable legacy LTE test cases were evaluated and if any test could be run with simple test environment change ( like broadcasting SIB1-BR on CAT M1\M2 bands) then we made it applicable, while all other tests were specifically marked NOT applicable for CAT M1 [2]

Similar approach can be taken for SNPN only UE, for applicable R15 NR test case can be run in an SNPN environment . Existing NR cells can be replaced with SNPN cells NR Cell broadcasting default SNPN IDs as indicated in TS 38.508-1 [3] Table 4.4.2-4. and with appropriate System information combination defined in TS 38.508-1 [3] clause 4.4.3.1.2.

To add, RAN5 has more R16 WI for verticals such as IIOT, NR-U or eURLLC and some of the test case in these areas might also become applicable for SNPN Only UE and needs to addressed for applicability to SNPN as well.

## 3.	Proposal/Way Forward

1. 	Explore and catalogue the applicable test cases from Rel-15 NR specifications which might be applicable to Rel-16 SNPN only UE and define applicability.

2. 	Explore the testability aspects without introdcing new test case for test cases in 1) above.

## 4.	References

[1]	TS 38.523-1: 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; 5GS; User Equipment (UE) conformance specification; Part 1: Protocol

[2]	 TS 36.523-1: 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; EUTRA; User Equipment (UE) conformance specification; Part 1: Protocol

[3]	TS 38.508-1: 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; User Equipment (UE) radio transmission and reception User Equipment (UE) conformance specification;