3GPP TSG-RAN5 Meeting #91-e	R5-212885

# Online, , 17th May 2021 - 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2234 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of clearing RPLMN in Idle mode TCs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc., Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-05-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to current implementation, AT/MMI command is used to clear the RPLMN in the preamble, but a UE may not have MMI or not support the AT+CRSM command which is optional in TS 27.007, therefore such UE will not be able to run these case. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated preamble of impacted test cases by using updated USIM configuration. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A UE not supporting AT/MMI command will not be able to run these cases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.1.2.3.1, 6.2.1.1.3.1, 6.2.1.2.3.1, 6.2.1.3.3.1, 6.3.1.1.3.1, 6.3.1.2.3.1, 6.3.1.3.3.1, 6.3.1.4.3.1, 6.3.1.5.3.1, 6.3.1.8.3.1 and 6.3.1.9.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.508-1 CR 1798 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN impacted. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

[Start of modification]

6.1.1.2.3	Test description

6.1.1.2.3.1	Pre-test conditions

System Simulator:

-	Four inter-frequency multi-PLMN cells as specified in TS 38.508-1 [4] clause 4.4.1.1.3 are configured broadcasting default PLMNs as indicated in TS 38.508-1 [4] Table 4.4.2-3.

-	The PLMNs are identified in the test by the identifiers in Table 6.1.1.2.3.1-1.

Table 6.1.1.2.3.1-1: PLMN identifiers

| NR Cell | PLMN name |
| --- | --- |
| 1 | PLMN1 |
| 12 | PLMN2 |
| 13 | PLMN3 |
| 11 | PLMN4 |

UE:

-	The UE is in Automatic PLMN selection mode.

-	The UE is equipped with a USIM configuration as defined in TS 38.508-1 [4] Table 6.4.1-4.

Preamble:

-	The UE is in state Switched OFF (state 0-A).

[Skip unchanged text]

6.2.1.1.3	Test description

6.2.1.1.3.1	Pre-test conditions

System Simulator

-	3 NR cells as specified in TS 38.508-1 [4] table 4.4.2-3. System information combination NR-1 as defined in Ts 38.508-1 [4] sub-clause 4.4.3.1.2 is applied to all the NR cells.

-	1 cell belongs to E-UTRA. System information combination 1 as defined in TS 36.508 [7] clause 4.4.3.1 is used.

-	PLMN settings are defined in TS 36.523-1 [13] table 6.0.1-1.

Table 6.2.1.1.3.1-1: Cell PLMN identities

| Cell | PLMN name |
| --- | --- |
| NR Cell 1 | PLMN2 |
| NR Cell 12 | PLMN13 |
| NR Cell 13 | PLMN14 |
| E-UTRA Cell 1 | PLMN13 |

UE

-	The UE is in Automatic PLMN selection mode.

-	The HPLMN is PLMN1.

-	USIM configuration 12 as specified in tables 6.4.1-12 in TS 38.508-1 [4] will be used.

Preamble

-	The UE is in state Switched OFF (state 0-A) according to TS 38.508-1 [4].

[Skip unchanged text]

6.2.1.2.3	Test description

6.2.1.2.3.1	Pre-test conditions

System Simulator

-	NR Cell 1 and NR Cell 12 as specified in TS 38.508-1 [4] table 4.4.2-3 are configured as shown in Table 6.2.1.2.3.2-1. System information combination NR-1 as defined in Ts 38.508-1 [4] sub-clause 4.4.3.1.2 is applied to all the NR cells.

-	E-UTRA Cell 1 belongs to E-UTRA (defined in TS 36.508 [7]). System information combination 1 as defined in TS 36.508 [7] clause 4.4.3.1 is used.

-	PLMN settings are defined in TS 36.523-1 [13] table 6.0.1-1.

Table 6.2.1.2.3.1-1: Cell PLMN identities

| Cell | PLMN name |
| --- | --- |
| NR Cell 1 | PLMN2 |
| NR Cell 12 | PLMN13 |
| E-UTRA Cell 1 | PLMN13 |

UE

-	The UE is in Automatic PLMN selection mode.

-	The HPLMN is PLMN1.

-	USIM configuration 7 as specified in tables 6.4.1-7 in TS 38.508-1 [4] will be used.

Preamble

-	The UE is in state Switched OFF (state 0N-B) according to TS 38.508-1 [4].

[Skip unchanged text]

6.2.1.3.3	Test description

6.2.1.3.3.1	Pre-test conditions

System Simulator

-	2 NR cells as specified in TS 38.508-1 [4] table 4.4.2-3 are configured as shown in Table 6.2.1.3.3.2-1. System information combination NR-1 as defined in Ts 38.508-1 [4] sub-clause 4.4.3.1.2 is applied to all the NR cells.

-	1 cell belongs to E-UTRA. System information combination 1 as defined in TS 36.508 [7] clause 4.4.3.1 is used.

-	PLMN settings are defined in TS 36.523-1 [13] table 6.0.1-1.

Table 6.2.1.3.3.1-1: Cell PLMN identities

| Cell | PLMN name |
| --- | --- |
| NR Cell 1 | PLMN16 |
|  | PLMN15 |
| NR Cell 12 | PLMN16 |
|  | PLMN17 |
| E-UTRA Cell 1 | PLMN17 |
|  | PLMN16 |

UE

-	The UE is in Automatic PLMN selection mode.

-	The HPLMN is PLMN1.

-	USIM configuration 8 as specified in tables 6.4.1-8 in TS 38.508-1 [4] will be used.

Preamble

-	The UE is in state Switched OFF (state 0N-B) according to TS 38.508-1 [4].

[Skip unchanged text]

6.3.1.1.3	Test Description

6.3.1.1.3.1	Pre-test conditions

System Simulator:

Three inter-frequency multi-PLMN NR Cells as specified in TS 38.508-1 [4] Table 4.4.2-1 are configured broadcasting PLMNs as indicated in Table 6.3.1.1.3.1-1.

The PLMNs are identified in the test by the identifiers in Table 6.3.1.1.3.1-1. The MCC and MNC values corresponding to the PLMN identifier shall understood as those specified in TS 36.523-1 [42], Table 6.0.1-1.

Table 6.3.1.1.3.1–1: PLMN identifiers

| NR Cell | PLMN names |
| --- | --- |
| NR Cell 11 | PLMN2 |
| NR Cell 12 | PLMN13 |
| NR Cell 13 | PLMN14 |

NR Cell 11 is set to ''Serving Cell”;

NR Cell 12 is set to ''Serving Cell'';

NR Cell 13 is set to “Serving Cell'';

System Information Combination NR-4 as defined in TS 38.508-1 [4] clause 4.4.3.1.3 is used in NR cells.

UE:

-	The UE is in Automatic PLMN selection mode.

-	USIM configuration as defined in Table 6.4.1-10 in TS 38.508-1 [4] will be used.

Preamble:

-	The UE is in Switched OFF (State 0-A) as defined in TS 38.508-1 [4] Table 4.4A.2-0.

[Skip unchanged text]

6.3.1.2.3	Test Description

6.3.1.2.3.1	Pre-test conditions

System Simulator:

-	Three inter-frequency multi-PLMN NR Cells as specified in TS 38.508-1 [4] Table 4.4.2-1 are configured broadcasting PLMNs as indicated in Table 6.3.1.2.3.1-1.

-	The PLMNs are identified in the test by the identifiers in Table 6.3.1.2.3.1-1. The MCC and MNC values corresponding to the PLMN identifier shall understood as those specified in TS 36.523-1 [42], Table 6.0.1-1.

Table 6.3.1.2.3.1–1: PLMN identifiers

| NR Cell | PLMN names |
| --- | --- |
| NR Cell 11 | PLMN2 |
| NR Cell 12 | PLMN13 |
| NR Cell 13 | PLMN14 |

NR Cell 11 is set to ''Serving Cell”;

NR Cell 12 is set to ''Serving Cell'';

NR Cell 13 is set to “Serving Cell'';

System Information Combination NR-4 as defined in TS38.508-1 [4] clause 4.4.3.1.3 is used in NR cells.

UE:

-	The UE is in Automatic PLMN selection mode.

-	USIM configuration as defined in Table 6.4.1-10 in TS 38.508-1 [4] will be used.

Preamble:

-	The UE is in Switched OFF (State 0-A) as defined in TS 38.508-1 [4] Table 4.4A.2-0.

[Skip unchanged text]

6.3.1.3.3	Test Description

6.3.1.3.3.1	Pre-test conditions

System Simulator:

-	Three inter-frequency multi-PLMN NR Cells as specified in TS 38.508-1 [4] Table 4.4.2-1 are configured broadcasting PLMNs as indicated in Table 6.3.1.3.3.1-1.

-	The PLMNs are identified in the test by the identifiers in Table 6.3.1.3.3.1-1. The MCC and MNC values corresponding to the PLMN identifier shall understood as those specified in TS 36.523-1 [42], Table 6.0.1-1.

Table 6.3.1.3.3.1-1: PLMN identifiers

| NR Cell | PLMN names |
| --- | --- |
| NR Cell 11 | PLMN2 |
| NR Cell 12 | PLMN13 |
| NR Cell 13 | PLMN14 |

NR Cell 11 is set to ''Serving Cell'';

NR Cell 12 is set to ''Serving Cell'';

NR Cell 13 is set to “Serving Cell'';

System Information Combination NR-4 as defined in TS 38.508-1 [4] clause 4.4.3.1.3 is used in NR cells.

UE:

-	The UE is in Automatic PLMN selection mode.

-	USIM configuration as defined in Table 6.4.1-10 of TS 38.508-1 [4] will be used.

Preamble:

-	The UE is in Switched OFF (State 0-A) as defined in TS 38.508-1 [4] Table 4.4A.2-0.

[Skip unchanged text]

6.3.1.4.3	Test Description

6.3.1.4.3.1	Pre-test conditions

System Simulator:

-	Two inter-frequency multi-PLMN NR Cells as specified in TS 38.508-1 [4] Table 4.4.2-1 are configured broadcasting PLMNs as indicated in Table 6.3.1.4.3.1-1.

-	The PLMNs are identified in the test by the identifiers in Table 6.3.1.4.3.1-1. The MCC and MNC values corresponding to the PLMN identifier shall understood as those specified in TS 36.523-1 [42], Table 6.0.1-1.

Table 6.3.1.4.3.1-1: PLMN identifiers

| NR Cell | PLMN names |
| --- | --- |
| NR Cell 11 | PLMN2 |
| NR Cell 12 | PLMN13 |

NR Cell 11 is set to "Serving Cell";

NR Cell 12 is set to "Serving Cell";

System Information Combination NR-4 as defined in TS 38.508 [4] clause 4.4.3.1.3 is used in NR cells.

UE:

-	The UE is in Manual PLMN selection mode.

-	USIM configuration as defined in Table 6.4.1-10 in TS 38.508-1 [4] will be used.

Preamble:

-	The UE is in Switched OFF (State 0-A) as defined in TS 38.508-1 [4] Table 4.4A.2-0.

[Skip unchanged text]

6.3.1.5.3	Test Description

6.3.1.5.3.1	Pre-test conditions

System Simulator:

-	Three inter-frequency multi-PLMN NR Cells as specified in TS 38.508-1 [4] Table 4.4.2-1 are configured broadcasting PLMNs as indicated in Table 6.3.1.5.3.1-1.

-	The PLMNs are identified in the test by the identifiers in Table 6.3.1.5.3.1-1. The MCC and MNC values corresponding to the PLMN identifier shall understood as those specified in TS 36.523-1 [42], Table 6.0.1-1.

Table 6.3.1.5.3.1-1: PLMN identifiers

| NR Cell | PLMN names |
| --- | --- |
| NR Cell 11 | PLMN2 |
| NR Cell 12 | PLMN13 |
| NR Cell 13 | PLMN14 |

NR Cell 11 is set to ''Serving Cell'';

NR Cell 12 is set to ''Serving Cell'';

NR Cell 13 is set to ''Serving Cell'';

System Information Combination NR-4 as defined in TS 38.508-1 [4] clause 4.4.3.1.3 is used in NR cells.

UE:

-	The UE is in Automatic PLMN selection mode.

-	USIM configuration as defined in Table 6.4.1-10 of TS 38.508-1 [4] will be used.

Preamble:

-	The UE is in Switched OFF (State 0-A) as defined in TS 38.508-1 [4] Table 4.4A.2-0.

[Skip unchanged text]

6.3.1.8.3	Test Description

6.3.1.8.3.1	Pre-test conditions

System Simulator:

-	Three inter-frequency multi-PLMN NR Cells as specified in TS 38.508-1 [4] Table 4.4.2-1 are configured broadcasting PLMNs as indicated in Table 6.3.1.8.3.1-1.

-	The PLMNs are identified in the test by the identifiers in Table 6.3.1.8.3.1-1. The MCC and MNC values of PLMN identifiers are specified in TS 36.523-1 [13], Table 6.0.1-1.

Table 6.3.1.8.3.1-1: PLMN identifiers

| NR Cell | PLMN names |
| --- | --- |
| NR Cell 11 | PLMN2 |
| NR Cell 12 | PLMN13 |
| NR Cell 13 | PLMN14 |

NR Cell 11 is set to "Serving Cell";

NR Cell 12 is set to "Serving Cell";

NR Cell 13 is set to "Serving Cell";

System Information Combination NR-4 as defined in TS 38.508-1 [4] clause 4.4.3.1.3 is used in NR cells.

UE:

-	The UE is in Automatic PLMN selection mode.

-	USIM configuration as defined in Table 6.4.1-21 of TS 38.508-1 [4] will be used.

Preamble:

-	The UE is in Switched OFF (State 0-A) as defined in TS 38.508-1 [4] Table 4.4A.2-0.

[Skip unchanged text]

6.3.1.9.3	Test Description

6.3.1.9.3.1	Pre-test conditions

System Simulator:

-	Two inter-frequency multi-PLMN NR Cells as specified in TS 38.508-1 [4] Table 4.4.2-1 are configured broadcasting PLMNs as indicated in Table 6.3.1.9.3.1-1.

-	The PLMNs are identified in the test by the identifiers in Table 6.3.1.9.3.1-1. The MCC and MNC values of PLMN identifiers are specified in TS 36.523-1 [13], Table 6.0.1-1.

Table 6.3.1.9.3.1-1: PLMN identifiers

| NR Cell | PLMN names |
| --- | --- |
| NR Cell 11 | PLMN2 |
| NR Cell 12 | PLMN13 |

NR Cell 11 is set to ''Serving Cell'';

NR Cell 12 is set to ''Serving Cell'';

System Information Combination NR-4 as defined in TS38.508 clause 4.4.3.1.3 is used in NR cells.

UE:

-	The UE is in Manual PLMN selection mode.

-	USIM configuration as defined in Table 6.4.1-21 of TS 38.508-1 [4] will be used.

Preamble:

-	The UE is in Switched OFF (State 0-A) as defined in TS 38.508-1 [4] Table 4.4A.2-0.

[End of modification]