3GPP TSG-RAN5 Meeting 91-e	R5-212960

# Electric Meeting, 17th  May  – 28th Mar. 2021

Title:							On MU for FR2 Blocker Test

Source:			Anritsu

Agenda Item:			5.3.2.17

Document for:			Endorsement

# 1.	Introduction

# In this paper, we propose to blocker signal ACLR impact for FR2 blocking test e.g. ACS and IBB.

# 2.	Discussion

# In general, the ACLR of the modulated blocker signal will become bigger for wider CBW as more intermodulation products arise. As of now, the FR2 blocker test case tested bandwidth is limited to 100MHz due to the testability issue for high DL power issue, then ACLR impact could not become so big comparing to FR1. Hence, we propose to apply +/- 0.7dB for ACLR effect for FR2 blocking test case for the current assumption for testability limit (i.e. with slight relaxation for interferer power and test only BW ≤ 100MHz). It was already agreed with [ ] in [1], hence we propose to remove [ ] and fix the MU for FR2 blocker test.

# Proposal 1 : Propose to remove [ ] from the blocker signal ACLR impact (adopt 0.7dB) for FR2 ACS and IBB

# 3.	Conclusion

# Proposal 1 : Propose to remove [ ] from the blocker signal ACLR impact (adopt 0.7dB) for FR2 ACS and IBB

# 4.	References

# [1] R5-212960, “On MU for FR2 Blocker Test”, Anritsu, RAN5#99