3GPP TSG-RAN5 Meeting #91-e	R5-212983

# Electronic Meeting, 17th – 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 1255 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updating Transmitter power for CA requiements for CA_n28A-n41A |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R16-UEConTest |  |  |  |  |  | Date: |  |  | 2021-05-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The minimum requirements and test requirement of MOP for CA and ΔTIB,c for CA is absent for CA_n28A-n41A. They need to be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updating clause 6.2A.1 and 6.2A.4.0 for CA_n28A-n41A |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Transmitter power for CA can’t be tested for CA_n28A-n41A |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2A.1, 6.2A.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Resubmission of R5-210913 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

6.2A.1	UE maximum output power for CA

6.2A.1.0	Minimum conformance requirements

6.2A.1.0.1	Void

6.2A.1.0.2	Void

6.2A.1.0.3	UE maximum output power for Inter-band CA

For inter-band carrier aggregation with one uplink carrier assigned to one NR band, the transmitter power requirements in subclause 6.2.1.3 apply.

For inter-band carrier aggregation with uplink assigned to two NR bands, UE maximum output power shall be measured over all component carriers from different bands. If each band has separate antenna connectors, maximum output power is measured as the sum of maximum output power at each UE antenna connector. The period of measurement shall be at least one sub frame (1 ms). The maximum output power is specified in Table 6.2A.1.0.3-1.

Table 6.2A.1.0.3-1 UE Power Class for uplink inter-band CA (two bands)

| NR CA Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n1A-n78A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n1A-n79A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n3A-n41A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n3A-n78A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n3A-n79A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n5A-n78A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n5A-n79A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n8A-n78A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n28A-n41A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n39A-n41A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n40A-n41A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n41A-n79A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n50A-n78A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n66A-n71A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n70A-n71A |  |  |  |  | 23 | +2/-32 |  |  |
| NOTE 1: Void NOTE 2: 2 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 3: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 4: For inter-band carrier aggregation the maximum power requirement should apply to the total transmitted power over all component carriers (per UE). NOTE 5: Power class 3 is the default power class unless otherwise stated |  |  |  |  |  |  |  |  |

<Unchanged Text Skipped>

6.2A.1.1.5	Test requirement

The maximum output power for CA, derived in step 6 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2A.1.1.5-1 for Inter-band 2 UL CA configuration. The test requirement of configurations for CA operating band including Band n41 also apply for the corresponding CA operating bands with Band n90 replacing Band n41.

Table 6.2A.1.1.5-1: Maximum Output Power test requirement for Power Class 3

| NR CA Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n1A-n78A |  |  |  |  | 23 | +2+TT/-32-TT |  |  |
| CA_n1A-n79A |  |  |  |  | 23 | +2+TT/-32-TT |  |  |
| CA_n3A-n41A |  |  |  |  | 23 | +2+TT/-32-TT |  |  |
| CA_n3A-n78A |  |  |  |  | 23 | +2+TT/-32-TT |  |  |
| CA_n3A-n79A |  |  |  |  | 23 | +2+TT/-32-TT |  |  |
| CA_n5A-n78A |  |  |  |  | 23 | +2+TT/-32-TT |  |  |
| CA_n5A-n79A |  |  |  |  | 23 | +2+TT/-32-TT |  |  |
| CA_n8A-n78A |  |  |  |  | 23 | +2+TT/-32-TT |  |  |
| CA_n28A-n41A |  |  |  |  | 23 | +2+TT/-32-TT |  |  |
| CA_n39A-n41A |  |  |  |  | 23 | +2+TT/-32-TT |  |  |
| CA_n40A-n41A |  |  |  |  | 23 | +2+TT/-32-TT |  |  |
| CA_n41A-n79A |  |  |  |  | 23 | +2+TT/-32-TT |  |  |
| CA_n50A-n78A |  |  |  |  | 23 | +2+TT/-32-TT |  |  |
| CA_n66A-n71A |  |  |  |  | 23 | +2+TT/-32-TT |  |  |
| CA_n70A-n71A |  |  |  |  | 23 | +2+TT/-32-TT |  |  |
| NOTE 1: Void NOTE 2: 2 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 3: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 4: For inter-band carrier aggregation the maximum power requirement should apply to the total transmitted power over all component carriers (per UE). NOTE 5: Power class 3 is the default power class unless otherwise stated NOTE 6: The TT for 2UL CA Maximum Output Power is in the maximum TT among all UL CCs. For TT of each UL CC refer to Table6.2A.1.1.5-2. |  |  |  |  |  |  |  |  |

Table 6.2A.1.1.5-2: Test Tolerance (UE maximum output power)

| TT for overall output power (dB) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | PCell |  |  |  |
|  |  |  | BW ≤ 40MHz |  | 40MHz < BW ≤ 100MHz |  |
|  |  |  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0GHz | f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0GHz |
| SCell | BW ≤ 40MHz | f ≤ 3.0GHz | 0.7 | 1.0 | 1.0 | 1.0 |
|  |  | 3.0GHz < f ≤ 6.0GHz | 1.0 | 1.0 | 1.0 | 1.0 |
|  | 40MHz < BW ≤ 100MHz | f ≤ 3.0GHz | 1.0 | 1.0 | 1.0 | 1.0 |
|  |  | 3.0GHz < f ≤ 6.0GHz | 1.0 | 1.0 | 1.0 | 1.0 |

For the UE which supports inter-band NR CA configuration, SUL configuration or inter-band EN-DC configuration, ΔTIB,c as specified in 6.2A.4.0.2 for NR CA, clause 6.2C.2 for SUL, or TS 38.101-3 [4] clause 6.2B.4.2 for EN-DC applies. Unless otherwise stated, ΔTIB,c is set to zero. In case the UE supports more than one of band combinations for CA, SUL or DC, and an operating band belongs to more than one band combinations then

<Unchanged Text Skipped>

6.2A.4.0.2	ΔTIB,c for CA

6.2A.4.0.2	ΔTIB,c for CA

For the UE which supports inter-band NR CA configuration, ΔTIB,c in tables below applies. Unless otherwise stated, ΔTIB,c is set to zero.

6.2A.4.0.2.1	FFS

6.2A.4.0.2.2	FFS

6.2A.4.0.2.3	ΔTIB,c for Inter-band CA

Table 6.2A.4.0.2.3-1: ΔTIB,c due to NR CA (two bands)

| Inter-band CA combination | NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| CA_n1-n77 | n1 | 0.6 |
|  | n77 | 0.8 |
| CA_n1-n78 | n1 | 0.3 |
|  | n78 | 0.8 |
| CA_n3-n41 | n3 | 0.5 |
|  | n41 | 0.34 |
|  |  | 0.85 |
| CA_n3-n77 | n3 | 0.6 |
|  | n77 | 0.8 |
| CA_n3-n78 | n3 | 0.6 |
|  | n78 | 0.8 |
| CA_n3-n79 | n3 | 0.3 |
|  | n79 | 0.8 |
| CA_n8-n75 | n8 | 0.3 |
| CA_n8-n78 | n8 | 0.6 |
|  | n78 | 0.8 |
| CA_n8-n79 | n8 | 0.3 |
|  | n79 | 0.8 |
| CA_n28-n41 | n28 | 0.3 |
|  | n41 | 0.3 |
| CA_n28-n75 | n28 | 0.3 |
| CA_n28-n78 | n28 | 0.5 |
|  | n78 | 0.8 |
| CA_n39-n41 | n39 | 02 |
|  | n41 | 02 |
|  | n39 | 0.53 |
|  | n41 | 0.53 |
| CA_n41-n781 | n41 | 0.3 |
|  | n78 | 0.8 |
| CA_n41-n79 | n41 | 0.3 |
|  | n79 | 0.8 |
| CA_n66-n70 | n66 | 0.5 |
|  | n70 | 0.5 |
| CA_n66-n71 | n66 | 0.3 |
|  | n71 | 0.3 |
| CA_n70-n71 | n70 | 0.3 |
|  | n71 | 0.6 |
| CA_n75-n78 | n78 | 0.8 |
| CA_n76-n78 | n78 | 0.8 |
| CA_n77-n79 | n77 | 0.5 |
|  | n79 | 0.5 |
| CA_n78-n79 | n78 | 0.5 |
|  |  | 1.56 |
|  | n79 | 0.5 |
|  |  | 1.56 |
| NOTE 1: The requirements only apply when the sub-frame and Tx-Rx timings are synchronized between the component carriers. In the absence of synchronization, the requirements are not within scope of these specifications. NOTE 2: Only applicable for UE supporting inter-band carrier aggregation with uplink in one NR band and without simultaneous Rx/Tx. NOTE 3: Applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx. NOTE 4: The requirement is applied for UE transmitting on the frequency range of 2515-2690 MHz. NOTE 5: The requirement is applied for UE transmitting on the frequency range of 2496-2515 MHz. NOTE 6: The requirements only apply for UE supporting inter-band carrier aggregation with simultaneous Rx/Tx capability, and NR UL carrier frequencies are confined to 3700 MHz-3800MHz for n78 and 4400 MHz-4500MHz for n79. Simultaneous Rx/Tx capability does not apply for UEs supporting band n78 with a n77 implementation. |  |  |

<End of Changes>