Title: 	Discussion on test point selection for TX spurious emission for intra-band EN-DC

Introduction

Generic initial test conditions, such as environmental conditions, test frequencies, channel bandwidths, subcarrier spacing (SCS) were discussed in previous RAN5 presentation [1] [2]. The general agreement is that FR1 testing mostly follows similar requirement as for LTE. A recent paper [3] also referenced to an agreed common uplink configuration table for all TX test cases.

In another RAN5 paper (R5-185892), it is proposed that NR test point selection shall follow what is specified in SA scenarios.

The purpose of this paper is to discuss test point selection of E-UTRA and NR carriers for TX spurious emission measurement for intra-band contiguous/non-contiguous EN-DC. Considering there are so many EN-DC combos expected to be supported, it is reasonable to carefully investigate the possibility of reducing test points. Since the objective of 38.521-3 specification is provide conformance testing requirement for NR in the case of EN-DC scenarios, the number of LTE test points can be reduced.

Discussion

From TS 38.521-1, the initial conditions used for NR general/UE co-existence spurious emission is given below:

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1. |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1. |  | Low range, Mid range, High range (NOTE 2) |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1. |  | Lowest, Mid, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  | Lowest |  |
| Test Parameters |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |
|  | N/A | Modulation | RB allocation (NOTE 1) |
| 1 |  | CP-OFDM QPSK | Outer_Full |
| 2 |  | CP-OFDM QPSK | Edge_1RB_Left |
| 3 |  | CP-OFDM QPSK | Edge_1RB_Right |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 Common UL configuration. NOTE 2: For NR band n28, 30MHz test channel bandwidth is tested with Low range and High range test frequencies. |  |  |  |

As a reference, the initial test condition used for LTE CA (intra-band contiguous DL CA and UL CA) UE co-existence spurious emission is listed below:

Table 6.6.3.2A.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] clause 4.1 |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] clause 4.3.1 for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  | Low range, High range |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in clause 5.4.2A.1 for the CA Configuration |  |  |  | Lowest NRB_agg Highest NRB_agg (Note 2) |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |
| CA Configuration / NRB_agg |  | DL Allocation | CC MOD | UL Allocation |  |  |  |  |
| PCC NRB | SCCs NRB | PCC & SCC RB allocation |  | NRB_alloc | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
| 25 | 50 | N/A | QPSK | 2 | P_1@0 | S_1@49 |  |  |
| 25 | 50 |  | QPSK | 75 | P_25@0 | S_50@0 |  |  |
| 50 | 25 |  | QPSK | 2 | P_1@0 | S_1@24 |  |  |
| 50 | 25 |  | QPSK | 75 | P_50@0 | S_25@0 |  |  |
| 50 | 50 |  | QPSK | 2 | P_1@0 | S_1@49 |  |  |
| 50 | 50 |  | QPSK | 100 | P_50@0 | S_50@0 |  |  |
| 75 | 75 |  | QPSK | 2 | P_1@0 | S_1@74 |  |  |
| 75 | 75 |  | QPSK | 150 | P_75@0 | S_75@0 |  |  |
| 100 | 25 |  | QPSK | 2 | P_1@0 | S_1@24 |  |  |
| 100 | 25 |  | QPSK | 125 | P_100@0 | S_25@0 |  |  |
| 100 | 50 |  | QPSK | 2 | P_1@0 | S_1@49 |  |  |
| 100 | 50 |  | QPSK | 150 | P_100@0 | S_50@0 |  |  |
| 100 | 75 |  | QPSK | 2 | P_1@0 | S_1@74 |  |  |
| 100 | 75 |  | QPSK | 175 | P_100@0 | S_75@0 |  |  |
| 100 | 100 |  | QPSK | 2 | P_1@0 | S_1@99 |  |  |
| 100 | 100 |  | QPSK | 200 | P_100@0 | S_100@0 |  |  |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1. Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg , only the combination with the highest NRB_PCC is tested. |  |  |  |  |  |  |  |  |

The initial test condition used for LTE CA (intra-band non-contiguous DL CA and UL CA) UE co-existence spurious emission is listed below:

Table 6.6.3.2A.3.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  |  | Refer to test points A: N/A (Range is not relevant), Maximum Wgap(low + high frequency) |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  | Refer to test points Test only test points with Lowest NRB for PCC and SCC Highest NRB for PCC and SCC |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |
| ID | CA Configuration / NRB_agg |  | Wgap [MHz] | DL Allocation |  | UL Allocation |  |  |  |
|  | PCC NRB | SCCs NRB |  | CC MOD | PCC & SCC RB allocation | CC MOD | PCC NRB_alloc (LCRB @ RBstart) | CC MOD | SCC NRB_alloc (LCRB @ RBstart) |
| Test Parameters for CA_4A-4A Configurations |  |  |  |  |  |  |  |  |  |
| 1 | 25 | 25 | 35 | N/A | N/A | QPSK | P_1@0 | QPSK | S_1@24 |
| 2 | 25 | 25 | 35 | N/A | N/A | QPSK | P_25@0 | QPSK | S_25@0 |
| 3 | 100 | 100 | 5 | N/A | N/A | QPSK | P_1@0 | QPSK | S_1@99 |
| 4 | 100 | 100 | 5 | N/A | N/A | QPSK | P_100@0 | QPSK | S_100@0 |

For spurious emissions, it is reasonable to make the test configuration for EN-DC follow what is specified in LTE CA scenarios or NR CA scenarios.

Test Environment

Considering EN-DC testing scenario is very similar to LTE CA test case, similar Test Environment should be used for EN-DC testing.

Proposal 1: Propose Test Environment as Normal for spurious emission test cases in EN-DC mode.

Test Frequencies selection

In LTE intra-band contiguous DL CA and UL CA TX scenario, both Low range and High range are selected for spurious emission test cases. In LTE intra-band non-contiguous DL CA and UL CA TX spurious emission testing, Maximum WGap is selected at both low and high range for spurious emission test cases.

For intra-band EN-DC, it is reasonable to follow similar test frequencies selection as for LTE CA to test spurious emissions.

Proposal 2: For intra-band contiguous EN-DC, both Low range and High range shall be selected for spurious emission test cases. For intra-band non-contiguous EN-DC, Maximum WGap is selected at both low and high range for spurious emission test cases.

Test Channel Bandwidth selection

Tx spurious emission testing is to verify that the unwanted spurious emissions out of band meet the minimum requirements defined by RAN4. In LTE CA scenario, two extreme bandwidth combinations corresponding to Lowest NRB_agg and Highest NRB_agg are selected for spurious emission test cases.

Similarly, lowest and highest aggregated channel bandwidth shall be selected for spurious emission test cases in EN-DC mode. If the UE supports multiple channel BW Combinations in the EN-DC Configuration with the same aggregated channel bandwidth, only the combination with highest NR channel BW is tested.

Proposal 3: Lowest and highest aggregated channel bandwidth combinations shall be selected for spurious emission test cases in EN-DC mode. (Lowest NRB_agg and Highest NRB_agg).

Proposal 4: If UE supports multiple channel BW Combinations in the EN-DC configuration with the same aggregated channel bandwidth, only the combination with highest NR channel BW shall be tested.

RB allocation and RB location selection

In LTE intra-band contiguous/non-contiguous DL CA and UL CA spurious emission testing, the uplink RB allocation is set as “1@0 for PCC, 1@RBmax for SCC” and “NRB_agg@0 for PCC, NRB_agg@0 for SCC”, respectively.

For intra-band contiguous/non-contiguous EN-DC, it is reasonable to follow similar RB allocation and RB location selection as for LTE CA to test spurious emissions. Therefore, it is proposed to use the RB configuration described in Table 2.4-1 for UE co-existence spurious emission for intra-band contiguous/non-contiguous EN-DC.

Table 2.4-1: Test configuration table

| Test Parameters |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test ID | Downlink Configuration | EN-DC Uplink Configuration |  |  |  |
|  |  | E-UTRA Cell |  | NR Cell |  |
|  |  | Modulation | RB allocation (NOTE 2) | Modulation | RB allocation (NOTE 1) |
| 1 | N/A for Spurious emission. | QPSK | Outer_1RB_Left | CP-OFDM QPSK | Edge_1RB_Right |
| 2 |  | QPSK | Outer_Full | CP-OFDM QPSK | Outer_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 in TS 38.521-1 [8]. NOTE 2: Outer_Full defined as the transmission bandwidth configuration NRB per channel bandwidth for the E-UTRA component as indicated in TS 36.521-1 [10] Table 5.4.2-1. Outer_1RB_Left defined as 1 RB allocated at the left edge of the E-UTRA component. Outer_1RB_Right defined as 1 RB allocated at the right edge of the E-UTRA component. |  |  |  |  |  |

Proposal 5: Propose the same RB configuration as for LTE CA test cases to test UE co-existence spurious emissions for intra-band contiguous/non-contiguous EN-DC as described in Table 2.4-1.

Modulation selections

For LTE and NR SA spurious emissions measurement, the modulation QPSK and CP-OFDM are used for the uplink modulation, respectively.

For intra-band EN-DC, there is no particular reason to deviate from the selected modulation in LTE and NR SA scenarios.

Proposal 6: Propose the same modulation scheme as for LTE and NR SA testing for E-UTRA carrier and NR carrier for EN-DC spurious emission test cases.

Number of Test Points for each EN-DC combo

Table 2.6-1 Maximum Number of Test Points for UE co-existence spurious emission for each EN-DC combo

| Environmental conditions | Maximum Number of Frequencies | Maximum Number of ChBW | Number SCS | Number of RB allocations | Maximum Number of Test Points |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 2 | 1 | 2 | 8 |

Conclusion

In conclusion, the following test points selection are proposed for intra-band EN-DC in spurious emission measurement:

Proposal 1: Propose Test Environment as Normal for spurious emission test cases in EN-DC mode.

Proposal 2: For intra-band contiguous EN-DC, both Low range and High range shall be selected for spurious emission test cases. For intra-band non-contiguous EN-DC, Maximum WGap is selected at both low and high range for spurious emission test cases.

Proposal 3: Lowest and highest aggregated channel bandwidth combinations shall be selected for spurious emission test cases in EN-DC mode. (Lowest NRB_agg and Highest NRB_agg).

Proposal 4: If UE supports multiple channel BW Combinations in the EN-DC configuration with the same aggregated channel bandwidth, only the combination with highest NR channel BW shall be tested.

Proposal 5: Propose the same RB configuration as for LTE CA test cases to test UE co-existence spurious emissions for intra-band contiguous/non-contiguous EN-DC as described in Table 2.4-1.

Proposal 6: Propose the same modulation scheme as for LTE and NR SA testing for E-UTRA carrier and NR carrier for EN-DC spurious emission test cases.

References

[1] R5-180065, “Discussion about Test parameters for NR TRx TCs“, NTT DOCOMO, RAN5 Ad hoc, Munich, Jan. 2018.

[2] 3GPP TS 38.101-1 v15.0.0 User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone (Release 15)

[3] 3GPP TS 38.521-1 v0.4.0 User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 1 Standalone (Release 15)

[4] 3GPP TR 38.817-01 v0.3.0 General aspects for UE RF for NR (Release 15)

[5] R5-181895, “TP for adding Mid channel BW definition in TS 38.508-1”

[6] R5-180886, “Discussion_NR_FR1_TestPoints_For_SEM”

[7] R5-191451r1, “Resolving ambiguity in LTE anchor agnostic approach”