3GPP TSG-RAN5 Meeting #91-e	R5-213094

# Online, , 17th May 2021 - 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2244 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR5GC test case 10.1.4.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-05-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As per the current Prose for TP1, the PDU Session Establishment retransmissions should contain same values of S-NSSAI and DNN which were present during the PDU Session Establishment in preamble. Quote with { the UE in 5GMM-REGISTERED state and at least one PDU session has been established. the SS sends PDU SESSION RELEASE COMMAND with #39 "reactivation requested", and the UE has sent a PDU SESSION ESTABLISHMENT REQUEST message } ensure that { when { On the 1st, 2nd, 3rd, 4th expiry of timer T3580 } then { the UE retransmits PDU SESSION ESTABLISHMENT REQUEST message for the same [S-NSSAI, DNN] values provided in PDU SESSION RELEASE COMMAND } } Unquote However, there is no specific message content for UL NAS Transport received during preamble that ensures that the S-NSSAI and DNN are present. It is proposed to add a special message content for UL NA Transport that checks S-NSSAI and DNN to be present. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added a new Table 10.1.4.1.3.3-3 that specifie S-NSSAI and DNN to be present in the UL NAS Transport received during preamble. Also corrected the derivation Table reference for Table 10.1.4.1.3.3-2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TP not properly tested. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | Associated with TTCN CR R5s210383 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 10.1.4	UE-requested PDU session establishment

#### 10.1.4.1	UE-requested PDU session establishment / Abnormal / T3580

10.1.4.1.1	Test Purpose (TP)

(1)

with { the UE in 5GMM-REGISTERED state and at least one PDU session has been established. the SS sends PDU SESSION RELEASE COMMAND with #39 "reactivation requested", and the UE has sent a PDU SESSION ESTABLISHMENT REQUEST message }

ensure that {

when { On the 1st, 2nd, 3rd, 4th expiry of timer T3580 }

then { the UE retransmits PDU SESSION ESTABLISHMENT REQUEST message for the same [S-NSSAI, DNN] values provided in PDU SESSION RELEASE COMMAND }

}

(2)

with { the UE in 5GMM-REGISTERED state and has sent a PDU SESSION ESTABLISHMENT REQUEST message }

ensure that {

when { on the maximum 5th expiry of timer T3580 }

then { the UE aborts the procedure }

}

10.1.4.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.501, clause 6.4.1.6. Unless otherwise stated these are Rel-15 requirements.

[TS 24.501, clause 6.4.1.6]

The following abnormal cases can be identified:

a)	Expiry of timer T3580

The UE shall, on the first expiry of the timer T3580, retransmit the PDU SESSION ESTABLISHMENT REQUEST message and the PDU session information which was transported together with the initial transmission of the PDU SESSION ESTABLISHMENT REQUEST message and shall reset and start timer T3580, if still needed. This retransmission can be repeated up to four times, i.e. on the fifth expiry of timer T3580, the UE shall abort the procedure, release the allocated PTI and enter the state PROCEDURE TRANSACTION INACTIVE.

10.1.4.1.3	Test description

10.1.4.1.3.1	Pre-test conditions

System Simulator:

-	NGC Cell A.

UE:

-	None.

Preamble:

-	The UE is in state 3N-A on NGC Cell A with at least one PDU session X (1 <= X <= 15) active according to 38.508-1 [4].

10.1.4.1.3.2	Test procedure sequence

Table 10.1.4.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The generic test procedure in TS 38.508-1 clause 4.9.21 for PDU Session Release is performed with PDU SESSION RELEASE COMMAND message includes 5GSM cause #39 "reactivation requested" to release PDU session X. | - | - | - | - |
| 2-15 | Void. | - | - | - | - |
| 16 | The UE transmits a PDU SESSION ESTABLISHMENT REQUEST message and the S-NSSAI and DNN in UL NAS TRANSPORT message are the same values as released (Note 2). | --> | 5GMM: UL NAS TRANSPORT 5GSM: PDU SESSION ESTABLISHMENT REQUEST | - | - |
| 17 | The SS waits 16 seconds (T3580). | - | - | - | - |
| 18 | Check: Does the UE re-transmit the PDU SESSION ESTABLISHMENT REQUEST message and the S-NSSAI and DNN in UL NAS TRANSPORT message are the same values as sent in step 16? | --> | 5GMM: UL NAS TRANSPORT 5GSM: PDU SESSION ESTABLISHMENT REQUEST | 1 | P |
| 19 | The SS waits 16 seconds (2nd expiry of T3580). | - | - | - | - |
| 20 | Check: Does the UE re-transmit the PDU SESSION ESTABLISHMENT REQUEST message and the S-NSSAI and DNN in UL NAS TRANSPORT message are the same values as sent in step 16? | --> | 5GMM: UL NAS TRANSPORT 5GSM: PDU SESSION ESTABLISHMENT REQUEST | 1 | P |
| 21 | The SS waits 16 seconds (3rd expiry of T3580). | - | - | - | - |
| 22 | Check: Does the UE re-transmit the PDU SESSION ESTABLISHMENT REQUEST message and the S-NSSAI and DNN in UL NAS TRANSPORT message are the same values as sent in step 16? | --> | 5GMM: UL NAS TRANSPORT 5GSM: PDU SESSION ESTABLISHMENT REQUEST | 1 | P |
| 23 | The SS waits 16 seconds (4th expiry of T3580). | - | - | - | - |
| 24 | Check: Does the UE re-transmit the PDU SESSION ESTABLISHMENT REQUEST message and the S-NSSAI and DNN in UL NAS TRANSPORT message are the same values as sent in step 16? | --> | 5GMM: UL NAS TRANSPORT 5GSM: PDU SESSION ESTABLISHMENT REQUEST | 1 | P |
| 25 | The SS waits 16 seconds (5th expiry of T3580). | - | - | - | - |
| 26 | Check: Does the UE re-transmit the PDU SESSION ESTABLISHMENT REQUEST message and the S-NSSAI and DNN in UL NAS TRANSPORT message are the same values as sent in step 16 in the next 20 seconds? (Note 1) | --> | 5GMM: UL NAS TRANSPORT 5GSM: PDU SESSION ESTABLISHMENT REQUEST | 2 | F |
| Note 1: The 20 seconds is chosen randomly as long as it is greater than T3580 = 16 seconds. Note 2 : At step 16 the SS waits for 1s for the UE to transmit the PDU Session Establishment REQ. Upon expiry of 1s, the PDU Session Establishment may be performed by MMI or AT command. |  |  |  |  |  |

10.1.4.1.3.3	Specific message contents

Table 10.1.4.1.3.3-0: PDU SESSION ESTABLISHMENT ACCEPT (preamble; step 3, Table 4.5A.2.2.2-1, TS 38.508-1 [4])

| Derivation path: TS 38.508-1 [4], Table 4.7.2-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Authorized QoS rules |  | 5GC QoS rule of the Config#1 in Table 4.8.4-1 |  |  |
| Mapped EPS bearer contexts |  |  |  |  |
| Mapped EPS bearer context |  |  |  |  |
| Mapped EPS QoS parameters |  | EPC default bearer context of the Config#1 in Table 4.8.4-1 |  |  |
| DNN |  | The same DNN value as sent in the UL NAS TRANSPORT message during (preamble; step 2a1, Table 4.5A.2.2.2-2, TS 38.508-1 [4]) |  |  |

Table 10.1.4.1.3.3-1: PDU SESSION RELEASE COMMAND (Step 1, Table 10.1.4.1.3.2-1; step 1, TS 36.508 [4] Table 4.9.21.2.2-1)

| Derivation path: TS 38.508-1 [4], table 4.7.2-14 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| PDU session ID | X | ID of the active PDU session in preamble |  |
| 5GSM cause | ‘0010 0111’B | Reactivation requested |  |

Table 10.1.4.1.3.3-2: UL NAS TRANSPORT (Step 16, 18, 20, 22, 24 Table 10.1.4.1.3.2-1)

| Derivation path: TS 38.508-1 [4], table 4.7.1-10 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| S-NSSAI | Same as the S-NSSAI of PDU session X in the Preamble |  |  |
| DNN | Same as the DN of PDU session X in the Preamble |  |  |

Table 10.1.4.1.3.3-3: UL NAS TRANSPORT (Preamble, step 2a1, Table 4.5A.2.2.2-2, TS 38.508-1 [4])

| Derivation Path: TS 38.508-1 [4] Table 4.7.1-10 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| S-NSSAI |  | Present but contents not checked |  |  |
| DNN |  | Present but contents not checked |  |  |