3GPP TSG-RAN5 Meeting # 91-e	R5-213168

# Electronic Meeting,  17th May - 28th May

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 1928 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Testing frequencies update for band n39 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-04-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | R5-206623 was not properly implemented in the specification regarding GSN and absoluteFrequencySSB [ARFCN] for SCS 15 kHz and channel bandwidth 5 MHz. Additionally, there is one testable frequency for such case closer to the mid of the band. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Mid frequency corrected for SCS 15 kHz and channel bandwidth 5 MHz. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Affected test frequency will remain untestable. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1.1.1.39 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of change >>

4.3.1.1.1.39	Reference test frequencies for NR operating band n39

Table 4.3.1.1.1.39-1: Test frequencies for NR operating band n39 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink | Low | 1882.5 | 376500 | 1880.25 | 376050 | 0 | 15 | 4707 | 376590 | 0 | 1 | 2 (4) | 5 |
|  |  | & | Mid | 1899.1 | 379820 | 1878.49 | 375698 | 102 |  | 4747 | 379730 | 0 | 0 | 0 (0) | 102 |
|  |  | Uplink | High | 1917.5 | 383500 | 1824.53 | 364906 | 504 |  | 4793 | 383530 | 4 | 1 | 1 (2) | 507 |
| 10 | 52 | Downlink | Low | 1885 | 377000 | 1880.32 | 376064 | 0 | 30 | 4714 | 377090 | 6 | 2 | 1 (6) | 8 |
|  |  | & | Mid | 1900 | 380000 | 1876.96 | 375392 | 102 |  | 4750 | 379970 | 2 | 3 | 0 (2) | 107 |
|  |  | Uplink | High | 1915 | 383000 | 1819.6 | 363920 | 504 |  | 4789 | 383090 | 6 | 2 | 1 (6) | 512 |
| 15 | 79 | Downlink | Low | 1887.5 | 377500 | 1880.39 | 376078 | 0 | 30 | 4712 | 377050 | 0 | 1 | 1 (6) | 7 |
|  |  | & | Mid | 1900 | 380000 | 1874.53 | 374906 | 102 |  | 4744 | 379490 | 4 | 3 | 0 (2) | 107 |
|  |  | Uplink | High | 1912.5 | 382500 | 1814.67 | 362934 | 504 |  | 4773 | 381870 | 0 | 0 | 0 (2) | 506 |
| 20 | 106 | Downlink | Low | 1890 | 378000 | 1880.46 | 376092 | 0 | 30 | 4713 | 377070 | 2 | 1 | 1 (6) | 7 |
|  |  | & | Mid | 1900 | 380000 | 1872.1 | 374420 | 102 |  | 4738 | 379010 | 6 | 3 | 0 (2) | 107 |
|  |  | Uplink | High | 1910 | 382000 | 1809.74 | 361948 | 504 |  | 4760 | 380890 | 2 | 0 | 0 (2) | 506 |
| 25 | 133 | Downlink | Low | 1892.5 | 378500 | 1880.53 | 376106 | 0 | 30 | 4714 | 377090 | 4 | 1 | 1 (6) | 7 |
|  |  | & | Mid | 1900 | 380000 | 1869.67 | 373934 | 102 |  | 4732 | 378530 | 8 | 3 | 0 (2) | 107 |
|  |  | Uplink | High | 1907.5 | 381500 | 1804.81 | 360962 | 504 |  | 4750 | 379970 | 0 | 2 | 0 (2) | 508 |
| 30 | 160 | Downlink | Low | 1895 | 379000 | 1880.6 | 376120 | 0 | 30 | 4712 | 377050 | 10 | 3 | 0 (2) | 5 |
|  |  | & | Mid | 1900 | 380000 | 1867.24 | 373448 | 102 |  | 4726 | 378050 | 10 | 3 | 0 (2) | 107 |
|  |  | Uplink | High | 1905 | 381000 | 1799.88 | 359976 | 504 |  | 4737 | 378990 | 2 | 2 | 0 (2) | 508 |
| 40 | 216 | Downlink | Low | 1900 | 380000 | 1880.56 | 376112 | 0 | 30 | 4714 | 377090 | 2 | 1 | 1 (6) | 7 |
|  |  | & | Mid | 1900 | 380000 | 1862.2 | 372440 | 102 |  | 4714 | 377090 | 2 | 1 | 1 (6) | 109 |
|  |  | Uplink | High | 1900 | 380000 | 1789.84 | 357968 | 504 |  | 4714 | 377090 | 2 | 1 | 1 (6) | 511 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 for SSB_SCS = 15kHz and Table 13-3 for SSB_SCS=30kHz in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.39-2: Test frequencies for NR operating band n39 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 1885 | 377000 | 1880.68 | 376136 | 0 | 30 | 4714 | 377090 | 6 | 0 | 3 (3) | 6 |
|  |  | & | Mid | 1900 | 380000 | 1858.96 | 371792 | 102 |  | 4750 | 379970 | 14 | 0 | 1 (1) | 206 |
|  |  | Uplink | High | 1915 | 383000 | 1729.24 | 345848 | 504 |  | 4789 | 383090 | 6 | 0 | 3 (3) | 1014 |
| 15 | 38 | Downlink | Low | 1887.5 | 377500 | 1880.66 | 376132 | 0 | 30 | 4712 | 377050 | 18 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 1900 | 380000 | 1856.44 | 371288 | 102 |  | 4744 | 379490 | 22 | 0 | 1 (1) | 206 |
|  |  | Uplink | High | 1912.5 | 382500 | 1724.22 | 344844 | 504 |  | 4773 | 381870 | 6 | 0 | 0 (0) | 1008 |
| 20 | 51 | Downlink | Low | 1890 | 378000 | 1880.82 | 376164 | 0 | 30 | 4713 | 377070 | 14 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 1900 | 380000 | 1854.1 | 370820 | 102 |  | 4738 | 379010 | 18 | 0 | 1 (1) | 206 |
|  |  | Uplink | High | 1910 | 382000 | 1719.38 | 343876 | 504 |  | 4760 | 380890 | 2 | 0 | 0 (0) | 1008 |
| 25 | 65 | Downlink | Low | 1892.5 | 378500 | 1880.8 | 376160 | 0 | 30 | 4714 | 377090 | 22 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 1900 | 380000 | 1851.58 | 370316 | 102 |  | 4732 | 378530 | 2 | 0 | 2 (2) | 208 |
|  |  | Uplink | High | 1907.5 | 381500 | 1714.36 | 342872 | 504 |  | 4750 | 379970 | 6 | 0 | 1 (1) | 1010 |
| 30 | 78 | Downlink | Low | 1895 | 379000 | 1880.96 | 376192 | 0 | 30 | 4712 | 377050 | 22 | 0 | 1 (1) | 2 |
|  |  | & | Mid | 1900 | 380000 | 1849.24 | 369848 | 102 |  | 4726 | 378050 | 22 | 0 | 1 (1) | 206 |
|  |  | Uplink | High | 1905 | 381000 | 1709.52 | 341904 | 504 |  | 4737 | 378990 | 2 | 0 | 1 (1) | 1010 |
| 40 | 106 | Downlink | Low | 1900 | 380000 | 1880.92 | 376184 | 0 | 30 | 4714 | 377090 | 14 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 1900 | 380000 | 1844.2 | 368840 | 102 |  | 4714 | 377090 | 14 | 0 | 2 (2) | 208 |
|  |  | Uplink | High | 1900 | 380000 | 1699.48 | 339896 | 504 |  | 4714 | 377090 | 14 | 0 | 2 (2) | 1012 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.39-3: Test frequencies for NR operating band n39 and SCS 60 kHz without CORESET#0

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Downlink | Low | 1885 | 377000 | 1881.04 | 376208 | 0 | 30 | - | 376928 |
|  |  | & | Mid | 1900 | 380000 | 1822.6 | 364520 | 102 |  | - | 379928 |
|  |  | Uplink | High | 1915 | 383000 | 1548.16 | 309632 | 504 |  | - | 382928 |
| 15 | 18 | Downlink | Low | 1887.5 | 377500 | 1881.02 | 376204 | 0 | 30 | - | 376924 |
|  |  | & | Mid | 1900 | 380000 | 1820.08 | 364016 | 102 |  | - | 379424 |
|  |  | Uplink | High | 1912.5 | 382500 | 1543.14 | 308628 | 504 |  | - | 381924 |
| 20 | 24 | Downlink | Low | 1890 | 378000 | 1881.36 | 376272 | 0 | 30 | - | 376992 |
|  |  | & | Mid | 1900 | 380000 | 1817.92 | 363584 | 102 |  | - | 378992 |
|  |  | Uplink | High | 1910 | 382000 | 1538.48 | 307696 | 504 |  | - | 380992 |
| 25 | 31 | Downlink | Low | 1892.5 | 378500 | 1881.34 | 376268 | 0 | 30 | - | 376988 |
|  |  | & | Mid | 1900 | 380000 | 1815.4 | 363080 | 102 |  | - | 378488 |
|  |  | Uplink | High | 1907.5 | 381500 | 1533.46 | 306692 | 504 |  | - | 379988 |
| 30 | 38 | Downlink | Low | 1895 | 379000 | 1881.32 | 376264 | 0 | 30 | - | 376984 |
|  |  | & | Mid | 1900 | 380000 | 1812.88 | 362576 | 102 |  | - | 377984 |
|  |  | Uplink | High | 1905 | 381000 | 1528.44 | 305688 | 504 |  | - | 378984 |
| 40 | 51 | Downlink | Low | 1900 | 380000 | 1881.64 | 376328 | 0 | 30 | - | 377048 |
|  |  | & | Mid | - | - | - | - | - |  | - | - |
|  |  | Uplink | High | - | - | - | - | - |  | - | - |
| Note: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |

<< End of change>>