3GPP TSG-RAN5 Meeting #91-e	R5-213180

# Electronic Meeting, 17th May 2021 – 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 1939 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update chapter 4.5.2 RRC_IDLE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-05-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The option without unrestricted PDNs is missing for connectivity E-UTRA/EPC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Additional procedure and generic procedure parameter have been added for E-UTRA/EPC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Connectivities procedures will be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of first modified section>

### 4.5.2	RRC_IDLE

#### 4.5.2.1	Initiation

The SS shall:

1>	if connectivity is EN-DC:

2>	use 1 E-UTRA cell and 1 NR cell, default parameters;

2>	if connected without release is not present:

3>	perform according to the table 4.5.2.2-1: E-UTRA RRC_IDLE;

1>	if connectivity is E-UTRA/EPC:

2>	use 1 E-UTRA cell, default parameters;

2>	if unrestricted nr PDN is not present:

3>	perform according to the table 4.5.2.2-1: E-UTRA RRC_IDLE;

2>	else:

3>	perform according to the table 4.5.2.2-5: E-UTRA RRC_IDLE Unrestricted nr PDN:

3>	if pc_noOf_PDNsNewConnection > 0:

4>	perform according to the table 4.5.2.2-6: E-UTRA RRC_IDLE Unrestricted nr PDN Extension;

1>	if connectivity is NR:

2>	use 1 NR cell, default parameters;

2>	perform according to the table 4.5.2.2-2: NR RRC_IDLE;

2> if pc_noOf_PDUsNewConnection > 0

3> perform according to the table 4.5.2.2-4: NR RRC_IDLE Extension;

1>	if connectivity is WLAN:

2>	use 1 WLAN cell, default parameters;

2>	if connected without release is not present:

3>	perform according to the table 4.5.2.2-3: WLAN Ipsec_SA_Released;

2>	else:

3>	Not defined:

1>	if connectivity is NR-DC:

2>	use 2 NR cells, default parameters;

2>	perform according to the table 4.5.2.2-2: NR RRC_IDLE.

Editor's Note:	The value of specify “default parameters” is FFS. For NR-DC the requirement is 2 cells, but details might be required for specific test cases. For other connectivities the same assumption applies for 1 cell.

<End of modified section>