3GPP TSG RAN Meeting #91-e	 	R5-213314

Electronic Meeting, 17th May – 28th May 2021

Source:		Rohde & Schwarz

Title:	On the SNR of FR2 TRx test cases for PC1

Agenda Item:		5.3.2.17

Document for:	Discussion and Endorsement

# Introduction

At previous meeting, estimation for the SNR for PC1 MOP TCs have been discussed [1]. In this document, we present the results of our analysis based on the common assumptions.

# Discussion

In Table 1, we present our results for SNRtotal under the assumption that the UE is allowed to transmit at a maximum peak EIRP of 55 dBm.

Table 1: SNRtotal based on core requirement and impact of noise

| Test case | Frequency (GHz) | Requirement (dBm) | Estimated SNRtotal(dB) | Impact of Noise w/o relaxation (dB) | Required Relaxation |
| --- | --- | --- | --- | --- | --- |
| MOP (Min Peak EIRP, PC1) | 23.45 GHz ≤ f ≤ 32.125 GHz | 40 | ≥ 31.9+ ChBW | 0.01 (uprounded) | N/A |
|  | 32.125 GHz ≤ f ≤ 40.8 GHz | 38 | ≥ 31.3+ ChBW | 0.01 (uprounded) | N/A |
| MOP (TRP, PC1) | Work in progress |  |  |  |  |
| MOP (Spherical Coverage, PC1) | Work in progress |  |  |  |  |
| NOTE 1: ChBW = 10log10(400MHz/ChBW), 9.03dB for ChBW=50MHz. |  |  |  |  |  |

Proposal 1: Apply the impact due to noise and the relaxation summarized in Table 1 for the derivation of the MU of these TCs for PC1.

# Assumptions

| ID | Description | Assumption |
| --- | --- | --- |
| #1 | Frequency ranges under consideration | All Rel-15 FR2 bands for in-band measurements |
| #2 | Size of QZ for IFF | 30 cm |
| #3 | UE power class | PC1 |
| #4 | Max EIRP | EIRP MAX= +55 dBm (rms) |
| #5 | Wanted signal PAPR | 13 dB |
| #6 | Temperature range of the test equipment | 20°C – 35°C |
| #7 | Channel bandwidth | 400 MHz |

# Conclusion

In summary, SNR estimations for PC1 test cases have been provided and the influence of noise has been analyzed.

Proposal 1: Apply the impact due to noise and the relaxation summarized Table 1 for the derivation of the MU of these TCs for PC1.

# References

- R5-211275, Measurement uncertainties for PC1 devices, Keysight Technologies, GPP TSG RAN WG5 Meeting #90-e, February 2021