3GPP TSG-RAN5 Meeting #91-e	R5-213315

# Electronic Meeting, 17th May – 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.508 | CR | 1363 | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of test frequencies for Bands 7 and 65 for NB-IoT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_bands_R16_M1_NB1-UEConTest |  |  |  |  |  | Date: |  |  | 2021-05-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Test frequencies for Bands 7 and 65 for NB-IoT are missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Test frequencies for Bands 7 and 65 have been introduced in clauses 8.1.3.1.1.7 and 8.1.3.1.1.65. Clause 8.1.3.1 has been updated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Band 7 and Band 65 cannot be tested for NB-IoT. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.3.1, 8.1.3.1.1.6, 8.1.3.1.1.7, 8.1.3.1.1.32, 8.1.3.1.1.65 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

### 8.1.3	NB-IoT Reference test conditions

This clause contains the reference test conditions, which apply to all NB-IoT test cases unless otherwise specified.

#### 8.1.3.1	NB-IoT Test frequencies

NB-IoT is designed to operate in the E-UTRA operating bands 1, 2, 3, 4, 5, 7, 8, 11, 12, 13, 14, 17, 18, 19, 20, 21, 25, 26, 28, 31, 65, 66, 70, 71, 72, 73, 74, 85, 87 and 88 which are defined in Table 5.5-1 in [21]. NB-IoT system operates in HD-FDD duplex mode.

The test frequencies are based on the E-UTRA frequency bands defined in the core specifications.

The raster spacing is 100 KHz.

E-UTRA/FDD is designed to operate in paired bands of 3GPP TS 36.101 [27]. The reference test frequencies for the RF and Signalling test environment for each of the operating bands are defined in sub clause 8.1.3.1.1.

##### 8.1.3.1.1	NB-IoT FDD Mode Test frequencies

< Unchanged sections omitted >

8.1.3.1.1.6	Void

8.1.3.1.1.7	NB-IoT FDD reference test frequencies for operating band 7

Table 8.1.3.1.1.7-1: NB-IoT standalone Test frequencies for operating band 7

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 20751 | 0 | 2500.1000 | 2751 | -0.5 | 2620.1000 |
| Mid Range | 21100 | 0 | 2535.0000 | 3100 | -0.5 | 2655.0000 |
| High Range | 21449 | 0 | 2569.9000 | 3449 | -0.5 | 2689.9000 |
| NOTE 1: Applicable to either 3.75 kHz or 15 kHz NB-IoT UL subcarrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.7-2: NB-IoT in-band Test frequencies for operating band 7

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range1 | 20757 | 0 | 2500.7000 | 2757 | -2 | 2620.6925 |
| Low Range2 | 20810 | -2 | 2505.9900 | 2810 | -1 | 2625.9975 |
| Low Range3 | 20819 | -2 | 2506.8900 | 2819 | -1 | 2626.8975 |
| Mid Range1 | 21091 | 0 | 2534.1000 | 3091 | -2 | 2654.0925 |
| Mid Range2 | 21110 | -2 | 2535.9900 | 3110 | -1 | 2655.9975 |
| Mid Range3 | 21119 | -2 | 2536.8900 | 3119 | -1 | 2656.8975 |
| High Range1 | 21443 | 0 | 2569.3000 | 3443 | 1 | 2689.2975 |
| High Range2 | 21410 | -2 | 2565.9900 | 3410 | -1 | 2685.9975 |
| High Range3 | 21419 | -2 | 2566.8900 | 3419 | -1 | 2686.8975 |
| NOTE 1: Related to LTE channel BW 5 MHz NOTE 2: Related to LTE channel BW 10 MHz and NB-IoT PRB 30 NOTE 3: Related to LTE channel BW 10 MHz and NB-IoT PRB 35 NOTE 4: Defined for NB-IoT UL subcarrier spacing 15 kHz. Also applicable for 3.75 kHz UL sub-carrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.7-3: NB-IoT guard-band Test frequencies for operating band 7

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 20751 | 0 | 2500.1000 | 2751 | 1 | 2620.1075 |
| Mid Range | 21076 | 0 | 2532.6000 | 3076 | 1 | 2652.6075 |
| High Range | 21449 | 0 | 2569.9000 | 3449 | -2 | 2689.8925 |
| NOTE 1: Related to LTE channel BW 5 MHz NOTE 2: Defined for NB-IoT UL subcarrier spacing 15 kHz. Also applicable for 3.75 kHz UL sub-carrier spacing |  |  |  |  |  |  |

< Unchanged sections omitted >

8.1.3.1.1.32 to 8.1.3.1.1.64	Void

8.1.3.1.1.65	NB-IoT FDD reference test frequencies for operating band 65

Table 8.1.3.1.1.65-1: NB-IoT standalone Test frequencies for operating band 65

| Test Frequency ID |  | NUL |  | MUL |  | Frequency of Uplink [MHz] |  | NDL |  | MDL |  | Frequency of Downlink [MHz] |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Low Range |  | 131073 |  | 0 |  | 1920.1000 |  | 65537 |  | -0.5 |  | 2110.1000 |  |
| Mid Range |  | 131522 |  | 0 |  | 1965.0000 |  | 65986 |  | -0.5 |  | 2155.0000 |  |
| High Range |  | 131971 |  | 0 |  | 2009.9000 |  | 66435 |  | -0.5 |  | 2199.9000 |  |
| NOTE 1: Applicable to either 3.75 kHz or 15 kHz NB-IoT UL subcarrier spacing |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 8.1.3.1.1.65-2: NB-IoT in-band Test frequencies for operating band 65

| Test Frequency ID |  |  | NUL |  |  | MUL |  |  | Frequency of Uplink [MHz] |  |  | NDL |  |  | MDL |  |  | Frequency of Downlink [MHz] |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Low Range1 |  |  | 131079 |  |  | 0 |  |  | 1920.7000 |  |  | 65543 |  |  | -2 |  |  | 2110.6925 |  |  |
| Low Range2 | 131132 | -2 | 1925.9900 | 65596 | -1 | 2115.9975 |
| Low Range3 |  |  | 131141 |  |  | -2 |  |  | 1926.8900 |  |  | 65605 |  |  | -1 |  |  | 2116.8975 |  |  |
| Mid Range1 |  |  | 131513 |  |  | 0 |  |  | 1964.1000 |  |  | 65977 |  |  | -2 |  |  | 2154.0925 |  |  |
| Mid Range2 |  |  | 131532 |  |  | -2 |  |  | 1965.9900 |  |  | 65996 |  |  | -1 |  |  | 2155.9975 |  |  |
| Mid Range3 |  |  | 131541 |  |  | -2 |  |  | 1966.8900 |  |  | 66005 |  |  | -1 |  |  | 2156.8975 |  |  |
| High Range1 |  |  | 131965 |  |  | 0 |  |  | 2009.3000 |  |  | 66429 |  |  | 1 |  |  | 2199.2975 |  |  |
| High Range2 |  |  | 131932 |  |  | -2 |  |  | 2005.9900 |  |  | 66396 |  |  | -1 |  |  | 2195.9975 |  |  |
| High Range3 |  |  | 131941 |  |  | -2 |  |  | 2006.8900 |  |  | 66405 |  |  | -1 |  |  | 2196.8975 |  |  |
| NOTE 1: Related to LTE channel BW 5 MHz NOTE 2: Related to LTE channel BW 10 MHz and NB-IoT PRB 30 NOTE 3: Related to LTE channel BW 10 MHz and NB-IoT PRB 35 NOTE 4: Defined for NB-IoT UL subcarrier spacing 15 kHz. Also applicable for 3.75 kHz UL sub-carrier spacing |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 8.1.3.1.1.65-3: NB-IoT guard-band Test frequencies for operating band 65

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 131073 | 0 | 1920.1000 | 65537 | 1 | 2110.1075 |
| Mid Range | 131498 | 0 | 1962.6000 | 65962 | 1 | 2152.6075 |
| High Range | 131971 | 0 | 2009.9000 | 66435 | -2 | 2199.8925 |
| NOTE 1: Related to LTE channel BW 5 MHz NOTE 2: Defined for NB-IoT UL subcarrier spacing 15 kHz. Also applicable for 3.75 kHz UL sub-carrier spacing |  |  |  |  |  |  |

< End of changes >