3GPP TSG-RAN5 Meeting #91-e	R5-213335
Electronic Meeting, 17 May– 28 May 2021

Agenda item:	5.3.2.17

Source:	Keysight Technologies

Title:	On Demod achievable SNR and Tx EVM

Document for:	Discussion and Endorsement

# Introduction

This contribution discusses maximum achievable SNR for IFF 30cm QZ system.

# Discussion

At RAN5#89e and RAN5#90e different papers were discussed to optimize maximum achievable SNR in an IFF 30cm QZ test system. It was agreed a maximum SNR of 19.2dB for FR2a at 100 MHz channel bandwidth where this value consider a total backoff of 17.7dB included signal PAPR and fading crest factor.

This maximum achievable SNR of 19.2 dB is pending to conclude if test equipments can meet with Tx DL EVM requirement and action point AP#89e.23 was defined.

| Action ID | sWG | Action | Responsible | Relevant Tdoc | Deadline | Status |
| --- | --- | --- | --- | --- | --- | --- |
| AP#89e.23 | RF | Provide input on acceptable clipping frequency due to fading and/or acceptable fading crest factor margin for FR2 Demod performance and CSI test cases Provide input on the impact to TxEVM (and consequently SNR) for higher probability of saturation of faded signal | Qualcomm, Anritsu, Keysight, E///, R&S | R5-206168, R5-205702 R5-211083 | RAN5#91e | Open |

However, maximum test system power level were discussed and agreed for IFF 30 cm as part of UE RF maximum input level requirement defined in TS 38.521-2 as follows:

Table 7.4.5-1: Maximum input level

| Rx Parameter | Units | Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| Power in Transmission Bandwidth Configuration | dBm | -51 (NOTE 2,3) for band n257, n258 and n261 -59 (NOTE 2,3) for band n260 -53 (NOTE 3,4) for band n257, n258 and n261 -61 (NOTE 3,4) for band n260 |  |  |  |
| NOTE 1: The transmitter shall be set to 4 dB below the PUMAX,f,c as defined in subclause 6.2.4, with uplink configuration specified in Table 7.3.2.3.1-2. NOTE 2: Reference measurement channel is specified in Annex A.3.3.2: QPSK, R=1/3 variant with one sided dynamic OCNG Pattern as described in Annex A. NOTE 3: The test requirements deviate from minimum requirements by 26dB relaxation for 24.25 ~ 29.5 GHz and 34 dB relaxation for 37 ~ 40 GHz. NOTE 4: Reference measurement channel is specified in Annex A.3.3.5: 256QAM, R=4/5 variant with one sided dynamic OCNG Pattern as described in Annex A. |  |  |  |  |  |

Where maximum power level of -51dBm is defined for FR2a and 400MHz CBW for meeting with QPSK modulation and -53 dBm for FR2a and 400MHz CBW for meeting with 256QAM modulation. Hence, these values already agreed meet with Tx DL EVM requirements. And considering FR2a worse case, -53dBm/400MHz should not be exceeded from DL Tx power level.

Observation 1: FR2 maximum input level requirement was analysed in UE RF reaching an agreement of -53 dBm/400MHz for meeting with 256QAM considering 13 dB backoff due to signal PAPR

Furthermore, these maximum input level value does not consider facing crest factor backoff which needs to be considered for Demod testing. Hence, considering signal PAPR and fading crest factor backoff of 17.7dB, max DL power level meeting with DL Tx EVM should be around -57.7 dBm/400MHz. Which interpolated in 100 MHz channel BW it should be around -51.7 dBm/100MHz

Proposal 1: To consider -52 dBm/100MHz as maximum DL power level to achieve DL EVM requirements for FR2a.

# Conclusion

The following observations and proposals were made in this contribution.

Observation 1: FR2 maximum input level requirement was analysed in UE RF reaching an agreement of -53 dBm/400MHz for meeting with 256QAM considering 13 dB backoff due to signal PAPR

Proposal 1: To consider -52 dBm/100MHz as maximum DL power level to achieve DL EVM requirements for FR2a

# References

- TS 38.521-2, User Equipment (UE) conformance specification; Radio transmission and reception; Part 2: Range 2 Standalone, V16.7.0 (2021-3)