3GPP TSG-RAN WG5 Meeting #91-e	R5-213348

Electronic Meeting, May 17 – May 28, 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.509 | CR | 0042 | rev | - | Current version: | 15.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding new test mode UE Tx prioritization test function |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-05-17 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This change allows to disable FR2 Tx power allocation prioritization for UL CA test cases |  |  |  |  |  |  |  |  |
| Summary of change: |  | New test function added to disable/enable FR2 Tx power allocation prioritization |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | For UL CA test cases the PCC gets all the power allocation and SCC is power starving which is not inline with the intent of UL CA testing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.8 (new); 6.8 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR is dependent on discussion paper: R5-212812 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### << Start of changes >>

## 5.8	UE Tx Prioritization test Function (TPF)

### 5.8.1	General

The UE Tx Prioritization test function (TPF) is intended to disable Tx power allocation prioritization in the UE during CA test cases following the SCC configuration in preparation for subsequent test procedures. Disabling Tx power allocation prioritization using TPF shall allow for equal power allocation between PCC and SCC.

The UE Tx Prioritization test function (TPF) is mandatory for applicable UEs operating in Frequency Range 2 CA configuration.

The SS uses the TPF disable prioritization procedure to command the UE to disable Tx power allocation prioritization.

DISBLE PRIORITIZATION

DISBLE PRIORITIZATION COMPLETE

Figure 5.8.1-1: TPF disable prioritization procedure

The SS uses the TPF enable prioritization procedure to command the UE to enable Tx power allocation prioritization.

ENABLE PRIORITIZATION

ENABLE PRIORITIZATION COMPLETE

Figure 5.8.1-2: TPF enable prioritization procedure

### 5.8.2	Disable Prioritization procedure

#### 5.8.2.1	Initiation

The SS requests the UE to disable Tx power allocation prioritization by transmitting DISABLE PRIORITIZATION message.

#### 5.8.2.2	Reception of DISABLE PRIORITIZATION message by UE

When UE receives DISABLE PRIORITIZATION message then the UE shall:

1>	if the UE is operating in FR2 AND is in RRC_CONNECTED state:

2>	Disable prioritization and transmit DISABLE PRIORITIZATION COMPLETE message

1>	else:

2>	the UE behaviour is unspecified.

### 5.8.3	Enable Prioritization procedure

#### 5.8.3.1	Initiation

The SS requests the UE to enable Tx power allocation prioritization by transmitting ENABLE PRIORITIZATION message. The SS should do this when the UE is in RRC_CONNECTED state.

#### 5.8.3.2	Reception of ENABLE PRIORITIZATION message by UE

When UE receives ENABLE PRIORITIZATION message then the UE shall:

1>	if the UE is operating in FR2 AND is in RRC_CONNECTED state AND the TPF disabled prioritization:

2>	Enable prioritization and transmit ENABLE PRIORITIZATION COMPLETE message;

1> else:

2>	the UE behaviour is unspecified.

#### 5.8.3.3	Release of disable prioritization by UE

When the UE leaves the RRC_CONNECTED state, the UE shall:

1>	if the UE is operating in FR2 AND the TPF disabled prioritization:

2>	Enable prioritization;

<< Unchanged portions skipped >>

## 6.8	Prioritization messages

### 6.8.1	DISABLE PRIORITIZATION

This message is only sent in the direction SS to UE.

| Information Element | Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- |
| Protocol discriminator | TS 24.007 [5], sub clause 11.2.3.1.1 | M | V | ½ |
| Skip indicator | TS 24.007 [5], sub clause 11.2.3.1.2 | M | V | ½ |
| Message type |  | M | V | 1 |

where message type is:

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | bit no. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 1 | 0 | 1 | 0 | 0 | 0 | octet 1 |

### 6.8.2	DISABLE PRIORITIZATION COMPLETE

This message is only sent in the direction UE to SS.

| Information Element | Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- |
| Protocol discriminator | TS 24.007 [5], sub clause 11.2.3.1.1 | M | V | ½ |
| Skip indicator | TS 24.007 [5], sub clause 11.2.3.1.2 | M | V | ½ |
| Message type |  | M | V | 1 |

where message type is:

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | bit no. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 1 | 0 | 1 | 0 | 0 | 1 | octet 1 |

### 6.8.3	ENABLE PRIORITIZATION

This message is only sent in the direction SS to UE.

| Information Element | Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- |
| Protocol discriminator | TS 24.007 [5], sub clause 11.2.3.1.1 | M | V | ½ |
| Skip indicator | TS 24.007 [5], sub clause 11.2.3.1.2 | M | V | ½ |
| Message type |  | M | V | 1 |

where message type is:

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | bit no. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | octet 1 |

### 6.8.4	ENABLE PRIORITIZATION COMPLETE

This message is only sent in the direction UE to SS.

| Information Element | Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- |
| Protocol discriminator | TS 24.007 [5], sub clause 11.2.3.1.1 | M | V | ½ |
| Skip indicator | TS 24.007 [5], sub clause 11.2.3.1.2 | M | V | ½ |
| Message type |  | M | V | 1 |

where message type is:

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | bit no. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 1 | 0 | 1 | 0 | 1 | 1 | octet 1 |

<< End of changes >>