3GPP TSG-RAN5 Meeting #91-e	R5-213439

# Electronic Meeting, 17th May 2021 - 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 1816 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to IMS call release sequences |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-04-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | For IMS call release sequences, the current contents of PDU SESSION MODIFICATION COMMAND are including the QoS rule and QoS flow configuration for IMS signalling bearer, with operation code “add new QoS rule / QoS flow”. This can give an error in UE processing as these IDs will already exist as part of the PDU session. The operations configured in the message should be a delta to the existing PDU session, and should delete the QoS rule and QoS flow that were added in the establishment of IMS voice traffic bearer, through sequences 4.9.15/4.9.16. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modify PDU SESSION MODIFICATION COMMAND contents to release QoS rule ID 3 and QoS flow ID 7. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail IMS test cases at call release step |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.9.17, 4.9.18 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Associated with TTCN CR R5s210387 R5-212378->R5-213439 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 4.9.17	Generic Test Procedure for IMS MO call release in 5GC

4.9.17.1	Scope

The purpose of this procedure is to make a UE initiated release of an ongoing IMS speech call.

4.9.17.2	Procedure description

4.9.17.2.1	Initial conditions

System Simulator:

-	1 NR Cell connected to 5GC, default parameters.

User Equipment:

-	The UE is in state 3N-A on the NR Cell with an active IMS call.

4.9.17.2.2	Procedure sequence

Table 4.9.17.2.2-1: IMS MO speech call release in 5GC

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U – S | Message |  |  |
| 1-2 | Generic test procedure for MO release of speech call, steps 1-2, as defined in annex A.7 of TS 34.229-5 [47] are performed. | - | - | - | - |
| 3 | The SS transmits an RRCReconfiguration message and an PDU SESSION MODIFICATION COMMAND | <-- | NR RRC: RRCReconfiguration 5GMM: DL NAS TRANSPORT 5GSM: PDU SESSION MODIFICATION COMMAND | - | - |
| 4 | The UE transmits an RRCReconfigurationComplete message | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 5 | Check: Does the UE transmit a ULInformationTransfer message, an UL NAS TRANSPORT message and an PDU SESSION MODIFICATION COMPLETE message? | --> | NR RRC: ULInformationTransfer 5GMM: UL NAS TRANSPORT 5GSM: PDU SESSION MODIFICATION COMPLETE | - | P |

4.9.17.2.3	Specific message contents

Table 4.9.17.2.3-1: RRCReconfiguration (step 3, Table 4.9.17.2.2-1)

| Derivation Path: Table 4.6.1-13. |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |  |
| radioBearerConfig SEQUENCE { |  |  |  |  |
| drb-ToReleaseList SEQUENCE (SIZE (1..maxDRB)) OF DRB-Identity { |  | 1 entry |  |  |
| DRB-Identity[1] |  | DRB-Identity linked to the IMS speech bearer | entry 1 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| masterCellGroup SEQUENCE { |  |  |  |  |
| rlc-BearerToReleaseList SEQUENCE (SIZE (1..maxLC-ID)) OF LogicalChannelIdentity { |  | 1 entry |  |  |
| LogicalChannelIdentity[1] |  | Same value as drb-Identity[1] above | entry 1 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 4.9.17.2.3-2: PDU SESSION MODIFICATION COMMAND (step 3, Table 4.9.17.2.2-1)

| Derivation Path: Table 4.7.1-14. |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PDU session ID |  | Same value as sent in PDU SESSION ESTABLISHMENT REQUEST message. |  |  |
| Authorized QoS rules |  |  |  |  |
| QoS rule |  |  |  |  |
| QoS rule identifier |  | ‘00000011’B | QoS rule 3 |  |
| Rule operation code |  | ‘010’B | Delete existing QoS rule |  |
| Authorized QoS flow descriptions |  |  |  |  |
| QoS flow descriptions |  |  |  |  |
| QFI |  | ‘00000111’B | QFI 7 |  |
| Operation code |  | ‘010’B | Delete existing QoS flow |  |

### 4.9.18	Generic Test Procedure for IMS MT call release in 5GC

4.9.18.1	Scope

The purpose of this procedure is to make the network release an ongoing IMS speech call.

4.9.18.2	Procedure description

4.9.18.2.1	Initial conditions

System Simulator:

-	1 NR Cell connected to 5GC, default parameters.

User Equipment:

-	The UE is in state 3N-A on the NR Cell with an active IMS call.

4.9.18.2.2	Procedure sequence

Table 4.9.18.2.2-1: IMS MT speech call release in 5GC

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U – S | Message |  |  |
| 1-2 | Generic test procedure for MT release of speech call, steps 1-2, as defined in annex A.8 of TS 34.229-5 [47] are performed. | - | - | - | - |
| 3 | The SS transmits an RRCReconfiguration message and an PDU SESSION MODIFICATION COMMAND | <-- | NR RRC: RRCReconfiguration 5GMM: DL NAS TRANSPORT 5GSM: PDU SESSION MODIFICATION COMMAND | - | - |
| 4 | The UE transmits an RRCReconfigurationComplete message | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 5 | Check: Does the UE transmit a ULInformationTransfer message, an UL NAS TRANSPORT message and an PDU SESSION MODIFICATION COMPLETE message? | --> | NR RRC: ULInformationTransfer 5GMM: UL NAS TRANSPORT 5GSM: PDU SESSION MODIFICATION COMPLETE | - | P |

4.9.18.2.3	Specific message contents

Table 4.9.18.2.3-1: RRCReconfiguration (step 3, Table 4.9.18.2.2-1)

| Derivation Path: Table 4.6.1-13. |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |  |
| radioBearerConfig SEQUENCE { |  |  |  |  |
| drb-ToReleaseList SEQUENCE (SIZE (1..maxDRB)) OF DRB-Identity { |  | 1 entry |  |  |
| DRB-Identity[1] |  | DRB-Identity linked to the IMS speech bearer | entry 1 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| masterCellGroup SEQUENCE { |  |  |  |  |
| rlc-BearerToReleaseList SEQUENCE (SIZE (1..maxLC-ID)) OF LogicalChannelIdentity { |  | 1 entry |  |  |
| LogicalChannelIdentity[1] |  | Same value as drb-Identity[1] above |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 4.9.18.2.3-2: PDU SESSION MODIFICATION COMMAND (step 3, Table 4.9.18.2.2-1)

| Derivation Path: Table 4.7.2-9. |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PDU session ID |  | Same value as sent in PDU SESSION ESTABLISHMENT REQUEST message. |  |  |
| Authorized QoS rules |  | . |  |  |
| QoS rule |  |  |  |  |
| QoS rule identifier |  | ‘00000011’B | QoS rule 3 |  |
| Rule operation code |  | ‘010’B | Delete existing QoS rule |  |
| Authorized QoS flow descriptions |  |  |  |  |
| QoS flow descriptions |  |  |  |  |
| QFI |  | ‘00000111’B | QFI 7 |  |
| Operation code |  | ‘010’B | Delete existing QoS flow |  |