3GPP TSG-RAN5 Meeting #91-e	R5-213549

# Electronic Meeting, 17th May– 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.508 | CR | 1357 | rev | 1 | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to RRC messages for LTE CHO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_feMob-UEConTest |  |  |  |  |  | Date: |  |  | 2021-05-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 1, Some IEs about conditional handover are not supplemented. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1, Add Some IEs about conditional handover |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The conditional handover related test cases may run incorrectly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.1, 4.6.5, 4.6.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from R5-212446r1. |  |  |  |  |  |  |  |  |

<Start of modified section 1>

4.6.1	Contents of RRC messages

…

–	RRCConnectionReconfiguration

Table 4.6.1-8: RRCConnectionReconfiguration

| Derivation Path: 36.331 clause 6.2.2 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Information Element |  |  | Value/remark |  | Comment |  | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |  |  |  |  |
| rrc-TransactionIdentifier |  |  | RRC-TransactionIdentifier-DL |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |  |  |  |
| c1 CHOICE{ |  |  |  |  |  |  |  |
| rrcConnectionReconfiguration-r8 ::= SEQUENCE { |  |  |  |  |  |  |  |
| measConfig |  |  | Not present |  |  |  |  |
|  |  |  | MeasConfig-DEFAULT |  |  |  | MEAS |
| mobilityControlInfo |  |  | Not present |  |  |  |  |
|  |  |  | MobilityControlInfo-HO |  |  |  | HO, HO-TO-EUTRA, HO-TO-EN-DC |
| dedicatedInfoNASList |  |  | Not present |  |  |  |  |
|  |  |  | Set according to specific message content |  |  |  | SRB2-DRB(n, m) DRB(n, m) SRB1-SRB2-DRB(n,m) AM-DRB-ADD(bid) UM-DRB-ADD(bid) DRB-REL(bid) DRB-Mod MCG_and_split EN-DC_SRB2-DRB |
| radioResourceConfigDedicated |  |  | Not present |  |  |  |  |
|  |  |  | RadioResourceConfigDedicated-SRB2-DRB(n, m) |  |  |  | SRB2-DRB(n, m) |
|  |  |  | RadioResourceConfigDedicated-DRB(n, m) |  |  |  | DRB(n, m) |
|  |  |  | RadioResourceConfigDedicated-AM-DRB-ADD(bid) |  |  |  | AM-DRB-ADD(bid) |
|  |  |  | RadioResourceConfigDedicated-UM-DRB-ADD(bid) |  |  |  | UM-DRB-ADD(bid) |
|  |  |  | RadioResourceConfigDedicated-DRB-REL(bid) |  |  |  | DRB-REL(bid) |
|  |  |  | RadioResourceConfigDedicated-HO |  |  |  | HO |
|  |  |  | RadioResourceConfigDedicated-HO-TO-EUTRA(n, m) |  |  |  | HO-TO-EUTRA(n,m)，,HO-TO-EN-DC(n,m) |
|  |  |  | RadioResourceConfigDedicated-DRB-Mod |  |  |  | DRB-Mod |
|  |  |  | RadioResourceConfigDedicated-SCell_AddMod |  |  |  | SCell_AddMod |
|  |  |  | RadioResourceConfigDedicated-PCell-PATTERN |  |  |  | PCell-PATTERN |
|  |  |  | RadioResourceConfigDedicated-EN-DC-Split |  |  |  | MCG_and_split |
|  |  |  | RadioResourceConfigDedicated-EN-DC_Split_DRB_Rel(bid) |  |  |  | EN-DC_Split_Rel(bid) |
|  |  |  | RadioResourceConfigDedicated-SRB4 |  |  |  | SRB4 |
|  |  |  | RadioResourceConfigDedicated-EN-DC-SRB2-DRB |  |  |  | EN-DC_SRB2-DRB |
| securityConfigHO |  |  | Not present |  |  |  |  |
|  |  |  | SecurityConfigHO-DEFAULT |  |  |  | HO, HO-TO-EUTRA，,HO-TO-EN-DC |
| nonCriticalExtension SEQUENCE { |  |  |  |  | RRCConnectionReconfiguration-v890-IEs |  |  |
| lateNonCriticalExtension SEQUENCE { |  |  | RRCConnectionReconfiguration-v8m0-IEs |  | 4Rx_PCC |  |
| lateNonCriticalExtension | Not present |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  | RRCConnectionReconfiguration-v10i0-IEs |  |  |  |
| antennaInfoDedicatedPCell-v10i0 SEQUENCE { |  |  |  |  |  |  |
| maxLayersMIMO-r10 | fourLayers |  |  |  |  |  |
| } |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE {} | Not present |  | RRCConnectionReconfiguration-v10l0-IEs |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  | RRCConnectionReconfiguration-v920-IEs |  |  |  |
| otherConfig-r9 | OtherConfig-r9 |  |  |  | OtherConfig |  |
| fullConfig-r9 | Not present |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  | RRCConnectionReconfiguration-v1020-IEs |  |  |  |
| sCellToReleaseList-r10 | Not present |  |  |  |  |  |
| sCellToAddModList-r10 SEQUENCE (SIZE (1..maxSCell-r10)) OF | The same number of entries as the configured SCell(s) |  | n denotes the index of the entry |  | SCell_AddMod |  |
| sCellToAddMod-r10[n] | SCellToAddMod-r10-DEFAULT |  |  |  |  |  |
| } |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  | RRCConnectionReconfiguration-v1130-IEs |  |  |  |
| systemInfomationBlockType1Dedicated-r11 | Not present |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  | RRCConnectionReconfiguration-v1250-IEs |  |  |  |
| wlan-OffloadInfo-r12 CHOICE {} | Not present |  |  |  |  |  |
| wlan-OffloadInfo-r12 CHOICE { |  |  |  |  |  |  |
| release | NULL |  |  |  | WLAN-OffloadRelease |  |
| setup SEQUENCE { |  |  |  |  | WLAN-OffloadSetup |  |
| wlan-OffloadConfigDedicated-r12 | WLAN-OffloadConfig-DEFAULT |  |  |  |  |  |
| t350-r12 | min10 |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| scg-Configuration-r12 | Not present |  |  |  |  |  |
| scg-Configuration-r12 | SCG-Configuration-r12-DEFAULT |  |  |  | PSCell_Release, PSCell_Mod, PSCell_Add_Split_DRB, PSCell_Add_SCG_DRB |  |
| sl-SyncTxControl-r12 | Not present |  |  |  |  |  |
| sl-DiscConfig-r12 | Not present |  |  |  |  |  |
| sl-CommConfig-r12 | Not present |  |  |  |  |  |
| nonCriticalExtension SEQUENCE {} | Not present |  | RRCConnectionReconfiguration-v1310-IEs |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  | RRCConnectionReconfiguration-v1310-IEs |  | LWA |  |
| lwa-configuration-r13 | Set as specified in test case |  |  |  |  |  |
| } |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  | RRCConnectionReconfiguration-v1310-IEs |  | LWIP |  |
| lwip-configuration-r13 | Set as specified in test case |  |  |  |  |  |
| } |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  | RRCConnectionReconfiguration-v1310-IEs |  | MCG_and_SCG, MCG_and _split, SCG-DRB(n,m), MCG_NR_PDCP, EN-DC_PSCell_Rel, SRB_NR_PDCP, EN-DC_EmbedNR_RRCRecon, EN-DC_Embed_RBConfig, EN-DC_PSCell_HO, CHO |  |
| nonCriticalExtension SEQUENCE { |  |  | RRCConnectionReconfiguration-v1430-IEs |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  | RRCConnectionReconfiguration-v1510-IEs |  |  |  |
| nr-Config-r15 CHOICE { |  |  |  |  |  |  |
| Release |  |  |  |  | EN-DC_PSCell_Rel |  |
| setup SEQUENCE { |  |  |  |  | MCG_and_SCG, MCG_and_split, SCG-DRB(n,m), MCG_NR_PDCP, EN-DC_EmbedNR_RRCRecon, EN-DC_Embed_RBConfig, EN-DC_PSCell_HO |  |
| endc-ReleaseAndAdd-r15 | false |  |  |  |  |  |
| nr-SecondaryCellGroupConfig-r15 | OCTET STRING including the RRCReconfiguration message and the IE secondaryCellGroup according TS 38.508-1 [67], table 4.6.1-13 with condition EN-DC. |  |  |  | MCG_and_SCG，HO-TO-EN-DC(n,m) |  |
|  | OCTET STRING including the RRCReconfiguration message, the IE secondaryCellGroup according TS 38.508-1 [67], table 4.6.1-13 with condition EN-DC and the IE PDCP-Config according TS 38.508-1 [67], table 4.6.3-99 with condition Split. |  |  |  | MCG_and_split |  |
|  | OCTET STRING including the RRCReconfiguration message and the IE secondaryCellGroup according TS 38.508-1 [67], table [4.8.1-2]. |  |  |  | SCG-DRB(n,m) |  |
|  | OCTET STRING including the test case specific RRCReconfiguration |  |  |  | EN-DC_EmbedNR_RRCRecon |  |
|  | OCTET STRING including the RRCReconfiguration message and the IE secondaryCellGroup according TS 38.508-1 [67], table 4.6.1-13 with condition EN-DC_HO |  |  |  | EN-DC_PSCell_HO |  |
| p-MaxEUTRA-r15 | Not present |  |  |  |  |  |
|  | 23 |  |  |  | SIG AND EN-DC AND pc_dynamicPowerSharing |  |
|  | 20 |  |  |  | SIG AND EN-DC AND (NOT pc_dynamicPowerSharing) |  |
|  | 10 |  |  |  | RF AND EN-DC_Tx_PC3 |  |
|  | 13 |  |  |  | RF AND EN-DC_Tx_PC2 |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| sk-Counter-r15 | 0 |  |  |  | MCG_and_SCG, MCG_and _split, SCG-DRB(n,m), HO-TO-EN-DC（n,m） |  |
|  | Not Present |  |  |  |  |  |
| nr-RadioBearerConfig1-r15 | OCTET STRING including RadioBearerConfig according TS 38.508-1 [67], [table 4.6.3-132] with condition EN-DC_DRB. |  |  |  | MCG_and_SCG, MCG_and_split，HO-TO-EN-DC（n,m） |  |
|  | OCTET STRING including RadioBearerConfig according TS 38.508-1 [67], table [4.8.1-3]. |  |  |  | SCG-DRB(n,m) |  |
|  | OCTET STRING including RadioBearerConfig according TS 38.508-1 [67], table [4.6.3-132] with condition MCG_NR_PDCP |  |  |  | MCG_NR_PDCP |  |
|  | OCTET STRING including RadioBearerConfig according TS 38.508-1 [67], table 4.6.3-132 with condition SRB_NR_PDCP |  |  |  | SRB_NR_PDCP |  |
|  | OCTET STRING including the test case specific RadioBearerConfig |  |  |  | EN-DC_Embed_RBConfig |  |
|  | OCTET STRING including RadioBearerConfig according TS 38.508-1 [67], table 4.6.3-132 with conditions EN-DC_DRB and Re-establish_PDCP |  |  |  | EN-DC_PSCell_HO AND RBConfig_KeyChange |  |
|  | OCTET STRING including RadioBearerConfig according TS 38.508-1 [67], table 4.6.3-132 with conditions EN-DC_DRB and Recover_PDCP |  |  |  | EN-DC_PSCell_HO AND RBConfig_NoKeyChange |  |
| nr-RadioBearerConfig2-r15 | Not present |  |  |  |  |  |
| tdm-PatternConfig-r15 | Not present |  |  |  |  |  |
| nonCriticalExtension SEQUENCE {} | Not present |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  | RRCConnectionReconfiguration-v1530-IEs |  | CHO |  |
| securityConfigHO-v1530 | Not present |  |  |  |  |  |
| sCellGroupToReleaseList-r15 | Not present |  |  |  |  |  |
| sCellGroupToAddModList-r15 | Not present |  |  |  |  |  |
| dedicatedInfoNASList-r15 | Not present |  |  |  |  |  |
| p-MaxUE-FR1-r15 | Not present |  |  |  |  |  |
| smtc-r15 | Not present |  |  |  |  |  |
| nonCriticalExtension | Not present |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  | RRCConnectionReconfiguration-v1610-IEs |  | CHO |  |
| conditionalReconfiguration-r16 | Not present |  |  |  |  |  |
|  | ConditionalReconfiguration-r16-HO |  |  |  | CHO |  |
| daps-SourceRelease-r16 | Not present |  |  |  |  |  |
| tdm-PatternConfig2-r16 | Not present |  |  |  |  |  |
| sl-ConfigDedicatedForNR-r16 | Not present |  |  |  |  |  |
| sl-SSB-PriorityEUTRA-r16 | Not present |  |  |  |  |  |
| nonCriticalExtension | Not present |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| SRB2-DRB(n,m) | Establishment of a SRB and DRB combination with n x AM DRB and m x UM DRB (including establishment of SRB2) |
| DRB(n,m) | Establishment of additional n x AM DRB:s and m x UM DRB:s (SRB2 should already be established) |
| AM-DRB-ADD(bid) | Establishment of a single additional AM DRB with bearer identity bid (SRB2 should already be established) |
| UM-DRB-ADD(bid) | Establishment of a single additional UM DRB with bearer identity bid (SRB2 should already be established) |
| DRB-REL(bid) | Release of the DRB with bearer identity bid |
| HO | Intra LTE handover |
| MEAS | A measurement is configured |
| HO-TO-EUTRA(n,m) | Inter-RAT handover to E-UTRA including the establishment of a SRB1, SRB2 and n x AM DRB plus m x UM DRB |
| nonFullConfig | The field is not present in case of handover within E-UTRA when the fullConfig is included or in case of handover of handover to E-UTRA; otherwise it is optional present, need ON. |
| DRB-Mod | Modification of already established DRB ID 2 and is used for sending Modify EPS Bearer Context Request message (SRB2 should already be established) |
| SCell_AddMod | Addition or modification of SCell(s) |
| PCell-PATTERN | eICIC Serving Cell Pattern |
| OtherConfig | Other Configuration is configured |
| WLAN-OffloadSetup | When Dedicated WLAN Offload parameters need to be configured in UE |
| WLAN-OffloadRelease | When Dedicated WLAN Offload parameters need to be released in UE |
| PSCell_Mod | Modification of PSCell(s) |
| PSCell_Release | Release of PSCell |
| PSCell_Add_Split_DRB | Add PSCell(s) and setup of Split DRB |
| PSCell_Add_SCG_DRB | Add PSCell(s) and setup of SCG DRB |
| 4Rx_PCC | For 4Rx testcases with more than 2 Layers on PCC |
| MCG_and_SCG | EN-DC with MCG and SCG. |
| MCG_and_split | EN-DC with MCG and split. |
| SCG-DRB(n,m) | EN-DC establishment of additional n x AM DRB:s and m x UM DRB:s in SCG. |
| EN-DC_PSCell_Rel | Release of EN-DC PSCell |
| EN-DC_Split_Rel(bid) | Release of E-UTRA leg of EN-DC Split DRB with bearer identity bid |
| MCG_NR_PDCP | EN-DC configuration of MCG DRB with NR PDCP. |
| SRB_NR_PDCP | EN-DC configuration of SRB1 and SRB2 with NR PDCP. |
| EN-DC_SRB2-DRB | Establishment of SRB2 and DRB combination for EN-DC |
| EN-DC_EmbedNR_RRCRecon | To modify NR Cell Configuration in EN-DC by including RRCReconfiguration |
| EN-DC_EmbedNR_RBConfig | To modify NR RB Configuration in EN-DC by including RadioBearerConfig |
| SRB4 | Establishment of SRB4 |
| LWA | LWA is being configured |
| LWIP | LWIP is being configured |
| HO-TO-EN-DC(n,m) | Inter-RAT handover to EN-DC including the establishment of a SRB1, SRB2 and n x AM DRB plus m x UM DRB |
| EN-DC_PSCell_HO | RRCReconfiguration to perform PSCell handover (SCG change) in EN-DC |
| RBConfig_KeyChange | RadioBearerConfig to perform PSCell handover (SCG change) with security key change in EN-DC |
| RBConfig _NoKeyChange | RadioBearerConfig to perform PSCell handover (SCG change) without security key change in EN-DC |
| EN-DC_NR_CA | Addition, modification or release of NR SCell |
| EN-DC_Tx_PC3 | Power Class 3 UE testing Power Class 3 requirements for EN-DC transmitter test cases |
| EN-DC_Tx_PC2 | Power Class 2 UE testing Power Class 2 requirements for EN-DC transmitter test cases |
| CHO | Conditional handover |

<End of modified section 1>

<Start of modified section 2>

4.6.5	Mobility control information elements

…

–	ConditionalReconfiguration-r16-HO

Table 4.6.5-2: ConditionalReconfiguration-r16-HO

| Derivation Path: 36.331 clause 6.3.4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ConditionalReconfiguration-r16-HO ::= SEQUENCE { |  |  |  |
| condReconfigurationToAddModList-r16 | Not present |  |  |
| condReconfigurationToRemoveList-r16 | Not present |  |  |
| attemptCondReconf-r16 | true |  |  |
| } |  |  |  |

<End of modified section 2>

<Start of modified section 3>

4.6.6	Measurement information elements

…

–	ReportConfigEUTRA-A3-CHO

Table 4.6.6-17: ReportConfigEUTRA-A3-CHO

| Derivation Path: 36.331 clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigEUTRA-A3 ::= SEQUENCE { |  |  |  |
| triggerType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  |  |
| purpose | reportStrongestCells |  |  |
| } |  |  |  |
| } |  |  |  |
| triggerQuantity | rsrp |  |  |
| reportQuantity | both |  |  |
| maxReportCells | 1 |  |  |
| reportInterval | min60 |  |  |
| reportAmount | r1 |  |  |
| si-RequestForHO-r9 | Not Present |  |  |
| ue-RxTxTimeDiffPeriodical-r9 | Not Present |  |  |
| includeLocationInfo-r10 | Not Present |  |  |
| reportAddNeighMeas-r10 | Not Present |  |  |
| alternativeTimeToTrigger-r12 | Not Present |  |  |
| useT312-r12 | Not Present |  |  |
| usePSCell-r12 | Not Present |  |  |
| aN-Threshold1-v1250 | Not Present |  |  |
| a5-Threshold2-v1250 | Not Present |  |  |
| reportStrongestCSI-RSs-r12 | Not Present |  |  |
| reportCRS-Meas-r12 | Not Present |  |  |
| triggerQuantityCSI-RS-r12 | Not Present |  |  |
| reportSSTD-Meas-r13 | Not Present |  |  |
| rs-sinr-Config-r13 | Not Present |  |  |
| useWhiteCellList-r13 | Not Present |  |  |
| measRSSI-ReportConfig-r13 | Not Present |  |  |
| includeMultiBandInfo-r13 | Not Present |  |  |
| ul-DelayConfig-r13 | Not Present |  |  |
| ue-RxTxTimeDiffPeriodicalTDD-r13 | Not Present |  |  |
| purpose-v1430 | Not Present |  |  |
| maxReportRS-Index-r15 | Not Present |  |  |
| includeBT-Meas-r15 | Not Present |  |  |
| includeWLAN-Meas-r15 | Not Present |  |  |
| purpose-r15 | Not Present |  |  |
| numberOfTriggeringCells-r15 | Not Present |  |  |
| a4-a5-ReportOnLeave-r15 | Not Present |  |  |
| condReconfigurationTriggerEUTRA-r16 ::= SEQUENCE { |  |  |  |
| condEventId-r16 CHOICE { |  |  |  |
| condEventA3-r16 SEQUENCE { |  |  |  |
| a3-Offset-r16 | 0 (0 dB) |  |  |
| hysteresis-r16 | 0 (0 dB) |  |  |
| timeToTrigger-r16 | ms640 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| ul-DelayValueConfig-r16 | Not Present |  |  |
| } |  |  |  |

–	ReportConfigEUTRA-A5-CHO

Table 4.6.6-18: ReportConfigEUTRA-A5-CHO(Thres1, Thres2)

| Derivation Path: 36.331 clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigEUTRA-A3 ::= SEQUENCE { |  |  |  |
| triggerType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  |  |
| purpose | reportStrongestCells |  |  |
| } |  |  |  |
| } |  |  |  |
| triggerQuantity | rsrp |  |  |
| reportQuantity | both |  |  |
| maxReportCells | 1 |  |  |
| reportInterval | min60 |  |  |
| reportAmount | r1 |  |  |
| si-RequestForHO-r9 | Not Present |  |  |
| ue-RxTxTimeDiffPeriodical-r9 | Not Present |  |  |
| includeLocationInfo-r10 | Not Present |  |  |
| reportAddNeighMeas-r10 | Not Present |  |  |
| alternativeTimeToTrigger-r12 | Not Present |  |  |
| useT312-r12 | Not Present |  |  |
| usePSCell-r12 | Not Present |  |  |
| aN-Threshold1-v1250 | Not Present |  |  |
| a5-Threshold2-v1250 | Not Present |  |  |
| reportStrongestCSI-RSs-r12 | Not Present |  |  |
| reportCRS-Meas-r12 | Not Present |  |  |
| triggerQuantityCSI-RS-r12 | Not Present |  |  |
| reportSSTD-Meas-r13 | Not Present |  |  |
| rs-sinr-Config-r13 | Not Present |  |  |
| useWhiteCellList-r13 | Not Present |  |  |
| measRSSI-ReportConfig-r13 | Not Present |  |  |
| includeMultiBandInfo-r13 | Not Present |  |  |
| ul-DelayConfig-r13 | Not Present |  |  |
| ue-RxTxTimeDiffPeriodicalTDD-r13 | Not Present |  |  |
| purpose-v1430 | Not Present |  |  |
| maxReportRS-Index-r15 | Not Present |  |  |
| includeBT-Meas-r15 | Not Present |  |  |
| includeWLAN-Meas-r15 | Not Present |  |  |
| purpose-r15 | Not Present |  |  |
| numberOfTriggeringCells-r15 | Not Present |  |  |
| a4-a5-ReportOnLeave-r15 | Not Present |  |  |
| condReconfigurationTriggerEUTRA-r16 ::= SEQUENCE { |  |  |  |
| condEventId-r16 CHOICE { |  |  |  |
| condEventA5-r16 SEQUENCE { |  |  |  |
| a5-Threshold1-r16 CHOICE{ |  |  |  |
| threshold-RSRP | Thres1 + 140 | Thres1 is actual threshold value in dBm | Not RSRQ |
| threshold-RSRQ | Thres1 * 2 + 40 | Thres1 is actual threshold value in dB | RSRQ |
| } |  |  |  |
| a5-Threshold2-r16 CHOICE{ |  |  |  |
| threshold-RSRP | Thres2 + 140 | Thres2 is actual threshold value in dBm | Not RSRQ |
| threshold-RSRQ | Thres2 * 2 + 40 | Thres2 is actual threshold value in dB | RSRQ |
| } |  |  |  |
| hysteresis-r16 | 0 (0 dB) |  |  |
| timeToTrigger-r16 | ms640 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| ul-DelayValueConfig-r16 | Not Present |  |  |
| } |  |  |  |

<End of modified section 3>