3GPP TSG-RAN5 Meeting #91-e	R5-213629

# Online, , 17th May 2021 - 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2197 | rev | 1 | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of MDT test case 8.1.6.1.2.9 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT-UEConTest |  |  |  |  |  | Date: |  |  | 2021-05-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The specific message contents should be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added specific message contents. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.6.1.2.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is an update of R5-212587. |  |  |  |  |  |  |  |  |

8.1.6.1.2.9	Logged MDT / Location information

8.1.6.1.2.9.1	Test Purpose (TP)

with { UE received LoggedMeasurementConfiguration message and has stored logMeasReport with detailed

location information in VarLogMeasReport}

ensure that {

when { receiving UEInformationRequest message }

then { UE transmits UEInformationResponse messages with a logMeasReport with locationCoordinates in the locationInfo in the logged measurement entries }

}

8.1.6.1.2.9.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 38.331, clauses 5.3.3.4, 5 5.5a.1.3 and 5.7.10.3.

[TS 38.331, clause 5.3.3.4]

The UE shall perform the following actions upon reception of the RRCSetup:

…

- set the content of RRCSetupComplete message as follows:

…

2>	if the UE has logged measurements available for NR and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

3>	include the logMeasAvailable in the RRCSetupComplete message;

2>	if the UE has Bluetooth logged measurements available and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

3>	include the logMeasAvailableBT in the RRCSetupComplete message;

2>	if the UE has WLAN logged measurements available and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

3>	include the logMeasAvailableWLAN in the RRCSetupComplete message;

…

[TS 38.331, clause 5.5a.1.3]

Upon receiving the LoggedMeasurementConfiguration message the UE shall:

1>	discard the logged measurement configuration as well as the logged measurement information as specified in 5.5a.2;

1>	store the received loggingDuration, reportType and areaConfiguration, if included, in VarLogMeasConfig;

1>	if the LoggedMeasurementConfiguration message includes plmn-IdentityList:

2>	set plmn-IdentityList in VarLogMeasReport to include the RPLMN as well as the PLMNs included in plmn-IdentityList;

1>	else:

2>	set plmn-IdentityList in VarLogMeasReport to include the RPLMN;

1>	store the received absoluteTimeInfo, traceReference, traceRecordingSessionRef, and tce-Id in VarLogMeasReport;

1>	store the received bt-NameList, if included, in VarLogMeasConfig;

1>	store the received wlan-Namelist, if included, in VarLogMeasConfig;

1>	store the received sensor-Namelist, if included, in VarLogMeasConfig;

1>	start timer T330 with the timer value set to the loggingDuration;

[TS 38.331, clause 5.7.10.3]

Upon receiving the UEInformationRequest message, the UE shall, only after successful security activation:

…

1>	if the logMeasReportReq is present and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

2>	if VarLogMeasReport includes one or more logged measurement entries, set the contents of the logMeasReport in the UEInformationResponse message as follows:

3>	include the absoluteTimeStamp and set it to the value of absoluteTimeInfo in the VarLogMeasReport;

3>	include the traceReference and set it to the value of traceReference in the VarLogMeasReport;

3>	include the traceRecordingSessionRef and set it to the value of traceRecordingSessionRef in the VarLogMeasReport;

3>	include the tce-Id and set it to the value of tce-Id in the VarLogMeasReport;

3>	include the logMeasInfoList and set it to include one or more entries from VarLogMeasReport starting from the entries logged first;

3>	if the VarLogMeasReport includes one or more additional logged measurement entries that are not included in the logMeasInfoList within the UEInformationResponse message:

4>	include the logMeasAvailable;

3>	if the VarLogMeasReport includes one or more additional logged Bluetooth measurement entries that are not included in the logMeasInfoList within the UEInformationResponse message:

4>	include the logMeasAvailableBT;

3>	if the VarLogMeasReport includes one or more additional logged WLAN measurement entries that are not included in the logMeasInfoList within the UEInformationResponse message:

4>	include the logMeasAvailableWLAN;

…

1>	if the logMeasReport is included in the UEInformationResponse:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB2;

2>	discard the logged measurement entries included in the logMeasInfoList from VarLogMeasReport upon successful delivery of the UEInformationResponse message confirmed by lower layers;

1>	else:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB1.

8.1.6.1.2.9.3	Test description

8.1.6.1.2.9.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1 and NR Cell 11.

-	System information combination NR-2 as defined in TS 38.508-1[4] clause 4.4.3.1.2 is used in NR cells.

UE:

-	None.

Preamble:

-	The UE is in state 3N-A according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-3 on Cell 1.

-	The UE's positioning engine (e.g. standalone GNSS receiver) should be provided with any necessary stimulus to allow it to provide the position. This shall be done by use of the test function Update UE Location Information defined (FFS) in TS 38.509 [6]. Or in addition any other suitable method may also be used.

8.1.6.1.2.9.3.2	Test procedure sequence

Table 8.1.6.1.2.9.3.2-1/2 illustrate the downlink power levels and other changing parameters to be applied for the cell at various time instants of the test execution. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.1.6.1.2.9.3.2-1: Time instances of cell power level for FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 11 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -78 | -85 | NR Cell 1 becomes the highest ranked cell. |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -85 | -78 | NR Cell 11 becomes the highest ranked cell. |

Table 8.1.6.1.2.9.3.2-2: Time instances of cell power level for FR2

|  | Parameter | Unit | NR Cell 1 | NR Cell 11 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | FFS | FFS | NR Cell 1 becomes the highest ranked cell. |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | FFS | FFS | NR Cell 11 becomes the highest ranked cell. |

Table 8.1.6.1.2.9.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits a LoggedMeasurementConfiguration message including to configure the UE to perform logging of measurement results while in RRC_IDLE. | <-- | LoggedMeasurementConfiguration | - | - |
| 2 | The SS transmits an RRCRelease message. | <-- | NR RRC:RRCRelease | - | - |
| 3 | Wait 5s to allow UE to activate logging | - | - | - | - |
| 4 | The SS changes Cell 1 and Cell 11 levels according to the row "T1" in table 8.1.6.1.2.9.3.2-1. (Note 1) | - | - | - | - |
| 5 | The UE transmits an RRCSetupRequest message on Cell 11 to initiate a tracking area update procedure. (Note 1) | --> | NR RRC: RRCSetupRequest | - | - |
| 6 | SS transmit an RRCSetup message. | <-- | NR RRC: RRCSetup | - | - |
| 7 | The UE include the IE logMeasAvailable in the RRCSetupComplete message. | --> | NR RRC: RRCSetupComplete 5GMM: REGISTRATION REQUEST | - | - |
| 8-10 | Steps 4 to 6 of generic test procedure in TS 38.508-1 [4] subclause 4.9.5.2.2-1 are performed on Cell 11. NOTE: The UE performs a REGISTRATION procedure update the registration of the actual tracking area and the RRC connection is released. | - | - | - | - |
| 11-18 | Steps 1 to 8 of the generic procedure in TS 38.508[4] Table 4.5.4.2-3 are executed to successfully complete the service request procedure. | - | - | - | - |
| 19 | The SS transmits a UEInformationRequest message on Cell 11. | <-- | UEInformationRequest | - | - |
| 20 | Check: Does the UE transmit a UEInformationResponse message where the IE CommonLocationInfo is present. | --> | UEInformationResponse | 1 | P |

8.1.6.1.2.9.3.3	Specific message contents

Table 8.1.6.1.2.9.3.3-1: LoggedMeasurementConfiguration (step 1, Table 8.1.6.1.2.9.3.2-3)

| Derivation path: 38.508-1 clause 4.6.1 table 4.6.1-5AA LoggedMeasurementConfiguration |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| LoggedMeasurementConfiguration-r16 ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| LoggedMeasurementConfiguration SEQUENCE { |  |  |  |  |
| reportType CHOICE { |  |  |  |  |
| periodical SEQUENCE { |  |  |  |  |
| loggingInterval-r16 |  | ms2560 | 2.56 seconds |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

T

Table 8.1.6.1.2.9.3.3-2: RRCSetupComplete (step 7, Table 8.1.6.1.2.9.3.2-3)

| Derivation path: 38.508-1 clause 4.6.1 table 4.6.1-23 RRCSetupComplete |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCSetupComplete ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcSetupComplete SEQUENCE { |  |  |  |  |
| nonCriticalExtension { |  |  |  |  |
| ue-MeasurementsAvailable-r16 SEQUENCE { |  |  |  |  |
| logMeasAvailable-r16 |  | True |  |  |
| logMeasAvailableBT-r16 |  | Not Present |  |  |
| logMeasAvailableWLAN-r16 |  | Not Present |  |  |
| connEstFailInfoAvailable-r16 |  | Not Present |  |  |
| rlf-InfoAvailable-r16 |  | Not Present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.1.6.1.2.9.3.3-3: UEInformationRequest (step 19, Table 8.1.6.1.2.9.3.2-3)

Derivation path: 38.508-1 clause 4.6.1 table 4.6.1-32A UEInformationRequest with condition LOG

Table 8.1.6.1.2.9.3.3-4: UEInformationResponse (step 20, Table 8.1.6.1.2.9.3.2-3)

| Derivation path: 38.508-1 clause 4.6.1 table 4.6.1-32B UEInformationResponse |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UEInformationResponse-r16 ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  |  |  |  |
| criticalExtensions{ |  |  |  |  |
| ueInformationResponse-r16 SEQUENCE { |  |  |  |  |
| logMeasReport-r16 SEQUENCE { |  |  |  |  |
| absoluteTimeStamp-r16 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| traceReference-r16 SEQUENCE { |  |  |  |  |
| plmn-Identity-r16 SEQUENCE { |  |  |  |  |
| mcc SEQUENCE (SIZE (3)) OF MCC-NMC-Digit |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| mnc SEQUENCE (SIZE (2..3)) OF MCC-NMC-Digit |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| traceRecordingSessionRef-r16 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| tce-Id-r16 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| logMeasInfoList-r16 SEQUENCE (SIZE (1..maxLogMeasReport-r16)) OF SEQUENCE { |  | At least 1 entrie where at least one entry complies to entry with index ‘x’ below. |  |  |
| locationInfo-r16[x] SEQUENCE { |  |  |  |  |
| locationCoordinates-r16 CHOICE { |  |  |  |  |
| ellipsoid-Point-r16 |  | Any allowed value |  |  |
| ellipsoidPointWithAltitude-r16 |  | Any allowed value |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| relativeTimeStamp-r16 [x] |  | SS record the value |  |  |
| servCellIdentity-r16 [x] |  | Same as Cell 1 |  |  |
| measResultServCell-16 [x] SEQUENCE { |  |  |  |  |
| resultsSSB-Cell-r16 |  | MeasQuantityResults of Cell 1 |  |  |
| resultsSSB SEQUENCE { |  |  |  |  |
| best-ssb-Index |  | Not checked |  |  |
| best-ssb-Results |  | Not checked |  |  |
| numberOfGoodSSB |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| measResultNeighCells-r16 [x] SEQUENCE { |  |  |  |  |
| MeasResultLogging2NR-r16 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  | 1 entry |  |  |
| carrierFreq-r16[x] |  | Same as Cell 11 |  |  |
| MeasResultListLoggingNR -r16[x] SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { |  | 1 entry |  |  |
| physCellId |  | Same as Cell 11 |  |  |
| resultsSSB-Cell-r16 |  | MeasQuantityResults of Cell 11 |  |  |
| numberOfGoodSSB-r16 |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| measResultListEUTRA-r16 |  | Not present |  |  |
| } |  |  |  |  |
| anyCellSelectionDetected-r16[x] |  | Not present |  |  |
| } |  |  |  |  |
| logMeasAvailable-r16 |  | Not present |  |  |
| logMeasAvailableBT-r16 |  | Not present |  |  |
| logMeasAvailableWLAN-r16 |  | Not present |  |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |