3GPP TSG-RAN5 Meeting #91-e	R5-213870

# Electronic Meeting, 17th – 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 1884 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Align Chapter 7 of TS 38.508-1 with Annex H of TS 38.533 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rohde & Schwarz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-05-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The CSI-RS related configuration in 7.3 is sometimes contradicting to Annex H of TS 38.533 or not complete Table 7.3.1-15: ControlResourceSet is not aligned to the CCR RMCs The SearchSpace is missing parameters required for FR2 RMCs |  |  |  |  |  |  |  |  |
| Summary of change: |  | Aligned 7.3 content to Annex H of TS 38.533 removing duplicated and contradicting settings Introduced new parameters in Table 7.3.1-15 to align to the CCR RMCs Added new parameters in Table 7.3.1-17 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The message contents will remain incorrect |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | R1: updated to revet change to CSI-MeasConfig and to clarify the change to the SearchSpace based on offline feedback |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# < Unchanged sections omitted >

### 7.3.1	Radio resource control information elements for RRM

As defined in clause 4.6.3 with the following exceptions:

#### –	TDD-UL-DL-ConfigCommon

Table 7.3.1-1: TDD-UL-DL-ConfigCommon

| Derivation Path: Table 4.6.3-192 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| TDD-UL-DL-ConfigCommon ::= SEQUENCE { |  |  |  |
| referenceSubcarrierSpacing | SubcarrierSpacing |  |  |
| pattern1 SEQUENCE { |  |  |  |
| dl-UL-TransmissionPeriodicity | ms0p625 |  | TDDConf.3.1 |
| nrofDownlinkSlots | 3 |  | TDDConf.2.1, TDDConf.3.1 |
|  | 1 |  | TDDConf.1.1 |
| nrofDownlinkSymbols | 10 |  | TDDConf.1.1, TDDConf.3.1 |
|  | 6 |  | TDDConf.2.1 |
| nrofUplinkSlots | 2 |  | TDDConf.1.1 |
|  | 1 |  | TDDConf.3.1 |
|  | 4 |  | TDDConf.2.1 |
| nrofUplinkSymbols | 4 |  | TDDConf.2.1 |
|  | 2 |  | TDDConf.1.1, TDDConf.3.1 |
| dl-UL-TransmissionPeriodicity-v1530 | Not present |  |  |
|  | ms4 |  | TDDConf.1.1, TDDConf.2.1 |
| } |  |  |  |
| pattern2 | Not present |  |  |
| pattern2 SEQUENCE { |  |  | TDDConf.1.1, TDDConf.2.1 |
| dl-UL-TransmissionPeriodicity | ms1 |  | TDDConf.1.1, TDDConf.2.1 |
| nrofDownlinkSlots | 1 |  | TDDConf.1.1 |
|  | 2 |  | TDDConf.2.1 |
| nrofDownlinkSymbols | 0 |  | TDDConf.1.1, TDDConf.2.1 |
| nrofUplinkSlots | 0 |  | TDDConf.1.1, TDDConf.2.1 |
| nrofUplinkSymbols | 0 |  | TDDConf.1.1, TDDConf.2.1 |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| TDDConf.1.1 | TDD UL/DL configuration for SCS=15kHz |
| TDDConf.2.1 | TDD UL/DL configuration for SCS=30kHz |
| TDDConf.3.1 | TDD UL/DL configuration for SCS=120kHz |

#### –	FilterCoefficient

Table 7.3.1-2: FilterCoefficient

| Derivation Path: Table 4.6.3-57 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| FilterCoefficient | fc0 | L3 filtering is not used |  |

#### –	SSB-MTC

Table 7.3.1-3: SSB-MTC

| Derivation Path: Table 4.6.3-185 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SSB-MTC ::= SEQUENCE { |  |  |  |
| periodicityAndOffset CHOICE { |  |  |  |
| sf20 | 0 |  | SMTC.1, SMTC.2 |
| sf20 | 10 |  | SMTC.4, SMTC.5 |
| sf160 | 0 |  | SMTC.3 |
| } |  |  |  |
| duration | sf1 |  | SMTC.1, SMTC.3, SMTC.4 |
|  | sf5 |  | SMTC.2, SMTC.5 |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SMTC.n | SMTC pattern n as defined in 38.533 Annex A.4 |

#### –	SubcarrierSpacing

Table 7.3.1-3a: SubcarrierSpacing

| Derivation Path: Table 4.6.3-188 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ssbSubcarrierSpacing | kHz15 |  | SSB.1 FR1, SSB.3 FR1 or SSB.5 FR1 |
|  | kHz30 |  | SSB.2 FR1, SSB.4 FR1 or SSB.6 FR1 |
|  | kHz120 |  | SSB.1 FR2, SSB.3 FR2, SSB.5 FR2 or SSB.7 FR2 |
|  | kHz240 |  | SSB.2 FR2, SSB.4 FR2, SSB.6 FR2 or SSB.8 FR2 |

| Condition | Explanation |
| --- | --- |
| SSB.n FR1 | SSB RMC n for FR1 as defined in 38.533 Annex A.3.1 |
| SSB.n FR2 | SSB RMC n for FR2 as defined in 38.533 Annex A.3.2 |

#### –	ServingCellConfigCommon

Table 7.3.1-4: ServingCellConfigCommon

| Derivation Path: Table 4.6.3-168 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommon ::= SEQUENCE { |  |  |  |
| ssb-PositionsInBurst CHOICE { |  |  |  |
| shortBitmap | 1000 | 1 SS Block in low FR1 frequencies | LOW_FREQ |
|  | 1100 | 2 SS Blocks in low FR1 frequencies | 2SSB AND LOW_FREQ |
| mediumBitmap | 10000000 | 1 SS Block in high FR1 frequencies | HIGH_FREQ |
|  | 11000000 | 2 SS Blocks in high FR1 frequencies | 2SSB AND HIGH_FREQ |
| longBitmap | 1000000000000000000000000000000000000000000000000000000000000000 | 1 SS Block in FR2 | FR2 |
|  | 1100000000000000000000000000000000000000000000000000000000000000 | 2 SS Blocks in FR2 | 2SSB AND FR2 |
| } |  |  |  |
| ssb-periodicityServingCell | ms20 |  |  |
| ssbSubcarrierSpacing | kHz15 |  |  |
|  | kHz30 |  | SCS30kHz |
|  | kHz120 |  | FR2 |
| } | kHz240 |  | FR2 AND SCS240kHz |

| Condition | Explanation |
| --- | --- |
| LOW_FREQ | Frequency <= 2.4 GHz for TDD or Frequency <= 3 GHz for FDD |
| HIGH_FREQ | FR1 and (Frequency > 2.4 GHz for TDD or Frequency > 3 GHz for FDD or CASE_C) |
| 2SSB | The SSB pattern as defined in TS 38.533 [18] Annex A.3.1 contain 2 SSBs within a burst |
| SCS30kHz | The SSB pattern as defined in TS 38.533 [18] Annex A.3.1 is for 30 kHz SCS |
| SCS240kHz | The SSB pattern as defined in TS 38.533 [18] Annex A.3.1 is for 240 kHz SCS |

#### –	ServingCellConfigCommonSIB

Table 7.3.1-5: ServingCellConfigCommonSIB-RRM

| Derivation Path: Table 4.6.3-169 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommonSIB ::= SEQUENCE { |  |  |  |
| ssb-PositionsInBurst SEQUENCE { |  |  |  |
| inOneGroup | ’1000 0000’B | When carrier frequency <= 3 GHz for FDD or <= 2.4 GHz for TDD, only the 4 leftmost bits are valid; |  |
|  | ’1100 0000’B |  | 2SSB |
| groupPresence | Not present |  |  |
|  | ’1000 0000’B |  | FR2 |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| FR2 | Frequency range 2 |
| 2SSB | For configuration with 2 SS Blocks |

#### -	CSI-MeasConfig

Table 7.3.1-6: CSI-MeasConfig for RRM

| Derivation Path: Table 4.6.3-38 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |  |
| CSI-MeasConfig::= SEQUENCE { |  |  |  |  |  |  |  |
| nzp-CSI-RS-ResourceToAddModList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-Resource { |  | n1+n2+n3+n4+n5 entries |  | n1=1 if CSI-RS for CSI is configured in test case, n1=0 otherwise; n2=2 if CSI-RS for BM is configured in test case, n2=0 otherwise; n3=4 if TRS is configured in test case, n3=0 otherwise; n4=4 if second resource set of TRS is configured in test case, n4=0 otherwise; n5=2 if aperiodic CSI-RS for BM is configured in test case, n5=0 otherwise; |  |  |  |
| NZP-CSI-RS-Resource[k, k=1..n1] |  | NZP-CSI-RS-Resource for CSI |  | entry 1 |  | n1>0 |  |
| NZP-CSI-RS-Resource[k, k=n1+1.. n1+n2] |  | NZP-CSI-RS-Resource for BM (k-n1-1) |  | entry ... |  | n2>0 |  |
| NZP-CSI-RS-Resource[k, k= n1+n2+1.. n1+n2+n3] |  | NZP-CSI-RS-Resource for TRS (k-n1-n2) |  | entry ... |  | n3>0 |  |
| NZP-CSI-RS-Resource[k, k= n1+n2+n3+1.. n1+n2+n3+n4] |  | NZP-CSI-RS-Resource for TRS (k-n1-n2-n3) with condition SECOND_SET |  | entry ... |  | n4>0 |  |
| NZP-CSI-RS-Resource[k, k= n1+n2+n3+n4+1.. n1+n2+n3+n4+n5] |  | NZP-CSI-RS-Resource for BM (k-n1-n2-n3-n4) with condition APERIODIC |  | entry ... |  | n5>0 |  |
| } |  |  |  |  |  |  |  |
| nzp-CSI-RS-ResourceSetToAddModList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSets)) OF NZP-CSI-RS-ResourceSet { |  | m1+m2+m3+m4+m5 entries |  | mi=1 if ni>0, mi=0 otherwise |  |  |  |
| NZP-CSI-RS-ResourceSet[k, k=1..m1] |  | NZP-CSI-RS-ResourceSet for CSI |  | entry 1 |  | n1>0 |  |
| NZP-CSI-RS-ResourceSet[k, k=m1+1.. m1+m2] |  | NZP-CSI-RS-ResourceSet for BM |  | entry ... |  | n2>0 |  |
| NZP-CSI-RS-ResourceSet[k, k= m1+m2+1.. m1+m2+m3] |  | NZP-CSI-RS-ResourceSet for TRS |  | entry ... |  | n3>0 |  |
| NZP-CSI-RS-ResourceSet[k, k= m1+m2+m3+1.. m1+m2+m3+m4] |  | NZP-CSI-RS-ResourceSet for TRS with condition SECOND_SET |  | entry ... |  | n4>0 |  |
| NZP-CSI-RS-ResourceSet[k, k= m1+m2+m3+m4+1.. m1+m2+m3+m4+m5] |  | NZP-CSI-RS-ResourceSet for BM with condition APERIODIC |  | entry ... |  | n5>0 |  |
| } |  |  |  |  |  |  |  |
| csi-IM-ResourceToAddModList SEQUENCE (SIZE (1..maxNrofCSI-IM-Resources)) OF CSI-IM-Resource { |  | 1 entry |  |  |  | n1>0 |  |
| CSI-IM-Resource[1] |  | CSI-IM-Resource-RRM |  | entry 1 |  |  |  |
| } |  |  |  |  |  |  |  |
| csi-IM-ResourceToAddModList |  | Not present |  |  |  | n1=0 |  |
| csi-IM-ResourceSetToAddModList SEQUENCE (SIZE (1..maxNrofCSI-IM-ResourceSets)) OF CSI-IM-ResourceSet { |  | 1 entry |  |  |  | n1>0 |  |
| CSI-IM-ResourceSet[1] |  | CSI-IM-ResourceSet-RRM |  | entry 1 |  |  |  |
| } |  |  |  |  |  |  |  |
| csi-IM-ResourceSetToAddModList |  | Not present |  |  |  | n1=0 |  |
| csi-SSB-ResourceSetToAddModList |  | Not present |  |  |  |  |  |
| csi-ResourceConfigToAddModList SEQUENCE (SIZE (1..maxNrofCSI-ResourceConfigurations)) OF CSI-ResourceConfig { |  | p1+m2+m3+m5 entries |  | p1=m1+1 if n1>0, p1=0 otherwise. |  |  |  |
| CSI-ResourceConfig[k, k=1..m1] |  | CSI-ResourceConfig for CSI |  | entry 1 |  | n1>0 |  |
| CSI-ResourceConfig[k, k=p1] |  | CSI-ResourceConfig for CSI-IM |  | entry ... |  | n1>0 |  |
| CSI-ResourceConfig[k, k=p1+1.. p1+m2] |  | CSI-ResourceConfig for BM |  | entry ... |  | n2>0 |  |
| CSI-ResourceConfig[k, k= p1+m2+1.. p1+m2+m3] |  | CSI-ResourceConfig for TRS |  | entry ... |  | n3>0 and n4=0 |  |
|  |  | CSI-ResourceConfig for TRS with condition SECOND_SET |  |  |  | n4>0 |  |
| CSI-ResourceConfig[k, k= p1+m2+m3+1.. p1+m2+m3+m5] |  | CSI-ResourceConfig for BM with condition APERIODIC |  | entry ... |  | n5>0 |  |
| } |  |  |  |  |  |  |  |
| csi-ReportConfigToAddModList SEQUENCE (SIZE (1..maxNrofCSI-ReportConfigurations)) OF CSI-ReportConfig { |  | m1+m2+m5 entries |  |  |  |  |  |
| CSI-ReportConfig[k, k=1..m1] |  | CSI-ReportConfig for CSI |  | entry 1 |  | n1>0 |  |
| CSI-ReportConfig[k, k=m1+1.. m1+m2] |  | CSI-ResourceConfig for BM |  | entry ... |  | n2>0 |  |
| CSI-ReportConfig[k, k=m1+m2+1.. m1+m2+m5] |  | CSI-ResourceConfig for BM with condition APERIODIC |  | entry ... |  | n5>0 |  |
| } |  |  |  |  |  |  |  |
| reportTriggerSize |  | Not present |  |  |  |  |  |
|  |  | 1 |  |  |  | n5>0 |  |
| aperiodicTriggerStateList CHOICE { |  | Not present |  |  |  |  |  |
| setup |  | CSI-AperiodicTriggerStateList |  |  |  | n5>0 |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |

#### –	NZP-CSI-RS-Resource for TRS

Table 7.3.1-7: NZP-CSI-RS-Resource for TRS(Id)

| Derivation Path: Table 4.6.3-45 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-Resource ::= SEQUENCE { |  |  |  |
| NZP-CSI-RS-ResourceId | NZP-CSI-RS-ResourceId for TRS(Id) |  |  |
|  | NZP-CSI-RS-ResourceId for TRS(Id) with Condition SECOND_SET |  | SECOND_SET |
| CSI-RS-ResourceMapping | CSI-RS-ResourceMapping for TRS(Id) |  |  |
|  | CSI-RS-ResourceMapping for TRS(Id) with condition SECOND_SET |  | SECOND_SET |
| powerControlOffsetSS | db-3 |  |  |
| scramblingID | PhysCellId | PCI of the cell sending the TRS |  |
| periodicityAndOffset | CSI-ResourcePeriodicityAndOffset for TRS(Id) |  |  |
| qcl-InfoPeriodicCSI-RS | TCI-StateId-RRM(2) |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SECOND_SET | For resource belong to the second resource set for TRS, only applies to FR2 test |

#### –	NZP-CSI-RS-Resource for CSI

Table 7.3.1-7A: NZP-CSI-RS-Resource for CSI

| Derivation Path: Table 4.6.3-45 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-Resource ::= SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceId | NZP-CSI-RS-ResourceId for CSI |  |  |
| resourceMapping | CSI-RS-ResourceMapping for CSI |  |  |
| powerControlOffset | 0 |  |  |
| powerControlOffsetSS | db0 |  |  |
| scramblingID | 0 |  |  |
| periodicityAndOffset | CSI-ResourcePeriodicityAndOffset for CSI |  |  |
| qcl-InfoPeriodicCSI-RS | TCI-StateId-RRM(0) |  |  |
| } |  |  |  |

#### –	NZP-CSI-RS-Resource for BM

Table 7.3.1-7B: NZP-CSI-RS-Resource for BM(Id)

| Derivation Path: Table 4.6.3-45 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-Resource ::= SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceId | NZP-CSI-RS-ResourceId for BM (Id) |  |  |
|  | NZP-CSI-RS-ResourceId for BM (Id) with condition APERIODIC |  | APERIODIC |
| resourceMapping | CSI-RS-ResourceMapping for BM (Id) |  |  |
| powerControlOffset | 0 |  |  |
| powerControlOffsetSS | db0 |  |  |
| scramblingID | 0 |  |  |
| periodicityAndOffset | CSI-ResourcePeriodicityAndOffset for BM |  |  |
|  | Not present |  | APERIODIC |
| qcl-InfoPeriodicCSI-RS | TCI-StateId-RRM(Id) |  |  |
|  | Not present |  | APERIODIC |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| APERIODIC | For apeiodic CSI-RS resources |

#### –	NZP-CSI-RS-Resource for TRS

Table 7.3.1-7C: NZP-CSI-RS-ResourceId for TRS(Id)

| Derivation Path: Table 4.6.3-86 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-ResourceId | n+Id-1 | n is the first NZP-CSI-RS-ResourceId allocated for TRS resource set. Value of n is left to internal implementation Id = 1,2,3,4 |  |
|  | n+Id+3 |  | SECOND_SET |

| Condition | Explanation |
| --- | --- |
| SECOND_SET | For the second TRS resource set configured in test, only applies to FR2 test |

#### –	NZP-CSI-RS-ResourceId for CSI

Table 7.3.1-7D: NZP-CSI-RS-ResourceId for CSI

| Derivation Path: Table 4.6.3-86 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-ResourceId | n | n is the NZP-CSI-RS-ResourceId allocated for CSI-RS for CSI report. Value of n is left to internal implementation |  |

#### –	NZP-CSI-RS-ResourceId for BM

Table 7.3.1-7E: NZP-CSI-RS-ResourceId for BM(Id)

| Derivation Path: Table 4.6.3-86 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-ResourceId | n+Id | n is the first NZP-CSI-RS-ResourceId allocated for CSI-RS for BM. Value of n is left to internal implementation Id = 0,1 |  |
|  | m+Id | m is the first NZP-CSI-RS-ResourceId allocated for aperiodic CSI-RS for BM. Value of m is left to internal implementation Id = 0,1 | APERIODIC |

| Condition | Explanation |
| --- | --- |
| APERIODIC | For apeiodic CSI-RS resources |

#### –	CSI-RS-ResourceMapping for TRS

Table 7.3.1-8: CSI-RS-ResourceMapping for TRS(Id)

| Derivation Path: Table 4.6.3-45 with condition TRS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-ResourceMapping ::= SEQUENCE { |  |  |  |
| frequencyDomainAllocation CHOICE { |  |  |  |
| row1 | 0001 | k0=0 for CSI-RS resource 1,2,3,4 |  |
| } |  |  |  |
| firstOFDMSymbolInTimeDomain | 5 | l0 = 5 for CSI-RS resource 1 and 3 | (Id = 1 or 3) AND FR1 |
|  | 9 | l0 = 9 for CSI-RS resource 2 and 4 | (Id = 2 or 4) AND FR1 |
|  | 1 |  | (Id = 1 or 3) AND FR2 |
|  | 2 |  | (Id = 1 or 3) AND FR2 AND SECOND_SET |
|  | 5 |  | (Id = 2 or 4) AND FR2 |
|  | 6 |  | (Id = 1 or 3) AND FR2 AND SECOND_SET |
| nrofPorts | p1 | 1 for CSI-RS resource 1,2,3,4 |  |
| freqBand | CSI-FrequencyOccupation-RRM |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SECOND_SET | For the second resource set for TRS configured in test, only applies to FR2 test |

#### –	CSI-RS-ResourceMapping for CSI

Table 7.3.1-8A: CSI-RS-ResourceMapping for CSI

| Derivation Path: Table 4.6.3-45 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-ResourceMapping ::= SEQUENCE { |  |  |  |
| frequencyDomainAllocation CHOICE { |  |  |  |
| other | 000001 |  |  |
| } |  |  |  |
| nrofPorts | p2 |  | FR1 |
|  | p1 |  | FR2 |
| firstOFDMSymbolInTimeDomain | 5 |  |  |
| freqBand | CSI-FrequencyOccupation-RRM |  |  |
| } |  |  |  |

#### –	CSI-RS-ResourceMapping for BM

Table 7.3.1-8B: CSI-RS-ResourceMapping for BM (Id)

| Derivation Path: Table 4.6.3-45 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-ResourceMapping ::= SEQUENCE { |  |  |  |
| frequencyDomainAllocation CHOICE { |  |  |  |
| row1 | 0001 |  |  |
| } |  |  |  |
| nrofPorts | p1 |  |  |
| firstOFDMSymbolInTimeDomain | 6 |  | Id = 0 |
|  | 10 |  | Id = 1 |
| cdm-Type | noCDM |  |  |
| density CHOICE { |  |  |  |
| three | NULL |  |  |
| } |  |  |  |
| freqBand | CSI-FrequencyOccupation-RRM |  |  |
| } |  |  |  |

#### –	CSI-ResourcePeriodicityAndOffset for TRS

Table 7.3.1-9: CSI-ResourcePeriodicityAndOffset for TRS(Id)

| Derivation Path: Table 4.6.3-43 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourcePeriodicityAndOffset ::= CHOICE { |  |  |  |
| slots80 | 40 |  | (Id = 1 or 3) AND SCS120 |
|  | 41 |  | (Id = 2 or 4) AND SCS120 |
| slots40 | 20 | Periodicity 40 slots and offset 20 for CSI-RS resource 1 and 2 | (Id = 1 or 3) AND SCS30 |
|  | 21 | Periodicity 40 slots and offset 21 for CSI-RS resource 3 and 4 | (Id = 2 or 3) AND SCS30 |
| slots20 | 10 | Periodicity 20 slots and offset 10 for CSI-RS resource 1 and 2 | (Id = 1 or 3) AND SCS15 |
|  | 11 | Periodicity 20 slots and offset 11 for CSI-RS resource 3 and 4 | (Id = 2 or 4) AND SCS15 |
| } |  |  |  |

#### –	CSI-ResourcePeriodicityAndOffset for CSI

Table 7.3.1-9A: CSI-ResourcePeriodicityAndOffset for CSI

| Derivation Path: Table 4.6.3-43 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourcePeriodicityAndOffset ::= CHOICE { |  |  |  |
| slots5 | 1 |  | SCS15 |
| slots10 | 2 |  | SCS30 |
| slots40 | 8 |  | SCS120 |
| } |  |  |  |

#### –	CSI-ResourcePeriodicityAndOffset for BM

Table 7.3.1-9B: CSI-ResourcePeriodicityAndOffset for BM

| Derivation Path: Table 4.6.3-43 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourcePeriodicityAndOffset ::= CHOICE { |  |  |  |
| slots10 | 1 |  | SCS15 |
| slots20 | 2 |  | SCS30 |
| slots80 | 8 |  | SCS120 |
| } |  |  |  |

#### –	CSI-FrequencyOccupation

Table 7.3.1-10: CSI-FrequencyOccupation-RRM

| Derivation Path: Table 4.6.3-33 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-FrequencyOccupation ::= SEQUENCE { |  |  |  |
| startingRB | 4*floor(n/4) | n is the start RB of active BWP floor() means rounding down to the nearest integer startingRB can only be multiple of 4 |  |
| nrofRBs | max(4*ceil(n/4-floor(n/4)+m/4),24) | m is the bandwidth of active BWP ceil() means rounding up to the nearest integer Bandwidth of CSI-RS used in RRM test is required to be same as active BWP according to 38.133. nrofRBs shall be no less than 24 and shall be multiple of 4 |  |
| } |  |  |  |

#### –	NZP-CSI-RS-ResourceSet for TRS

Table 7.3.1-11: NZP-CSI-RS-ResourceSet for TRS

| Derivation Path: Table 4.6.3-87 with Condition TRS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-Resource ::= SEQUENCE { |  |  |  |
| nzp_CSI_ResourceSetId | NZP-CSI-RS-ResourceSetId-TRS |  |  |
|  | NZP-CSI-RS-ResourceSetId-TRS with condition SECOND_SET |  | SECOND_SET |
| nzp-CSI-RS-Resources SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerSet)) OF NZP-CSI-RS-ResourceId { | 4 entries |  |  |
| NZP-CSI-RS-ResourceId[1] | NZP-CSI-RS-ResourceId for TRS(1) | entry 1 |  |
| NZP-CSI-RS-ResourceId[2] | NZP-CSI-RS-ResourceId for TRS(2) | entry 2 |  |
| NZP-CSI-RS-ResourceId[3] | NZP-CSI-RS-ResourceId for TRS(3) | entry 3 |  |
| NZP-CSI-RS-ResourceId[4] | NZP-CSI-RS-ResourceId for TRS(4) | entry 4 |  |
| } |  |  |  |
| nzp-CSI-RS-Resources SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerSet)) OF NZP-CSI-RS-ResourceId { | 4 entries |  | SECOND_SET |
| NZP-CSI-RS-ResourceId[1] | NZP-CSI-RS-ResourceId for TRS(1) with condition SECOND_SET | entry 1 |  |
| NZP-CSI-RS-ResourceId[2] | NZP-CSI-RS-ResourceId for TRS(2) with condition SECOND_SET | entry 2 |  |
| NZP-CSI-RS-ResourceId[3] | NZP-CSI-RS-ResourceId for TRS(3) with condition SECOND_SET | entry 3 |  |
| NZP-CSI-RS-ResourceId[4] | NZP-CSI-RS-ResourceId for TRS(4) with condition SECOND_SET | entry 4 |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SECOND_SET | For the second TRS resource set configured in test, only applies to FR2 test |

#### –	NZP-CSI-RS-ResourceSet for CSI

Table 7.3.1-11A: NZP-CSI-RS-ResourceSet for CSI

| Derivation Path: Table 4.6.3-87 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-Resource ::= SEQUENCE { |  |  |  |
| nzp_CSI_ResourceSetId | NZP-CSI-RS-ResourceSetId-CSI |  |  |
| nzp-CSI-RS-Resources SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerSet)) OF NZP-CSI-RS-ResourceId { | 1 entry |  |  |
| NZP-CSI-RS-ResourceId[1] | NZP-CSI-RS-ResourceId for CSI | entry 1 |  |
| } |  |  |  |
| repetition | Not present |  |  |
| } |  |  |  |

#### –	NZP-CSI-RS-ResourceSet for BM

Table 7.3.1-11B: NZP-CSI-RS-ResourceSet for BM

| Derivation Path: Table 4.6.3-87 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-ResourceSet ::= SEQUENCE { |  |  |  |
| nzp-CSI-ResourceSetId | NZP-CSI-RS-ResourceSetId-BM |  |  |
|  | NZP-CSI-RS-ResourceSetId-BM with condition APERIODIC |  | APERIODIC |
| nzp-CSI-RS-Resources SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerSet)) OF NZP-CSI-RS-ResourceId { | 2 entries |  |  |
| NZP-CSI-RS-ResourceId[1] | NZP-CSI-RS-ResourceId for BM (0) | entry 1 |  |
|  | NZP-CSI-RS-ResourceId for BM (0) with condition APERIODIC |  | APERIODIC |
| NZP-CSI-RS-ResourceId[2] | NZP-CSI-RS-ResourceId for BM (1) | entry 2 |  |
|  | NZP-CSI-RS-ResourceId for BM (1) with condition APERIODIC |  | APERIODIC |
| } |  |  |  |
| aperiodicTriggeringOffset | 4 |  | APERIODIC |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| APERIODIC | For apeiodic CSI-RS resources |

#### –	NZP-CSI-RS-ResourceSetId for TRS

Table 7.3.1-11C: NZP-CSI-RS-ResourceSetId-TRS

| Derivation Path: Table 4.6.3-88 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-ResourceSetId | n | n is the first NZP-CSI-RS-ResourceSetId allocated for TRS resource set. Value of n is left to internal implementation |  |
|  | n+1 |  | SECOND_SET |

| Condition | Explanation |
| --- | --- |
| SECOND_SET | For the second TRS resource set configured in test, only applies to FR2 test |

#### –	NZP-CSI-RS-ResourceSetId for CSI

Table 7.3.1-11D: NZP-CSI-RS-ResourceSetId-CSI

| Derivation Path: Table 4.6.3-88 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-ResourceSetId | n | n is the NZP-CSI-RS-ResourceSetId allocated for resource set of CSI-RS for CSI reporting. Value of n is left to internal implementation |  |

#### –	NZP-CSI-RS-ResourceSetId for BM

Table 7.3.1-11E: NZP-CSI-RS-ResourceSetId-BM

| Derivation Path: Table 4.6.3-88 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-ResourceSetId | n | n is the NZP-CSI-RS-ResourceSetId allocated for resource set of CSI-RS for BM. Value of n is left to internal implementation |  |
|  | m | m is the NZP-CSI-RS-ResourceSetId allocated for resource set of aperiodic CSI-RS for BM. Value of m is left to internal implementation | APERIODIC |

| Condition | Explanation |
| --- | --- |
| APERIODIC | For apeiodic CSI-RS resources |

#### –	CSI-ResourceConfig for TRS

Table 7.3.1-12: CSI-ResourceConfig for TRS

| Derivation Path: TS 38.508-1 Table 4.6.3-41 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourceConfig ::= SEQUENCE { |  |  |  |
| csi-ResourceConfigId | CSI-ResourceConfigId-TRS |  |  |
| csi-RS-ResourceSetList CHOICE { |  |  |  |
| nzp-CSI-RS-SSB SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceSetList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig)) OF NZP-CSI-RS-ResourceSetId { | 1 entry |  |  |
| NZP-CSI-RS-ResourceSetId[1] | NZP-CSI-RS-ResourceSetId-TRS |  |  |
| } |  |  |  |
| nzp-CSI-RS-ResourceSetList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig)) OF NZP-CSI-RS-ResourceSetId { | 2 entries |  | SECOND_SET |
| NZP-CSI-RS-ResourceSetId[1] | NZP-CSI-RS-ResourceSetId-TRS | entry 1 |  |
| NZP-CSI-RS-ResourceSetId[2] | NZP-CSI-RS-ResourceSetId-TRS with condition SECOND_SET | entry 2 |  |
| } |  |  |  |
| } |  |  |  |
| bwp-Id | BWP-Id of active BWP |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SECOND_SET | For the second resource set for TRS configured in test, only applies to FR2 test |

#### –	CSI-ResourceConfig for CSI

Table 7.3.1-12A: CSI-ResourceConfig for CSI

| Derivation Path: TS 38.508-1 Table 4.6.3-41 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourceConfig ::= SEQUENCE { |  |  |  |
| csi-ResourceConfigId | CSI-ResourceConfigId-CSI |  |  |
| csi-RS-ResourceSetList CHOICE { |  |  |  |
| nzp-CSI-RS-SSB SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceSetList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig)) OF NZP-CSI-RS-ResourceSetId { | 1 entry |  |  |
| NZP-CSI-RS-ResourceSetId[1] | NZP-CSI-RS-ResourceSetId-CSI | entry 1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| bwp-Id | BWP-Id of active BWP |  |  |
| } |  |  |  |

#### –	CSI-ResourceConfig for BM

Table 7.3.1-12B: CSI-ResourceConfig for BM

| Derivation Path: Table 4.6.3-41 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourceConfig ::= SEQUENCE { |  |  |  |
| csi-ResourceConfigId | CSI-ResourceConfigId-BM |  |  |
|  | CSI-ResourceConfigId-BM with condition APERIODIC |  | APERIODIC |
| csi-RS-ResourceSetList CHOICE { |  |  |  |
| nzp-CSI-RS-SSB SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceSetList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig)) OF NZP-CSI-RS-ResourceSetId { | 1 entry |  |  |
| NZP-CSI-RS-ResourceSetId[1] | NZP-CSI-RS-ResourceSetId-BM |  |  |
|  | NZP-CSI-RS-ResourceSetId-BM with condition APERIODIC |  | APERIODIC |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| bwp-Id | BWP-Id of active BWP |  |  |
| resourceType | aperiodic |  | APERIODIC |
|  | periodic |  | PERIODIC |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| APERIODIC | For aperiodic CSI-RS resources |
| PERIODIC | For periodic CSI-RS resources |

#### –	CSI-ResourceConfig for CSI-IM

Table 7.3.1-12BA: CSI-ResourceConfig for CSI-IM

| Derivation Path: Table 4.6.3-41 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourceConfig ::= SEQUENCE { |  |  |  |
| csi-ResourceConfigId | CSI-ResourceConfigId-CSI-IM |  |  |
| csi-RS-ResourceSetList CHOICE { |  |  |  |
| csi-IM-ResourceSetList SEQUENCE (SIZE (1..maxNrofCSI-IM-ResourceSetsPerConfig)) OF CSI-IM-ResourceSetId { | 1 entry |  |  |
| CSI-IM-ResourceSetId[1] | CSI-IM-ResourceSetId-RRM | entry 1 |  |
| } |  |  |  |
| } |  |  |  |
| bwp-Id | BWP-Id |  |  |
| resourceType | periodic |  |  |
| } |  |  |  |

#### –	CSI-ResourceConfigId for TRS

Table 7.3.1-12C: CSI-ResourceConfigId-TRS

| Derivation Path: Table 4.6.3-42 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourceConfigId | n | n is the CSI-ResourceConfigId allocated for resource config of TRS. Value of n is left to internal implementation |  |

#### –	CSI-ResourceConfigId for CSI

Table 7.3.1-12D: CSI-ResourceConfigId-CSI

| Derivation Path: Table 4.6.3-42 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourceConfigId | n | n is the CSI-ResourceConfigId allocated for resource config of CSI-RS for CSI reporting. Value of n is left to internal implementation |  |

#### –	CSI-ResourceConfigId for BM

Table 7.3.1-12E: CSI-ResourceConfigId-BM

| Derivation Path: Table 4.6.3-42 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourceConfigId | n | n is the CSI-ResourceConfigId allocated for resource config of CSI-RS for BM. Value of n is left to internal implementation |  |
|  | m | m is the CSI-ResourceConfigId allocated for resource config of aperiodic CSI-RS for BM. Value of m is left to internal implementation | APERIODIC |

| Condition | Explanation |
| --- | --- |
| APERIODIC | For apeiodic CSI-RS resources |

#### –	CSI-ResourceConfigId-CSI-IM

Table 7.3.1-12EA: CSI-ResourceConfigId-CSI-IM

| Derivation Path: Table 4.6.3-42 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourceConfigId | n | n is the first CSI-ResourceConfigId allocated for CSI-IM resource configuration Value of n is left to internal implementation |  |

#### –	CSI-ReportConfig for CSI

Table 7.3.1-12F: CSI-ReportConfig for CSI

| Derivation Path: Table 4.6.3-39 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ReportConfig ::= SEQUENCE { |  |  |  |
| reportConfigId | CSI-ReportConfigId-CSI |  |  |
| resourcesForChannelMeasurement | CSI-ResourceConfigId-CSI |  |  |
| csi-IM-ResourcesForInterference | CSI-ResourceConfigId-CSI-IM |  |  |
| nzp-CSI-RS-ResourcesForInterference | Not present |  |  |
| reportConfigType CHOICE { |  |  |  |
| periodic | Set according to parameters given in test cases |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	CSI-ReportConfig for BM

Table 7.3.1-12G: CSI-ReportConfig for BM

| Derivation Path: Table 4.6.3-39 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ReportConfig ::= SEQUENCE { |  |  |  |
| reportConfigId | CSI-ReportConfigId-BM |  |  |
|  | CSI-ReportConfigId-BM with condition APERIODIC |  | APERIODIC |
| resourcesForChannelMeasurement | CSI-ResourceConfigId-BM |  |  |
|  | CSI-ResourceConfigId-BM with condition APERIODIC |  | APERIODIC |
| reportConfigType CHOICE { |  |  |  |
| periodic | Set according to parameters given in test cases |  |  |
| aperiodic SEQUENCE { |  |  | APERIODIC |
| reportSlotOffsetList SEQUENCE (SIZE (1..maxNrofUL-Allocations)) OF { | 1 entry |  |  |
| INTEGER[1] | 8 |  |  |
| } |  |  |  |
| } |  |  |  |
| reportQuantity CHOICE { |  |  |  |
| cri-RSRP | NULL |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| APERIODIC | For aperiodic CSI-RS resources |

#### –	CSI-ReportConfigId for CSI

Table 7.3.1-12H: CSI-ReportConfigId-CSI

| Derivation Path: Table 4.6.3-40 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ReportConfigId | n | n is the CSI-ReportConfigId allocated for report config of CSI-RS for CSI. Value of n is left to internal implementation |  |

#### –	CSI-ReportConfigId for BM

Table 7.3.1-12I: CSI-ReportConfigId-BM

| Derivation Path: Table 4.6.3-40 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ReportConfigId | n | n is the CSI-ReportConfigId allocated for report config of CSI-RS for BM. Value of n is left to internal implementation |  |
|  | m | m is the CSI-ReportConfigId allocated for report config of aperiodic CSI-RS for BM. Value of m is left to internal implementation | APERIODIC |

| Condition | Explanation |
| --- | --- |
| APERIODIC | For apeiodic CSI-RS resources |

#### –	CSI-AperiodicTriggerStateList

Table 7.3.1-12J: CSI-AperiodicTriggerStateList

| Derivation Path: Table 4.6.3-32 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-AperiodicTriggerStateList ::= SEQUENCE (SIZE (1..maxNrOfCSI-AperiodicTriggers)) OF SEQUENCE { | 1 entry |  |  |
| associatedReportConfigInfoList[1] SEQUENCE (SIZE(1..maxNrofReportConfigPerAperiodicTrigger)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | CSI-ReportConfigId-BM with condition APERIODIC |  |  |
| resourcesForChannel[1] CHOICE { |  |  |  |
| nzp-CSI-RS SEQUENCE { |  |  |  |
| resourceSet | NZP-CSI-RS-ResourceSetId-BM with condition APERIODIC |  |  |
| qcl-info SEQUENCE (SIZE(1..maxNrofAP-CSI-RS-ResourcesPerSet)) OF { | 2 entries |  |  |
| TCI-StateId[1] | TCI-StateId-RRM(0) | QCL Type C+D to SSB #0 |  |
| TCI-StateId[2] | TCI-StateId-RRM(1) | QCL Type C+D to SSB #1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| csi-IM-ResourcesForInterference[1] | Not present |  |  |
| nzp-CSI-RS-ResourcesForInterference[1] | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	RACH-ConfigCommon

Table 7.3.1-13: RACH-ConfigCommon

| Derivation Path: TS 38.508-1 Table 4.6.3-128 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigCommon::= SEQUENCE { |  |  |  |
| rach-ConfigGeneric | RACH-ConfigGeneric |  |  |
| totalNumberOfRA-Preambles | 48 |  |  |
| ssb-perRACH-OccasionAndCB-PreamblesPerSSB CHOICE { |  |  |  |
| oneFourth | n48 |  |  |
| } |  |  |  |
| groupBconfigured | Not present |  |  |
| ra-ContentionResolutionTimer | sf48 |  |  |
| rsrp-ThresholdSSB | 51 |  |  |
| rsrp-ThresholdSSB-SUL | Not present |  |  |
| prach-RootSequenceIndex CHOICE { |  |  |  |
| l139 | Set according to table 4.4.2-2 for the NR Cell. |  |  |
| } |  |  |  |
| msg1-SubcarrierSpacing | SubcarrierSpacing |  |  |
| restrictedSetConfig | unrestrictedSet |  |  |
| msg3-transformPrecoder | Not present | transform precoding is disabled for Msg3 PUSCH transmission and any PUSCH transmission scheduled with DCI format 0_0 |  |
| } |  |  |  |

#### –	RACH-ConfigGeneric

Table 7.3.1-14: RACH-ConfigGeneric

| Derivation Path: TS 38.508-1 Table 4.6.3-130 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigGeneric ::= SEQUENCE { |  |  |  |
| prach-ConfigurationIndex | 102 |  | FR1 |
|  | 190 |  | FR2 |
| msg1-FDM | one |  |  |
| msg1-FrequencyStart | 0 |  |  |
| zeroCorrelationZoneConfig | 11 |  |  |
| preambleReceivedTargetPower | -120 |  |  |
| preambleTransMax | n6 |  |  |
| powerRampingStep | dB2 |  |  |
| ra-ResponseWindow | sl10 |  |  |
| } |  |  |  |

#### –	ControlResourceSet

Table 7.3.1-15: ControlResourceSet

| Derivation Path: Table 4.6.3-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ControlResourceSet ::= SEQUENCE { |  |  |  |
| controlResourceSetId | ControlResourceSetId |  |  |
| frequencyDomainResources | 11111111 00000000 00000000 00000000 00000000 00000 |  | CCR.3.7 OR SCS240 |
| duration | 1 |  | CCR.3.x |
| cce-REG-MappingType CHOICE { |  |  |  |
| interleaved ::= SEQUENCE { |  |  | CCR.X.Y |
| reg-BundleSize | n6 |  |  |
| interleaverSize | n2 |  |  |
| shiftIndex | 0 |  |  |
| } |  |  |  |
| tci-StatesPDCCH-ToAddList SEQUENCE(SIZE (1..maxNrofTCI-StatesPDCCH)) OF TCI-StateId { | 1 entry |  |  |
| TCI-StateId[1] | TCI-StateId-RRM(0) | TCI State #0, QCLed to SSB index #0 |  |
|  | TCI-StateId-RRM(2) | TCI State #2, QCLed to TRS resource #4 in the first resource set | TRS |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| CCR.x.y | Refers to CCR.x.y as defined in A.1.3 of TS 38.533 [18] |
| TRS | When at least one TRS resource set is configured. |

#### -	SchedulingRequestResourceConfig

Table 7.3.1-16: SchedulingRequestResourceConfig

| Derivation Path: Table 4.6.3-157 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SchedulingRequestResourceConfig ::= SEQUENCE { |  |  |  |
| periodicityAndOffset CHOICE { |  |  |  |
| sl10 | 7 | With SCS = kHz15 results in repetition every 10 ms | SCS_15kHz |
| sl20 | 7 | With SCS = kHz30 results in repetition every 10 ms | SCS_30kHz |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SCS_15kHz | SCS=15kHz for frequency of the cell according to clause 6.2.3 for signalling test cases and clause 4.3.1 otherwise |
| SCS_30kHz | SCS=30kHz for frequency of the cell according to clause 6.2.3 for signalling test cases and clause 4.3.1 otherwise |

#### -	SearchSpace

Table 7.3.1-17: SearchSpace

| Derivation Path: Table 4.6.3-162 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| monitoringSlotPeriodicityAndOffset CHOICE { |  |  |  |
| sl10 | 1 |  | SISS |
| sl160 | 0 |  | (CCR.3.1 OR CCR.3.2 OR CCR.3.4 OR CCR.3.5 OR CCR.3.7) AND NOT_CONT_PDCCH |
|  | 80 |  | (CCR.3.3 OR CCR.3.6) AND NOT_CONT_PDCCH |
| } |  |  |  |
| monitoringSymbolsWithinSlot | 11000000000000 |  | (CCR.3.1 OR CCR.3.3 OR CCR.3.4 OR CCR.3.6 OR CCR.3.7) AND NOT_CONT_PDCCH |
|  | 00110000000000 |  | (CCR.3.2 OR CCR.3.5) AND NOT_CONT_PDCCH |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SISS | SearchSpace for SI |
| CCR.x.y | Refers to CCR.x.y as defined in A.1.3 of TS 38.533 [18] |
| NOT_CONT_PDCCH | The cell shall be configured with the default CCR.3.x settings and not transmit PDCCH continuously |

#### –	PDSCH-Config

Table 7.3.1-18: PDSCH-Config

| Derivation Path: Table 4.6.3-100 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-Config ::= SEQUENCE { |  |  |  |
| tci-StatesToAddModList SEQUENCE(SIZE (1.. maxNrofTCI-States)) OF TCI-State { | 1+n1+n2+n3 entries | n1 = 1 if SSB configuration used in test case contains two SSBs in a burst, n1 = 0 otherwise n2 = 1 if TRS is configured in test case, n1 = 0 otherwise n3 = 1 if two resource sets of TRS are configured in test case, n3 = 0 otherwise |  |
| TCI-State[1] | TCI-State(0) | entry 1 QCLed to SSB index #0 |  |
| TCI-State[k, k=2..1+n1] | TCI-State(1) | entry ... QCLed to SSB index #1 | SECOND_SSB |
| TCI-State[k, k=2+n1..1+n1+n2] | TCI-State(2) | entry ... QCLed to TRS resource #4 in the first resource set | TRS |
| TCI-State[k, k=2+n1+n2..1+n1+n2+n3] | TCI-State(3) | entry ... QCLed to TRS resource #4 in the second resource set | SECOND_SET |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SECOND_SSB | SSB configuration used in test case contain two SSBs in a burst |
| TRS | One resource set for TRS is configured in test case |
| SECOND_SET | Two resource sets for TRS are configured in test case, only applies to FR2. |

#### –	TCI-State

Table 7.3.1-19: TCI-State(Id)

| Derivation Path: Table 4.6.3-190 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| TCI-State ::= SEQUENCE { |  |  |  |
| tci-StateId | TCI-StateId-RRM(Id) |  |  |
| qcl-Type1 SEQUENCE { |  |  |  |
| bwp-Id | BWP-Id of the active BWP |  | Id = 2 or 3 |
| referenceSignal CHOICE { |  |  |  |
| ssb | SSB-Index of SSB #0 |  | Id = 0 |
|  | SSB-Index of SSB #1 |  | Id = 1 |
| csi-rs | NZP-CSI-RS-ResourceId for TRS (4) |  | Id = 2 |
|  | NZP-CSI-RS-ResourceId for TRS (4) with condition SECOND SET |  | Id = 3 |
| } |  |  |  |
| qcl-Type | typeC |  | Id = 0 or 1 |
|  | typeA |  | Id = 2 or 3 |
| } |  |  |  |
| qcl-Type2 | Not present |  |  |
| qcl-Type2 SEQUENCE { |  |  | FR2 |
| cell | Not present |  |  |
| bwp-Id | Not present |  | Id = 0 or 1 |
|  | BWP-Id of the active BWP |  | Id = 2 or 3 |
| referenceSignal CHOICE { |  |  |  |
| ssb | SSB-Index of SSB #0 |  | Id = 0 |
|  | SSB-Index of SSB #1 |  | Id = 1 |
| csi-rs | NZP-CSI-RS-ResourceId for TRS (4) |  | Id = 2 |
|  | NZP-CSI-RS-ResourceId for TRS (4) with condition SECOND SET |  | Id = 3 |
| } |  |  |  |
| qcl-Type | typeD |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	TCI-StateId

Table 7.3.1-20: TCI-StateId(Id)

| Derivation Path: Table 4.6.3-191 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| TCI-StateId | n+Id | n is the first TCI-StateId allocated for TCI-State configured in RRM test. Value of n is left to internal implementation Id = 0,1,2,3 |  |

#### –	PUSCH-Config

Table 7.3.1-21: PUSCH-Config

| Derivation Path: Table 4.6.3-118 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-Config ::= SEQUENCE { |  |  |  |
| pusch-TimeDomainAllocationList CHOICE { |  |  | APERIODIC |
| setup | PUSCH-TimeDomainResourceAllocationList-BM |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| APERIODIC | For apeiodic CSI-RS resources |

#### –	PUSCH-TimeDomainResourceAllocationList

Table 7.3.1-22: PUSCH-TimeDomainResourceAllocationList-BM

| Derivation Path: Table 4.6.3-122 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-TimeDomainResourceAllocationList ::= SEQUENCE (SIZE(1..maxNrofUL-Allocations)) OF SEQUENCE { | 1 entriy | same number of entries as reportSlotOffsetList in Table 7.3.1-12G |  |
| k2[1] | 4 | Same with k2 configrued in reportSlotOffsetList in Table 7.3.1-12G |  |
| mappingType[1] | typeA |  |  |
| startSymbolAndLength[1] | 27 | Start symbol(S)=0, Length(L)=14 |  |
| } |  |  |  |

#### – 	ServingCellConfig

Table 7.3.1-23: ServingCellConfig

| Derivation Path: Table 4.6.3-167 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| csi-MeasConfig CHOICE { |  |  |  |
| setup | csi-MeasConfig |  |  |
| } |  |  |  |

#### –	CSI-IM-Resource-RRM

Table 7.3.1-24: CSI-IM-Resource-RRM

| Derivation Path: Table 4.6.3-34 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-IM-Resource ::= SEQUENCE { |  |  |  |
| csi-IM-ResourceId | CSI-IM-ResourceId-RRM |  |  |
| csi-IM-ResourceElementPattern CHOICE { |  |  |  |
| pattern1 SEQUENCE { |  |  |  |
| subcarrierLocation-p1 | s8 |  |  |
| symbolLocation-p1 | 4 |  |  |
| } |  |  |  |
| } |  |  |  |
| freqBand | CSI-FrequencyOccupation-RRM |  |  |
| periodicityAndOffset | CSI-ResourcePeriodicityAndOffset for CSI |  |  |
| } |  |  |  |

#### –	CSI-IM-ResourceId-RRM

Table 7.3.1-25: CSI-IM-ResourceId-RRM

| Derivation Path: Table 4.6.3-35 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-IM-ResourceId | n | n is the first CSI-IM-ResourceId allocated for CSI-IM resource. Value of n is left to internal implementation |  |

#### –	CSI-IM-ResourceSet-RRM

Table 7.3.1-26: CSI-IM-ResourceSet-RRM

| Derivation Path: Table 4.6.3-36 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-IM-ResourceSet ::= SEQUENCE { |  |  |  |
| csi-IM-ResourceSetId | CSI-IM-ResourceSetId-RRM |  |  |
| csi-IM-Resources SEQUENCE (SIZE (1..maxNrofCSI-IM-ResourcesPerSet)) OF CSI-IM-ResourceId { | 1 entry |  |  |
| CSI-IM-ResourceId[1] | CSI-IM-ResourceId-RRM | entry 1 |  |
| } |  |  |  |
| } |  |  |  |

#### –	CSI-IM-ResourceSetId-RRM

Table 7.3.1-27: CSI-IM-ResourceSetId-RRM

| Derivation Path: Table 4.6.3-37 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-IM-ResourceSetId | n | n is the first CSI-IM-ResourceSetId allocated for CSI-IM resource set. Value of n is left to internal implementation |  |

#### –	SSB-Index

Table 7.3.1-28: SSB-Index

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SSB-Index | 0 |  |  |
|  | 1 |  | SECOND_SSB |

| Condition | Explanation |
| --- | --- |
| SECOND_SSB | SSB configuration used in test case contain two SSBs in a burst |

#### –	CSI-SSB-ResourceSet

Table 7.3.1-29: CSI-SSB-ResourceSet

| Derivation Path: Table 4.6.3-47 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-SSB-ResourceSet ::= SEQUENCE { |  |  |  |
| csi-SSB-ResourceSetId | CSI-SSB-ResourceSetId |  |  |
| csi-SSB-Resource SEQUENCE (SIZE (1..maxNrofCSI-SSB-ResourcePerSet)) OF SSB-Index { | 1 entry |  |  |
| SSB-Index[1] | SSB-Index | entry 1 |  |
| } |  |  |  |
| csi-SSB-Resource SEQUENCE (SIZE (1..maxNrofCSI-SSB-ResourcePerSet)) OF SSB-Index { | 2 entries |  | SECOND_SSB |
| SSB-Index[1] | SSB-Index | entry 1 |  |
| SSB-Index[2] | SSB-Index with condition SECOND_SSB | entry 2 |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SECOND_SSB | SSB configuration used in test case contain two SSBs in a burst |

#### –	SSB-ToMeasure

Table 7.3.1-31: SSB-ToMeasure

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SSB-ToMeasure ::= CHOICE { |  |  |  |
| shortBitmap | 1000 |  | 2.3GHz<FREQ<=3GHz AND (FDD OR (TDD AND SCS15)) OR FREQ<=2.3GHZ |
|  | 1100 |  | SECOND_SSB AND (2.3GHz<FREQ<=3GHz AND (FDD OR (TDD AND SCS15)) OR FREQ<=2.3GHZ) |
| mediumBitmap | 10000000 |  |  |
|  | 11000000 |  | SECOND_SSB |
| longBitmap | 10000000 00000000 00000000 00000000 00000000 00000000 00000000 00000000 |  | FR2 |
|  | 11000000 00000000 00000000 00000000 00000000 00000000 00000000 00000000 |  | FR2 AND SECOND_SSB |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| FREQ<=2.3GHz | Frequency range <= 2.4GHz |
| 2.3GHz<FREQ<=3GHz | Frequency range > 2.3GHz and <= 3GHz |
| FREQ>3GHz | Frequency range > 3GHz |
| SECOND_SSB | SSB configuration used in test case contain two SSBs in a burst |

< End of changes >