3GPP TSG-RAN5 Meeting #91-e	R5-212082r1

# Online, , 17th May 2021 - 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 1798 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of USIM Configurations 4, 7, 8, 10, 12 and 21 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc., Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-04-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | For TC 6.1.1.2, 6.2.1.1/2/3 and 6.3.1.1/2/3/4/5/8/9, as clearing RPLMN is needed in the preamble, AT/MMI command is needed to do so in current implementation, but such operation may be optional to UEs. Therefore it is proposed to update the USIM configurations to not contain any RPLMN information. And USIM Configurations #4, 7, 8, 10, 12 and 21 are used by these cases only. |  |  |  |  |  |  |  |  |
| Summary of change: |  | USIM Configurations #4, 7, 8, 10, 12 and 21 were updated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | AT/MMI command will be unsupported for some UEs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.523-1 CR 2334 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | r1: Update the encoding of EFLOCI & EFPSLOCI accoding to TS 31.102 and 24.008. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

[Start of change]

6.4	Signalling Test Case specific USIM Configurations

6.4.1	General

The default USIM fields are specified in section 4.8.3. Specific USIM fields are set according to the USIM configuration specified in the tables below. PLMN settings are defined in TS 36.523-1 [42] Table 6.0.1-1.

Note:	Changes to any existing USIM configuration can be done only if the change WILL NOT HAVE IMPACT on any of the tests which are referring to the configuration! To establish whether this might be the case, the test case author needs to review all tests in all RAN5 test specifications, which refer to the particular USIM configuration e.g. all test cases in TS 38.523-1 [12].

Table 6.4.1-1: USIM Configuration 1

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EFIMSI |  | The HPLMN (MCC+MNC) of the IMSI is set to PLMN1. |  |
| EFPLMNwAcT | 1 | Default | Default |
|  | 2 | PLMN17 | All specified |
|  | 3 | PLMN16 | NG-RAN |
|  |  | Remaining mandatory entries use default values |  |
| EFOPLMNwACT | 1 | PLMN15 | All specified |
|  |  | Remaining defined entries use default values |  |
| EFHPLMNwAcT | 1 | PLMN1 | NG-RAN |
| EFUST |  | Services 20, 42, 43 and 74 are supported. Service 71 is not supported (there is no EHPLMN list). |  |
| EFHPPLMN |  | 1 (6 minutes) |  |

Table 6.4.1-2: USIM Configuration 2

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EF5GS3GPPLOCI |  | PLMN4 |  |
| EFPLMNwAcT |  | Empty |  |
| EFIMSI |  | The HPLMN (MCC+MNC) of the IMSI is set to PLMN1. |  |
| EFUST |  | Service n°71 and n°74 are "available" |  |
| EFEHPLMN | 1 | PLMN15 |  |
|  | 2 | PLMN1 |  |
| EFLRPLMNSI |  | 01 |  |

Table 6.4.1-3: USIM Configuration 3

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EF5GS3GPPLOCI |  | PLMN4 |  |
| EFPLMNwAcT |  | Empty |  |
| EFIMSI |  | The HPLMN (MCC+MNC) of the IMSI is set to PLMN1. |  |
| EFUST |  | Service n°74 is "available" |  |
| EFEHPLMN |  | Empty |  |
| EFLRPLMNSI |  | 01 |  |

Table 6.4.1-4: USIM configuration 4

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EFEHPLMN | 1 | PLMN1 |  |
|  |  | Remaining mandatory entries use default values |  |
| EFPLMNwAcT | 1 | PLMN2 | NG-RAN |
|  |  | Remaining mandatory entries use default values |  |
| EFOPLMNwACT | 1 | PLMN3 | NG-RAN |
|  |  | Remaining mandatory entries use default values |  |
| EFUST |  | Services 20, 42 and 71 are supported. |  |
| EF5GS3GPPLOCI |  | FF FF…FF FE 01 (20 Bytes) |  |
| EFEPSLOCI |  | FF FF…FF FE 01 (18 Bytes) |  |
| EFPSLOCI |  | FF FF…FE FF 01 (14 Bytes) |  |
| EFLOCI |  | FF FF…FE FF 01 (11 Bytes) |  |

Table 6.4.1-5: USIM configuration 5

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EF5GS3GPPLOCI |  | PLMN4 (See preamble) |  |
| EFPLMNwAcT |  | Empty |  |
| EFIMSI |  | The HPLMN (MCC+MNC) of the IMSI is set to PLMN1. |  |
| EFUST |  | Service 71 is not supported |  |
|  |  | Service 74 is supported. |  |
| EFLRPLMNSI |  | 00 |  |
| EFEHPLMN |  | 0xFF..FF |  |

Table 6.4.1-6: USIM configuration 6

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EF5GS3GPPLOCI |  | PLMN1 (See preamble) |  |
| EFIMSI |  | The HPLMN (MCC+MNC) of the IMSI is set to PLMN3. |  |
| EFPLMNwAcT | 1 | PLMN1 | NG-RAN |
|  |  | Remaining mandatory entries use default values |  |
| EFOPLMNwACT | 1 | PLMN2 | NG-RAN |
|  | 2 | PLMN4 | NG-RAN |
|  |  | Remaining defined entries use default values |  |
| EFUST |  | Service 71 is not supported |  |

Table 6.4.1-7: USIM configuration 7

| USIM field | Priority | Value | Access technology | Comment |
| --- | --- | --- | --- | --- |
| E.FPLMNwAcT | 1 | PLMN13 | NG-RAN |  |
|  | 2 | PLMN13 | E-UTRAN |  |
| EFOPLMNwAcT | 1 | PLMN2 | All |  |
|  | 2 | PLMN14 | E-UTRAN |  |
|  | 3 | PLMN13 | NG-RAN |  |
| EF5GS3GPPLOCI |  | FF FF…FF FE 01 (20 Bytes) |  |  |
| EFEPSLOCI |  | FF FF…FF FE 01 (18 Bytes) |  |  |
| EFPSLOCI |  | FF FF…FE FF 01 (14 Bytes) |  |  |
| EFLOCI |  | FF FF…FE FF 01 (11 Bytes) |  |  |

Table 6.4.1-8: USIM configuration 8

| USIM field | Priority | Value | Access technology | Comment |
| --- | --- | --- | --- | --- |
| EFOPLMNwAcT | 1 | PLMN15 | NG-RAN |  |
|  | 2 | PLMN15 | E-UTRAN |  |
|  | 3 | PLMN17 | E-UTRAN |  |
|  | 4 | PLMN16 | NG-RAN |  |
| EF5GS3GPPLOCI |  | FF FF…FF FE 01 (20 Bytes) |  |  |
| EFEPSLOCI |  | FF FF…FF FE 01 (18 Bytes) |  |  |
| EFPSLOCI |  | FF FF…FE FF 01 (14 Bytes) |  |  |
| EFLOCI |  | FF FF…FE FF 01 (11 Bytes) |  |  |

Table 6.4.1-9: USIM configuration 9

| USIM field | Priority | Value | Access technology | Comment |
| --- | --- | --- | --- | --- |
| EFPLMNwAcT | 1 | PLMN1 | NG-RAN |  |
|  | 2 | PLMN15 | E-UTRAN |  |
| EFHPPLMN |  | 1(=6 min) |  | The HPLMN Search Period on the USIM shall be set to 6 minutes. |

Table 6.4.1-10: USIM configuration 10

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EFOPLMNwACT | 1 | PLMN14 | NG-RAN |
|  | 2 | PLMN13 | NG-RAN |
|  | 3 | PLMN2 | NG-RAN |
|  |  | Remaining defined entries use default values |  |
| EFUST |  | Service n°127 is "available" |  |
| EFHPPLMN |  | 1(=6 min) |  |
| EF5GS3GPPLOCI |  | FF FF…FF FE 01 (20 Bytes) |  |
| EFEPSLOCI |  | FF FF…FF FE 01 (18 Bytes) |  |
| EFPSLOCI |  | FF FF…FE FF 01 (14 Bytes) |  |
| EFLOCI |  | FF FF…FE FF 01 (11 Bytes) |  |

Table 6.4.1-11: USIM configuration 11

| USIM field |  | Priority |  | Value |  | Access Technology Identifier |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EF5GS3GPPLOCI |  |  |  | PLMN15 (See preamble) |  |  |  |
| EFIMSI |  |  |  | The HPLMN (MCC+MNC) of the IMSI is set to PLMN1 |  |  |  |
| EFPLMNwAcT |  | 1 |  | Default |  | Default |  |
|  |  | 2 |  | PLMN16 |  | NG-RAN |  |
|  |  |  |  | Remaining defined entries use default values |  | Default |  |
| EFOPLMNwACT |  | 1 |  | PLMN15 |  | NG-RAN |  |
|  |  |  |  | Remaining defined entries use default values |  | Default |  |
| EFHPLMNwAcT |  | 1 |  | PLMN1 |  | NG-RAN |  |
| EFUST |  |  |  | Services 20, 42, 43, 74 and 96 are supported. Service 71 is not supported (there is no EHPLMN list) |  |  |  |
| EFHPPLMN |  |  |  | 1 (6 minutes) |  |  |  |
| EFNASCONFIG |  |  |  | MinimumPeriodicSearchTimer set to 7 minutes |  |  |  |

Table 6.4.1-12: USIM configuration 12

| USIM field | Priority | Value | Access technology | Comment |
| --- | --- | --- | --- | --- |
| EFPLMNwAcT |  | 3GPP TS 31.102, Annex E |  | The EF is empty. |
| EFOPLMNwAcT | 1 | PLMN2 | NG-RAN |  |
|  | 2 | PLMN13 | E-UTRAN |  |
|  | 3 | PLMN13 | NG-RAN |  |
| EF5GS3GPPLOCI |  | FF FF…FF FE 01 (20 Bytes) |  |  |
| EFEPSLOCI |  | FF FF…FF FE 01 (18 Bytes) |  |  |
| EFPSLOCI |  | FF FF…FE FF 01 (14 Bytes) |  |  |
| EFLOCI |  | FF FF…FE FF 01 (11 Bytes) |  |  |

Table 6.4.1-13: USIM configuration 13

| USIM field | Priority | Value | Access technology | Comment |
| --- | --- | --- | --- | --- |
| EFOPLMNwAcT | 1 | PLMN2 | NG-RAN |  |
|  | 2 | PLMN2 | E-UTRAN |  |
|  | 3 | PLMN13 | NG-RAN |  |

Table 6.4.1-14: Void

Table 6.4.1-15: USIM Configuration 15

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EFIMSI |  | The HPLMN (MCC+MNC) of the IMSI is set to PLMN1. |  |
| EFFPLMN |  | PLMN2 |  |

Table 6.4.1-16: Void

Table 6.4.1-17: USIM Configuration 17

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EFIMSI |  | The HPLMN (MCC+MNC) of the IMSI is set to PLMN1. |  |
| EFPLMNwAcT | 1 | PLMN1 | NG-RAN |
| EFUST |  | Service n°126 is "available". |  |
| EFEHPLMN |  | PLMN1 |  |
| EFUAC_AIC and EFACC |  | For Bits b4 and b8 in byte 1 of EFACC (defined in TS 31.102 clause 4.2.15), only single bit is set to 1. Bits b1 and b2 in byte 1 of EFUAC_AIC (defined in TS 31.102 clause 4.4.11.7), and all remaining bits of EFACC and EFUAC_AIC are set to 0. |  |

Table 6.4.1-18: USIM Configuration 18

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EFIMSI |  | The HPLMN (MCC+MNC) of the IMSI is set to PLMN1. |  |
| EFPLMNwAcT | 1 | PLMN2 | NG-RAN |
| EFHPLMNwAcT | 1 | PLMN1 | NG-RAN |
| EFUST |  | Service n°126 (for UAC Access Identities Configuration) defined in TS 31.102 clause 4.2.8 is declared "available" |  |
| EFUAC_AIC |  | Bit b1 in byte 1 defined in TS 31.102 clause 4.4.11.7 is set to 1 and bit b2 in byte 1 is set to 0. |  |
| EFACC |  | For Bit b1 in byte 2 defined in TS 31.102 clause 4.2.15, only single bit set to 1. All remaining bits of byte 1 and byte 2 are set to 0. |  |

Table 6.4.1-19: USIM Configuration 19

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EFIMSI |  | The HPLMN (MCC+MNC) of the IMSI is set to PLMN1. |  |
| EFPLMNwAcT | 1 | PLMN2 | NG-RAN |
| EFHPLMNwAcT | 1 | PLMN1 | NG-RAN |
| EFUST |  | Service n°126 (for UAC Access Identities Configuration) defined in TS 31.102 clause 4.2.8 is declared "available" |  |
| EFUAC_AIC |  | Bit b2 in byte 1 defined in TS 31.102 clause 4.4.11.7 is set to 1 and bit b1 in byte 1 is set to 0. |  |
| EFACC |  | For Bit b2 in byte 2 defined in TS 31.102 clause 4.2.15, only single bit set to 1. All remaining bits of byte 1 and byte 2 are set to 0. |  |

Table 6.4.1-20: USIM Configuration 20

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EFECC |  | 144, 117 |  |

Table 6.4.1-21: USIM configuration 21

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EFOPLMNwACT | 1 | PLMN14 | NG-RAN |
|  | 2 | PLMN13 | NG-RAN |
|  | 3 | PLMN2 | NG-RAN |
|  |  | Remaining defined entries use default values |  |
| EFUST |  | Service n°127 is not "available" |  |
| EFHPPLMN |  | 1(=6 min) |  |
| EF5GS3GPPLOCI |  | FF FF…FF FE 01 (20 Bytes) |  |
| EFEPSLOCI |  | FF FF…FF FE 01 (18 Bytes) |  |
| EFPSLOCI |  | FF FF…FE FF 01 (14 Bytes) |  |
| EFLOCI |  | FF FF…FE FF 01 (11 Bytes) |  |

Table 6.4.1-22: USIM Configuration 22

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EFUST |  | Service n°19 and n°51 defined in TS 31.102 clause 4.2.8 is declared "service not available" |  |

[End of change]