3GPP TSG-RAN5 Meeting #91-e	R5-212190r1

# Online, , 17th May 2021 - 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 1208 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update p-Max of PCC to 6.2A.1.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Guangdong OPPO Mobile Telecom. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-04-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Configure p-Max of PCC to avoid SCC dropping during measurement. |  |  |  |  |  |  |  |  |
| Summary of change: |  | p-Max of PCC is set to 20dBm for UE maximum output power for inter-band CA |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | SCC might drop during measurement. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2A.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | r1: Fix 3GU issue of incorrect release. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Unchanged sections omitted >>

6.2A.1.1	UE maximum output power for CA (2UL CA)

Editor’s Note:

No test points are defined for intra-band contiguous UL CA since there is no configuration satisfying MPR=0dB requirements in RAN4. Testing with lowest value of MPR will be covered in 6.2A.2.

6.2A.1.1.1	Test purpose

To verify that the error of the UE maximum output power in two uplink carrier aggregation does not exceed the range prescribed by the specified nominal maximum output power and tolerance.

An excess maximum output power has the possibility to interfere to other channels or other systems. A small maximum output power decreases the coverage area.

6.2A.1.1.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that support NR 2UL CA.

NOTE: Testing for intra-band contiguous CA can’t be performed due to lack of appropriate test points.

6.2A.1.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.2A.1.0.

6.2A.1.1.4	Test description

6.2A.1.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of e nvironmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR CA configurations specified in 5.5A. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in table 6.2A.1.1.4.1-1 and table 6.2A.1.1.4.1-2. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2A.1.1.4.1-1: Inter-band CA Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Low range for PCC and SCC High range for PCC and SCC |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Lowest NRB_agg, Highest NRB_agg |  |  |
| Test SCS as specified in Table 5.5A.3-1 |  | Lowest, Highest |  |  |
| Test Parameters |  |  |  |  |
| Test ID | Downlink Configuration for PCC & SCC | Uplink Configuration |  |  |
|  |  | Modulation for all CCs | RB allocation (NOTE 1) |  |
|  |  | (NOTE 2) | PCC | SCC |
| 1 | N/A for this test | DFT-s-OFDM Pi/2 BPSK | Inner Full | Inner Full |
| 2 |  | DFT-s-OFDM Pi/2 BPSK | Inner 1RB Left | Inner 1RB Left |
| 3 |  | DFT-s-OFDM Pi/2 BPSK | Inner 1RB Right | Inner 1RB Right |
| 4 |  | DFT-s-OFDM QPSK | Inner Full | Inner Full |
| 5 |  | DFT-s-OFDM QPSK | Inner 1RB Left | Inner 1RB Left |
| 6 |  | DFT-s-OFDM QPSK | Inner 1RB Right | Inner 1RB Right |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports Pi/2 BPSK in FR1. |  |  |  |  |

Table 6.2A.1.1.4.1-2: Intra-band CA Test Configuration Table

NOTE: No test points are defined since there is no configuration satisfying MPR=0dB requirements in RAN4.

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.3 for TE diagram and section A.3.2  for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement Channel is set according to Table 6.2A.1.1.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.2A.1.1.4.3.

6.2A.1.1.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1, C.2 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 38.508-1 [5] clause 5.5.1. Message contents are defined in clause 6.2A.1.1.4.3.

3.	SS activates SCC by sending the activation MAC CE (Refer TS 38.321 [18], clauses 5.9, 6.1.3.10). Wait for at least 2 seconds (Refer TS 38.133[19], clause 9.3).

4.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.2A.1.1.4.1-1 on both PCC and SCC. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

5.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level corresponding to Power Class 3.

6.	Measure the sum of mean transmitted power over all component carriers in the CA configuration of the radio access mode. The period of measurement shall be at least the continuous duration of one active sub-frame (1ms) and in the uplink symbols. For TDD symbols with transient periods are not under test.

6.2A.1.1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 and 5.4 with the following exceptions.

Table 6.2A.1.1.4.3-1: PUSCH-Config

Derivation Path: TS 38.508-1 [5], Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED

Table 6.2A.1.1.4.3-2: FrequencyInfoUL-SIB

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 20 |  | Power class 3 and Inter-band CA |

6.2A.1.1.5	Test requirement

The maximum output power for CA, derived in step 6 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2A.1.1.5-1 for Inter-band 2 UL CA configuration. The test requirement of configurations for CA operating band including Band n41 also apply for the corresponding CA operating bands with Band n90 replacing Band n41.

Table 6.2A.1.1.5-1: Maximum Output Power test requirement for Power Class 3

| NR CA Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n1A-n78A |  |  |  |  | 23 | +2+TT/-32-TT |  |  |
| CA_n1A-n79A |  |  |  |  | 23 | +2+TT/-32-TT |  |  |
| CA_n3A-n41A |  |  |  |  | 23 | +2+TT/-32-TT |  |  |
| CA_n3A-n78A |  |  |  |  | 23 | +2+TT/-32-TT |  |  |
| CA_n3A-n79A |  |  |  |  | 23 | +2+TT/-32-TT |  |  |
| CA_n5A-n78A |  |  |  |  | 23 | +2+TT/-32-TT |  |  |
| CA_n5A-n79A |  |  |  |  | 23 | +2+TT/-32-TT |  |  |
| CA_n8A-n78A |  |  |  |  | 23 | +2+TT/-32-TT |  |  |
| CA_n39A-n41A |  |  |  |  | 23 | +2+TT/-32-TT |  |  |
| CA_n40A-n41A |  |  |  |  | 23 | +2+TT/-32-TT |  |  |
| CA_n41A-n79A |  |  |  |  | 23 | +2+TT/-32-TT |  |  |
| CA_n50A-n78A |  |  |  |  | 23 | +2+TT/-32-TT |  |  |
| CA_n66A-n71A |  |  |  |  | 23 | +2+TT/-32-TT |  |  |
| CA_n70A-n71A |  |  |  |  | 23 | +2+TT/-32-TT |  |  |
| NOTE 1: Void NOTE 2: 2 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 3: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 4: For inter-band carrier aggregation the maximum power requirement should apply to the total transmitted power over all component carriers (per UE). NOTE 5: Power class 3 is the default power class unless otherwise stated NOTE 6: The TT for 2UL CA Maximum Output Power is in the maximum TT among all UL CCs. For TT of each UL CC refer to Table6.2A.1.1.5-2. |  |  |  |  |  |  |  |  |

<< End of changes >>