3GPP TSG-RAN5 Meeting # 91-e	R5-212217r2

# Online, May 17th -28th, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 1215 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Cleanup for TS 38.521-1 spurious emission for UE co-existence table (CA Bands) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-04-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Protected bands in CA combinations are missing harmonic exception. CA_n39-n79. Spec 38.101 (Rel-15, Rel-16, Rel-17) clause 6.5A.3.2.3 changed with CRs R4-2102595, R4-2103160 and R4-2102603. Following the clean up changes in 38.101 it is also required update 38.521-1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In Table 6.5A.3.2.0.3-2 band CA_n39-n79 row, band n78 is missing harmonic exception as found in single band n39. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE may fail the co-existence compliance tests for CA_n39-n79 due to CA combination missing harmonic exception. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5A.3.2.0 and 6.5A.3.2.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1:Corrected unchecking “proposed change affects” boxes. r2: removed word “Table” r3: updated with CA_n39-n79 related change in 6.5A.3.2.1.5 |  |  |  |  |  |  |  |  |

#### 6.5A.3.2	Spurious emission for UE co-existence

Editor’s note:  The following aspects are either missing or not yet determined:

- Extending the coverage of the TCs with intra-band CA scenarios is FFS.

##### 6.5A.3.2.0	Minimum conformance requirements

###### 6.5A.3.2.0.1	Spurious emissions for UE co-existence for intra-band contiguous CA

This clause specifies the requirements for the specified intra-band contiguous carrier aggregation configurations for coexistence with protected bands, the requirements in Table 6.5A.3.2.0.1-1 apply.

NOTE:	For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth defined for the protected band.

Table 6.5A.3.2.0.1-1: Requirements for uplink intra-band contiguous carrier aggregation

| NR CA combination | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected Band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n7 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 10, 12, 13, 14, 17, 20, 22, 26, 27, 28, 29, 30, 31, 32, 33, 34, 40, 42, 43, 50, 51, 52, 65, 66, 67, 68, 72, 74, 75, 76, 85, NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 1, 2, 3 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 1, 2, 3 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 1, 2 |
| CA_n41 | E-UTRA Band 1, 2, 3, 4, 5, 8, 10, 12, 13, 14, 17, 24, 25, 26, 27, 28, 29, 30, 34, 39, 42, 44, 45, 48, 50, 51, 52, 65, 66, 70, 71, 73, 74, 85, NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | E-UTRA Band 9, 11, 18, 19, 21 | FDL_low | - | FDL_high | -50 | 1 | 6 |
|  | Frequency range | 1884.5 |  | 1915.7 | -41 | 0.3 | 5, 6 |
| CA_n48 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 50, 51, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
| CA_n77 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 39, 40, 41, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 5 |
| CA_n78 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 39, 40, 41, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 5 |
| CA_n79 | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 28, 34, 39, 40, 41, 42, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 5 |
| NOTE 1: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. NOTE 2: This requirement is applicable for any channel bandwidths within the range 2500 - 2570 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2560.5 - 2562.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2552 - 2560 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 3: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 4: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x RBsize kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 5: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz. NOTE 6: This requirement applies when the NR carrier is confined within 2545 – 2575 MHz or 2595 – 2645 MHz and the channel bandwidth is 10 or 20 MHz |  |  |  |  |  |  |  |

###### 6.5A.3.2.0.2	Void

###### 6.5A.3.2.0.3	Spurious emissions for UE co-existence for Inter-band CA

For inter-band carrier aggregation with the uplink assigned to two NR bands, the requirements in Table 6.5A.3.2.0.3-1 and Table 6.5A.3.2.0.3-2 apply on each component carrier with both component carriers are active.

NOTE:	For inter-band carrier aggregation with uplink assigned to two NR bands, the requirements in Table 6.5A.3.2.0.3-1 and Table 6.5A.3.2.0.3-2 could be verified by measuring spurious emissions at the specific frequencies where second and third order intermodulation products generated by the two transmitted carriers can occur; in that case, the requirements for remaining applicable frequencies in Table 6.5A.3.2.0.3-1 and Table 6.5A.3.2.0.3-2 would be considered to be verified by the measurements verifying the one uplink inter-band CA UE to UE co-existence requirements.

Table 6.5A.3.2.0.3-1: Requirements for uplink inter-band carrier aggregation (two bands)

| NR CA combination | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected Band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n3-n78 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 39, 40, 41, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n8-n78 | E-UTRA Band 1, 8, 20, 28, 34, 39, 40, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 7, 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| NOTE 1: FDL_low and FDL_high refer to each frequency band specified in Table 5.2-1 in TS 38.101-1 or Table 5.5-1 in TS 36.101 NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 -1915.7 MHz NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. NOTE 5: Void. |  |  |  |  |  |  |  |

NOTE:	To simplify Table 6.5A.3.2.0.3-1, E-UTRA band numbers are listed for bands which are specified only for E-UTRA operation or both E-UTRA and NR operation. NR band numbers are listed for bands which are specified only for NR operation.

Table 6.5A.3.2.0.3-2: Requirements for uplink inter-band carrier aggregation (two bands) Rel-16

| NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected Band | Frequency range (Mhz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n1-n78 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 40, 41, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 4, 6 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 4, 6, 7 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 4, 6, 7 |
| CA_n1-n79 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 21, 26, 28, 34, 40, 41, 42, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 4, 6 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 4, 6, 7 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 4, 6, 7 |
| CA_n3-n78 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 39, 40, 41, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n8-n39 | E-UTRA Band 1, 28, 34, 40, 50, 51, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 22, 41, 42 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 8 | FDL_low | - | FDL_high | -50 | 1 | 4 |
| CA_n8-n78 | E-UTRA Band 1,8, 11, 20, 21, 28, 34, 39, 40, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 41, 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n39-n40 | E-UTRA Band 1, 8, 22, 26, 28, 34, 41, 42, 44, 45, 50, 51, 52, 73, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1805 |  | 1855 | -40 | 1 | 8 |
|  | Frequency range | 1855 |  | 1880 | -15.5 | 5 | 4, 7, 8 |
| CA_n39-n41 | E-UTRA Band 1, 8, 26, 28, 34, 40, 42, 44, 45, 50, 51, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1805 | - | 1855 | -40 | 1 | 4 |
|  | Frequency range | 1855 | - | 1880 | -15.5 | 5 | 4, 7, 8 |
| CA_n39-n79 | E-UTRA Band 1, 8, 28, 34, 40, 41, 44, 45 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1805 | - | 1855 | -40 | 1 | 4, 8 |
|  | Frequency range | 1855 | - | 1880 | -15.5 | 5 | 4, 7, 8 |
| CA_n41-n79 | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 28, 34, 40, 42, 44, 45, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| NOTE 1: FDL_low and FDL_high refer to each frequency band specified in Table 5.2-1 or Table 5.5-1 in TS 36.101 NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 and Table 6.5A.3.1-1 from the edge of the channel bandwidth. NOTE 5: Void. NOTE 6: This requirement is applicable for any channel bandwidths within the range 1920 – 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 – 1938 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 7: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 8: This requirement is only applicable for carriers with bandwidth confined within 1885-1920 MHz (requirement for carriers with at least 1RB confined within 1880 - 1885 MHz is not specified). This requirement applies for an uplink transmission bandwidth less than or equal to 54 RB for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1892.5 - 1894.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1895 - 1903 MHz. |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 38.101-1 [2] subclause 6.5A.3.2.

##### 6.5A.3.2.1	Spurious emissions for UE co-existence for CA (2UL CA)

Editor’s Note: TP Analysis related to n28 in Table 6.5A.3.2.1.5-1a need FFS in TR 38.905

6.5A.3.2.1.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to co-existing systems for the specified bands which has specific requirements in terms of transmitter spurious emissions for 2UL CA.

6.5A.3.2.1.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that support 2DL and 2UL CA.

6.5A.3.2.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in subclause 6.5A.3.2.0.

6.5A.3.2.1.4	Test description

6.5A.3.2.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR CA configuration specified in clause 5.5A. All of these configurations shall be tested with applicable test parameters for each CA configuration, are shown in Table 6.5A.3.2.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5A.3.2.1.4.1-1: Inter band CA Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | NC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1.1.3 for inter band CA in FR1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | For test frequencies refer to “Range” columns. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Refer to “PCC NRB@SCS”and “SCC NRB@SCS ” columns |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test SCS as specified in Table 5.5A.3-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Lowest SCS for default test points. CA configuration specific test points: Refer to “PCC NRB@SCS”and “SCC NRB@SCS ” columns |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID |  |  |  | CA Configuration / NRB_agg (Note 4) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | DL Allocation |  |  |  |  |  |  |  |  | UL Allocation (Note 2,3) |  |  |  |  |  |  |
|  |  |  |  | CA Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PCC NRB @SCS |  |  |  | SCC NRB@SCS |  |  | CC MOD |  |  |  | PCC & SCC RB allocation |  |  |  |  | CC MOD |  | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |  |
|  |  |  |  | PCC |  |  |  |  |  |  |  | SCC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PCC | SCC |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Band |  |  |  | Range |  |  |  | Band |  |  |  | Range |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Default Test Settings for a CA_XA-YA Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | X |  |  |  | Low |  |  |  | Y |  |  |  | Low |  |  |  | Highest NRB@SCS |  |  | Highest NRB@SCS |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  |  | 1@0 |  |  |  | 1@0 |  |
| 2 |  | X |  |  |  | High |  |  |  | Y |  |  |  | High |  |  |  | Highest NRB@SCS |  |  | Highest NRB@SCS |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  |  | 1@RBmax |  |  |  | 1@RBmax |  |
| Test Settings for CA_n1A-n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n1 |  |  |  | High |  |  |  | n78 |  |  |  | High |  |  |  | 106@15kHz |  |  |  |  |  | 273@30kHz |  |  | CP-OFDM QPSK |  |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  |  | 1@105 |  |  | 1@272 |
| 2 | n1 |  |  |  | Low |  |  |  | n78 |  |  |  | 3455MHz |  |  |  | 106@15kHz |  |  |  |  |  | 273@30kHz |  |  | CP-OFDM QPSK |  |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  |  | 1@0 |  |  | 1@0 |
| 3 | n1 |  |  |  | Low |  |  |  | n78 |  |  |  | 3475MHz |  |  |  | 106@15kHz |  |  |  |  |  | 273@30kHz |  |  | CP-OFDM QPSK |  |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  |  | 1@0 |  |  | 1@0 |
| 4 | n1 |  |  |  | High |  |  |  | n78 |  |  |  | High |  |  |  | 106@15kHz |  |  |  |  |  | 106@30kHz |  |  | CP-OFDM QPSK |  |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  |  | 1@105 |  |  | 106@0 |
| 5 | n1 |  |  |  | Low |  |  |  | n78 |  |  |  | High |  |  |  | 106@15kHz |  |  |  |  |  | 106@30kHz |  |  | CP-OFDM QPSK |  |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  |  | 1@0 |  |  | 106@0 |
| Test Settings for CA_n3A-n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  | n3 |  |  |  | Mid |  |  |  | n78 |  |  |  | Mid |  |  |  | 160@15kHz |  |  | 270@15kHz |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@160 |  |  | 1@270 |  |  |
| 2 |  |  | n3 |  |  |  | Mid |  |  |  | n78 |  |  |  | Mid |  |  |  | 78@30KHz |  |  | 273@30KHz |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@78 |  |  | 1@273 |  |  |
| 3 |  |  | n3 |  |  |  | Mid |  |  |  | n78 |  |  |  | Mid |  |  |  | 38@60KHz |  |  | 135@60KHz |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@38 |  |  | 1@135 |  |  |
| Test Settings for CA_n8A-n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  | n8 |  |  |  | Mid |  |  |  | n78 |  |  |  | Mid |  |  |  | 106@15kHz |  |  | 270@15kHz |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  | 1@0 |  |  |
| 2 |  |  | n8 |  |  |  | Mid |  |  |  | n78 |  |  |  | Mid |  |  |  | 106@15kHz |  |  | 270@15kHz |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@106 |  |  | 1@270 |  |  |
| 3 |  |  | n8 |  |  |  | Low |  |  |  | n78 |  |  |  | Low |  |  |  | 51@30KHz |  |  | 273@30KHz |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@51 |  |  | 1@273 |  |  |
| 4 |  |  | n8 |  |  |  | Mid |  |  |  | n78 |  |  |  | Mid |  |  |  | 51@30KHz |  |  | 273@30KHz |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@51 |  |  | 1@273 |  |  |
| Test Settings for CA_n41A-n79A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  | n41 |  |  |  | High |  |  |  | n79 |  |  |  | 4870MHz |  |  |  | 273@30KHz |  |  | 273@30KHz |  |  |  | QPSK |  |  | NA |  |  |  |  | QPSK / CP-OFDM QPSK |  |  | 1@272 |  |  | 1@0 |  |  |
| 2 |  |  | n41 |  |  |  | Mid |  |  |  | n79 |  |  |  | Low |  |  |  | 273@30KHz |  |  | 273@30KHz |  |  |  | QPSK |  |  | NA |  |  |  |  | QPSK / CP-OFDM QPSK |  |  | 1@136 |  |  | 1@136 |  |  |
| 3 |  |  | n41 |  |  |  | High |  |  |  | n79 |  |  |  | Low |  |  |  | 273@30KHz |  |  | 273@30KHz |  |  |  | QPSK |  |  | NA |  |  |  |  | QPSK / CP-OFDM QPSK |  |  | 1@272 |  |  | 1@272 |  |  |
| 4 |  |  | n41 |  |  |  | High |  |  |  | n79 |  |  |  | 4450MHz |  |  |  | 273@30KHz |  |  | 273@30KHz |  |  |  | QPSK |  |  | NA |  |  |  |  | QPSK / CP-OFDM QPSK |  |  | 1@272 |  |  | 1@136 |  |  |
| 5 |  |  | n41 |  |  |  | High |  |  |  | n79 |  |  |  | 4500MHz |  |  |  | 273@30KHz |  |  | 273@30KHz |  |  |  | QPSK |  |  | NA |  |  |  |  | QPSK / CP-OFDM QPSK |  |  | 1@272 |  |  | 1@136 |  |  |
| 6 |  |  | n41 |  |  |  | Mid |  |  |  | n79 |  |  |  | Low |  |  |  | 273@30KHz |  |  | 273@30KHz |  |  |  | QPSK |  |  | NA |  |  |  |  | QPSK / CP-OFDM QPSK |  |  | 1@136 |  |  | 1@0 |  |  |
| 7 |  |  | n41 |  |  |  | High |  |  |  | n79 |  |  |  | Mid |  |  |  | 273@30KHz |  |  | 273@30KHz |  |  |  | QPSK |  |  | NA |  |  |  |  | QPSK / CP-OFDM QPSK |  |  | 1@272 |  |  | 1@136 |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, in Figure A.3.1.1.3 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [6] subclause4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.5A.3.2.1.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5.  Message contents are defined in clause 6.5A.3.2.1.4.3.

6.5A.3.2.1.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1, and Annex C.2 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 38.508-1 [5] clause 5.5.1. Message contents are defined in clause 6.5A.3.2.1.4.3.

3.	SS activates SCC by sending the activation MAC CE (Refer TS 38.321 [18], clauses 5.9, 6.1.3.10). Wait for at least 2 seconds (Refer TS 38.133[19], clause9.3).

4.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.5A.3.2.1.4.1-1 on both PCC and SCC. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

5.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE until the UE transmits at PUMAX level.

6.	Measure the power of the transmitted signal with a measurement filter of bandwidths according to Table 6.5.3.2.3-1. The centre frequency of the filter shall be stepped in contiguous steps according to Table 6.5.3.2.3-1. The measured power shall be verified for each step. The measurement period shall capture the active time slots.

6.5A.3.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6.

6.5A.3.2.1.5	Test requirement

Test requirements for Spurious Emissions UE Co-existence are the same as the minimum requirements. The measured average power of spurious emission, derived in step 6, shall not exceed the described value in Table 6.5A.3.2.1.5-1. The test requirement of configurations for CA operating band including Band n41 also apply for the corresponding CA operating bands with Band n90 replacing Band n41.

Table 6.5A.3.2.1.5-1: Requirements for uplink inter-band carrier aggregation (two bands) Rel-15

| NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected Band | Frequency range (Mhz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n3-n78 | E-UTRA Band 3, 34, 39 | FDL_low | - | FDL_high | -50+TT | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41+TT | 0.3 | 3 |
| CA_n8-n78 | E-UTRA Band 8, 20, 28, 34, 39, 40 | FDL_low | - | FDL_high | -50+TT | 1 |  |
|  | E-UTRA Band 3, 7,41 | FDL_low | - | FDL_high | -50+TT | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41+TT | 0.3 | 3 |
| CA_n41-n79 | E-UTRA Band 1, 3, 5, 8, 9, 18, 19, 28, 34, 40, 44, 65 or NR Band n1, n3, n8, n28, n34, n40 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| NOTE 1: FDL_low and FDL_high refer to each frequency band specified in Table 5.2-1 or Table 5.5-1 in TS 36.101 NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 and Table 6.5A.3.1-1 from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

Table 6.5A.3.2.1.5-1a: Requirements for uplink inter-band carrier aggregation (two bands) Rel-16

| NR CA Configuration | Spurious emission |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected Band |  | Frequency range (Mhz) |  |  |  |  | Maximum Level (dBm) |  |  | MBW (MHz) |  | NOTE |
| CA_n1-n78 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 40, 41, 65 |  | FDL_low |  | - |  | FDL_high |  |  | -50+TT |  | 1 |  |
|  | Frequency range |  | 1880 |  | - |  | 1895 |  |  | -40+TT |  | 1 | 4, 6 |
|  | Frequency range |  | 1895 |  | - |  | 1915 |  |  | -15.5+TT |  | 5 | 4, 6, 7 |
|  | Frequency range |  | 1915 |  | - |  | 1920 |  |  | +1.6+TT |  | 5 | 4, 6, 7 |
| CA_n1-n79 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 21, 26, 28, 34, 40, 41, 42, 65, 74 | FDL_low |  | - |  | FDL_high |  |  | -50+TT |  | 1 |  |  |
|  | Frequency range | 1880 |  | - |  | 1895 |  |  | -40+TT |  | 1 |  | 4, 6 |
|  | Frequency range | 1895 |  | - |  | 1915 |  |  | -15.5+TT |  | 5 |  | 4, 6, 7 |
|  | Frequency range | 1915 |  | - |  | 1920 |  |  | +1.6+TT |  | 5 |  | 4, 6, 7 |
| CA_n3-n78 | E-UTRA Band 3, 34, 39 |  | FDL_low |  | - |  | FDL_high | -50+TT |  |  | 1 |  |  |
|  | Frequency range |  | 1884.5 |  | - |  | 1915.7 | -41+TT |  |  | 0.3 |  | 3 |
| CA_n8-n39 | E-UTRA Band 8, 20, 28, 34, 39, 40 |  | FDL_low |  | - |  | FDL_high | -50+TT |  |  | 1 |  |  |
|  | E-UTRA Band 3, 7,41 |  | FDL_low |  | - |  | FDL_high | -50+TT |  |  | 1 |  | 2 |
|  | E-UTRA Band 11, 21 |  | FDL_low |  | - |  | FDL_high | -50+TT |  |  | 1 |  | 5 |
|  | Frequency range |  | 860 |  | - |  | 890 | -40+TT |  |  | 1 |  | 4,5 |
|  | Frequency range |  | 1884.5 |  | - |  | 1915.7 | -41+TT |  |  | 0.3 |  | 3 |
| CA_n8-n78 | E-UTRA Band 8, 20, 28, 34, 39, 40 |  | FDL_low |  | - |  | FDL_high | -50+TT |  |  | 1 |  |  |
|  | E-UTRA Band 3, 7,41 |  | FDL_low |  | - |  | FDL_high | -50+TT |  |  | 1 |  | 2 |
|  | E-UTRA Band 11, 21 |  | FDL_low |  | - |  | FDL_high | -50+TT |  |  | 1 |  | 5 |
|  | Frequency range |  | 860 |  | - |  | 890 | -40+TT |  |  | 1 |  | 4,5 |
|  | Frequency range |  | 1884.5 |  | - |  | 1915.7 | -41+TT |  |  | 0.3 |  | 3 |
| CA_n39-n40 | E-UTRA Band 1, 8, 22, 26, 28, 34, 41, 42, 44, 45, 50, 51, 52, 73, 74 |  | FDL_low |  | - |  | FDL_high | -50 |  |  | 1 |  |  |
|  | NR Band n77, n78, n79 |  | FDL_low |  | - |  | FDL_high | -50 |  |  | 1 |  | 2 |
|  | Frequency range |  | 1805 |  |  |  | 1855 | -40 |  |  | 1 |  | 8 |
|  | Frequency range |  | 1855 |  |  |  | 1880 | -15.5 |  |  | 5 |  | 4, 7, 8 |
| CA_n39-n41 | E-UTRA Band 1, 8, 26, 28, 34, 40, 42, 44, 45, 50, 51, 74 |  | FDL_low |  | - |  | FDL_high | -50 |  |  | 1 |  |  |
|  | NR Band n77, n78, n79 |  | FDL_low |  | - |  | FDL_high | -50 |  |  | 1 |  | 2 |
|  | Frequency range |  | 1805 |  | - |  | 1855 | -40 |  |  | 1 |  | 4 |
|  | Frequency range |  | 1855 |  | - |  | 1880 | -15.5 |  |  | 5 |  | 4, 7, 8 |
| CA_n39-n79 | E-UTRA Band 1, 8, 28, 34, 40, 41, 44, 45 |  | FDL_low |  | - |  | FDL_high | -50 |  |  | 1 |  |  |
|  | NR Band n78 |  | FDL_low |  | - |  | FDL_high | -50 |  |  | 1 |  | 2 |
|  | Frequency range |  | 1805 |  | - |  | 1855 | -40 |  |  | 1 |  | 4, 8 |
|  | Frequency range |  | 1855 |  | - |  | 1880 | -15.5 |  |  | 5 |  | 4, 7, 8 |
| CA_n41-n79 | E-UTRA Band 1, 3, 5, 8, 9, 18, 19, 28, 34, 40, 44, 65 or NR Band n1, n3, n8, n28, n34, n40 |  | FDL_low |  | - |  | FDL_high | -50 |  |  | 1 |  |  |
|  | Frequency range |  | 1884.5 |  | - |  | 1915.7 | -41 |  |  | 0.3 |  | 3 |
| NOTE 1: FDL_low and FDL_high refer to each frequency band specified in Table 5.2-1 or Table 5.5-1 in TS 36.101 NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 and Table 6.5A.3.1-1 from the edge of the channel bandwidth. NOTE 5: This requirement is applicable only for the following cases: - for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 902.5 MHz ≤ Fc < 907.5 MHz with an uplink transmission bandwidth less than or equal to 20 RB - for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 907.5 MHz ≤ Fc ≤ 912.5 MHz without any restriction on uplink transmission bandwidth. - for carriers of 10 MHz channel bandwidth when carrier centre frequency (Fc) is Fc = 910 MHz with an uplink transmission bandwidth less than or equal to 32 RB with RBstart> 3. NOTE 6: This requirement is applicable for any channel bandwidths within the range 1920 – 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 – 1938 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 7: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 8: This requirement is only applicable for carriers with bandwidth confined within 1885-1920 MHz (requirement for carriers with at least 1RB confined within 1880 - 1885 MHz is not specified). This requirement applies for an uplink transmission bandwidth less than or equal to 54 RB for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1892.5 - 1894.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1895 - 1903 MHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |

<<< unchanged parts are skipped >>>

### 6.5A.4	Transmit intermodulation for CA

Editor’s Note: The following aspects are either missing or not yet determined:

- Extending the coverage of the TCs with intra-band CA scenarios is FFS

#### 6.5A.4.0	Minimum conformance requirements

For inter-band carrier aggregation with uplink assigned to two NR bands, the transmit intermodulation requirement is specified in Table 6.5.4-1 which shall apply on each component carrier with both component carriers active.

#### 6.5A.4.1	Transmit intermodulation for CA (2UL CA)

##### 6.5A.4.1.1	Test purpose

To verify that the UE transmit intermodulation does not exceed the described value in the test requirement.

The transmit intermodulation performance is a measure of the capability of the transmitter to inhibit the generation of signals in its non-linear elements caused by presence of the wanted signal and an interfering signal reaching the transmitter via the antenna.