3GPP TSG-RAN WG5 Meeting #91-e	R5-212286r1

# Online, , 17th May 2021 - 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-1 | CR | 0126 | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to Default Message content HTTP POST, PUT and DELETE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCPTT-ConTest |  |  |  |  |  | Date: |  |  | 2021-05-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - Condition for temporary group creation in HTTP POST and message body missing - HTTP DELETE has no message body – TS 24.481 clause 6.3.5.2.1 - Condition for Message Body of HTTP PUT missing |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Condition for temporary group creation in HTTP POST added - Correction of message content HTTP DELETE - Condition in HTTP PUT added - Typo corrected in Table 5.5.4.4-1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misleading message contents may result in incorrect MCPTT test case implementation, test cases cannot be validated |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.4.3, 5.5.4.4, 5.5.4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 36.579-1 CR 0148 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Revision r1: Typo corrected in Table 5.5.4.4-1 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 5.5.4.3	POST

Table 5.5.4.3-1: HTTP POST

| Derivation Path: RFC 2616 [26] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Status-Line |  |  |  |  |
| Method | "POST" |  |  |  |
| Request-URI |  |  |  |  |
| uri | tsc_MCX_IdMS_auth_UriPath | points to the Authorisation endpoint of the IdM Server | TS 33.180 [94] | AUTH, USERAUTH |
|  | tsc_MCX_IdMS_userauth_UriPath | points to the endpoint verifying the user authentication; same URI as provided to the UE in the action attribute of the HTML login form | TS 33.180 [94] HTML 4.01 Specification [105] | USERAUTH |
|  | tsc_MCX_IdMS_token_UriPath | points to the Token endpoint of the IdM Server | TS 33.180 [94] | TOKEN |
|  | tsc_MCX_KMS_init_UriPath | "KMS Initialize" request according to TS 33.180 [94] D.2.3 | TS 33.180 [94] | KMSINIT |
|  | tsc_MCX_KMS_keyprov_UriPath | "KMS KeyProvision" request according to TS 33.180 [94] D.2.4 | TS 33.180 [94] | KMSKEY |
|  | "/" & tsc_MCX_GMSXCAPRootURI & "/" & "org.openmobilealliance.groups/global/byGroupID/" & px_MCPTT_Group_T_ID | Points to the temporary group configuration document to be created | TS 24.481[11] clause 6.3.14.2 | TEMPGROUP |
| HTTP-Version | "HTTP/1.1" |  |  |  |
| Cache-Control |  |  | RFC 2616 [26] |  |
| cache-directive | "no-cache" |  |  |  |
| Authorization |  |  | RFC 2617 [72] | KMSINIT, KMSKEY |
| authentication-scheme | “Bearer” |  | RFC 6750 [104] |  |
| b64token | Access token as assigned to the UE by Token Response |  | RFC 6750 [104] |  |
| Content-Type |  |  |  | AUTH, USERAUTH, TOKEN |
| media-type | "application/x-www-form-urlencoded" |  |  |  |
| Content-Type |  | present in case of KMS request security |  | (KMSINIT OR KMSKEY) AND pc_MCX_KMS_RequestSecurity |
| media-type | "application/xml" |  | RFC 7303 [112] |  |
| Content-Type |  |  |  | TEMPGROUP |
| media-type | application/vnd.oma.poc.groups+xml |  |  |  |
| Message-body |  |  |  | AUTH |
| Authentication Request | As described in Table 5.5.4.10.1-1 |  |  |  |
| Message-body |  |  | HTML 4.01 Specification [105] | USERAUTH |
| user | px_MCX_User_A_username |  |  |  |
| password | px_MCX_User_A_password |  |  |  |
| Message-body |  |  |  | TOKEN |
| Token request | As described in Table 5.5.4.10.3-1 |  |  |  |
| Message-body |  | present in case of KMS request security |  | (KMSINIT OR KMSKEY) AND pc_MCX_KMS_RequestSecurity |
| Signed KMS Request | As described in Table 5.5.4.10.9-1 |  |  |  |
| Message-body |  |  |  | TEMPGROUP |
| Temporary Group Creation Document" | As described in Table 5.5.7.1-3 |  |  |  |

| Condition | Explanation |
| --- | --- |
| TEMPGROUP | Message/IE sent only in temporary group creation scenario |
| NOTE: For further conditions see table 5.5.1-1 |  |

#### 5.5.4.4	PUT

Table 5.5.4.4-1: HTTP PUT

| Derivation Path: RFC 2616 [26] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Request-line |  |  |  |  |
| Method | "PUT" |  |  |  |
| Request-URI | "/" & tsc_MCX_GMSXCAPRootURI & "/" & "org.openmobilealliance.groups/global/byGroupID/" & px_MCPTT_Group_B_ID | XCAP URI in users tree where the XUI is set to a group creation XUI configuration parameter | TS 24.481 [11] clause 6.3.2.2.1 | GROUPCREATE |
| Content-Type |  |  |  | GROUPCREATE |
| media-type | application/vnd.3gpp.GMOP+xml |  |  |  |
| Message-body |  |  |  | GROUPCREATE |
| Group Creation Document | As described in Table 5.5.7.1-2 |  |  |  |

| Condition | Explanation |
| --- | --- |
| GROUPCREATE | Message/IE sent only in group creation scenario |
| NOTE: For further conditions see table 5.5.1-1 |  |

#### 5.5.4.5	DELETE

Table 5.5.4.5-1: HTTP DELETE

| Derivation Path: RFC 2616 [26] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Request-line |  |  |  |  |
| Method | "DELETE" |  |  |  |
| Request-URI | "/" & tsc_MCX_GMSXCAPRootURI & "/" & "org.openmobilealliance.groups/global/byGroupID/" & px_MCPTT_Group_T_ID | Points to the group configuration document | TS 24.481 [11] |  |