3GPP TSG-RAN5 Meeting #91-e	R5-212437r1

# Electronic Meeting, 17th – 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2172 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR TC 11.4.9-Emergency call establishment and release |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, MediaTek |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-05-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - After the emergency call is released, there is no spec specify the behaviour of the UE, so we need to cover all possible UE implementation. Currently, the UE is allowed to enter DEREGISTERED state by sending DEREGISTRATION REQUEST, or the SS sends a DEREGISTRATION REQ, but if a UE choose to perform local deregistration, the UE will fail this test case. This behaviour shall also be covered. - TP2 is trying to verify the behaviour after the registration rejection, however, in the SS initiated deregistration procedure, the 5GMM cause is re-sent, then the following behaviour will be the abnormal behaviour caused by the deregistration, which is not the TP2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Voided the original SS initiated deregistration procedure. - Deleted the related content for the SS initiated DEREGISTRATION REQ. - Added step 12b2A1 to trigger the UE to deregistration, if the UE is still in REGISTERED state the UE will send a DEREGISTRATION REQUEST, otherwise, it means the UE has performed local dergistration. - Editorial corrections of RRC msg. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A Conformant UE may fail the TC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.4.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: - Added description for IMS into the UE configuration. - Merge R5-212102 and added MediaTek as co-source. |  |  |  |  |  |  |  |  |

<Start of modified section >

11.4.9	5GMM-DEREGISTERED.LIMITED-SERVICE No suitable cells in tracking area / Emergency call establishment and release

11.4.9.1	Test Purpose (TP)

(1)

with { UE in 5GMM-DEREGISTERED.LIMITED-SERVICE state after receiving REGISTRATION REJECT message with 5GMM cause value #15 'No suitable cells in tracking area' }

ensure that {

when { UE is requested to make an Emergency call }

then { UE establishes an RRC connection with the RRC establishmentCause set to "emergency", and, attempts an Initial registration for emergency services by sending a REGISTRATION REQUEST message with IE Service type set to "emergency services", and, accepts and applies security with NULL security and integrity algorithms, and, after successful completion of the registration for emergency services establishes an emergency PDU session by sending an UL NAS TRANSPORT message with Request type set to "initial emergency request" and a PDU SESSION ESTABLISHMENT REQUEST }

}

(2)

with { UE in 5GMM-DEREGISTERED.LIMITED-SERVICE state after receiving REGISTRATION REJECT message with 5GMM cause value #15 'No suitable cells in tracking area' and having established an Emergency call }

ensure that {

when { UE is requested to release the Emergency call }

then { UE releases the Emergency call, and, the UE considers the current cell as belonging to 5GS forbidden tracking areas for roaming }

}

11.4.9.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.331 [12], subclause 5.3.3.3, TS 23.501 [37], subclause 5.16.4.1, TS 23.122 [38], subclause 3.4.2, TS 24.501 [22], subclauses 5.5.1.2.5, 4.4.4.1, 5.1.3.2.1.3.3, 5.3.2, 5.4.2.3, 5.5.1.2.2, 6.4.1.2, TS 22.101 [42], subclause 10.1.1. Unless otherwise stated these are Rel-15 requirements.

[TS 24.501, subclause 5.5.1.2.5]

If the initial registration request cannot be accepted by the network, the AMF shall send a REGISTRATION REJECT message to the UE including an appropriate 5GMM cause value.

...

The UE shall take the following actions depending on the 5GMM cause value received in the REGISTRATION REJECT message.

...

#15	(No suitable cells in tracking area).

The UE shall set the 5GS update status to 5U3 ROAMING NOT ALLOWED (and shall store it according to subclause 5.1.3.2.2). The UE shall reset the registration attempt counter and shall enter the state 5GMM-REGISTERED.LIMITED-SERVICE. The UE shall search for a suitable cell in another tracking area according to 3GPP TS 38.304 [28].

The UE shall store the current TAI in the list of "5GS forbidden tracking areas for roaming" and shall remove the current TAI from the stored TAI list, if present.

If the UE is operating in single-registration mode, the UE shall handle the EMM parameters EMM state, EPS update status and tracking area updating attempt counter as specified in 3GPP TS 24.301 [15] for the case when the normal tracking area updating procedure is rejected with the EMM cause with the same value.

[TS 23.122, subclause 3.4.2]

The MS is not allowed to request 5GS services except emergency services when camped on a cell of a TA of which belongs to the list of "5GS forbidden tracking areas for regional provision of service".

[TS 36.331, subclause 5.3.3.3]

The UE shall set the contents of RRCSetupRequest message as follows:

...

1>	set the establishmentCause in accordance with the information received from upper layers;

The UE shall submit the RRCSetupRequest message to lower layers for transmission.

[TS 23.501, subclause 5.16.4.1]

UEs that are in limited service state, as specified in TS 23.122 [17], or that camp normally on a cell but failed to register successfully to the network under conditions specified in TS 24.501 [47], initiate the Registration procedure by indicating that the registration is to receive Emergency Services, referred to as Emergency Registration, and a Follow-on request is included in the Registration Request to initiate PDU Session Establishment procedure with a Request Type indicating "Emergency Request". UEs that had registered for normal services and do not have emergency PDU Sessions established and that are subject to Mobility Restriction in the present area or RAT (e.g. because of restricted tracking area) shall initiate the UE Requested PDU Session Establishment procedure to receive Emergency Services, i.e. with a Request Type indicating "Emergency Request". Based on local regulation, the network supporting Emergency Services for UEs in limited service state provides Emergency Services to these UE, regardless whether the UE can be authenticated, has roaming or Mobility Restrictions or a valid subscription.

[TS 24.501, subclause 4.4.4.1]

The use of "null integrity protection algorithm" 5G-IA0 (see subclause 9.11.3.32) in the current 5G NAS security context is only allowed for an unauthenticated UE for which establishment of emergency services is allowed. For setting the security header type in outbound NAS messages, the UE and the AMF shall apply the same rules irrespective of whether the "null integrity protection algorithm" or any other integrity protection algorithm is indicated in the 5G NAS security context.

If the "null integrity protection algorithm"5G-IA0 has been selected as an integrity protection algorithm, the receiver shall regard the NAS messages with the security header indicating integrity protection as integrity protected.

[TS 24.501, subclause 5.1.3.2.1.3.3]

The substate 5GMM-DEREGISTERED.LIMITED-SERVICE is chosen in the UE, when it is known that a selected cell for 3GPP access or TA for non-3GPP access is unable to provide normal service (e.g. the selected cell over 3GPP access is in a forbidden PLMN or is in a forbidden tracking area or TA for non-3GPP access is forbidden).

[TS 24.501, subclause 5.3.2]

The UE provides the SUPI to the network in concealed form. The SUCI is a privacy preserving identifier containing the concealed SUPI. When the SUPI contains a network specific identifier, the SUCI shall take the form of an NAI as specified in 3GPP TS 23.003 [4].

A UE supporting N1 mode includes a SUCI:

a)	in the REGISTRATION REQUEST message when the UE is attempting initial registration procedure and a valid 5G-GUTI is not available; or

...

The UE shall use the "null-scheme" as specified in 3GPP TS 33.501 [24] to generate the SUCI, if the following applies:

a)	the UE performs a registration procedure for emergency services or initiates a de-registration procedure before the registration procedure for emergency services was completed successfully; and

[TS 24.501, subclause 5.4.2.3]

If the UE is registered for emergency services, performing initial registration for emergency services or establishing an emergency PDU session and the SECURITY MODE COMMAND message is received with ngKSI value "000" and 5G-IA0 and 5G-EA0 as selected 5G NAS security algorithms, the UE shall locally derive and take in use 5G NAS security context. The UE shall delete existing current 5G NAS security context.

The UE shall accept a SECURITY MODE COMMAND message indicating the "null integrity protection algorithm" 5G-EA0 as the selected 5G NAS integrity algorithm only if the message is received when the UE is registered for emergency services, performing initial registration for emergency services or establishing an emergency PDU session.

[TS 24.501, subclause 5.5.1.2.2]

The UE in state 5GMM-DEREGISTERED shall initiate the registration procedure for initial registration by sending a REGISTRATION REQUEST message to the AMF,

...

b)	when the UE performs initial registration for emergency services;

...

If the UE initiates an initial registration for emergency services or needs to prolong the established NAS signalling connection after the completion of the initial registration procedure (e.g. due to uplink signalling pending), the UE shall set the Follow-on request indicator to 1.

[TS 24.501, subclause 6.4.1.2]

In order to initiate the UE-requested PDU session establishment procedure, the UE shall create a PDU SESSION ESTABLISHMENT REQUEST message.

NOTE 0:	When IMS voice is available over either 3GPP access or non-3GPP access, the "voice centric" UE in 5GMM-REGISTERED state will receive a request from upper layers to establish the PDU session for IMS signalling, if the conditions for performing an initial registration with IMS indicated in 3GPP TS 24.229 [14] subclause U.3.1.2 are satisfied.

If the UE requests to establish a new PDU session, the UE shall allocate a PDU session ID which is not currently being used by another PDU session over either 3GPP access or non-3GPP access.

The UE shall allocate a PTI value currently not used and shall set the PTI IE of the PDU SESSION ESTABLISHMENT REQUEST message to the allocated PTI value.

...

If the UE requests to establish a new emergency PDU session, the UE shall set the SSC mode IE of the PDU SESSION ESTABLISHMENT REQUEST message to "SSC mode 1".

...

The UE shall transport:

a)	the PDU SESSION ESTABLISHMENT REQUEST message;

b)	the PDU session ID of the PDU session being established, or being handed over or being transferred;

..

e)	the request type which is set to:

...

3)	"initial emergency request", if the UE requests to establish a new emergency PDU session; and

...

If the request type is set to "initial emergency request" or "existing emergency PDU session", neither DNN nor S-NSSAI is transported by the UE using the NAS transport procedure as specified in subclause 5.4.5.

[TS 22.101, subclause 10.1.1]

The ME shall identify an emergency number dialled by the end user as a valid emergency number and initiate emergency call establishment if it occurs under one or more of the following conditions. If it occurs outside of the following conditions, the ME should not initiate emergency call establishment but normal call establishment. Emergency number identification takes place before and takes precedence over any other (e.g. supplementary service related) number analysis.

a)	112 and 911 shall always be available. These numbers shall be stored on the ME.

11.4.9.3	Test description

11.4.9.3.1	Pre-test conditions

System Simulator:

-	1 NR Cell

-	NR Cell 1, as defined in TS 38.508-1 [4], Table 4.4.2-3.

-	System information combination NR-1 as defined in TS 38.508-1 [4], subclause 4.4.3.1.2. SIB1 indicates ims-EmergencySupport.

UE:

-	The UE is configured to register for IMS after switching on.

Preamble:

-	The UE is in test state 0N-B (Switched Off) as defined in TS 38.508-1 [4], subclause 4.4A.2.

11.4.9.3.2	Test procedure sequence

Table 11.4.9.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Switch the UE on. | - | - | - | - |
| 2-4 | Steps 2-4 of Table 4.5.2.2-2 NR RRC_IDLE in TS38.508-1 [4] take place (the UE initiates an initial registration procedure). | - | - | - | - |
| 5 | SS sends a REGISTRATION REJECT message containing 5GMM cause value = #15 (No suitable cells in tracking area). | <-- | NR RRC: DLInformationTransfer 5GMM: REGISTRATION REJECT | - | - |
| 6 | The SS transmits an RRC Release message. | <-- | NR RRC: RRCRelease | - | - |
| 7 | Make the UE attempt an IMS emergency call dialling a number which is stored on the ME (e.g. 112 or 911). (NOTE 1) | - | - | - | - |
| 8 | Check: Does the UE performs Generic Test Procedure for IMS Emergency call establishment without IMS emergency registration as specified in TS 38.508-1 [4], subclause 4.9.12? | - | - | 1 | - |
| 9 | Make the UE release the emergency call. (NOTE 1) | - | - | - | - |
| 10 | The Generic test procedure for IMS MO Emergency call release as specified in TS 38.508-1 [4], subclause 4.9.12A takes place. | - | - | - | - |
| 11 | Start Timer=10 sec. NOTE: This is an arbitrary value to wait for UE initiated detach. | - | - | - | - |
| - | EXCEPTION: Steps 12a1-12b3 describes optional behaviour that depends on the UE implementation. | - | - | - | - |
| 12a1 | The UE transmits a DEREGISTRATION REQUEST message with De-registration type IE set to "Normal de-registration". | --> | NR RRC: ULInformationTransfer 5GMM: DEREGISTRATION REQUEST | - | - |
| 12a2 | The SS transmits a DEREGISTRATION ACCEPT message. | <-- | NR RRC: DLInformationTransfer 5GMM: DEREGISTRATION ACCEPT | - | - |
| 12a3 | Stop Timer=10. | - | - | - | - |
| 12b1 | Timer=10 sec expires | - | - | - | - |
| 12b2A | Cause UE to de-register for non Switch off reason using MMI or AT commands. | - | - | - | - |
| 12b2B | Start T_Delay=5 sec. | - | - | - | - |
| 12b2 | Void. | - | - | - | - |
| 12b3 | Void. | - | - | - | - |
| - | EXCEPTION: Steps 12b4a1-12b4b1 describes optional behaviour that depends on the UE implementation. | - | - | - | - |
| 12b4a1 | The UE transmits a DEREGISTRATION REQUEST message with De-registration type IE set to "Normal de-registration". | --> | NR RRC: ULInformationTransfer 5GMM: DEREGISTRATION REQUEST | - | - |
| 12b4a2 | Stop T_Delay | - | - | - | - |
| 12b4a3 | The SS transmits a DEREGISTRATION ACCEPT message. | <-- | NR RRC: DLInformationTransfer 5GMM: DEREGISTRATION ACCEPT | - | - |
| 12b4b1 | T_Delay expires. (NOTE 2) | - | - | - | - |
| 13 | SS releases the RRC connection | <-- | NR RRC: RRCRelease | - | - |
| 14 | Check: Does the UE transmit an RRCSetupRequest message in the next 30sec? NOTE: This is an arbitrary value to wait for catching not allowed UE behaviour. | --> | NR RRC: RRCSetupRequest | 2 | F |
| NOTE 1: This could be done by e.g. MMI or AT command. NOTE 2: The UE has performed local deregistration, so the SS shall also perform the deregistration procedure. |  |  |  |  |  |

11.4.9.3.3	Specific message contents

Table 11.4.9.3.3-1: REGISTRATION REJECT (step 5, Table 11.4.9.3.2-1)

| Derivation path: TS 38.508-1 [4] table 4.7.1-9. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| 5GMM cause | ‘00001111’B | #15 (No suitable cells in tracking area). |  |

Table 11.4.9.3.3-2: REGISTRATION REQUEST (step 8, Table 11.4.9.3.2-1; step 3, TS 38.508-1 [4], Table 4.9.12.2.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.7.1-6, condition EMERGENCY. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| 5GS mobile identity | SUCI | The UE shall use the "null-scheme" as specified in 3GPP TS 33.501 [20] to generate the SUCI |  |

Table 11.4.9.3.3-3: DEREGISTRATION REQUEST (Step 12a1, Table 11.4.9.3.2-1)

Derivation Path: TS 38.508-1 [4], Table 4.7.1-12, Condition NORMAL.

Table 11.4.9.3.3-4: Void

<End of modified section >