Discussion on test point selection for Relative Power Tolerance in FR2

Introduction

A default baseline for test point selection for TRx test cases was presented in [1] at RAN5#1 5G NR Adhoc meeting, where some proposals for environmental conditions, test frequencies, channel bandwidths, subcarrier spacing and RB allocation for FR2 were based on LTE. Finally, it was stated that each test case would require specific analysis to conform its test configuration table based on the study for Frequency, Channel BW, SCS, RB allocation (including modulations).

The purpose of this contribution is to provide the complete analysis for each parameter included in the Test Configuration Table and propose test points selection for Relative Power Tolerance test case in FR2.

Discussion

Relative Power Tolerance for FR2 is defined in Power Control section in [2] where its scope is described as follows:

The relative power tolerance is the ability of the UE transmitter to set its output power in a target sub-frame relatively to the power of the most recently transmitted reference sub-frame if the transmission gap between these sub-frames is 20 ms

We also use the test point choice from the relevant LTE testcase from 36.521-1 (6.3.5.2) as inspiration:

Table 6.3.5.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  | Low range |  |  |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  | Lowest, 5MHz, Highest |  |  |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |
|  | Downlink Configuration |  | Uplink Configuration |  |  |  |
| Ch BW | N/A for Power Control Relative power tolerance test case |  | Mod'n | RB allocation |  |  |
|  |  |  |  | FDD |  | TDD |
| 1.4MHz |  |  | QPSK |  | See table 6.3.5.2.5-1 6.3.5.2.5-2 6.3.5.2.5-13 | See table 6.3.5.2.5-1 6.3.5.2.5-2 6.3.5.2.5-13 |
| 3MHz |  |  | QPSK |  | See table 6.3.5.2.5-3 6.3.5.2.5-4 6.3.5.2.5-13 | See table 6.3.5.2.5-3 6.3.5.2.5-4 6.3.5.2.5-13 |
| 5MHz |  |  | QPSK |  | See table 6.3.5.2.5-5 6.3.5.2.5-6 6.3.5.2.5-13 | See table 6.3.5.2.5-5 6.3.5.2.5-6 6.3.5.2.5-13 |
| 10MHz |  |  | QPSK |  | See table 6.3.5.2.5-7 6.3.5.2.5-8 6.3.5.2.5-13 | See table 6.3.5.2.5-7 6.3.5.2.5-8 6.3.5.2.5-13 |
| 15MHz |  |  | QPSK |  | See table 6.3.5.2.5-9 6.3.5.2.5-10 6.3.5.2.5-13 | See table 6.3.5.2.5-9 6.3.5.2.5-10 6.3.5.2.5-13 |
| 20MHz |  |  | QPSK |  | See table 6.3.5.2.5-11 6.3.5.2.5-12 6.3.5.2.5-13 | See table 6.3.5.2.5-11 6.3.5.2.5-12 6.3.5.2.5-13 |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1 Note 2: The starting resource block shall be RB# 0. |  |  |  |  |  |  |

The MPR requirement has an impact on selection of test points for relative power tolerance. To rule out any implication due to MPR when getting close to maximum output power, it is desired to select test points, giving 0 dB MPR. The MPR requirement in FR2 from 38.101-2 [2] is given below as reference below.

For all power classes, the waveform defined by BW = 100 MHz, SCS = 120 kHz, DFT-S-OFDM QPSK, 20RB23 is the reference waveform with 0 dB MPR and is used for the power class definition.

Table 6.2.2.1-1 MPRWT for power class 1, BWchannel ≤ 200 MHz

| Modulation |  | MPRWT (dB), BWchannel ≤ 200 MHz |  |  |
| --- | --- | --- | --- | --- |
|  |  | Outer RB allocations | Inner RB allocations |  |
|  |  |  | Region 1 | Region 2 |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 5.5 | 0.0 | ≤ 3.0 |
|  | QPSK | ≤ 6.5 | 0.0 | ≤ 3.0 |
|  | 16 QAM | ≤ 6.5 | ≤ 4.0 | ≤ 4.0 |
|  | 64 QAM | ≤ 6.5 | ≤ 5.0 | ≤ 5.0 |
| CP-OFDM | QPSK | ≤ 7.0 | ≤ 4.5 | ≤ 4.5 |
|  | 16 QAM | ≤ 7.0 | ≤ 5.5 | ≤ 5.5 |
|  | 64 QAM | ≤ 7.5 | ≤ 7.5 | ≤ 7.5 |

Table 6.2.2.1-2 MPRWT for power class 1, BWchannel = 400 MHz

| Modulation |  | MPRWT (dB), BWchannel = 400 MHz |  |  |
| --- | --- | --- | --- | --- |
|  |  | Outer RB allocations | Inner RB allocations |  |
|  |  |  | Region 1 | Region 2 |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 5.5 | 0.0 | ≤ 3.0 |
|  | QPSK | ≤ 6.5 | 0.0 | ≤ 3.5 |
|  | 16 QAM | ≤ 6.5 | ≤ 4.5 | ≤ 4.5 |
|  | 64 QAM | ≤ 6.5 | ≤ 6.5 | ≤ 6.5 |
| CP-OFDM | QPSK | ≤ 7.0 | ≤ 5.0 | ≤ 5.0 |
|  | 16 QAM | ≤ 7.0 | ≤ 6.5 | ≤ 6.5 |
|  | 64 QAM | ≤ 9.0 | ≤ 9.0 | ≤ 9.0 |

…

Table 6.2.2.2-1 Void

…

Table 6.2.2.3-1 MPRWT for power class 3, BWchannel ≤ 200 MHz

| Modulation |  | MPRWT, BWchannel ≤ 200 MHz |  |
| --- | --- | --- | --- |
|  |  | Inner RB allocations, Region 1 | Edge RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | 0.0 | ≤ 2.0 |
|  | QPSK | 0.0 | ≤ 2.0 |
|  | 16 QAM | ≤ 3.0 | ≤ 3.5 |
|  | 64 QAM | ≤ 5.0 | ≤ 5.5 |
| CP-OFDM | QPSK | ≤ 3.5 | ≤ 4.0 |
|  | 16 QAM | ≤ 5.0 | ≤ 5.0 |
|  | 64 QAM | ≤ 7.5 | ≤ 7.5 |

…

Table 6.2.2.3-2 MPRWT for power class 3, BWchannel = 400 MHz

| Modulation |  | MPRWT, BWchannel = 400 MHz |  |
| --- | --- | --- | --- |
|  |  | Inner RB allocations, Region 1 | Edge RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | 0.0 | ≤ 3.0 |
|  | QPSK | 0.0 | ≤ 3.0 |
|  | 16 QAM | ≤ 4.5 | ≤ 4.5 |
|  | 64 QAM | ≤ 6.5 | ≤ 6.5 |
| CP-OFDM | QPSK | ≤ 5.0 | ≤ 5.0 |
|  | 16 QAM | ≤ 6.5 | ≤ 6.5 |
|  | 64 QAM | ≤ 9.0 | ≤ 9.0 |

Furthermore, RB allocation “Inner_Full” is defined as the largest RB allocation within region 1.

Table 6.1-1: Common Uplink Configuration PC2, for PC3 and PC4

| Channel Bandwidth | SCS(kHz) | OFDM | RB allocation |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Outer_Full | Outer_1RB_Left | Outer_1RB_Right | Inner_Full (Note 1) | Inner_1RB_Left | Inner_1RB_Right | Inner_Partial_Left | Inner_Partial_Right |
| 50MHz | 60 | DFT-s | 64@0 | 1@0 | 1@65 | 20@22 | 1@22 | 1@43 | 4@22 | 4@40 |
|  |  | CP | 66@0 | 1@0 | 1@65 | 22@22 | 1@22 | 1@43 | 4@22 | 4@40 |
|  | 120 | DFT-s | 32@0 | 1@0 | 1@31 | 10@11 | 1@11 | 1@21 | 4@11 | 4@18 |
|  |  | CP | 32@0 | 1@0 | 1@31 | 11@113 10@104 | 1@11 | 1@21 | 4@11 | 4@18 |
| 100MHz | 60 | DFT-s | 128@0 | 1@0 | 1@131 | 40@44 | 1@44 | 1@87 | 4@44 | 4@84 |
|  |  | CP | 132@0 | 1@0 | 1@131 | 44@44 | 1@44 | 1@87 | 4@44 | 4@84 |
|  | 120 | DFT-s | 64@0 | 1@0 | 1@65 | 20@23 | 1@22 | 1@43 | 4@22 | 4@40 |
|  |  | CP | 66@0 | 1@0 | 1@65 | 22@22 | 1@22 | 1@43 | 4@22 | 4@40 |
| 200MHz | 60 | DFT-s | 256@0 | 1@0 | 1@263 | 81@88 | 1@88 | 1@175 | 4@88 | 4@172 |
|  |  | CP | 264@0 | 1@0 | 1@263 | 88@88 | 1@88 | 1@175 | 4@88 | 4@172 |
|  | 120 | DFT-s | 128@0 | 1@0 | 1@131 | 40@44 | 1@44 | 1@87 | 4@44 | 4@84 |
|  |  | CP | 132@0 | 1@0 | 1@131 | 44@44 | 1@44 | 1@87 | 4@44 | 4@84 |
| 400MHz | 60 | DFT-s | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  |  | CP | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 120 | DFT-s | 256@0 | 1@0 | 1@263 | 64@66 | 1@66 | 1@197 | 4@66 | 4@194 |
|  |  | CP | 264@0 | 1@0 | 1@263 | 66@66 | 1@66 | 1@197 | 4@66 | 4@194 |
| Note 1: RB allocation is left aligned within inner region. Note 2: Inner_Full allocation is selected as the largest RB allocation within Region 1 inner allocation defined in 6.2.2.3.3; Inner_Partial_Left and Inner_Partial_Right are selected as partial allocation within Region 1 inner allocation which are not impacted by MPRnarrow defined in 6.2.2.3.3. Note 3: Applicable to Rel-15 only. Note 4: Applicable to Rel-16 and forward. |  |  |  |  |  |  |  |  |  |  |

Observation 1: The configuration giving 0 dB MPR, when the UE transmitted power is close to maximum output power, is Inner_Full allocation, DFT-S-OFDM pi/2 BPSK or QPSK.

Following subclauses introduce study for test environment, test frequencies, test bandwidth, test subcarrier spacing and uplink configuration aspects.

Test Environment

According to general comment in [2] core spec, the power control testcases shall be tested “under normal conditions” only:

### 6.3.4	Power control

#### 6.3.4.1	General

The requirements on power control accuracy apply under normal conditions and are defined as a directional requirement. The requirements are verified in beam locked mode on beam peak direction.

Proposal 1: Select Test Environment as Normal Conditions (NC) for Relative Power Tolerance measurement in FR2.

Test Subcarrier Spacing

Currently, some of the actual UEs do not support SCS = 60 kHz. Therefore, SCS = 120 kHz is required.

Proposal 2: Select Highest for Relative Power Tolerance measurement in FR2.

  - Test Frequencies

In LTE, this testcase is tested for low frequency. We believe this is also reasonable for NR.

Proposal 3: Select Low Frequency for Relative Power Tolerance measurement in FR2.

Test Channel Bandwidth

In LTE, this testcase is tested for Lowest, 5 MHz, and Highest channel bandwidths. This assumption seems to be reasonable to test different behaviours of the UE transmitter related to different frequency selectivities with different channel bandwidths. However, to have 0 dB MPR throughout the test, BW = 100 MHz could be selected.

Proposal 4: Select Channel Bandwidth 100MHz for Relative Power Tolerance measurement in FR2.

Modulations

CP-OFDM waveform, independent of Modulation, has highest PAPR. However, based on observation 1, to have 0 dB MPR throughout the test, DFT-S-OFDM QPSK is selected. Pi/2 BPSK is also a candidate for Modulation, but is not selected here because it is optional in the future.

Proposal 5: Select DFT-s-OFDM QPSK in the Relative Power Tolerance measurement in FR2.

Uplink Configuration

LTE uses RB change for ramping up, ramping down and alternating pattern, see 36.521-1 [4] clause 6.3.5.2.5 in addition to 1 dB TPC step size. 1 dB step size is used for ramping up/ramping down sub-test and 0 dB for alternating sub-test. A similar approach is reasonable for NR.

Proposal 6: Use 1 dB TPC cmd in all slots for ramping up/ramping down sub-test and 0 dB for alternating sub-test.

LTE uses 1 RB allocation in all sub-frames except for RB change sub-frames. It is desired to test as many DeltaP intervals as possible, defined in Table 6.3.4.3-1 and Table 6.3.4.3-2 [2]. Therefore, the RB allocation will deviate from the 1 RB approach for some test points. However, for the case where 1 RB allocation is tested, we suggest to use inner allocation to avoid MPRnarrow. For RB allocation greater than one, we also suggest to test with inner allocation. Based on observation 1 and proposal 2, 4, 5 to have 0 dB MPR throughout the test, 20RBs should be allocated when the UE transmitted power is getting close to maximum output power, illustrated in figure 1 below.

X RBs <= 40RBs

Figure 1. FDD ramping up power pattern with RB change

Proposal 7: In case of 1 RB allocation, use inner allocation

Proposal 8: Use 20RB allocation in all slots after RB change in ramp up sub-test and in all slot before RB change in ramp down sub-test.

The RB allocation at RB change is chosen as such, to cover all possible power step ranges given in the minimum requirement Table 6.3.4.3-1 and Table 6.3.4.3-2 (column 1) in TS 38.101-2 [2].

Table 6.3.4.3-1: Relative power tolerance, Pint ≥ P ≥ Pmin

| Power step ∆P (Up or down) (dB) | All combinations of PUSCH and PUCCH, PUSCH/PUCCH and SRS transitions between sub-frames, PRACH (dB) |
| --- | --- |
| ΔP < 2 | [±5.0] |
| 2 ≤ ΔP < 3 | [±6.0] |
| 3 ≤ ΔP < 4 | [±7.0] |
| 4 ≤ ΔP < 10 | [±8.0] |
| 10 ≤ ΔP < 15 | [±10.0] |
| 15 ≤ ΔP | [±11.0] |

Table 6.3.4.3-2: Relative power tolerance, PUMAX ≥ P > Pint

| Power step ∆P (Up or down) (dB) | All combinations of PUSCH and PUCCH, PUSCH/PUCCH and SRS transitions between sub-frames, PRACH (dB) |
| --- | --- |
| ΔP < 2 | [±3.0] |
| 2 ≤ ΔP < 3 | [±4.0] |
| 3 ≤ ΔP < 4 | [±5.0] |
| 4 ≤ ΔP < 10 | [±6.0] |
| 10 ≤ ΔP < 15 | [±8.0] |
| 15 ≤ ΔP | [±9.0] |

Proposal 9: For ramp down sub-test; use appropriate inner RB allocation at RB change and all succeeding slots to cover all possible DeltaP as specified in Table 6.3.4.3-1 and Table 6.3.4.3-2 (column 1) in TS 38.101-2 [2].

Proposal 10: For ramp up sub test; use appropriate inner RB allocation at RB change and all foregoing slots to cover all possible DeltaP as specified in Table 6.3.4.3-1 and Table 6.3.4.3-2 (column 1) in TS 38.101-2 [2].

Conclusion

It is proposed that RAN5 discusses and agrees the following proposals for open areas described in section 2 of this document to progress on SA FR2 Relative Power Tolerance test case definition:

Proposal 1: Select Test Environment as Normal Conditions (NC) for Relative Power Tolerance measurement in FR2.

Proposal 2: Select Highest for Relative Power Tolerance measurement in FR2.

Proposal 3: Select Low Frequency for Relative Power Tolerance measurement in FR2.

Proposal 4: Select Channel Bandwidth 100MHz for Relative Power Tolerance measurement in FR2.

Proposal 5: Select DFT-s-OFDM QPSK in the Relative Power Tolerance measurement in FR2.

Proposal 6: Use 1 dB TPC cmd in all slots for ramping up/ramping down sub-test and 0 dB for alternating sub-test.

Proposal 7: In case of 1 RB allocation, use inner allocation

Proposal 8: Use 20RB allocation in all slots after RB change in ramp up sub-test and in all slot before RB change in ramp down sub-test.

Proposal 9: For ramp down sub-test; use appropriate inner RB allocation at RB change and all succeeding slots to cover all possible DeltaP as specified in Table 6.3.4.3-1 and Table 6.3.4.3-2 (column 1) in TS 38.101-2 [2].

Proposal 10: For ramp up sub test; use appropriate inner RB allocation at RB change and all foregoing slots to cover all possible DeltaP as specified in Table 6.3.4.3-1 and Table 6.3.4.3-2 (column 1) in TS 38.101-2 [2].

Number of test points

Ramp up sub test:

| Environmental conditions | Maximum Number of Frequencies | Maximum Number of ChBW | Number SCS | Number of steps (mod and RB) | Maximum Number of Test Steps |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 | 1 | 1 | 1 | 1 |

Ramp down sub test:

| Environmental conditions | Maximum Number of Frequencies | Maximum Number of ChBW | Number SCS | Number of steps (mod and RB) | Maximum Number of Test Steps |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 | 1 | 1 | 1 | 1 |

Alternating sub test:

| Environmental conditions | Maximum Number of Frequencies | Maximum Number of ChBW | Number SCS | Number of steps (mod and RB) | Maximum Number of Test Steps |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 | 1 | 1 | 1 | 1 |

References

[1] R5-180065, “Discussion about Test parameters for NR TRx TCs“, NTT DOCOMO, RAN5 Ad hoc, Munich, Jan. 2018.

[2] 3GPP TS 38.101-2 v15.4.0 User Equipment (UE) radio transmission and reception; Part 1: Range 2 Standalone (Release 15)

[3] R5-180115, 3GPP TS 38.521-1 v0.1.0 User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 1 Standalone (Release 15)

[4] 3GPP TS 36.521-1 v15.2.0 User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Conformance Testing (Release 15)